TSG SA4#67 meeting	Tdoc S4 (12)0221

30 January – 3 February, 2012, Edinburgh, Great Britain                            update to S4-120021

Source: 	Samsung Electronics Co., Ltd.

Title: 	Proposed Evaluation and Selection Criteria for Source Block Construction (EMM-EFEC)

Document for:	Proposal and Approval

Agenda item: 	7

Introduction

A work item on “Enhancement to FEC for MBMS” in SP-110555 has been approved during SA#53.

This document proposes evaluation and selection criteria related to the structure of source block for an application layer FEC scheme for MBMS. The concrete proposal is provided in clause 4.

Use Case

Introduction

According to a generic mechanism for applying MBMS Forward Error Correction (FEC) defined in TS26.346 Clause 8.2.2, the mechanism consists of three components:

- Construction of an FEC source block from the source media packets

- Modification of source packets (to indicate the position of the source data from the source packet within the source block)

- Definition of repair packets (FEC coding)

This clause addresses the importance of efficient construction of the FEC source block which is the first component of FEC mechanism. Note that the terminology used in the description of the FEC framework and scheme is aligned with what is defined in TS26.346 Clause 8.2.2 and Annex B.1.1.

FEC Encoding

An example of a systematic FEC encoding process for source data is depicted in Figure 1. The FEC encoding process begins by creating a source block, more precisely, a source block builder in Figure 1 generates a source block from an ordered sequence of source packets (or source data) to be protected.

After constructing the source block, the source block builder passes the source symbols to the FEC encoder as the input. Here, source symbol is a unit of data within a source block upon which the FEC scheme operates. It is clear that the form of source symbols is determined by the operation of the source block builder. Furthermore, for given source packets, the number of source symbols, namely K, can differ depending on the operation. The current construction method for the FEC source block defined in TS26.346 needs a zero-padding operation for a proper form of the input to FEC encoder, even though the zero-padded data are not transmitted. Therefore, an increase of the number of source symbols may be caused by the zero-padding operation.

The FEC encoder generates the desired amount of encoding symbols from the source symbols. In the case of a systematic FEC scheme, the encoding symbols consist of K source symbols and NR repair symbols. These repair symbols may be sent using the FEC repair packet format to the receiver.

Note that the source blocking algorithm and the FEC coding are relatively independent operation, furthermore, this FEC encoding process is applied to each source block independently.

Figure 1 Conceptual FEC Encoding with the Current Source Blocking Algorithm

Efficiency of Source Block Structure

From an FEC viewpoint, the coding rate K/(K + NR) is closely related to robustness of source data protection. For example, assuming that K’ < K and there is another source blocking algorithm generating K’ source symbols as depicted in Figure 2.A decrease of the number of source symbols can be achieved by a reduction of zero-padding portion in the source block. It is clear that K’/(K’ + NR) is smaller than K/(K + NR) for fixed NR. Therefore, if the source block builder generates K’ source symbols from the source data (or source packets), they can be more robustly protected.

Figure 2 Example of Source Blocking Algorithm Generating K’(< K) Source Symbols (K’=7, NR =9)

For easy understanding, we describe a simple example in Figures 3 and 4. In Figure 3, we assume that 10 (= K) source symbols are generated by the current source blocking algorithm and 9 (= NR) repair symbols are generated by an FEC encoder. Any FEC encoder is available. Then, if all source packets are lost on the network, we cannot reconstruct the source symbols, that is, we lose 10 source symbols as well. Therefore, we cannot recover the source packets since the number of lost source symbols, 10, is larger than the number of repair symbols, 9.

However, in the case of the source blocking algorithm in Figure 2, we lose 7 (= K’) source symbols for the same lost source packets, as described in Figure 4. Therefore, we can recover all the source packets since the number of repair symbols, 9, is larger than the number of lost source symbols 7. Consequently, the source blocking algorithm in Figure 2 has more robust protection level than the current source blocking algorithm. Note that if the number of repair symbols, NR, is fixed for the given source data, the overhead is not changed since the overhead is defined as the extra encoding data beyond “the size of the source data”, not the number of source symbols, K or K’.

The example in Figure 4 is no accident, in general, the less source symbols are generated by a source blocking algorithm, the more robust protection can be provided on any FEC code.

Figure 3 Example of Unrecoverable Error in the Current Source Block Structure

Figure 4 Example of Enhanced Robustness for Source Block Structure in Figure 2

In other words, since repair packets fewer than NR are needed to get the same protection level with the case of coding rate K/(K + NR), the number of repair packets required for recovery of source data can be reduced.

Moreover, a well-designed FEC scheme allows the recovery of the original K source symbols from any K + A of the encoding symbols with high probability, where A is a small positive integer. Therefore, if the number of source symbols is reduced to K’, we may need a smaller number of encoding symbols to recover source data, namely K’+ A. As a result, the decoding complexity and latency to recover the source data can be reduced.

In summary, if the source block builder creates source symbols as few as possible for given source data (or source packets) and source symbol size, any FEC code can provide more robust protection level without additional overhead. Furthermore, the decoding complexity can be reduced.

Therefore, evaluation of source blocking mechanism is needed and it shall be in isolation from evaluation of FEC coding scheme.

Evaluation Criteria for Source Blocking Mechanism

Measurement of Efficiency of Source Blocking Mechanism

A construction method of source block should be regarded as an important factor of efficient system. To measure the efficiency of source block, we propose a simple criterion as follows:

It is clear that the smaller K’(T) is, the larger ESB(T) is. Therefore, a larger ESB(T) means that the relevant source block is better constructed in terms of robustness and decoding complexity.

Simulation Conditions

An SPB means a block of source packets which are inputted to “Source Block Builder” to generate K source symbols in Figure 1 and its size (in bytes) is defined as Bs. As source packets are belonging to one or several UDP packet flows, source packets are assumed as UDP payloads with variable size.

For simulation purpose, SPBs are generated under following conditions and restriction.

- The maximum size (in bytes) of source packets, Smax is restricted to 1024 and the minimum size, Smin is restricted to 32.

- Each SPB consists of source packets whose sizes are randomly selected from Smin up to Smax.

Test Cases

The following test cases are determined for the purpose of evaluating efficiency of source blocking mechanism. For each case, 10,000 trials are required.

Table 1 Test Cases for Efficiency of Source Blocking Mechanism

| Number | Source Symbol Size T (in bytes) | The number of Source Packets for an SPB (SPBL) |
| --- | --- | --- |
| ESB1 | 1024 | 100 |
| ESB2 |  | 400 |
| ESB3 |  | 800 |
| ESB4 | 512 | 100 |
| ESB5 |  | 400 |
| ESB6 |  | 800 |
| ESB7 | 256 | 100 |
| ESB8 |  | 400 |
| ESB9 |  | 800 |
| ESB10 | 128 | 100 |
| ESB11 |  | 400 |
| ESB12 |  | 800 |

Performance Metric

For each of the above test cases the following performance metrics shall be reported.

Table 2 Reporting format for Efficiency of Source Blocking Mechanism

| Number | Average Value of ESB | Report [K; K’; Bs/SPBL] values |
| --- | --- | --- |
| ESB1 |  |  |
| ESB2 |  |  |
| ESB3 |  |  |
| ESB4 |  |  |
| ESB5 |  |  |
| ESB6 |  |  |
| ESB7 |  |  |
| ESB8 |  |  |
| ESB9 |  |  |
| ESB10 |  |  |
| ESB11 |  |  |
| ESB12 |  |  |
| Averaged (ESB,total) |  |  |

Selection of Source Blocking Mechanism

Introduction

Even though this document proposes evaluation of source blocking mechanism in isolation from evaluation of FEC coding schemes, we may need to re-check proposed improvements of source blocking mechanism once a new FEC coding scheme or no new FEC coding scheme have been selected. Especially if the FEC coding scheme is proposing a new source blocking mechanism, we need to compare efficiencies of proposed source blocking mechanisms.

Therefore, we propose selection criteria for source blocking mechanism based on possible selection results of FEC coding scheme.

Selection Criteria

Possible selection results of FEC coding scheme are listed as follows:

- Case1: No new FEC coding scheme is selected

- Case2: A new FEC coding scheme with the current source blocking mechanism is 
      selected.

- Case3: A new FEC coding scheme with a new source blocking mechanism is  
      selected.

Selection Criteria for Case1

A candidate mechanism shall satisfy the following requirements:

- The candidate mechanism shall have ESB,total equal or bigger than 1.05 and ESB equal or bigger than 1.0 for each test case as compared with the current source blocking mechanism in MBMS.

- Each original packet does not have to be placed itself in the source block. But, in this case, the information on how to recover the source block at the receiver side shall be provided.

- The candidate mechanism with a negative effect compared to the current one may be acceptable only if it gives relatively more benefits than the negative effect and all participants agree with its necessity.

Selection Criteria for Case2

A candidate mechanism shall satisfy the following requirements:

- The candidate mechanism shall have ESB,total equal or bigger than 1.03 and ESB equal or bigger than 1.0 for each test case, as compared with the current source blocking mechanism in MBMS.

- Each original packet does not have to be placed itself in the source block. But, in this case, the information on how to recover the source block at the receiver side shall be provided.

- The candidate mechanism with a negative effect compared to the current one may be acceptable only if it gives relatively more benefits than the negative effect and all participants agree with its necessity.

Selection Criteria for Case3

A candidate mechanism shall satisfy the following requirements:

- The candidate mechanism shall have ESB,total bigger than ESB,total of the new source blocking mechanism.

- Each original packet does not have to be placed itself in the source block. But, in this case, the information on how to recover the source block at the receiver side shall be provided.

- The candidate mechanism with a negative effect compared to the new one may be acceptable only if it gives relatively more benefits than the negative effect and all participants agree with its necessity.

To select the best source blocking mechanism, if one mechanism has a highest value of ESB,total on the assumption that it satisfies the above requirements, it shall replace or supplement the current source blocking mechanism in MBMS or new source blocking mechanism

Conclusion

It is proposed

:to include the evaluation criteria in clause 3 to Permanent Document for EMM-EFEC

:to use the selection criteria in clause 4 for the selection of the source blocking mechanism for an application layer FEC for MBMS.