Source:	InterDigital Communications Inc

Title:	Downsampling filters for test sequences generation for the evaluation of HEVC

## Document for:	Information

## Agenda Item:	9

## Introduction

In 3GPP TSG-SA4#72 meeting, the HEVC test plan was updated and corresponding test cases, test conditions, test sequences, software and quality metrics were dicussed and agreed for the evaluation of HEVC compared to AVC for 3GPP multimedia services [1][2]. Besides the test sequences (Class B/C/D) used by JCT-VC for development of HEVC, additional test sequences were added to Class C, D and E as shown in Table 1. The Class E test sequences are down-sampled from Class B 1080p test sequences using the same downsampling filter as currently being used by JCT-VC to generate the anchor sequences for the scalable extension of HEVC. The additonal test sequences in Class C and Class D are also downsampled from Class B 1080p test sequences, but with different downsampling filters because the picture aspect ratio (PAR) is changed and the downsampling ration is neither 1.5x nor 2x for these sequences.

- New test sequences

| Class | Sequence | Spatial resolution | Frame rate |
| --- | --- | --- | --- |
| Class-C | Kimono_480p | 832x480 | 24 fps |
|  | ParkScene_480p | 832x480 | 24 fps |
|  | Cactus_480p | 832x480 | 50 fps |
|  | BasketballDrive_480p | 832x480 | 50 fps |
|  | BQTerrace_480p | 832x480 | 60 fps |
| Class-D | Kimono_240p | 416x240 | 24 fps |
|  | ParkScene_240p | 416x240 | 24 fps |
|  | Cactus_240p | 416x240 | 50 fps |
|  | BasketballDrive_240p | 416x240 | 50 fps |
|  | BQTerrace_240p | 416x240 | 60 fps |
| Class E | Downsampled 1080p sequences will be used. The filter used for downsampling is the same as currently being used in JCT-VC for scalable extension of HEVC. |  |  |

## Description of Downsampling Filters

According to the fundamentals of digital signal processing, ideal impulse response hn is shown in (1),

hn=aMNSincπnaN,   n=-∞, …, +∞                                       (1)

where a is the cut-off frequency with the value in [0, 1], Sinc(.) is defined in (2), and NM is equal to the downsampling ratio, i.e., 1.5 for 1.5X downsampling and 2 for the 2.0X case.

Sincx=1,           x=0sin⁡(x)x,   x≠0                                                    (2)

hn, which has unlimited length, can be clipped by a window function to limit the filter size. The types of the window functions include but are not limited to the commonly used, such as Hanning, Hamming, triangular, Gaussian, and Blackman windows. The downsampling filter is the product of the ideal impulse response hn and the window wn, as in (3).

fn=hnwn,  n=-L-12, …, L-12                                          (3)

## Downsampling Filters for Class E

The filters used to generate Class E test sequences support 1.5x down-sampling ratio. The filters are Cosine windowed Sinc function with cut-off frequency at 0.9π in the downsampling domain to preserve high frequency details. The filters were designed in odd-length symmetric such that the down-sampled videos have zero phase shift compard with the original videos [4].

The coefficients of 1.5x downsampling filter, are shown in Table 2. The corresponding impulse and frequency responses are shown in Figure 1.

The details of the downsampling filters used to generate Class E test sequences can also be found in [3].

- 1.5X downsampling filter

| Phase | Filter Coefficients |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Integer | [ 0 | 5 | -6 | -10 | 37 | 76 | 37 | -10 | -6 | 5 | 0 | 0] /128 |
| 1 | [-1 | 5 | -3 | -12 | 29 | 75 | 45 | -7 | -8 | 5 | 0 | 0] /128 |
| 2 | [-1 | 4 | -1 | -13 | 22 | 73 | 52 | -3 | -10 | 4 | 1 | 0] /128 |
| 3 | [-1 | 4 | 1 | -13 | 14 | 70 | 59 | 2 | -12 | 3 | 2 | -1] /128 |
| 4 | [-1 | 3 | 2 | -13 | 8 | 65 | 65 | 8 | -13 | 2 | 3 | -1] /128 |
| 5 | [-1 | 2 | 3 | -12 | 2 | 59 | 70 | 14 | -13 | 1 | 4 | -1] /128 |
| 6 | [ 0 | 1 | 4 | -10 | -3 | 52 | 73 | 22 | -13 | -1 | 4 | -1] /128 |
| 7 | [ 0 | 0 | 5 | -8 | -7 | 45 | 75 | 29 | -12 | -3 | 5 | 0] /128 |

(a)                                                                           (b)

- Impulse response and (b) frequency response of the 1.5X downsampling filter

## Downsampling Filters for Class C and Class D

The downsampling filters used to generate the additional test sequences in Class C and Class D are different from the filters used to generate Class E test sequences. Instead of even down-sampling ratio applied to both width and height, the down-sampling ratio is 3013 horizontally and 94 vertically for Class C, and 6013 horizontally and 92 vertically for Class D. The uneven sampling ratio of the width and height also changes the picture aspect ratio (PAR) of the down-sampled test sequences.

The downsampling filters used here are Gaussian windowed Sinc function with the cut-off frequency π.  Gaussian window function is shown in (4), where N denotes the length of the filter and σ is the standard variance of the Gaussian function. Figure 2 gives an example of the window function with (N=71, σ=1.5).

wn=e-12n-(N-1)/2σ(N-1)/22                                                                         (4)

The coefficients of the horizontal and vertical downsampling filters used to generate Class C test sequences are shown in Table 2, and the coefficients of the horizontal and vertical downsampling filters used to generate Class D test sequences are shown Table 3. Although the downsampling filters are 2-D separable, the horizontal and vertical filters are different. So the impulse and frequency responses of the dwonsampling filters are plotted in a 2-D manner, as shown in Figures 3 and 4.

- Gaussian window function (N=71, σ=1.5)

- Downsampling filter for Class C test sequences

| Horizontal filters | 0 | [ 21 -60 -119 112 619 901 619 112 -119 -60 21]/2048 |
| --- | --- | --- |
|  | 1 | [-52 -123 82 587 909 664 147 -116 -69 19 ] /2048 |
|  | 2 | [-44 -124 53 547 905 701 183 -109 -78 15 ] /2048 |
|  | 3 | [-35 -123 25 506 896 735 220 -101 -86 11 ] /2048 |
|  | 4 | [-27 -121 0 465 883 768 259 -90 -94 6 ] /2048 |
|  | 5 | [-20 -118 -23 423 867 797 299 -77 -101 0 ] /2048 |
|  | 6 | [-13 -113 -43 381 847 824 340 -61 -107 -6 ] /2048 |
|  | 7 | [-6 -107 -61 340 824 847 381 -43 -113 -13] /2048 |
|  | 8 | [ 0 -101 -77 299 797 867 423 -23 -118 -20 ] /2048 |
|  | 9 | [6 -94 -90 259 768 883 465 0 -121 -27 ] /2048 |
|  | 10 | [11 -86 -101 220 735 896 506 25 -123 -35 ] /2048 |
|  | 11 | [15 -78 -109 183 701 905 547 53 -124 -44 ] /2048 |
|  | 12 | [19 -69 -116 147 664 909 587 82 -123 -52 ] /2048 |
| Vertical filters | 0 | [28 -52 -127 94 621 920 621 94 -127 -52 28] /2048 |
|  | 1 | [-24 -126 0 492 914 754 214 -111 -82 17] /2048 |
|  | 2 | [ 0 -109 -69 350 852 852 350 -69 -109 0] /2048 |
|  | 3 | [17 -82 -111 214 754 914 492 0 -126 -24] /2048 |

- Downsampling filter for Class D test sequences

| Horizontal filters | 0 | [-11 32 128 265 390 441 390 265 128 32 -11] /2048 |
| --- | --- | --- |
|  | 1 | [27 118 252 380 438 394 274 137 37 -9] /2048 |
|  | 2 | [22 109 241 371 437 401 284 147 43 -7] /2048 |
|  | 3 | [18 101 230 363 435 407 295 156 49 -5] /2048 |
|  | 4 | [13 92 219 354 433 413 305 166 55 -3] /2048 |
|  | 5 | [10 84 208 344 430 418 315 177 62 0] /2048 |
|  | 6 | [6 77 198 335 426 422 325 187 69 3] /2048 |
|  | 7 | [3 69 187 325 422 426 335 198 77 6] /2048 |
|  | 8 | [0 62 177 315 418 430 344 208 84 10] /2048 |
|  | 9 | [-3 55 166 305 413 433 354 219 92 13] /2048 |
|  | 10 | [-5 49 156 295 407 435 363 230 101 18] /2048 |
|  | 11 | [-7 43 147 284 401 437 371 241 109 22] /2048 |
|  | 12 | [-9 37 137 274 394 438 380 252 118 27] /2048 |
| Vertical filters | 0 | [-14 27 124 265 397 451 397 265 124 27 -14] /2048 |
|  | 1 | [0 68 190 334 433 433 334 190 68 0] /2048 |

(a)                                                                           (b)

Figure 3 Downsampling filter for Class C. (a) Impulse response, and (b) frequency response.

(a)                                                                           (b)

Figure 4 Downsampling filter for Class D. (a) Impulse response, and (b) frequency response.

## References

- S4-130160, Proposed update to HEVC test plan, 3GPP TSG-SA4#72, February 2013.

- S4-130165, Proposed update to the draft TR on Evaluation of HEVC for 3GPP services, 3GPP TSG-SA4#72, February 2013.

- Jie Dong, Yong He, and Yan Ye, "Downsampling filter for anchor generation for scalable extensions of HEVC," m24499, 100th MPEG meeting, Geneva, USA, May 2012.

- Ajay Luthra, Jill Boyce, Jens-Rainer Ohm, Jörn Ostermann, “Draft Call for Proposals on the Scalable Video Coding Extensions of HEVC,” m24482, 100th MPEG meeting, Geneva, Switzerland, Apr. 2012.

Contact : Yong He, InterDigital Communications Inc,  Yong.He@Interdigital.com