Source:	Audience, Inc., Apple (UK) Limited, Blackberry UK Limited, Samsung Telecommunications, SPRINT Corporation

Title:	Open Architecture Rebuffering Delay

Document for:	Discussion and Agreement

Agenda Item:	9.5

# Introduction

In SA4 #79, Qualcomm presented S4-140612, a “Block Diagram for LTE delay components”.  While that diagram and document was very useful in helping to understand the pieces and nature of VoLTE UE Delay, it represented a homogenous, monolithic stack based on a closed architecture where all components are provided by a single vendor, often in a single system-on-a-chip (SoC) and therefore able to leverage a shared memory buffer to accomplish all speech processing.

This contribution presents a more common scenario, where original device manufacturers (ODMs) make architectural decisions based on open architectures.

## Why Open Architectures?

Open architectures allow ODMs to integrate best-of-breed componentry from multiple vendors, and at the same time, strategically avoid the potential business disadvantages of single-sourcing a given component ,or worse, compounding the risk of single-sourcing by having to acquire all components from a single vendor.  Sourcing multiple vendors for the same component ensures competition, which keeps quality high and prices in check. It also prevents a discontinuity of your business if your chosen vendor discontinues the part or goes out of business. For these reasons prevention of single-sourcing is a common practice of ODMs and carriers alike.

# High Level Block Diagram of Media Path delay components in an LTE to LTE call based on an Open Architecture.

## 2.1 Review of S4-140612’s closed, monolithic architecture

The high level block diagram of the media path delay components in an LTE to LTE call scenario of S4-140612 is shown below in Figure 1-1.  The green rectangles overlaid on the diagram represents the UE portion of the drawing that we will be addressing with this contribution.

Figure 1-Qualcomm original block diagram of MTSI based speech services media path delay components in an LTE-LTE call, overlaid to illustrate single SOC architecture of UE.

## 2.2 Open Architecture LTE to LTE Media Path (Worst Case Example)

As mentioned above, open architectures allow for the integration of chipsets from multiple vendors implementing the same bus protocol (e.g. SLIMbus, Soundwire, I2S, etc.).

Below is an example of a UE with a media path comprising three physical chips on both Tx and Rx, a baseband, vocoder and acoustic/speech enhancement.  The physical boundaries are indicated by the blue dashed lines. These could all be separate SoCs, or encompass a transfer back and forth to the application processor (AP) to handle some of the processing.

Figure 2-Open Architecture UE with 3 Separate Chips comprising the Tx and Rx media paths

This architecture requires the data to be transferred from chip to chip over a bus before it can be processed further.  In the case of three separate chips, and in the absence of shared memory, two additional rebuffers are required in each direction.  The buffer depth of each chip varies from vendor to vendor, but is usually a factor of 20 to divide the 20ms AMR frame cleanly into equal chunks.  Three chips (with four rebuffers total) is less common, and shown primarily to identify the boundaries where rebuffering can happen, but does happen and there are commercially available HDVoice-qualified phones with this architecture.   This scenario could add as much as 80ms in rebuffering to the Implementation Dependent Delay (aka “TBD1” in our discussions).

Two chips (with two rebuffers total) is a very common scenario with as much or more market share for HDVoice-qualified phones than monolithic closed stacks.  This could add as much as 40ms to TBD1.

## 2.2.1 Rebuffering Delay can be virtually eliminated with bursting, but it won’t happen for awhile

It’s very important to understand that rebuffering delay is not an inherent penalty of using an open architecture, it’s the result of chips not having knowledge of each other to negotiate a faster transfer speed. Buffering delay can be dramatically reduced by as much as 88% in some cases by bursting the transmission between cooperating chips.

This functionality is already being developed between some vendors collaborating in a proprietary fashion, and already being requested by some operators in their handset specs, but a hardware bus protocol bursting standard is needed to ensure proliferation of the feature.

This feature has been proposed in the MIPI Alliance LML SWG for SoundWire and the next version of SLIMbus, however it is meeting resistance from some closed stack SoC vendors who benefit from this disadvantage.

If ODMs/Operators were to request bursting in the phones they build/sell, they could have their open architecture without the rebuffering delay.  However, developing a standard, and the chips and phones that leverage the standard won’t happen overnight, and we must therefore account for these rebuffers in our TBD1 for the Rel 12 timeframe

# Proposal

- It is proposed that 80ms be added to our TBD1 value to account for rebuffering.

- It is proposed that if the closed architecture represented in S4-140612 is added to a permanent document, TR, or annex to TS 26.132. to document the block diagram and delay component descriptions, then the version presented in this document based on an open architecture with components from multiple vendors must also be included to help the reader understand the nature of rebuffering delay.

- It is proposed that 3GPP open a liaison agreement with MIPI and LS to MIPI-LML identifying the need for, and request the status of, the ability to eliminate rebuffering via bursted transfers.

# Works Cited

| [1] | S4-140612 - Block Diagram for LTE Components, Qualcomm Incorporated, 3GPP TSG SA WG4 #79, Sophia Antipolis, France |
| --- | --- |