Source:	Qualcomm Inc.

Title:	Proposals for EVSoCS

Document for:	Discussion and Approval

Agenda Item:	6

- Introduction

In this contribution, we discuss basic parameters of EVSoCS such as codec type, code point, bandwidth control, mode sets and encoding of Single Codec IE to be included in TS 26.103.

- Considerations

- Preference for one codec type and code point.

- Mode sets have to be contiguous to be compatible with the EVS VoLTE side because of SDP parameters.

- A general principle is to keep total number of mode sets as small as possible, to avoid implementation and testing effort.

- For capacity reasons, a basic proposal is that a default configuration shall allow a DL SF = 128, and the proposal is to define Configuration 0 including 5.9VBR, 7.2, 8, 9.6, 13.2 kb/s.

- Additional optional mode sets:

  - Configuration 1 targets maximum capacity savings, e.g. allowing the DL PHY spreading factor to be SF=256. It may include 5.9 VBR, 7.2 CBR, 8 CBR modes. The inclusion of AMR-WB IO 6.6 kb/s mode is rather questionable because of its limited quality when running a call at that bit rate after a cell handover, for example; setting up a new call is an alternative option.

  - Configuration 2 targets a wider range of codec rates and covers bit rates where super-wideband (SWB) speech and high-quality audio may be supported; also fullband (FB) would be supported at the bit rates (16.4 and 24.4 kbps). For this, PHY DL SF=64 will be needed.

  - Based on input in AHEVS-377, potentially configuration 3 for SWB operation with EVS primary modes and in addition, inclusion of AMR-WB IO modes (they do not provide SWB).

- Bandwidth control

  - Ensuring the availability of SWB operation at call set-up can be achieved by defining a corresponding mode set, e.g. including codec rates above 9.6 kbps (note that subsequent handover after call set-up may require a lower codec rate, and thus a lower bandwidth, to be used).

  - As higher audio bandwidths at a given bit rate provide increased quality over lower bandwidths in general, specifying a requirement for the UE to use the maximum possible audio bandwidth at a given bit rate is adequate to ensure the highest possible audio quality. On this way, by knowing the bit rate, the audio bandwidth is known as well.

- Codec Type and Code Point in the Codec List

The modified Table 4.1 of TS 26.103 shows the inclusion of UMTS_EVS codec type.

Modified Table 4.1: Support of Codec Types in Radio Access Technologies

|  | TDMA EFR | UMTS AMR 2 | UMTS AMR | (GSM) HR AMR | (GSM) FR AMR | GSM EFR | GSM HR | GSM FR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CoID | 0x07 | 0x06 | 0x05 | 0x04 | 0x03 | 0x02 | 0x01 | 0x00 |
| GERAN GMSK | not defined | not possible | not possible | yes, 1..4 modi | yes, 1..4 modi | yes | yes | Yes |
| GERAN 8PSK | not defined | not possible | not possible | not defined | not defined | not defined | not defined | not defined |
| UTRAN | not defined | yes, 1..8 modi 1..4 modi recomm. | R99, UTRAN-only UEs | not defined | not defined | not defined | not defined | not defined |

|  | Codec Extension | UMTS EVS | OHR AMR-WB | OFR AMR-WB | OHR AMR | UMTS AMR-WB | FR AMR-WB | PDC EFR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CoID | 0x0F | 0x0E | 0X0D | 0x0C | 0x0B | 0x0A | 0x09 | 0x08 |
| GERAN GMSK | reserved | not defined | not defined | not defined | not defined | not possible | yes3 modi | not defined |
| GERAN 8PSK | reserved | not defined | yes, 3 modi | yes, 3 modi | yes, 1..4 modi | not possible | not defined | not defined |
| UTRAN | reserved | Yes [TBD] modi | not defined | not defined | not defined | yes 3..4 modi | not defined | not defined |

The modified Table 4.2 of TS 26.103 shows the code point for EVS on UTRAN radio interface.

Modified Table 4.2: Defined Code Points

| Hexadecimal Notation | Binary Notation | Codec Name | Remark |
| --- | --- | --- | --- |
| 0x00h | 0x0000.0000 | GSM_FR |  |
| 0x01h | 0x0000.0001 | GSM_HR |  |
| 0x02h | 0x0000.0010 | GSM_EFR |  |
| 0x03h | 0x0000.0011 | (GSM) FR_AMR |  |
| 0x04h | 0x0000.0100 | (GSM) HR_AMR |  |
| 0x05h | 0x0000.0101 | UMTS_AMR |  |
| 0x06h | 0x0000.0110 | UMTS_AMR2 |  |
| 0x07h | 0x0000.0111 | TDMA_EFR |  |
| 0x08h | 0x0000.1000 | PDC_EFR |  |
| 0x09h | 0x0000.1001 | (GSM) FR_AMR-WB |  |
| 0x0Ah | 0x0000.1010 | UMTS_AMR-WB |  |
| 0x0Bh | 0x0000.1011 | OHR_AMR |  |
| 0x0Ch | 0x0000.1100 | OFR_AMR-WB |  |
| 0x0Dh | 0x0000.1101 | OHR_AMR-WB |  |
| 0x0Eh | 0x0000.1110 | UMTS_EVS |  |
| 0x0Fh | 0x0000.1111 | Codec Extension | For AoIP and TFO |
| 0x10h … 0xFCh | 0x0001.0012 … 0x1111.1100 | Spare, for future use |  |
| 0xFDh | 0x1111.1101 | CSData | For AoIP only |
| 0xFEh | 0x1111.1110 | MuMe2 | For OoBTC only |
| 0xFFh | 0x1111.1111 | MuMe | For OoBTC only |

- Mode Sets for EVS

The following Table X to be included in TS 26.103 and the note below provides a proposal for agreement on allowed codec mode sets, similarly to other codecs in TS 26.103.

Table X: Allowed Configurations for the UMTS_EVS Codec Type

| Configuration → (Config-EVS-Code) ↓ Codec Mode | 0 | 1 | 2 | 3 |  |
| --- | --- | --- | --- | --- | --- |
| EVS Primary 24.4 |  |  | 1 |  |  |
| EVS Primary 16.4 |  |  | 1 |  |  |
| EVS Primary 13.2 | 1 |  | 1 | 1 |  |
| EVS Primary 9.6 | 1 |  | 1 | 1 |  |
| EVS Primary 8 | 1 | 1 | 1 |  |  |
| EVS Primary 7.2 | 1 | 1 | 1 |  |  |
| EVS Primary 5.9VBR | 1 | 1 | 1 |  |  |
| AMR-WB IO 12.65 | 1 |  | 1 | 1 |  |
| AMR-WB IO 8.85 | 1 |  | 1 | 1 |  |
| AMR-WB IO 6.60 | 1 | 1 | 1 | 1 |  |

Notes:

- Configuration 0 is mandatory.

- Requirement for UE: use the maximum possible audio bandwidth at a given bit rate.

- Typical SF for configurations 0, 1, 2 and 3 on DL are respectively 128, 256, 64 and 128.

- From a transmission point of view, 5.9 VBR mode (5.9 kb/s average gross bit rate) includes bit rates of 2.8, 7.2, 8.0 kb/s.

- Single Codec IE for EVS

The following Table Y to be included in TS 26.103 and the notes below provide a proposal for agreement on the coding of the “Single Codec” information element for EVS.

Table Y: Coding of "Single Codec" for the EVS Codec

| Octet | Parameter | MSB 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 LSB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 m | Single Codec | Single Codec (see ITU-T Q.765.5) |  |  |  |  |  |  |  |
| 2 m | Length Indication | 4 |  |  |  |  |  |  |  |
| 3 m | Compat. Info | Compatibility Information |  |  |  |  |  |  |  |
| 4 m | OID | ETSI OID (See ITU-T Q.765.5 [6]) |  |  |  |  |  |  |  |
| 5 m | CoID | UMTS_EVS_CoID |  |  |  |  |  |  |  |
| 6 m | Config-EVS | (spare) | (spare) | (spare) | (spare) | Config-EVS-Code |  |  |  |

- Since only 5 allowed configurations are proposed to be defined, 3 bits would be sufficient to encode the value of the Config-EVS-Code field (which contains the configuration index). However it is proposed to set its length to 4 bits in order to have spare values in case additional allowed configurations are needed in the future.

- There is no need to add additional fields in octet 6 m for signaling of the bit rate, the bandwidth, or whether EVS primary or AMR-WB IO mode is used because:

  - the bit rates will be derived from the configuration indicated in Config-EVS-Code

  - the bandwidth will be derived as the highest supported bandwidth for the bit rate in use

  - whether EVS primary mode or AMR-WB IO mode is used will be derived from the bit rate

- In order to avoid impacts on existing NAS and RRC signaling, it is proposed, not to add any additional field in octet 6m for signaling of the following:

  - Whether Compact or Header-Full format is used. Default value (both are allowed) will apply.

  - Whether DTX is enabled. Default value (DTX is enabled) will apply.

  - Number of channels. Default value (mono) will apply.

- In-call Adaptation by Using CMR

Contribution AHEVS-381 addressed adjusting codec mode to system conditions, due to change in radio network quality or in case of cell handover. The contribution also addresses the case of adaptation with regards to audio bandwidth.

In our document, we propose the principle of using the maximum possible audio bandwidth at any time because maximizing audio bandwidth maximizes audio quality. For this, audio front end of the UE is assumed to be prepared for all bandwidths of EVS. The encoder determines the audio bandwidth automatically if the input signal had limited audio bandwidth, as compared to the maximum at a given bit rate. The bandwidth at the receiving side may change for example due to connection to (plugging in) external devices or switching to another mode in the UE. In such situation, the audio bandwidth will be automatically limited on the receiving side. An alternative solution is signaling the audio bandwidth using in-band CMR commands as proposed in AHEVS-381. The advantages are not fully explored yet. On the other hand, this comes at the cost of having to support CMR in the CS domain. As such it can be considered as an optimization and if found beneficial, it should be introduced as an optional mechanism.

In case of CMR as an optional solution, different (reduced) encoding of CMR in CS as compared to CMR in LTE could be investigated, with the motivation to reduce the overhead, especially for cases of high spreading factors like 256. On the other hand, re-using the LTE CMR encoding to facilitate interworking with LTE and reduce complexity can also be seen as a priority. There will have to be a trade-off between these two aspects when defining the encoding for optional in-band CMR in CS.