3GPP TSG SA4#86 meeting	S4-151237

San Jose del Cabo, Mexico, 26 - 30 October, 2015

Agenda item: 	9.9

Source: 	Huawei Technologies Co Ltd

Title: 	On the Complexity of the EVS Codec

Document for	Discussion & Approval

___________________________________________________________________

1	Summary

In this document we analyse the available complexity information for the AMR, AMR-WB and EVS codecs.

From this analysis we see that the significant performance benefits, in terms of intelligibility/quality, error resilience, bandwidth efficiency of the EVS codec are provided with very modest increases in algorithmic complexity compared to EFR, AMR and AMR-WB.

Text for inclusion in the MCPTT TR 26.879 [2] is proposed.

2	On the Complexity of the EVS Codec

The complexity of the EVS codec is described in subclause 13.2 of TR 26.952 [1]. Tables 13.2a, 13.2c and 13.2d from [1], reproduced below as Tables 1, 2,and 3, provide a comparison of the mean of the per-audio-frame complexity for EVS and AMR-WB, analysed with a source file comprising 8.5 minutes of mixed speech and music. In the case of the decoders the complexity is measured in the presence of 30% frame erasures and the EVS operation is analysed by bandwidth and sample rate.

Table 1: Highest values of the mean complexity for different sample rates and coded bandwidths

| Coded Bandwidth | Sample Rate (kHz) | Encoder Complexity (WMOPS) | Decoder Complexity (WMOPS) | Balanced Combined Complexity (WMOPS) | Worst/Worst Combined Complexity (WMOPS) |
| --- | --- | --- | --- | --- | --- |
| NB | 8 | 29.98 | 15.40 | 43.97 | 45.38 |
|  | 16 | 31.52 | 16.38 | 47.35 | 47.90 |
|  | 32 | 37.34 | 19.37 | 56.71 | 56.71 |
| WB | 16 | 38.25 | 18.29 | 51.38 | 56.54 |
|  | 32 | 43.32 | 22.88 | 60.92 | 66.20 |
| SWB | 32 | 45.12 | 22.67 | 65.56 | 67.80 |

Table 2: Highest values of the mean complexity for AMR-WB

| Coded Bandwidth | Sample Rate (kHz) | Encoder Complexity (WMOPS) | Decoder Complexity (WMOPS) | Balanced Combined Complexity (WMOPS) | Worst/Worst Combined Complexity (WMOPS) |
| --- | --- | --- | --- | --- | --- |
| WB | 16 | 32.30 | 7.80 | 38.73 | 40.11 |

Table 3: Incremental mean complexity for EVS relative to AMR-WB

| Coded Bandwidth | Sample Rate (kHz) | Encoder Complexity (WMOPS) | Decoder Complexity (WMOPS) | Balanced Combined Complexity (WMOPS) | Worst/Worst Combined Complexity (WMOPS) |
| --- | --- | --- | --- | --- | --- |
| NB | 8 | -7.18% | +97.4% | +13.5% | +13.1% |
|  | 16 | -2.41% | +110% | +22.3% | +19.4% |
|  | 32 | +15.6% | +148% | +46.4% | +41.4% |
| WB | 16 | +18.4% | +134% | +32.7% | +41.0% |
|  | 32 | +34.1% | +193% | +57.3% | +65.0% |
| SWB | 32 | +39.8% | +191% | +69.3% | +69.0% |

From Table 3 it can be seen that under the same conditions, the EVS codec requires just 69% more processing on average to reproduce twice the audio bandwidth in a SWB capable UE than the AMR-WB codec providing just WB. It can also be seen that in a WB-only capable UE, the EVS codec requires just 41% more processing on average to provide the significantly enhanced WB quality for both speech and music signals and robustness to frame erasures described in subclause 5.1.1.3 (of [2]).

The worst-case observed complexity of EFR and AMR is given in [4] as 15.21 and 15.33 WMOPs respectively but both the bit-exact instruction weights and methodology have changed in arriving at the complexity figures for EVS and for AMR-WB above. Having said this though, if the complexity of the 3GPP Codecs, beginning with GSM EFR in 1995 [4 & 6], followed by AMR in 1999 [4], AMR-WB in 2001 [5] and EVS in 2015 [7] is plotted (see Figure 1) we see that the complexity follows a trend of approximately doubling every 9.3 years.

Figure 1: Complexity of 3GPP Codecs over time

Over the same 20 year period, Koomey’s Law [8] (an extension of Moore’s Law) for computing energy efficiency has delivered a huge reduction in the power consumption of semiconductors per WMOPs (roughly 1/6800 = 2-12.7) and battery technology advances have led to an approximate doubling of battery energy densities in Wh/kg [9] or Wh/l. Taken together these two trends would suggest that the battery weight [or volume] necessary to power the EVS codec in 2015 is roughly equivalent to 1/3000 of the battery weight [or volume] that was necessary to power the EFR codec in 1995. Even taking into account that these trends do not include all relevant factors, it is hard to reach any other conclusion than that the complexity of the EVS codec is readily implementable with today’s technology.

3	Text Recommendations

It is recommended that a new sub-clause 5.1.2 be added as follows and the other sub-clauses renumbered accordingly

5.1.2	Review of Codec Alternatives and their Relative Complexity

It is recommended that the text of Clause 2 of this contribution be included here along with reference [5].

4	References

[1]	3GPP TR 26.952 “Codec for Enhanced Voice Services (EVS); Performance characterization”

[2]	3GPP Draft TR 26.879 v.0.2.0 “Mission Critical Push To Talk; Media, codecs and MBMS enhancements for Mission Critical Push to Talk over LTE”

[3]	3GPP TS 22.179: "Mission Critical Push To Talk (MCPTT) over LTE; Stage 1"

[4]	3GPP TR 26 975 “Performance characterization of the Adaptive Multi-Rate (AMR) speech codec”

[5]	3GPP TR 26 976 “Performance characterization of the Adaptive Multi-Rate Wideband (AMR-WB) speech codec”

[6]	ETSI ETR 305 “Performance characterization of the GSM Enhanced Full Rate (EFR) speech codec (GSM 06.55)”

[7]	3GPP TR 26.952: " Codec for Enhanced Voice Services (EVS);Performance Characterization"

[8]	Koomey, J.G., "Outperforming Moore's Law", IEEE Spectrum, 26 Feb 2010.

[9]	Van Noorden, R., "A Better Battery", Nature, Vol 507, 6 March 2014, pp26-28.