Source:	Qualcomm Inc.

Title:	Proposals for EVSoCS

Document for:	Discussion

Agenda Item:	8.5

- Introduction

SA4 concluded in working assumptions on aspects related to EVSoCS Work Item and sent an LS to RAN and CT groups from the Rennes meeting. The LS replies contain useful advice from the RAN and CT groups to SA4 work, and questions that need to be answered. In this contribution, we summarize the working assumptions and the LS replies and make proposals on next steps.

- Working Assumptions and Agreements

In this section, first we summarize the working assumptions and agreements reached so far:

- EEP (Equal Error Protection) will be used.

- Defining a single code point for EVS with one reservation (signaling a single audio bandwidth is requested)

- On mode sets, the working assumption is that for each SF in DL 64, 128, 256, at least one mode set will be defined that contains all modes that can be supported, also further discussions will take place on additional mode sets that do not contain certain of these bit rates.

- A working assumption was reached on sending in-band CMR only and that CMR from UE will be combined in the MGW with the max rate command from the RNC.

- Liaison Activity

We highlight important items based on liaison activity and make proposals for next steps.

SA4 has sent a comprehensive LS from the Rennes meeting to CT1, CT3, CT4, RAN1, RAN2 in S4-150870 on the working assumptions reached in SA4.

LS reply from RAN1 in S4-151192 (R1-154798)

- confirmed that mode sets A and B (A*, B*) should have maximum bit rates of 8 and 13.2 kbps, respectively;

- found that there are not many impacts to the RAB configuration due to addition of few extra CMR bits to EVS payload size. RAN1 leaves the final decision on the number of bits used in CMR to SA4, provided this number is small (e.g. equal or less than 7);

- states that using EEP is suitable;

- recommends the inclusion of low codec rates in EVS mode sets as beneficial mainly from capacity and RAB configuration aspects.

LS reply from RAN2 group in S4-151378 (R2-153904)

- confirmed usage of EEP;

- found that there are not many impacts to the RAB configuration due to addition of few extra CMR bits to EVS payload size. RAN2 leaves the final decision on the number of bits used in CMR to SA4, provided this number is small (e.g. equal or less than 7);

- RAN2 recommends the inclusion of low codec rates in EVS mode sets as beneficial mainly from capacity and RAB configuration aspects. Removing lower codec rates can impact coverage.

LS reply from CT1 in S4-151042 (C1-153239)

- confirms that defining one code point for EVS codec implies no impact to the encoding of the Supported Codec List IE;

- regarding the potential inclusion of more parameters in the Setup and Confirm messages emphasizes that impacts on legacy CS signaling should be avoided if at all possible.

LS reply from CT3 in S4-151044 (C3-153456)

- says that combining CMR from UE and may rate command from RNC puts a new requirement on the MGW and this needs to be reflected in an update of TS 29.414 and TS 29.415;

- request SA4 to confirm that the CMR will be sent in-band; CT3 will need to update interworking procedures in TS 29.163.

LS reply from CT4 in S4-151050 (C4-151290)

- assumes that SA4 will not change the principle used today for all other codecs in UE-MSC signaling and TS 23.153 needs to be updated if new parameters will be introduced to allow differentiating between audio bandwidths and/or bit rate limitations;

- confirms that one code point is sufficient for BICC signaling between MSCs;

- confirms that configurations A, B, C are TrFO-compatible and suggests that SA4 should try to avoid specifying non-TrFO-compatible mode sets additionally to A, B, C.

The LS replies referenced here indicate that the working assumptions and decisions reached in SA4 are suitable from a RAN and CT point of view. In order for CT1, CT3, CT4, RAN1, RAN2 to be able to progress the related normative work, they need a reply LS from SA4 confirming that SA4 has agreed to proceed with these working assumptions and decisions. Moreover, CT3’s request to confirm that CMR will be sent in-band also needs to be answered.

- Proposals

  - Mode Sets

There was much discussion in the group on the pros and cons of the so-called onion model. An argument on the cons side was that the possibility of a single audio bandwidth (super-wideband) in EVS primary modes would be excluded with the onion model.

Specifically, inclusion of a mode set consisting of 9.6 and 13.2 kbps primary modes (and 6.6, 8.85, 12.65 kbps IO modes) was proposed as a so-called single bandwidth (SWB) set. We note in this context that EVS supports three bandwidths at both 9.6 and 13.2 kbps: narrowband, wideband and super-wideband. In combination with our proposed requirement that the UE uses the maximum possible bandwidth at a given bit rate, inclusion of a mode set consisting of 9.6 and 13.2 kbps (besides IO modes) maximizes the chance of SWB operation during the call although it does not guarantee that; on the other hand, there are no fallback modes (besides IO modes) in this case so the lifetime of the call may be expected shorter at limited radio/network conditions.

For sake of compromise, we propose to agree on the mode sets for EVS as summarized in Table 5.8-1.

Table 5.8-1: Allowed Configurations for the EVS Codec Type

| Configuration → (Config-EVS-Code) ↓ Codec Mode | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| EVS Primary 24.4 |  |  | 1 |  |
| EVS Primary 16.4 |  |  | 1 |  |
| EVS Primary 13.2 | 1 |  | 1 | 1 |
| EVS Primary 9.6 | 1 |  | 1 | 1 |
| EVS Primary 8 | 1 | 1 | 1 |  |
| EVS Primary 7.2 | 1 | 1 | 1 |  |
| EVS Primary 5.9VBR | 1 | 1 | 1 |  |
| AMR-WB IO 12.65 | 1 |  | 1 | 1 |
| AMR-WB IO 8.85 | 1 |  | 1 | 1 |
| AMR-WB IO 6.60 | 1 | 1 | 1 | 1 |

The "1" in the table indicates that the Codec Mode is included in the Active Codec Set of the Configuration. 

Notes:

- Configuration 0 is conditionally mandatory for all networks supporting UMTS_EVS.

- All Configurations are conditionally mandatory for all terminals supporting UMTS_EVS.

- The maximum audio bandwidth at a given bit rate shall be used. If bandwidth adaptation is allowed for EVS Primary modes (e.g. by CMR), this specified audio bandwidth may be reduced to a lower bandwidth during the call; if the input signal has a lower audio bandwidth, the coded audio bandwidth may also be lower in some cases.

- Typical SF for configurations 0, 1, 2, 3 on DL are respectively 128, 256, 64, 128.

- From a transmission point of view, 5.9 VBR mode (5.9 kb/s average gross bit rate) includes bit rates of 2.8, 7.2, 8.0 kb/s.

  - CMR

Contribution AHEVS-396 proposed using 7 bits for CMR at all bit rates in all frames (provided SA4 companies checked the 504-question).

This proposal is in line with RAN1 and RAN2 liaisons.

We propose to agree at SA4#86 that 7 bit in-band CMR will be used at all bit rates.

  - Single Codec IE

Table X: Coding of "Single Codec" for the EVS Codec Type

| Octet | Parameter | MSB 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 LSB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 m | Single Codec | Single Codec (see ITU-T Q.765.5) |  |  |  |  |  |  |  |
| 2 m | Length Indication | 4, 5 |  |  |  |  |  |  |  |
| 3 m | Compat. Info | Compatibility Information |  |  |  |  |  |  |  |
| 4 m | OID | ETSI OID (See ITU-T Q.765.5 [6]) |  |  |  |  |  |  |  |
| 5 m | CoID | UMTS_EVS_CoID |  |  |  |  |  |  |  |
| 6 m | Config-EVS | (spare) | (spare) | (spare) | (spare) | Config-EVS-Code |  |  |  |
| 7 o | dtx/dtx-recv | (spare) | (spare) | (spare) | (spare) | (spare) | (spare) | dtx-recv | dtx |

with "m" = mandatory

Normative for UTRAN terminals for UMTS_EVS:
If an UTRAN terminal offers Codec Type UMTS_EVS in the Supported Codec List Information Element, then all allowed EVS Configurations shall be supported by this terminal.

Normative for UTRAN infrastructures for UMTS_EVS:
If an UTRAN infrastructure supports Codec Type UMTS_EVS, then at least EVS Configuration 0 shall be supported. The other EVS Configurations are not normative, but optional.

  - Liaison

We propose to send an LS to CT and RAN groups from this meeting; Annex A contains a proposed draft LS.

Annex A

Draft LS reply from SA4 to CT1, CT3, CT4, RAN1, RAN2 groups is proposed.

-------------------------------------------------------start--------------------------------------------------------------

Title:	DRAFT LS reply on EVS over UTRAN

Response to:	R1-154798, R2-153904, C1-153239, C3-153456, C4-151290

Release:	Rel-13

Work Item:	EVSoCS

Source:	SA4

To:	RAN1, RAN2, CT1, CT3, CT4

Cc:	RAN3

Contact Person:

#### Name:

E-mail Address:

Attachments:

1. Overall Description

SA4 thanks RAN1, RAN2, CT1, CT3 and CT4 for their LSs (R1-154798, R2-153904, C1-153239, C3-153456 and C4-151290) related to EVS over UTRAN CS.

SA4 would like to provide the following answers.

To CT1 group:

UE<=>MSC signalling:

Currently it is assumed that the principle used today for all other Codecs is not changed. The UE can only send in Call Setup and/or Call Confirm the Codec Type “EVSoCS” in the bit map of the Supported Codec List (TS 26.103). This would not allow differentiating between different audio bandwidths and/or bit rate limitations.

It may be feasible to include more parameters in the Setup and Confirm message, if needed. However, it could have impacts on legacy CS signalling.

CT1 notes that the bitmap in the Supported Codec List specified in TS 26.103 contains 1 spare value, and assumes this value will be used for the new Codec Type “EVSoCS”. As a result, the length of the bitmap will not need to be increased, and there will be no impact to the encoding of the Supported Codec List IE in CT1 specification TS 24.008.

SA4 thanks CT1 for confirming that defining one code point for EVS codec implies no impact to the encoding of the Supported Codec List IE.

SA4 decided to define one code point for EVS in 0x0000.1110 UMTS_EVS.

Regarding the potential inclusion of more parameters in the Setup and Confirm messages, CT1 would like to emphasize that impacts on legacy CS signalling should be avoided if at all possible, as they would require additional specification and testing efforts both on the UE and the core network sides.

SA4 acknowledges the answer.

To CT3 group:

Maximum Rate Control in the MGW:

If CMR within the payload is accepted, then potential maximum audio bandwidth control coming from the local UE in CMR would go transparently through the RNC. The MGW would have to combine this CMR and the rate control from the RNC (conveyed as PDU type 14 control message, see above), obeying the lower limit of both, before sending the (potentially) modified full CMR forward.

CT3 notes that this would put a new requirement on the MGW to combine the CMR received from the local UE and the rate control from the RNC. This new requirement would need to be reflected in an update to CT3 specification TS 29.414 and 29.415.

SA4 would like to inform CT3 that SA4 has agreed to use CMR within the payload for EVS over UTRAN, and requests CT3 to update TS 29.414 and TS 29.415 according to the above requirement. SA4 would like to emphasize that the require functionality to combine CMR and the rate control messages from the RNC is only required for the first MGW terminating the Iu interface. Other MGWs in the call path do not need to support this functionality.

MGW<=>MGW transport (Nb):

Between the CS MGWs and between CS- and IMS-networks RTP is used for transporting the EVS payload, including CMR for Rate and Bandwidth adaptation. This is exactly as specified for MTSI in TS 26.114.

(...) However if the CMR is sent out-of-band, then existing procedures for the IuFP will not work. In case the CMR is sent in-band, CT3 will need to update interworking procedures in TS 29.163.

CT3 asks SA4 to kindly take the above comments into account and to confirm whether the CMR will be sent in-band or out-of-band.

SA4 confirms that CMR will be sent in-band, and requests CT3 to update TS 29.163 according to the above requirement.

SA4 would also like to inform CT3 that SA4 has agreed the encoding of the Single Codec IE for EVS proposed in S4-151369, and requests CT3 to specify the mapping between Single Codec subfields and SDP parameters for the EVS codec in TS 29.163 accordingly.*

*Editor’s Note: assuming the Single Codec IE is agreed at SA4#86.

To CT4 group:

MSC<=>MSC signalling (Nc):

One Code Point, "EVSoCS" (0x0Eh) is sufficient for the BICC signalling between MSCs, considering the additional IEs (Information Elements) included in the BICC IAM and APM can convey all relevant configuration parameters, such as bit rates, audio bandwidths, DTX, and more.

CT4 noted that for "EVSoCS" SA4 will use one of the spare values from table 4.2 in TS 26.103. CT4 would like to confirm that one Code Point "EVSoCS" will be sufficient for the BICC signalling between MSCs.

SA4 decided to define one code point for EVS in 0x0000.1110 UMTS_EVS.

CT4 would like to emphasize that impacts on signalling in the CS Core Network, i.e. between the MSCs, should be minimized.

SA4 acknowledges the answer.

Codec parameter negotiation (call setup)

SDP parameters for EVS are mapped to CS-signalling parameters and vice versa, as needed.

The CS-Control Plane signalling may be more constrained than the SIP Control Plane signalling for some EVS Codec parameters, e.g. audio bandwidth selection. Assuming that only Configurations A, B, C would be supported, then compatibility would exist to a subset of the signalling space in the SIP/SDP, as defined in TS 26.114.

CT4 agrees that the standardized SDP parameters for EVS allow many ways to deploy the EVS codec.

CT4 noted that SA4 is planning to introduce three Configurations A, B, C which are all TrFO-compatible to each other under all conditions.

Regarding the potential additional Configurations with other mode-combinations which are not TrFO-compatible to the Configurations A, B, and C, CT4 would like to emphasize that unnecessary transcoding should be avoided and that any additional Configuration requires additional specifications and testing efforts both on the core network side and on the UE side. A call setup and any succeeding handover should result in transcoding free continuation whenever possible.

CT4 suggestion would be that SA4 should try to avoid specifying of additional Configurations which are not TrFO-compatible.

SA4 acknowledges the answer by CT4 group that the configurations A,B,C are TrFO-compatible to each other under all conditions. Note that SA4 is discussing potential additional Configurations.

SA4 has agreed on the following mode sets:*

Table 5.8-1: Allowed Configurations for the EVS Codec Type

| Configuration → (Config-EVS-Code) ↓ Codec Mode | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| EVS Primary 24.4 |  |  | 1 |  |
| EVS Primary 16.4 |  |  | 1 |  |
| EVS Primary 13.2 | 1 |  | 1 | 1 |
| EVS Primary 9.6 | 1 |  | 1 | 1 |
| EVS Primary 8 | 1 | 1 | 1 |  |
| EVS Primary 7.2 | 1 | 1 | 1 |  |
| EVS Primary 5.9VBR | 1 | 1 | 1 |  |
| AMR-WB IO 12.65 | 1 |  | 1 | 1 |
| AMR-WB IO 8.85 | 1 |  | 1 | 1 |
| AMR-WB IO 6.60 | 1 | 1 | 1 | 1 |

The "1" in the table indicates that the Codec Mode is included in the Active Codec Set of the Configuration. 

Notes:

- Configuration 0 is conditionally mandatory for all networks supporting UMTS_EVS.

- All Configurations are conditionally mandatory for all terminals supporting UMTS_EVS.

- The maximum audio bandwidth at a given bit rate shall be used. If bandwidth adaptation is allowed for EVS Primary modes (e.g. by CMR), this specified audio bandwidth may be reduced to a lower bandwidth during the call; if the input signal has a lower audio bandwidth, the coded audio bandwidth may also be lower in some cases.

- Typical SF for configurations 0, 1, 2, 3 on DL are respectively 128, 256, 64, 128.

- From a transmission point of view, 5.9 VBR mode (5.9 kb/s average gross bit rate) includes bit rates of 2.8, 7.2, 8.0 kb/s.

*Editor’s note: assuming these mode sets are agreed at SA4#86

To RAN1 and RAN2 groups:

- On possible extensions of EVS configurations to higher codec rates (fitting the same SF), it is suggested to exclude the bit-rates in brackets from the EVS configurations A & B (and/or A* & B*) identified by SA4. i.e., configs A & B (and/or A* & B*) should have maximum bit-rates of 8 and 13.2 kbps, respectively, in order to fulfill the SF requirements.

SA4 confirms that configurations A, B (A*, B*) will have maximum bit rates of 8 and 13.2 kbps, respectively.

- RAN1 would leave the final decision on the number of bits to be used for CMR to SA4, provided this number is small (e.g. less or equal than 7) and provided that SA4 is aware that it is preferable to use as few bits as possible.

SA4 confirms that a 7-bit CMR in-band signaling mechanism will be used at all bit rates.

- Initial simulation results and analysis show that UEP gains over EEP seem limited, thus using EEP for EVS is overall suitable.

SA4 acknowledges that its decision to use EEP for EVS matches the understanding of RAN1.

To RAN2 group:

- It is considered beneficial to include low codec rates in EVS configurations, mainly from a capacity and RAB reconfiguration standpoints. Removing the lower codec rates can also impact coverage and retainability, e.g. if a UE transmitting at maximum power is forced to stay on a high codec rate. Therefore RAN2 recommends SA4 to preserve the low codec rates within the set of EVS codec rates to be supported over UTRAN CS.

SA4 has the working assumption that for each SF in DL 64, 128, 256, at least one mode set will be defined that contains all modes that can be supported, also further discussions will take place on additional mode sets that do not contain certain of these bit rates.

SA4 has agreed on the following mode sets:*

Table 5.8-1: Allowed Configurations for the EVS Codec Type

| Configuration → (Config-EVS-Code) ↓ Codec Mode | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| EVS Primary 24.4 |  |  | 1 |  |
| EVS Primary 16.4 |  |  | 1 |  |
| EVS Primary 13.2 | 1 |  | 1 | 1 |
| EVS Primary 9.6 | 1 |  | 1 | 1 |
| EVS Primary 8 | 1 | 1 | 1 |  |
| EVS Primary 7.2 | 1 | 1 | 1 |  |
| EVS Primary 5.9VBR | 1 | 1 | 1 |  |
| AMR-WB IO 12.65 | 1 |  | 1 | 1 |
| AMR-WB IO 8.85 | 1 |  | 1 | 1 |
| AMR-WB IO 6.60 | 1 | 1 | 1 | 1 |

The "1" in the table indicates that the Codec Mode is included in the Active Codec Set of the Configuration. 

Notes:

- Configuration 0 is conditionally mandatory for all networks supporting UMTS_EVS.

- All Configurations are conditionally mandatory for all terminals supporting UMTS_EVS.

- The maximum audio bandwidth at a given bit rate shall be used. If bandwidth adaptation is allowed for EVS Primary modes (e.g. by CMR), this specified audio bandwidth may be reduced to a lower bandwidth during the call; if the input signal has a lower audio bandwidth, the coded audio bandwidth may also be lower in some cases.

- Typical SF for configurations 0, 1, 2, 3 on DL are respectively 128, 256, 64, 128.

- From a transmission point of view, 5.9 VBR mode (5.9 kb/s average gross bit rate) includes bit rates of 2.8, 7.2, 8.0 kb/s.

*Editor’s note: assuming these mode sets are agreed at SA4#86

2. Actions:

To CT1, CT4:

ACTION: SA4 asks CT1 and CT4 to take SA4 reply into account.

To CT3:

ACTION: SA4 asks CT3 to take SA4 reply into account and to update TS 29.414, TS 29.415 and TS 29.163.

To RAN1, RAN2:

ACTION: SA4 asks RAN1 and RAN2 to take SA4 reply into account.

3. Date of Next SA4 Meetings:

SA4#87	25th – 29th January 2016, Sophia Antipolis, France

-------------------------------------------------------end--------------------------------------------------------------