3GPP TSG-SA4 Meeting #88 	S4-160436

# Memphis, USA, 2016-04-18

| CR-Form-v9.9 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.103 | CR | 0043 | rev | - | Current version: | 13.1.0 |  |
| For HELP on using this form look at the pop-up text over the symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | X | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Mandatory EVS Configuration for CS Networks |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM; Swisscom; Qualcomm Incorporated; |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | EVSoCS |  |  |  |  |  |  | Date: |  | 2016-04-12 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change: |  | EVS may be deployed in a multitude of call scenarios in CS- and IMS-networks and between these. The standard for UMTS_EVS was completed for REL-13, with some minor points left FFS. One such point is the mandatory Codec Configuration for UMTS_EVS in CS networks. The discussion focused on the way, how to guarantee that a call with EVS can be setup and maintained in SWB quality, regardless of implementation options for the UE or radio link variations. One approach 1 brough forward is, to negotiate UMTS_EVS (Set 3) at call setup as Selected Codec, which by specification contains only EVS-SWB audio bandwidth. Nevertheless, it must be remembered that the EVS Encoder may use also EVS-WB or even EVS-NB modes to encode the speech signal - at the same bit rate. So also this approach is not 100% watertight. The other approach 2 brought forward is, to negotiate EVS (Set 1) or EVS (Set 2) as Selected Codec and control the audio bandwidth by EVS-CMR, commanded by the network during the call. A set of rules have been discussed and agreed in TS 26.454, clause 7, where it is explicitely stated: the UE must follow the audio bandwidth, as commanded by the received EVS-CMR. The network has exactly the same right to freeze the audio bandwidth.On the other hand the network has also the means to escape to a more robust codec mode, if worse comes to worse and the link quality cannot be kept. The sources of this CR share the opinion that this approach 2 is at least as good for the EVS-SWB only Service, as approach 1. Since it allows far more flexibility to react on network load conditions and allows a wider range for TrFO-compatibility with IMS networks, this CR is brought forward to resolve the last remaining FFS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The remaining FFS in a Note is removed and the mandatory EVS Configuration for CS Networks is defined as UMTS_EVS (Set 1). Some minor typing errors and formatting errors are corrected, too. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The introduction of EVS in CS networks may be delayed. Interoperability problems between networks will lead unavoidably to transcoding. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Begin of change

## 5.7A	EVS Codec Type (UMTS_EVS)

The UMTS_EVS Codec Type [25]…[38] is applied in UTRAN and the CS core network, with the Codec IDentification (CoID) code as follows: 
UMTS_EVS_CoID	:= 0x0000.1110.

The UMTS_EVS Codec Type can be used in conversational speech telephony services in a number of different configurations. These allowed configurations for CS networks are defined in Table 5.7A-1.

The combination of the UMTS_EVS Codec Type with one of these allowed configurations for CS networks is abbreviated with "UMTS_EVS (Set x)", e.g. UMTS_EVS (Set 1).

Table 5.7A-1: Allowed Configurations for the UMTS_EVS Codec Type

| Configuration→ (Config-EVS-Code or Set) ↓Codec Rate (kbps) | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| Typical SF on downlink | 256 | 128 | 64 | 128 |
| EVS Primary 24.4 |  |  | nb-fb |  |
| EVS Primary 16.4 |  |  | nb-fb |  |
| EVS Primary 13.2 |  | nb-swb | nb-swb | swb |
| EVS Primary 9.6 |  | nb-swb | nb-swb | swb |
| EVS Primary 8 | nb-wb | nb-wb | nb-wb |  |
| EVS Primary 7.2 | nb-wb | nb-wb | nb-wb |  |
| EVS Primary 5.9VBR (NOTE 2) | nb-wb | nb-wb | nb-wb |  |
| EVS Primary SID | nb-wb | nb-swb | nb-fb | swb |
| EVS AMR-WB IO 12.65 |  | wb | wb | wb |
| EVS AMR-WB IO 8.85 |  | wb | wb | wb |
| EVS AMR-WB IO 6.60 | wb | wb | wb | wb |
| EVS AMR-WB IO SID (NOTE 3) | wb | wb | wb | wb |
| NOTE 1: Every Speech or SID payload field is complemented by appending the 7-bit EVS-CMR field for Maximum Rate and Bandwidth Control. For details, see TS 26.454 [38]. NOTE 2: From a transmission point of view, the EVS Primary 5.9 VBR mode of operation includes three bit rates of 2.8, 7.2 and 8.0 kb/s to achieve the 5.9 kb/s average bit rate in active speech segments. Together with EVS Primary SID in speech pauses and the EVS-CMR appended in every Speech and SID frame, the total average net bit rate, assuming 40% speech pause and 60% active speech is in the order of 4.4 kb/s. |  |  |  |  |

Table 5.7A-2 defines the coding of the "Single Codec" information element for the UMTS_EVS Codec Type in a BICC-based CS core network.

Table 5.7A-2: Coding of "Single Codec" for the UMTS_EVS Codec Type

| Octet | Parameter | MSB 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 LSB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 m | Single Codec | Single Codec (see ITU-T Q.765.5) |  |  |  |  |  |  |  |
| 2 m | Length Indication | 4, 5 |  |  |  |  |  |  |  |
| 3 m | Compat. Info | Compatibility Information |  |  |  |  |  |  |  |
| 4 m | OID | ETSI OID (See ITU-T Q.765.5 [6]) |  |  |  |  |  |  |  |
| 5 m | CoID | UMTS_EVS_CoID |  |  |  |  |  |  |  |
| 6 m | Config-EVS | (spare) | (spare) | (spare) | (spare) | Config-EVS-Code |  |  |  |

Config-EVS-Code has the following allowed values: 0, 1, 2, 3. All other values are reserved for future use.

Mandatory for UTRAN terminals supporting UMTS_EVS:
If an UTRAN terminal offers Codec Type UMTS_EVS in the Supported Codec List Information Element, then all allowed EVS Configurations for CS networks shall be supported by this UTRAN terminal. The UTRAN terminal shall always apply Discontinuous Transmission (DTX) in uplink direction and shall accept in downlink direction all received EVS Codec Modes and DTX.

Mandatory for UTRAN and CS Network infrastructures supporting UMTS_EVS:
If an UTRAN or a CS Core Network infrastructure supports UMTS_EVS, then the mandatory EVS Configuration is UMTS_EVS (Set 1).

NOTE:	The rules for interworking on user plane between the allowed EVS Configurations for CS networks and the other EVS Configurations (e.g. for MTSI) are defined in TS 26.454 [38]. Also the rules for Maximum Rate and Bandwidth Control in CS networks are defined in TS 26.454 [38].

End of changes