3GPP TSG-SA4 Meeting #89 	S4-160647

# Kista, Sweden, 27. Jun - 1. Jul 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.454 | CR | xxxx | rev | - | Current version: | 13.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 11.4 Interworking between Nb and Mb |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM; |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | EVSoCS |  |  |  |  |  |  | Date: |  | 2016-06-21 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | - Similar, overlapping and partly contradicting text elements describing the User Plane interworking within MGWs and between MGW terminations are currently found in TS 26.454 (this specification) and in TS 29.414, TS 29.415 and TS 29.163. This is a potential source for interworking problems. Especially the handling of DTX is currently inconsistent and contradicting. - Some terms are aligned for consistency with other clauses. - The various alternatives for rate and mode control on Mb as well as the options for N-frame packing on Mb have not been considered so far. - Per definition in TS 29.163 the IM-MGW interworks only between Mb and Nb and not other CS-network interfaces. This is corrected. - Some text is still in "[]" and these must be removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR includes all relevant text elements from the 29 series in TS 26.454. The missing interworking cases are described. The [] are removed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Interworking problems may arise, which may delay the market introduction of EVSoCS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Begin of first change

## 11.4	Interworking between Nb and Mb

### 11.4.1	Interworking for EVS Rate Control

#### 11.4.1.1	General

On the Nb interface, EVS-CMR, as specifies in the present document, provides for in-band rate and mode control via the EVS-CMR field of every Codec frame (Speech or SID) and CMR-Only frame.

On the Mb interface, CMR as specified in 3GPP TS 26.445 [xx] subclause A.2.2.1.1 provides for in-band rate and mode control via the Codec Mode Request (CMR) field in RTP, or via rate and mode control commands in RTCP-APP packets. Clause 10.3 of the present document gives and overview of various alternatives for rate and mode control for EVS on Mb.

NOTE: 	TS 26.114 specifies the RTCP-APP for EVS without deploying the EVS-CMR. Instead, TS 26.114 specifies new rate and mode control commands for EVS. The present document does not describe the Interworking between EVS-CMR in RTP and these RTCP-APP commands.

Interworking of rate and mode control procedures at an IM-MGW between an Mb interface and a corresponding Nb interface only applies, when the IM-MGW bridges compatible codec configurations between the interfaces without applying a transcoding function and if the SDP offer answer negotiation has not disabled the rate and mode control on Mb (Alt 4 in clause 10.3). In all other cases transcoding applies and the rate and mode control loops on Nb and Mb are independent.

#### 11.4.1.2	 RTP on Mb contains one Speech or one SID frame

Transcoding free Interworking between Nb and Mb is easiest, when exactly one Speech or SID frame or one CMR-Only command is included in RTP on Mb, as is the case on Nb.

An IM-MGW, receiving an EVS-CMR from an incoming Nb interface in either a Speech or SID or CMR-Only packet, may filter and modify the EVS-CMR contents based on operator policies and shall map the resulting EVS-CMR onto the EVS Configuration on the outgoing Mb interface. Depending on the negotiated alternative for CMR on Mb, the IM-MGW shall send the resulting CMR either

Alt 1: 	in the corresponding Speech or SID or CMR-Only packet,
 	in which case every outgoing Speech or SID packet contains the active CMR; or

Alt 2: 	only if the resulting CMR-value is different to the previously sent CMR-value ("CMR on demand"), 
 	in which case the new CMR should be repeated at least [3] times within the next [100ms]
	to provide at least a minimum of error robustness, if necessary using also CMR-Only packets; or

Alt 3:	a corresponding RTCP-APP command, if CMR in RTP is disabled and RTCP-APP is enabled,
 	in which case also some redundancy should be foreseen for a minimum of error robustness
 	and note should be taken that RTP and RTCP-APP packets are not synchronized with respect to each 
 	other.

An IM-MGW receiving a rate or mode control command from an incoming Mb interface in either a Speech or SID or CMR-Only packet may filter and modify the EVS-CMR contents based on operator policies and shall map the resulting EVS-CMR onto the EVS Configuration on the outgoing Nb interface. In all cases, the IM-MGW shall send an active CMR in every Speech or SID or CMR-Only frame on Nb.

Depending on the negotiated alternative for CMR on Mb, the IM-MGW may have to translate the incoming rate and mode control procedure. If the negotiated Rate and Mode control procedure on Mb is

Alt 1:	then the IM-MGW gets in every incoming Speech or SID or CMR-Only packet the active CMR,
or - depending on the implementation in the  media-sender - the IM-MGW may get sometimes an active CMR and sometimes a NO_REQ code in the CMR field (see TS 26.445 [], Table A.3: Structure of CMR);

NOTE:	The handling of NO_REQ in the IM-MGW is FFS, because the meaning of NO_REQ is undefined in TS 26.445.

Alt 2:	then the IM-MGW gets sometimes ("CMR on demand") an active CMR within a Speech or SID or CMR-Only packet on Mb, in which case the IM-MGW must store the most recent active CMR for future use, until it is replaced by a new active CMR;

Alt 3: 	then the IM-MGW gets sometimes rate and/or mode control commands on Mb via RTCP-APP, in which case the IM-MGW must translate these into corresponding EVS-CMR and handle them as in Alt 2.

#### 11.4.1.3	 RTP on Mb contains multiple Speech or SID frames

Transcoding free Interworking between Nb and Mb is also possible, if multiple Speech or SID frames are packed into one RTP packet on Mb.

If N frames (Speech or SID or CMR-Only) are received on Nb and are packed into one RTP packet on Mb, then the most recent CMR-value is packed into the RTP packet, all other, older CMR-values are ignored. The most recent EVS-CMR is filtered, modified and mapped as described above.

### If such an N-frame RTP packet is received on Mb, then the received CMR is valid for all Speech or SID frames inside the RTP Packet. This CMR is filtered, modified and mapped as described above and then the IM-MGW sends it in every outgoing Speech or SID or CMR-Only frame on Nb.

### 11.4.2	Interworking for Discontinuous Transmission

In CS Networks DTX is always "on" in the uplink direction for AMR, AMR-WB and EVS, i.e. in direction CS => IMS. The CS-Networks is always able to receive DTX (SID Frames and pauses) in direction CS <= IMS.

On the Mb interface, it is, however, possible to disallow DTX in send and/or receive direction.

If DTX is disallowed on the Mb interface by the SDP Offer-Answer negotiation, then a media-receiver on the remote end of the Mb-link shall still be able to handle DTX (SID Frames and pauses and generate Comfort Noise) in its receive direction (CS => IMS). Transcoding is not necessary in the IM-MGW for DTX interworking. The IM-MGW shall send SID frames on Mb even if the SDP negotiation disabled DTX on Mb.

End of changes