Source:	Samsung Electronics Co., Ltd.

Title:	API Notification and Security Considerations

Document for:	discussion and agreement

Agenda Item:	9.6

# Introduction

3GPP SA4 is working on the definition of an API [1] between the MBMS client and applications that wish to use MBMS for data reception. The API provides different ways to access services that are delivered over MBMS such as service discovery, single file reception, file carousels, DASH over MBMS, RTP streaming, and other generic applications.

In this contribution, we address two issues that to the best of our knowledge have not been handled by the study item:

- Asynchronous notifications that are sent back to the application

- Restricting access to the downloaded content after transport layer protection is removed

# Notification Callbacks

Several MBMS API calls need to be performed asynchronously in a non-blocking fashion. This applies to the following non-exhaustive list:

- Request for download of one file and potentially its updates with a notification when the download of a version is complete

- Notifications about MPD updates in a live DASH session

- Receive a notification about the start of a user service or a delivery session

- Receive a notification about the update of a user service

- Receive notifications about the status of a download or about the reception

In fact, the MBMS API relies heavily on notification callbacks. However, different implementations of the API in different environments will have to use different callback mechanisms. The following table summarizes the most widely used callback mechanisms for inter-process communications:

| Environment | Callback Mechanism | Description |
| --- | --- | --- |
| Android | Broadcast Intents | The MBMS client registers and creates a new instance of the Intent class. It populates the intent’s extras with information about the event (e.g. the URL of the downloaded file and its cache location) using the putExtra method. Then it broadcasts the event using the sendBroadcast method. On the receiver side, a subclass of BroadcastReceiver is created and the onReceive method is overwritten to receive the broadcast intent. The newly created broadcast receiver must be registered in the Manifest of the application. |
| iOS | URL scheme | The MBMS client registers a new URL for each event type. When an event needs to be sent to the application, the MBMS client populates the URL with the extra parameters as part of the URL query part and triggers the opening of the URL. The receiver application registers a URL scheme handler for the corresponding event by including a CFBundleURLName in the Info.plist file of the application. When a URL is invoked, the receiver application is invoked and it checks if it is interested in processing the event or not. In the MBMS API, the URL may actually point to a downloaded file or to a document that describes the event. |
| Web Environment | Event Handlers | The MBMS client will create an instance of the Event class specific to the event type that needs to be communicated. The event may be populated with custom parameters as part of the event callback function. Finally, the event is triggered by calling the trigger function. The receiver application, on the other hand, will have to implement the event handler and register or add it for that specific event. |
| HTTP | SSE | The application receiver creates a new EventSource and passes the URL of the event source as a parameter. Whenever a new event is sent by the server, the onmessage callback function will be triggered. The MBMS client may for instance be implemented in a server scripting language such as PHP. It keeps the connection to the client up and running and writes events when they become available. An event message has to have the “text/event-stream” MIME type and the boundaries of an event is indicated by 2 line breaks. The event must have an event field, a data field, and may have an id and retry count. |

The previous table showed how different the callback function implementation may be, even when running on the same operating system. It is imperative that the realization of the notification callbacks remains a matter for implementation. What needs to be specified by the API should be limited to the event information parameters and their formats.

# Security Aspects

MBMS offers transport security to ensure that content is not accessible by non-authorized UEs that might be listening to the broadcast. However, the API opens up new security threats as the transport security/encryption ends already at the MBMS client.

Malware applications may for instance register for notifications and access broadcast resources, which actually should be restricted to authorized applications.

A mechanism to restrict access to authorized applications needs to be built in in the MBMS API.

# Conclusion

We propose to take the information in sections 2 and 3 into account when designing the MBMS API. In particular, we propose that the API shall not define a particular callback mechanism and it shall define means to ensure restricted access to the MBMS distributed resources.

# References

[1]	SA-700054, WID: MBMS Transport Protocol and APIs