3GPP TSG-SA4 Meeting #92                                                                                              S4-170031

Talinn, Estonia, 23 – 27 Jan 2017

# Agenda item: 	10.6

# Source: 	Fraunhofer HHI

Title: 	VR Video Systems

Document for	Discussion and agreement

# 1. Introduction

# This contribution gives an overview of the building blocks of VR video systems.

# 2. VR Video Systems

# Figure X0: Overview of the VR video processing chain

# 2.1. Capture:

VR content may be represented in different formats, e.g. panoramas or spheres depending on the capabilities of the capture systems. Many systems capture spherical videos covering the full 360°x180°. Capturing of such content is typically done by multiple cameras. Various camera configurations can be used for recording 2D and 3D content.

For 2D content, cameras can be mounted on a ring to capture horizontal panoramas or mounted on a sphere to capture spherical (360°) video. Multi-camera arrangements for capturing panoramic videos require focal points of all camera views to coincide at a common point so that stitching can easily be performed with minimum parallax effect.

3D content can be captured by stereo camera pairs with a relatively small overlap arranged in a star configuration (“segmented stereo”). However, such camera systems may suffer from serious parallax errors. On the other hand, mirror-based systems can capture 3D images using camera pairs reducing parallax errors drastically. Another option is “stereo by extreme overlap” in which stereoscopic content is created from overlapping images captured by either fish-eye or wide-angle lenses, or by clusters of cameras. During processing each image sensor is split into a left and right section and corresponding left and right panoramas are stitched from these sections.

# Figure X1. (Left) Segmented stereo (right) Stereo by extreme overlap

Light field rendering is another promising approach for creating 3D content [R0]. However, light-field rendering requires unrealistically dense camera grids. Hence, existing approaches use depth-based rendering to generate the intermediate camera views and reduce the required number of cameras (Figure X2). The disadvantage of this approach is that very accurate depth maps and sophisticated depth-based processing are needed which increases the required computational power and makes the approach error-prone.

# Figure X2. (Left) Depth-based light field processing (Right) Generation of intermediate views

# 2.2. Sphere stitching:

Captured views from each camera are stitched together to combine the single views of the omnidirectional camera systems to a single panorama or sphere. The stitching process should avoid parallax errors and visible transitions between the single views. Parallax errors occur because cameras do not have a common optical center when arranged in a star configuration. This results in blind regions (gaps) between the camera views. Also, in the overlap areas, objects recorded from different viewpoints appear on different positions in the single views. Blind regions and artefacts in overlap areas impede the stitching process (Figure X3). Stitching captured camera views parallax-free is much less complex for camera arrangements with a common focal-point, e.g., mirror-based systems. Such systems can reduce parallax down to objects within 1m [R1].

# Figure X3. Illustration showing how parallax errors occur [R1]

Stitching can be done offline during post-production stage or in real-time. For live transmission, a real time stitching process is required which should be able to process a large amount of data from multiple cameras and provide a high quality, error free panorama or sphere. Real time stitching is highly dependent on the omnidirectional camera system and is still a big challenge especially in terms of avoiding the parallax errors.

Taking into account the camera arrangements and stitching techniques, existing camera systems can roughly be classified into: mirror-based systems (direct stitching), systems with depth-aware stitching (segmented stereo and stereo by extreme overlap), and systems with depth-enabled light field rendering. Table T1 gives a summary of the advantages and disadvantages of such camera systems, respectively:

| System | Pros | Cons |
| --- | --- | --- |
| Direct stitching by mirror-based systems | - Parallax-free setup - Easy stitching - Almost no overlap - High resolution (>10k, 60fps) - Full lens control - Real-time 2D&3D processing - Capable of live transmission | - Bulky system - Calibration needed - Sensitive to damages |
| Depth-aware stitching (Segmented stereo, stereo by extreme overlap) | - Small form factor, light weight - Robust and compact systems - Easy handling - No calibration at setup - Existing stitching software tools - Established post-production | - Usually closed system, no lens control - Restricted real-time processing, limited use for live events - Reduced resolutions (due to overlap) - Anisotropic resolution of wide-angle and fisheye lenses causing sometimes extreme distortions in the resulting panoramas (for stereo by extreme overlap) |
| Depth-enabled light field rendering | - Parallax-free rendering - Enables producing novel views of the scene thus moving freely inside the scene | - Complex computing - Supervised post-production - Still an error-prone process - No real-time capabilities yet |

Table T1: Comparison of existing VR capture/stitching systems

# 2.3. Projection:

Modern video coding standards are not designed to handle spherical content. Therefore, in VR systems, projection is used for conversion of a spherical (or 360°) video into a two-dimensional rectangular video before the encoding stage. A sphere can be projected onto a plane in various ways [R2]. However, no projection method can be distortion free. The distortion caused by the conversion from spherical to planar domain is referred to as “sampling distortion”. Final reconstruction quality of a spherical video is a function of both sampling and coding distortions.

# 2.3.1. Common projection methods

The most commonly used projection method is equirectangular projection, in which the horizontal and vertical coordinates simply correspond to longitude and latitude, respectively, with no transformation or scaling applied [R3]. However, equirectangular projected images have large redundancy near the poles because they are stretched in latitude direction. This causes a redundant number of bits to be spent to encode the poles of image (relative to the actual information content).

A projection that reduces the sampling distortion (compared to equirectangular) is cubic projection in which a portion or whole of the sphere is projected to planar images. The images are arranged as the faces of a cube each of which has a 90°x90° FoV. Cube mapping is a sub-case of rectilinear (gnomonic) projections in which straight lines in real 3D space are mapped to straight lines in the projected space. Hence, each cube face retains straight lines [R4].

Figure X4: Equirectangular projection (left) vs Cubic mapping (right) of the same spherical content. Cube maps provide an approximately equal-area projection whereas equirectangular projection contains redundant samples towards poles of the image.

# 2.3.2. Projection methods in MPEG OMAF:

Working Draft of Omnidirectional Media Application Format (OMAF) developed by MPEG specifies a list of projection methods that can be used for conversion of a spherical video into a two-dimensional rectangular video.

In OMAF, projection methods are classified according to the type of geometry used for rendering the omnidirectional video. These types are:

- Sphere

- Squished Sphere

- Cylinder

- Platonic Solid

  - Cube (6 surfaces)

  - Octahedron (8 surfaces)

  - Icosahedrons (20 surfaces)

- Truncated Pyramid

- Segmented Sphere

- Direct Fisheye

A detailed explanation of these projection methods can be found in [R5].

# 2.4. Region-wise mapping (packing)

After projection, the obtained two-dimensional rectangular image can be partitioned into regions that can be rearranged to generate “packed” frames.

The operations to produce packed frames from projected frames (denoted as “packing” or “region-wise mapping”) might include translation, scaling, rotation, padding, affine transform, etc. Reasons to perform region-wise mapping include increasing coding efficiency or viewport dependent stream arrangement (see multi-stream approach in Section 2.5.2).

Figure X5: Example of a multi-resolution cube map. Some of the rectangular areas in the projected frame can be downsampled to construct the packed frame [R6].

# If region-wise mapping is not used, the packed VR frame is identical to the projected frame.

# 2.5. Encoding & Decoding :

Current 360 video services offer a limited user experience since the resolution in the user viewport and hence the visual quality are not on par with traditional video services. Multiple times UHD resolution is needed to cover the full-360 surroundings in a visually sufficient resolution. This poses a major challenge to the established video processing chain and to the available end devices.

# There are mainly three approaches that can be considered for 360 video delivery. All solutions can be grouped into:

- Single stream approach

- Multi-stream approach

- Tiled stream approach

# 2.5.1 Single stream approach

For HMDs, one straightforward approach would be to encode an exact or over-provisioned viewport for each user, i.e. crop the interesting part (e.g. viewport) for the user at the server side and encode it. However, although this approach minimizes the number of non-viewport samples to be decoded, it comes at the cost of a massive encoding overhead when considered for large-scale deployments. Another option that is considered as a single stream approach is to encode the full 360 video, transmit it to the receiver and decode the full 360 video while showing only the viewport.

Therefore, solutions that lie within this group have the drawback that either they are not scalable or they impose a big challenge in terms of required network resources (high bitrate of high resolution video) and required processing at the client side (decode a very high resolution video).

Mobile devices typically contain hardware video decoders tailored to resolutions used in traditional video services (HD or UHD). Therefore, it is important to limit the overall resolution to be transmitted and decoded in the mobile devices.

Using single stream approach, the receiver decodes the entire video (corresponding to either the viewport (exact or over-provisioned) or the full 360 video).

# 2.5.2 Multi-stream approach

The multi-stream approach consists of encoding several streams, each of them emphasizing a given viewport and making them available for the receiver, so that the receiver decides which stream is delivered at each time instance. The number of the streams made available can vary and be optimized; with a larger number of streams, a better match to the users’ viewport can be obtained. However, this requires more storage capacity at the server side. Even though multiple streams are encoded and made available, only a single stream needs to be decoded depending on the users’ viewport.

# There are two ways of generating video bitstreams for the multi-stream approach:

- Projection/Mapping based: A viewport dependent projection (e.g. Truncated Pyramid [R5]) or a projection (e.g. cubic) plus a viewport dependent mapping/packing (e.g. multi-resolution cubemap (Fig. X5)) is used so that the number of samples is higher at the viewport and lower at surrounding areas. The encoding is done as usual, i.e. viewport unaware.

- Encoding based: The encoder is configured so that the samples of the viewport are encoded at a higher quality, e.g. with a lower quantization parameter (QP).

Using multi-stream approach, the receiver decodes the corresponding entire video which results in different resolution or different quality areas. Mobile devices typically contain hardware video decoders tailored to resolutions used in traditional video services (HD or UHD). Therefore, it is important to limit the overall resolution to be transmitted and decoded in the mobile devices.

# 2.5.3 Tiled stream approach

Another approach is to use HEVC tiles or separate video streams for 360 video delivery. It allows emphasizing the current user viewport through transmitting non-viewport samples with decreased resolution, i.e. selecting the tiles from the viewport at a high-resolution version and the tiles that do not belong to the viewport at a lower resolution [R7]. Hence, the full 360° surroundings are always available on the end device but the number of samples that lie outside the user FoV is reduced. For this purpose, at the encoder side the full 360 video is projected into a frame, mapped (e.g. cube map) and encoded into several tiles at different resolutions.

When this approach is followed each of the tiles can be encoded as motion-constrained HEVC tiles or as separate video streams.

Using motion-constrained tile HEVC streams, the varying spatial resolution in video picture can be achieved by merging motion-constrained HEVC tiles of different resolutions into a single common bitstream and therefore this approach allows to use a single decoder.

Using separately encoded video streams, several decoders are required at the receiver side, as many as transmitted video streams.

Using tiled-streaming approach, the receiver can choose to decode only a subset of the received video stream depending on the current viewport position and/or device capabilities (e.g. video decoder capabilities).

# 2.6. File/DASH encapsulation/decapsulation

360 video might require additional signaling in DASH. For instance, projection and mapping formats might be required to be signaled at the MPD so that client can request the appropriate Representations and/or Adaptation Sets [R8].

File/DASH encapsulation is performed differently depending on the type of the considered solution (single-stream, multi-stream, tiled stream).

The receiver can choose to decapsulate only a subset of the received video stream depending on the current viewport position and/or device capabilities (e.g. video decoder capabilities).

# 2.7. Delivery

A panoramic or 360 video can be delivered in unicast, multicast or broadcast mode. In all of these modes, the delivery can be realized in the form of download or streaming and in real-time or non-real time.

For unicast delivery, DASH can be used. For multicast or broadcast delivery, DASH over MBMS can be used.

For unicast delivery, all three approaches mentioned in Section 2.5 (single stream, multi-stream, tiled stream) could be used. Whereas for MBMS, multi-stream is not considered as a viable solution due to the bandwidth requirements for supporting all viewports.

For unicast and MBMS delivery, the DASH client requests appropriate segments depending on the viewport position, available network throughput, device capabilities and service requirements. E.g. for multi-stream approach, the DASH client requests the stream (representation) that matches best to the expected viewport position (subject to network latency and user movement).

# 2.8. Rendering:

Considering the limited FoVs of the existing VR displays, only a part of the video frame needs to be displayed to the user. For instance, FoVs of the most currently available HMDs range between 100°-110° [R9].

Since the actual rendered video will cover only a portion of the scene, the renderer should receive metadata related to the current user viewport. The renderer also uses the video characteristics such as projection and mapping formats.

# 3. Proposal

We propose to include the Section 2 to Section 4.2 on video systems in TR 26.918.

# References

[R0] Levoy, Marc, and Pat Hanrahan. "Light field rendering." Proceedings of the 23rd annual conference on Computer graphics and interactive techniques. ACM, 1996.

[R1] DVB Report of CM-VR-SMG v 22, November 2016.

[R2] J. P. Snyder, “Flattening the Earth: Two Thousand Years of Map Projections,” University of Chicago Press, 1993.

[R3] http://wiki.panotools.org/Equirectangular_Projection

[R4] http://wiki.panotools.org/Cubic_Projection

[R5] N16439. WD on ISO/IEC 23000-20 Omnidirectional Media Application Format, October 2016, Chengdu, CN

[R6] M38945v2. Virual reality video metadata in ISO base media file format, October 2016, Chengdu, CN.

[R7] Skupin, R., Sanchez, Y., Hellge, C., & Schierl, T. Tile Based HEVC Video for Head Mounted Displays.

[R8] N16520. Initial Gap Analysis for Simple Streaming of 360 Media with DASH, October 2016, Chengdu, CN.

[R9] http://newatlas.com/best-vr-headsets-comparison-2016/45984/, accessed on January 9, 2017.