3GPP TSG-SA4 #92	S4-170106

Tallinn, Estonia 23-27 Oct 2016

Agenda item: 	8.9, 8.13

Source: 	Qualcomm Incorporated

Title: 	xMB-U Content and Protocol Format for ATSC 3.0 TV Services

Document for	Discussion and Agreement

# Introduction

The SA4 Rel-14 work item AE_enTV-S4, “eMBMS Delivery of Media and TV Services”, defined in SP-160956 [1], was recently approved during SA#74. The primary objective work item is to “specify the content formats provided over the xMB interface and the mapping of such content to MBMS User Services and delivery methods by the BMSC to the MBMS client as well as the aspects of the necessary control and user-plane APIs in the UE to provide the content to the application by the MBMS client”.

This discussion paper addresses one of the specific objectives and subordinate task of the work item, to identify/define:

- Permitted content formats provided by the content provider over xMB-U and their mapping to MBMS User Services and delivery methods.

  - Properties and restrictions for content formats for transport-only mode

The scope/context of this document is regarding the content format and protocol stack model in the delivery of ATSC 3.0 broadcast TV service content and metadata over the xMB User Plane, or xMB-U interface. The format and protocol models pertain to both the “Transport only mode” or MBMS Service Type 1 as defined in TR 23.746 [2] (hereafter referred to as the transport-only mode), as well as the “Full MBMS service mode” or MBMS Service Type 2 as defined in [2] (hereafter referred to as the MBMS User Service mode), with the further assumption in the latter case that the associated reformatting/translation to MBMS service layer functionality is performed by the BM-SC.

# ATSC 3.0 Overview

The Advanced Television Systems Committee (ATSC) is developing ATSC 3.0 as the next generation ATSC Digital Television (DTV) transmission standard for use over terrestrial broadcast, cable and satellite networks. ATSC 3.0 systems will be deployed in South Korea in 2017 and later in the USA, and likely in the future in other major North American countries such as Canada and Mexico. ATSC 3.0 comprises a family of 19 component specifications which include the physical layer, IP-based transport protocols, service and application level signaling, content formats for carriage of streaming media services, security and content protection, and a W3C-compliant and Web application based runtime environment. The specific specification which defines, among other functionality, the media content format and application transport protocols of ATSC 3.0 services is A/331, “Signaling, Delivery, Synchronization, and Error Protection” [3].

Media content produced by ATSC 3.0-compliant audio and video codecs and closed captioning systems are encapsulated in ISO BMFF (Base Media File Format) container structures. Two types of ISO BMFF compliant file formats are defined in A/331:

- MPEG-DASH as defined by ISO/IEC 23009-1 [4] and further profiled by the DASH-IF Interoperability Point for ATSC 3.0 [5], and

- MPU (Media Processing Unit) as defined by ISO-IEC 23008-1 [6], the latter specification also known as MMT or MPEG Media Transport.

Broadcast transmission of ATSC 3.0 streaming services, e.g. linear TV services, may employ either the ROUTE (Real-Time Object Delivery over Unidirectional Transport) protocol, as defined in A/331 [3], for the delivery of DASH-formatted content, or the MMTP protocol (MPEG Media Transport Protocol) as defined in ISO/IEC 23008-1 [6] for the delivery of MPU-formatted content. File delivery services, as well as other Non-Real-Time (NRT) content such as ROUTE-specific service signaling, application signaling, application content, Electronic Service Guide (ESG) and emergency alert messages, are strictly delivered using the ROUTE protocol. The protocol stack model of an ATSC 3.0 reference receiver is shown in Figure 1 below.

Figure 1: ATSC 3.0 Reference Receiver Protocol Stack

Note that broadcast delivery of ATSC 3.0 Real-Time (RT) and NRT services/content is natively IP-based. There is no use of MPEG-2 Transport Stream for the encapsulation and packetization of streaming media content in the form of Packetized Elementary Streams (PES) as done in DVB-T. For broadcast RT or streaming content, encoded samples of video and/or audio elementary stream are encapsulated in ISO BMFF and transported by ROUTE or MMTP over UDP/IP. For broadcast NRT content of different types, the associated file objects are delivered by ROUTE over UDP/IP. IP packets containing either RT or NRT content are encapsulated in ALP (ATSC Link-Layer Protocol) packets as defined by A/330 [7], which are in turn enclosed in Baseband Packets of Physical Layer Pipes (PLP) as defined by A/322 [8]. The protocol stack models for RT and NRT content delivery are shown in Figures 2a and 2b, respectively.

Figure 2a: ATSC 3.0 Protocol Model for Broadcast Delivery of RT Content

Figure 2b: ATSC 3.0 Protocol Model for Broadcast Delivery of NRT Content

# Content Format and Protocol Model for ATSC 3.0 Service and Metadata Delivery over xMB

This section and its sub-sections describe the format and protocol structure of user plane data, in the form of ATSC 3.0 service content and service metadata, delivered from the TV Content Provider to the BM-SC of the MBMS network operator, across the xMB interface. As previously indicated, these formats and protocol stack models apply to both the transport-only mode, as well as the MBMS User Service mode with content formatting/translation to MBMS service layer functionality performed by the BM-SC. In addition, the content format and protocol model for the exchange over xMB of native ATSC 3.0 service metadata and auxiliary contents such as application signaling, service announcement/discovery signaling, ESG and other auxiliary information, are also described.

## ATSC 3.0 Service Data Content Format and Protocol Model

For streaming TV services such as live events, linear TV programs and video-on-demand (VoD), ATSC 3.0 specifies HEVC for video encoding and includes features such as Spatial Scalable Coding, High Dynamic Range, Wide Color Gamut, 3D, and temporal layering, as defined in ATSC A/341 [9]. For audio, the specified ATSC 3.0 codecs are AC-4 and MPEG-H, as defined in the ATSC A/342 Parts 2 and 3 standards [10], [11]. Video encoding is typically expected to be set to operate in Variable Bit Rate (VBR) mode. Audio encoding is expected to be set to either VBR or CBR (Constant Bite Rate) mode. ATSC 3.0’s captioning and subtitling mechanism as defined in ATSC A/343 [12] is based on SMPTE Timed Text (SMPTE-TT) as specified in SMPTE 2052-1 [13], and in particular, employs the subset of SMPTE-TT features defined by W3C’s “TTML Text and Image Profiles for Internet Media Subtitles and Captions (IMSC1) [14].

NRT content such as download files of an ATSC 3.0’s App-based Service, or files of an App-based feature bound to a linear service, for display to the user under application control, are delivered over the ROUTE protocol, defined in A/331 [3], as file objects described by file metadata in the form of the Extended FDT, similar to the FDT in the FLUTE protocol as specified by RFC 6726 [15].

### ATSC 3.0 Service Content Protocol Stack over xMB-U

The user plane protocol stack model for ATSC 3.0 service data delivery across xMB-U is depicted below in Figure 3a.

Figure 3a: ATSC 3.0 Service Content Protocol Stack over xMB-U reference point and in the EPC

Due to the encapsulation of user plane data in IP, this protocol model has similarity to the MB2-U interface between the Group Communication Service Application Server (GCS AS) and the Broadcast-Multicast Service Centre (BM-SC) as defined in TS 29.468 [17]. In broadcast delivery of ATSC 3.0 streaming services over xMB-U, user plane data comprises ISO base media files containing encoded media samples and carried as DASH Media Segments or MPUs by the ROUTE or MMTP protocol, respectively. This is shown in Figure 3b. The user plane protocol model for broadcast delivery of NRT file objects over xMB-U is shown in Figure 3c. Similar to the operation of GCS application data delivery over MBMS, ATSC 3.0 User Plane data are transparently transported between the TV Content Provider’s Application Server (AS) and the UE. The BM-SC shall forward these protocol layers transparently.

Figure 3b: ATSC 3.0 Streaming Service Protocol Stack over xMB-U

Figure 3c: ATSC 3.0 NRT Service Protocol Stack over xMB-U

## ATSC 3.0 Service Metadata Content Format and Protocol Model

The content format and protocol model for the exchange over xMB-U of ATSC 3.0 service metadata such as application signaling, service announcement/discovery signaling, ESG and emergency alert messages are identical to that of NRT file objects as described in Sec. 3.1.1 and illustrated in Figure 3c.

# Summary and Recommendation

The above sections provide an overview of ATSC 3.0 broadcast TV services and metadata delivery over the xMB-U interface. This document describes the content format and protocol model for service content and service metadata exchanged across xMB-U pertaining to both the transport-only mode and the MBMS User Service mode, with the further assumption for the latter that the associated reformatting/translation to MBMS service layer functionality is performed by the BM-SC.

It is proposed that these descriptions be discussed by the MBS SWG, and if agreed, a CR will be submitted to Rel-14 TS 26.346 to incorporate the contents of Section 3. In addition, it is proposed that the contents of Sections 2 and 3.1 be added to TR 26.917.

# References

SP-160956, “eMBMS Delivery of Media and TV Services (AE_EnTV-S4)”, SA4 Rel-14 Work Item.

TR 23.746, “Study on System Architecture Enhancements to eMBMS for Television Video Service (Release 14)”.

ATSC Candidate Standard A/331, “Signaling, Delivery, Synchronization, and Error Protection”, 01/05/2016, http://atsc.org/wp-content/uploads/2016/01/A331S33-174r5-Signaling-Delivery-Sync-FEC.pdf.

ISO/IEC 23009-1, “Information technology - Dynamic adaptive streaming over HTTP (DASH) — Part 1: Media presentation description and segment formats,” International Organization for Standardization, Draft 3rd Edition, 7/26/2016.

DASH Industry Forum, “Guidelines for Implementation: DASH-IF Interoperability Point for ATSC 3.0”, v1.0.

ISO/IEC 23008-1, “Information Technology – High efficiency coding and media in heterogeneous environments – Part 1: MPEG media transport (MMT)”, 10/23/2015.

ATSC Standard A/330:2016, “Link-Layer Protocol”, 9/19/2016, http://atsc.org/wp-content/uploads/2016/10/A330-2016-Link-Layer-Protocol.pdf.

ATSC Standard A/322:2016, “Physical Layer Standard”, 9/7/2016, http://atsc.org/wp-content/uploads/2016/10/A322-2016-Physical-Layer-Protocol.pdf

ATSC Candidate Standard A/341, “Video”, 11/24/2015, http://atsc.org/wp-content/uploads/2015/12/A341S34-168r5-Video-HEVC.pdf

ATSC Candidate Standard A/342 Part 2, “AC-4 System”, 12/26/2016, http://atsc.org/wp-content/uploads/2016/05/A342S34-188r8-Audio-Part-2.pdf.

ATSC Candidate Standard A/342 Part 3, “MPEG-H System”, 12/27/2016, http://atsc.org/wp-content/uploads/2016/05/A342S34-189r6-Audio-Part-3.pdf.

ATSC Standard A/343:2016, “Captions and Subtitles”, 12/21/2016, http://atsc.org/wp-content/uploads/2016/12/A343-2016-Captions-and-Subtitles.pdf.

SMPTE ST 2052-1:2013, “Timed Text Format (SMPTE-TT)”, available at https://www.smpte.org/standards.

W3C Recommendation, “TTML Profiles for Internet Media Subtitles and Captions 1.0 (IMSC1)”, https://www.w3.org/TR/ttml-imsc1/.

IETF RFC 6726, “FLUTE - File Delivery over Unidirectional Transport", T. Paila, R. Walsh, M. Luby, V. Roca, R. Lehtonen, November 2012.