3GPP TSG-SA4 #93	S4-170408

Busan, Korea 24 – 28 Apr 2017	revision to S4-170305

Agenda item: 	8.6

Source: 	Qualcomm Incorporated

Title: 	File Delivery Manifest Parameters

Document for	Discussion and Agreement

# Introduction

The AE_enTV-S4 work item [1] includes the objective, under the category “Permitted content formats provided by the content provider over xMB-U and their mapping to MBMS User Services and delivery methods”:

Description formats for file delivery sessions that permit efficient use of the download delivery modes in MBMS User services, such as keep-updated, carousels, etc.

Such “description format for file delivery sessions” corresponds to a file delivery manifest referenced by the Session resource property “File delivery manifest URL” of the xMB reference point, as described in TS 26.346, clause 5.4A.2, under Table 5.4A-6. This contribution is a follow-up to companion document S4-170395 [2], which proposes key use cases and derived requirements for the delivery of file-based service from an external content providers over MBMS. Based on the cited requirements in [2] and attributes of the ‘File List’ property of the Session resource as shown TS 26.346, Table 5.4A-6, a proposal is provided on the relevant parameters of the file delivery manifest.

# Background

The file delivery manifest may be implemented as an XML or JSON encoded document and is provided by the Content Provider (CP) to the BM-SC across xMB-C. It contains a set of properties and requirements pertaining to the announcement, ingestion, and MBMS delivery of files over MBMS. Those files might represent content components of a file-based TV service, or correspond to contents affiliated with a streaming service (for example, application files of an interactivity application associated with a linear TV service). It is indicated in Table 5.4A-6 of TS 26.346 that the relevant parameters in the file delivery manifest are expressed by the attributes in the ‘File List’ properties of the Session resource, and may additionally include file schedule information. For ease of reference, Table 1 below reproduces Table 5.4A-6 in TS 26.346:

Table 1: Additional properties for Files (identical to Table 5.4A-6 in TS 26.346)

| Property Name |  | Property Description | C I | C O | G I | G O | U I | U O | T I |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ingest Mode |  | The ingest mode enumerates how resources are ingested into the BM-SC via xMB-U. - Push: The Content Provider shall push the file to the BM-SC that will immediately process and deliver as soon as it is ready. The BM-SC may be configured to ignore all files that are pushed before session active time, or stage them. In case of Push mode, the BM-SC shall provide back to the content provider the URL the Content Provider shall use to push the files. - Pull: In this case, the Content Provider provides the resource location from which the BM-SC will fetch the file. The Content Provider may tell the BM-SC when to start fetching the file Type; Unit; Default. String; None; Pull. |  |  |  | M | O |  |  |
| File List |  | List of files to be sent. In the Push mode, the file list is not used since the BM-SC will monitor its push folder and send the files it receives on a first-come first-served basis. In Pull mode, the file list contains the following information per file entry: - file URL: the URL to the file the BM-SC will use to fetch the content - file display URL: the URL to the file as seen by the UE - file earliest fetch time: The BM-SC shall fetch the file no sooner than this UTC timestamp. If absent, then the file shall be present on the Content Provider server and the BM-SC may fetch it when it wants. - file size (optional): The content provider may provide the precise or a file size estimate as input. The BM-SC may update the file size once it has started to fetch the file. - file status: Enumeration stating the state of the file. Possible values are pending, fetched, prepared, transmitting, sent. - Target reception completion time (on the MBMS Client): hint on the due date, when the file should be completely received by the UE. The BM-SC should schedule and order the transmission etc accordingly. - Keep Update Interval: The BM-SC checks the file resources with the given interval for changes. - File repeat / Duration: The number of times the file shall be sent on the session (a value of 1 means the file shall be sent only once). This counter shall be decreased each time the file has been transmitted. When equals to zero, no more file repeat is scheduled. The BM-SC may send FEC instead of source information. |  |  |  | O | O |  |  |
| File delivery manifest URL |  | Alternative to the file list. The resource may additionally describe scheduling information for the file. Type; Unit; Default. String; None; “”. |  |  |  | O | O |  |  |
| Push URL |  | A resource locator for ingesting content using HTTP via xMB-U. This is a read-only property managed by the BM-SC and only present when Ingest Mode is set to Push. Type; Unit; Default. String; None; “”. |  |  |  | O |  |  |  |
| Display Base URL |  | When ingest mode is set to Push, the Base URL is seen by the UE. Type; Unit; Default. String; None; “”. |  |  |  | O | O |  |  |

As seen in the table, characteristics of files to be delivered over MBMS may include file location, file size, time information on file availability at the content provider or in the UE, status, update interval or repetition information for MBMS delivery. In addition to the ‘File List’ properties, it is indicated by the ‘File delivery manifest URL’ entry in Table 5.4A-6 that file delivery schedule information may also be included in the file delivery manifest.

We will first check for validity and completeness of the indicated parameters under the ‘File List’ and ‘File delivery manifest URL’ properties in Table 5.4A-6 of TS 26.346 for representing parameters of the file delivery manifest. Based on the derived requirements for file-based services originating from external content providers for delivery over MBMS as indicated in S4-170304 [2], the file manifest should logically convey the following three types of information:

- Information required by the BM-SC from the CP to enable content ingestion such as:

  - size/size range of the content files,

  - earliest/latest times a given file is available for acquisition by the BM-SC,

  - resource location (fileURL) of the content files for pull-based ingestion.

- Information pertaining to requirements imposed on the BM-SC by the CP in support of the CP’s service objectives for the announcement and delivery of its file-based services via MBMS or unicast bearers. From that information, the corresponding USD information and MBMS delivery requirements to be supported by the BM-SC can be derived. Examples of this information include:

  - the earliest and latest times that a given file is expected to be received at the UE;

  - if the files are also available on the CP server for unicast acquisition, their resource locations in the form of HTTP(s) URLs;

  - whether the file-based service must be delivered as a Datacasting service, and if so, the requisite parameters pertaining to the specific delivery method for the Datacasting service (‘back-to-back’ or ‘scheduled-and-periodic’).

- General information:

  - if the service is intended by the CP to be delivered to specific geographical areas via MBMS or unicast bearers, the BM-SC should be informed by the CP on the target/restricted delivery area(s).

In general, the information pertaining to type b) should not contain detailed delivery aspects, as such properties should be solely determined or derived by the BM-SC from the high-level service requirements specified by the CP. It should also be checked whether the ‘File List’ parameters in Table 5.4A-6 convey sufficient information for all three types of metadata as indicated above. This assessment is described in Section 3 below.

# Analysis of File List Parameters

Table 5.4A-6 indicates that the set of parameters in the File Delivery Manifest should contain those entries under the ‘File List’ property of the Session resource, as well as including file delivery schedule information. Comparing against the information identified as types a) and c) in Sec. 2, the following observations are made:

- File repeat/Duration in Table 5.4A-6 refers to the number of repetitions of file delivery to be specified in the file schedule of service announcement. Logically, this parameter does not belong in xMB, as the scheduling for file delivery on the MBMS bearer is strictly the prerogative of the BM-SC upon taking into account any temporal requirements on MBMS delivery provided by the CP. In other words, it is inappropriate for the CP to dictate how the BM-SC should send the content via MBMS, as implied by the semantics of this parameter.

- It is not completely clear the meaning of and need for the file status attribute in ‘File List’. For example, what is the meaning of the states “pending” and “prepared”, and how relevant or necessary is it to provide this information to the BM-SC via the manifest? For example, the CP server could delay its response to the BM-SC’s HTTP request for a file when it is not yet available at the CP server, or it might return an error message to the request containing the reason (e.g., “file not ready”) in the message body, as alternative to sending such status information via the file delivery manifest. It is proposed not to include the file status entry in the file manifest.

- A number of parameters pertaining to types b) and c) as described in Sec. 2 are missing in the ‘File List’, as summarized below (non-exhaustive list):

  - Earliest content reception on UE – if required by the CP, the BM-SC should be informed by the CP of the earliest wall-clock time that a given file is expected to be available on the UE for subsequent rendering;

  - Content change – The BM-SC should be aware of whether dynamic changes may occur over a given period in the composition of the contents of the file-based service, and if so, for pull-based content ingestion, it should be notified by the CP of such changes, to enable subsequently informing the UE of such changes and scheduling MBMS delivery of the modified/new content;

  - Type of Datacasting service – if a file-based service will be delivered over MBMS as a Datacasting service, the BM-SC should be informed by the CP on the expected periodic update interval of the stock prices in the reported collection (in order to set the value of the child element interval and attribute @mode of the r12:recurrenceAndMonitoring element in the session schedule, in defining the use of the scheduled-and-periodic or back-to-back delivery mode);

- The following parameters pertaining to file content ingestion (type a) are missing in the ‘File List’, for example:

  - Although file earliest fetch time, indicating the earliest absolute time that the BM-SC is allowed to fetch the file (via pull mode), is included in ‘File List’, the related parameter indicating the latest absolute time that the BM-SC is allowed to fetch the file, call it file latest fetch time, is not included.

  - Although the BM-SC may be informed of the actual size of the file, via the file size attribute, it may additionally wish to know about the size range of files in the file-based service. This information could be given in the form of minimum file size and maximum file size.

4	Candidate Parameters of File Delivery Manifest

This section builds upon the analysis in Section 3 in compiling a comprehensive set of relevant properties of content files which belong to or are associated with TV services (and more broadly, generic file-based 3rd-party application/content services) for delivery over MBMS.

Table 2 below summarizes the desired contents or parameters of the file delivery manifest. The table is intended to convey a simplified, pseudo-schema representation of the manifest in the form of an XML document, but is not meant to preclude alternative content formats such as JSON. For ease of understanding, file properties as defined in TS 26.346, Table 5.4A-6 are shown as normal text, whereas additionally proposed parameters are shown as underlined text.

Note: the use of regular vs. underlined text in Table 2 is intended to differentiate between parameters proposed for the file delivery manifest which correspond to existing entries under the ‘file list’ property of the “Session” resource as shown Table 5.4A-6 in TS 26.346 (not underlined), vs. additionally proposed parameters that are currently not included under ‘file list’ (underlined).

Table 2: Proposed Parameters of the File Delivery Manifest

| Element Name |  |  |  | Card-inality | Data Type | Description |
| --- | --- | --- | --- | --- | --- | --- |
| FileManifest |  |  |  | 1 |  | The File Delivery Manifest. |
|  | FileList |  |  | 1..N |  | Container for the set of files, for delivery over MBMS, whose properties or transport parameters are given in the File Delivery Manifest. Note that in the following description of the attributes and child elements of FileList, the word “file” is a shorthand for “the file in this instance of FileList” |
|  |  | @fileURL |  | 0..1 | anyURI | In the case of pull-based content ingestion, this attribute represents the resource location of the file to be requested by the BM-SC from the Content Provider server. |
|  |  | @fileDisplayURL |  | 0..1 | anyURI | URL of the file as provided to the MBMS-aware application. Presence of this attribute is an indication to the application that the file can be fetched via unicast at this resource location. |
|  |  | @fileEarliestFetchTime |  | 0..1 | unsignedInt | In the case of pull-based content ingestion, the absolute time, expressed as the 32-bit integer portion of an NTP time stamp, representing the earliest time that the BM-SC is allowed to request the file from the Content Provider server. Absence of this attribute is an indication that the BM-SC may submit a request for the file at a time of its choosing. |
|  |  | @fileLatestFetchTime |  | 0..1 | unsignedInt | In the case of pull-based content ingestion, the absolute time, expressed as the 32-bit integer portion of an NTP time stamp, representing the latest time that the BM-SC is allowed to request the file from the Content Provider server. Absence of this attribute is an indication that the BM-SC may submit a request for this file at a time of its choosing. |
|  |  | @fileSize |  | 0..1 | unsignedInt | Size of the file (a precise or estimated value) in units of bytes. The value of @fileSize should be in the range between @fileSizeMinimum and @fileSizeMaximum. If this attribute absent, it means that the file size is not known to the Content Provider. |
|  |  | @targetReceptionCompletionTime |  | 0..1 | unsignedInt | If present, this attribute represents a hint from the CP to the BM-SC on the nominally expected time, expressed as the 32-bit integer portion of an NTP time stamp, at which the file should be completely received by the MBMS client and made available to the MBMS-aware application. The BM-SC is expected to schedule transmission of the file in accordance to this attribute. Absence of this attribute is an indication that there is no explicit requirement by the CP on when the reception of the file at the MBMS client should occur, other than it should not be earlier than @earliestReceptionCompletionTime, if present, in the file delivery manifest. |
|  |  | @earliestReceptionCompletionTime |  | 0..1 | unsignedInt | If present, this attribute represents a hint from the CP to the BM-SC on the earliest time, expressed as the 32-bit integer portion of an NTP time stamp, at which the file is permitted to be received by the MBMS client, to be in turn provided to the MBMS-aware application. The BM-SC is expected to schedule the transmission of the file in accordance to this attribute. Absence of this attribute is an indication that there is no explicit requirement by the CP on the earliest time that reception of the file at the MBMS client should occur. If both @targetReceptionCompletionTime and @earliestReceptionCompletionTime are present and should their values conflict, the @earliestReceptionCompletionTime shall take precedence. |
|  |  | @keepUpdatedInterval |  | 0..1 | unsignedInt | Interval that the BM-SC is expected to check for update of the file in the event that it belongs to a Keep-Updated Service, as expressed in units of milliseconds. |
|  |  | @unicastAvailability |  | 0..1 | boolean | If set to “true” or “1”, it is an indication that the file is also available for unicast acquisition by the application at a Content Provider server whose location is given by the HTTP(S) URL corresponding to the value of the attribute ‘@fileDisplayURL’. |
|  |  | @fileRepetition |  | 0..1 | unsignedInt | Indication of the number of times the file shall be sent in the session. A value of “1” means that the file shall be sent only once. This counter shall be decreased by one each time the file has been transmitted. When the counter reaches zero, the file shall cease to be delivered. The BM-SC may send FEC instead of source information. |
|  |  | Datacasting |  | 0..1 | boolean | If set to “true” or “1”, it is an indication that the file-based service to which the file belongs is a Datacasting service. If this element is absent, or is set to “false” or “0”, it implies that the file does not belong to a Datacasting service. |
|  |  |  | @mode | 0..1 | boolean | If Datacasting is set to “true” or “1”, - @mode, when set to "true" or "1", is an indication that the BM-SC should employ the ‘scheduled-and-periodic’ delivery mode of the Datacasting service. - @mode, when to "false" or "0", or if this attribute is absent, is an indication that the BM-SC should employ the ‘back-to-back’ delivery mode of the Datacasting service. |
|  |  |  | @interval | 0..1 | duration | If Datacasting is set to “true” or “1”, and - when @mode = "true" or "1", @interval represents the time duration between successive scheduled transmissions - when @mode = "false" or "0", @interval represents the nominal duration of successive updates of files in the back-to-back delivery mode. |

5	Summary and Proposal

It is proposed that the above parameters of the file delivery manifest be discussed and agreed for inclusion in TS 26.346, towards fulfilling the associated objective in the AE_enTV-S4 work item on specification of a file delivery manifest for delivery across xMB-U, as an alternative to delivering the equivalent information via xMB-C. A CR corresponding to this discussion paper is represented by Tdoc S4-170409 [3].

6	References

[1]	SP-160956, Rel-14 Work Item AE_enTV-S4, “eMBMS Delivery of Media and TV Services”.

[2]	Tdoc S4-170395, “Use Cases and Requirements for File-based Services over MBMS”, Discussion Paper contribution from Qualcomm to SA4 #93.

[3]	Tdoc S4-170409, “File Delivery Manifest for xMB-U”, Change Request contribution from Qualcomm to SA4 #93.