3GPP TSG-SA4 Meeting #95	S4-170994

# Belgrade, Serbia, 9th Oct 2017 - 13th Oct 2017						 Revision of S4-170820

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.247 | CR | 0112 | rev | 1 | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SAND Network Assistance mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sony Corporation, Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | SAND |  |  |  |  |  | Date: |  |  | 2017-10-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Avoid potential confusion with the Network Assistance session initiation request message parameters |  |  |  |  |  |  |  |  |
| Summary of change: |  | Rename one parameter, provide introductory text and remove the note on FFS for the network assistance initiation request message, and make parameter naming consistent for the description of the network initiation response message |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Disclarity about the parameters of the Network Assistance session initiation request and request messages |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13.6.5.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** CHANGE 1 ***

##### 13.6.5.3.1		Network Assistance session initiation

The 3GP-DASH client initiates a Network Assistance session by sending the NetworkAssistanceInitiation SAND extension message to the DANE within a SAND message envelope. The generic procedure for the Network Assistance session initiation request is shown in Table 13-4.

Table 13-4: Network Assistance session initiation generic procedure

| Function call | Originator destination | Parameters |
| --- | --- | --- |
| Network Assistance session initiation request | UE DANE | Media server IP address, Media delivery port number |
| Network Assistance session initiation response | DANE UE | session id, port number, websocket requirement |

In the Network Assistance session initiation request message, the following parameters shall be set by the UE:

-	MediaServerIPAddress – the IP address of the media server;

-	MediaDeliveryPortNumber – the port number used for the delivery of the media segments.

The Network Assistance session initiation request message syntax is shown in Table 13-5.

Table 13-5: Network Assistance session initiation request message syntax

| Parameter |  |  | Type | Cardinality | Description |
| --- | --- | --- | --- | --- | --- |
|  | SandMessage = NetworkAssistanceInitiationRequest |  | Object | 1 |  |
|  |  | MediaServerIPAddress | String | 1 | IP address of the media server delivering content segments in the present Network Assistance session. |
|  |  | MediaDeliveryPortNumber | Integer | 1 | Port number for media server delivering content segments in the present Network Assistance session. |

The NetworkAssistanceInitiationRequest message shall be carried in a SAND message envelope, as specified in ISO/IEC 23009-5 [54], with the following constraints:

-	The senderID element shall be included, in order to provide a reference by which the session initiation request can be authenticated or otherwise authorised at the DANE or elsewhere in the network, relayed by the DANE receiving the message;

-	The generationTime element may be omitted;

-	The messageId element may be omitted.

The NetworkAssistanceInitiationRequest message is defined in the schema whose @schemeIdUri is:

"urn:3gpp:dash: schema:sandmessageextension:2017"

In the Network Assistance session initiation response message, the following parameters are set by the DANE:

-	SessionID – this is used to identify the context of any future messages relating to this Network Assistance session;

-	PortNumber – the DANE sets the port number at which all further NA communications in this session shall occur;

-	WebSocketRequirement – the DANE indicates whether the UE shall set up a WebSocket connection to the DANE for all further NA communications in this session.

- The Network Assistance session initiation response SAND extension message syntax is shown in Table 13-6.

Table 13-6: Network Assistance session initiation response message syntax

| Parameter |  |  | Type | Cardinality | Description |
| --- | --- | --- | --- | --- | --- |
|  | SandMessage = NetworkAssistanceInitiationResponse |  | Object | 1 |  |
|  |  | SessionID | Integer | 1 | Reference to the network assistance session, allocated by the DANE. A value of 0 is reserved to indicate failure, or refusal of the NA session initiation. In case of failure, no further parameters shall be added to this message. |
|  |  | PortNumber | Integer | 0..1 | The DANE informs the 3GP-DASH client of the port number that shall be used for NA communications in the present session. |
|  |  | WebSocketRequirement | Object | 0..1 | The DANE informs the 3GP-DASH client whether a websocket shall be set up for the present session. |

The NetworkAssistanceInitiationResponse message shall be carried in a SAND message envelope, as specified in ISO/IEC 23009-5 [54], with the following constraints:

-	The senderID element shall be included, and shall be the same as that used in the corresponding session initiation request;

-	The generationTime element may be omitted;

-	The messageId element may be omitted.The NetworkAssistanceInitiationResponse message is defined in the schema whose @schemeIdUri is:

"urn:3gpp:dash: schema:sandmessageextension:2017"

*** END OF CHANGE 1 ***

*** END OF CHANGES ***