TSG SA4#97 meeting	Tdoc S4-180007

5-9 February 2018, Fukuoka, Japan	Agenda Item: 5.2

3GPP TSG-SA WG1 Meeting #80	S1-174513

Reno, Nevada, USA, 27 Nov - 1 Dec 2017	(revision of S1-174438, S1-174087)

Title:	Reply LS on QCIs for EPC based ULLC

Response to:	SA2 LS (S1-174xxx/ S2-178150)

Release:	Rel-15

Work Item:	NR_newRAT-Core, New Radio (NR) Access Technology, LTE_HRLLC, LTE_sTTIandPT, EDCE5

Source:	SA1

To:	SA2, RAN, RAN1

Cc:	RAN2, RAN3, SA4, CT4

Contact Person:

#### Name:	Alice Li

E-mail Address:	alice.li@vodafone.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

SA1 thanks SA2 for the LS on QCIs for EPC based ULLC. SA1 is pleased to note that SA2 has made use of table 7.2.2-1 in TS 22.261 for the purposes of defining new QCIs.  Please find below the answer to your question.

Question from SA2:

SA2 requests SA1 to provide feedback on whether the latencies in the above QCI table should be reduced to cater for e.g. application processing, and if so, to provide information such that SA2 can document the resulting latency between SGi and UE.

SA1's answer:

No application processing time is included in the SA1 definition of “end-to-end latency”. For a typical discrete automation service scenario, in the case that the communication path is completely within the 5G system the end-end latency indicated in table 7.2.2-1 of TS 22.261 can be mapped into the latencies for the new QCIs as defined in the QCI table. Note that TS 22.261 defines the service requirements for the 5G system.

SA1 notes the latency and reliability requirements for URLLC use cases defined by RAN in TR 38.913 are as follow:

A general URLLC reliability requirement for one transmission of a packet is 1-10-5 for 32 bytes with a user plane latency of 1ms.

SA1 would like to point out that the TS 22.261 requirements for the typical 5G URLLC scenario “discrete automation – motion control” are as follow: for such service scenario the reliability requirement for transmission of a small packet (with a payload size typically ≤ 256 bytes) is 99.9999% with an end to end latency of 1 ms.

2. Actions:

To SA WG2:

ACTION: 	SA1 kindly asks SA2 to take into account the above answer to your question when proceeding with their work.

To TSG RAN:

ACTION: 	SA1 kindly asks RAN to take into account the above TS 22.261 requirements for the typical 5G URLLC scenario “discrete automation – motion control” when proceeding with their work.

3. Date of Next TSG SA WG1 Meetings:

SA1#81	5-9 Feb 2018	Fukuoka, Japan

SA1#82	7-11 May 2018	Dubrovnik, Croatia