Source:	Ericsson LM

Title:	Discussion on MTSI speech in 5G

Document for:	Discussion and Agreement

Agenda Item:	11.8

# 1 Introduction

In GSMA VoLTE (IR.92), narrowband and wideband speech support are both mandatory since v9.0 (April 2015), while support for super-wideband and fullband speech are optional. In current MTSI, only narrowband speech is mandatory (when supporting speech communication).

At 3GPP SA4#96, the source presented a pCR to TR 26.919 [1] on mandating support for super-wideband speech in 5G MTSI. This was not agreed, the main concern being that some 5G MTSI devices may not be capable to support super-wideband acoustics, e.g. due to device waterproofing.

# 2 Discussion

If it is in general desirable to support an increased speech quality of experience for 5G MTSI devices supporting speech, a few ways forward can be identified that would address the above concern:

- Keeping the proposed mandatory requirement on 5G MTSI super-wideband speech support in terms of codecs and general media processing, but relaxing the device super-wideband acoustic requirement such that it is sufficient to support a single acoustic interface with super-wideband capability, for example through a wired headphone connection.

- Keeping the previously mandatory MTSI requirement for narrowband speech, but adding a new mandatory requirement for wideband speech MTSI clients in terminal to support EVS-WB primary mode (supporting only EVS-IO mode not sufficient), being the best, currently known wideband speech codec.

# 3 Proposal

It is proposed to include text in TR 26.919 aligned with bullet 1 above.

# 4 References

[1] S4-171279 pCR 26.919 5G MTSI Speech: http://www.3gpp.org/ftp/tsg_sa/WG4_CODEC/TSGS4_96/Docs/S4-171279.zip