3GPP TSG-SA4 Meeting #97 	S4-180299

revision of S4-180282

# 05-09 Jan 2018, Fukuoka, Japan

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.247 | CR | 0132 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on segment duration information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | HUAWEI Technologies Japan K.K., Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI-15 |  |  |  |  |  |  | Date: |  | 2018-2-04 |
| Category: | A |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In Table K-1 and K-2, Segment duration for video MOS calculation is mapped to @duration in MPD, but actually the timing required by ITU-T P.1203 Mode 0 may be influenced by actions such as play/pause. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the description and remark in Table K-1 and Table K-2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusion exists, the defition of Segment duration in ITU-T P.1203 Mode 0 differs from the meaning of @duration in MPD. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | K.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

=====  CHANGE  =====

K.2	Calculation

The PSS server should support MOS estimation according to ITU-T P.1203 [49] Mode 0. To enable the MOS estimation, the QoE reporting should include the following metrics in either MPD@metrics or OMA DM QoE Management Object ‘metrics’:

-	Initial Playout Delay (section 10.2.5)

-	Play List (section 10.2.7)

-	MPD Information, unless already available by other means (section 10.2.8)

-	Device Information (section 10.2.10)

The attribute MPD.Metrics.Range@duration or the OMA DM QoE Management Object ‘duration’ shall be set to a value between 1 and 5 minutes.

The value of MPD.Metrics.Reporting@reportinginterval or OMA DM QoE Management Object ‘Interval’ shall be larger or equal than the value of MPD.Metrics.Range@duration or OMA DM QoE Management Object ‘duration’.

The DASH client reports collected metrics using quality metric reporting protocol defined in clause 10.6.

The mapping between QoE metrics and input to ITU-T P.1203 [49] Mode 0 is provided in table K-1 to K-4 below.

Table K-1 mapping between QoE metrics and input in ITU-T P.1203 mode 0 model for video stream

| Video Metrics needed for ITU-T P.1203[49] model |  | QoE metrics defined in clause 10.3 and 10.4 |  | Remark |
| --- | --- | --- | --- | --- |
| Description | Value | Metric | Description |  |
| Target Video bit-rate | Bit-rate in kbps. | Mpdinfo | Provides the MPD information for the representation or subrepresentation identified by representationid and subreplevel, if present. Related attributes: @bandwidth, @width, @height, and @codecs. Note: codec attribute includes video codec profile information and video frame rate information | Target Video bit-rate has been supported by @bandwidth attribute. Video frame-rate and Video codec and profile have been supported by @ codec attribute. Video encoding resolution has been supported by @width and @height attributes. |
| Video frame-rate | Frame rate in frames per second. |  |  |  |
| Segment duration | Duration in seconds |  |  |  |
| Video encoding resolution | Number of pixels (WxH) in transmitted video |  |  |  |
| Video codec and profile | One of: H264-baseline, H264-high, H264-main |  |  |  |
|  |  | Play List | Provides information on when and how long different representations or subrepresentations have been used during the session. | Note the the actual timing of the required input for the P.1203 quality model may be influenced by actions such as play/pause. |

Table K-2 mapping between QoE metrics and input in ITU-T P.1203 mode 0 model for audio stream

| Audio Metrics needed for ITU-T P.1203 model |  | QoE metrics defined in clause 10.3 and 10.4 |  | Remark |
| --- | --- | --- | --- | --- |
| Description | Value | Metric | Description |  |
| Target Audio bit-rate | Bit-rate in kbps. | Mpdinfo | Provides the MPD information for the representation or subrepresentation identified by representationid and subreplevel, if present. Related attributes: @bandwidth, and @codecs. Note: codec attribute includes audio codec profile information, audio sampling frequency and Number of audio channels information | Target Audio bit-rate has been supported by @bandwidth attribute. Audio codec, Audio sampling frequency and Number of audio channels have been supported by @ codecs attribute. |
| Segment duration | Duration in seconds |  |  |  |
| Audio codec | One of: AAC-LC, AAC-HEv1, AAC-HEv2, AC3 |  |  |  |
| Audio sampling frequency | In Hz |  |  |  |
| Number of audio channels | 2 |  |  |  |
|  |  | Play List | Provides information on when and how long different representations or subrepresentations have been used during the session. | Note the the actual timing of the required input for the P.1203 quality model may be influenced by actions such as play/pause. |