TSG SA4#98 meeting	Tdoc S4-180333

9-13 April 2018, Kista, Sweden	Agenda Item: 5.2

3GPP TSG-SA WG2 Meeting #126	S2-182398

Montreal, Canada, 26th Feburay – 2nd March 2018

Title:	Reply LS on Group Message Delivery via MBMS

Response to:	LS on Group Message Delivery via MBMS (S2-181442/C3-180292)

Release:	Rel-15

Work Item:	NAPS, NAPS-CT

Source:	SA2

To:	CT3

Cc:	SA4

Contact Person:

#### Name:	Fenqin, zhu

Tel. Number:

E-mail Address:	Zhufenqin@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

1. Overall Description:

SA2 would like to thank CT3 for their LS on Group Message Delivery via MBMS (S2-181442/C3-180292).

SA2 discussed the raised issues and have following answers.

Question 1: Does SA2 assume that all the detailed requirements for the Group Message Delivery via MBMS defined in 3GPP TS 26.346 apply for the T8 Group Message Delivery via MBMS procedures in subclause 5.5.1 of TS 23.682?

SA2 answer: SA2 does not assume that all the detailed requirements for the Group Message Delivery via MBMS defined in 3GPP TS 26.346 apply for the T8 Group Message Delivery via MBMS procedures in subclause 5.5.1 of TS 23.682.  For example, the authentication and authorization procedure over xMB interface as defined in subclause 5.4A.3 in 3GPP TS 26.346 is not applicable to the T8 Group Message Delivery.

Question 2: Is it feasible to hide differences of southbound interfaces for Group Message Delivery via MBMS from the SCS/AS?

SA2 answer: From SA2 point of view, as SCS/AS has different behaviours at the application layer when MB2 or xMB is used as a southbound interface, the SCS/AS need be aware which interface is used at southbound interface. However, SA2 encourages CT3 to find, if possible, a stage 3 solution which hides the differences of the southbound interface and which meets the stage 2 requirements (e.g., one unified T8 API containing different parameters for MB2 or xMB cases).

2. Actions:

To CT3.

SA2 kindly asks CT3 take the above agreement into account.

3. Date of Next TSG-SA WG2 Meetings:

SA WG2 Meeting #127		16th – 20th April 2018, Sanya, China

SA WG2 Meeting #127bis	28th May – 1st June 2018, Newport Beach, CA, USA