Source:	Sony Mobile Communications

Title:	Reply to S4-180708 and S4-180709 on requirements, objectives and test methods for SWB and FB terminals (SPAN)

Document for:	Agreement

Agenda Item:		9.7

Contents

1	Test methods	1

2	Lab-to-lab variation	1

3	Requirements and performance objectives	1

4	References	2

# Test methods

ETSI selected not one but two models for super-wideband and fullband P.835 MOS prediction. At SA4 #93, Sony views were expressed in [1]:

“It is proposed to reference only one of the models described in TS 103 281.”

However, SA4 agreed to use a combination of results from both models, model A and model B. Approximately one year later, at SA4 #98, it was been observed that only one of the models is commercially available. If the situation remains at SA4#99:

Independent model implementation by each company in the industry is not realistic why the conclusion is that we have the following choices:

- Revert to using only one ETSI model

- Delay the usage of 3GPP SWB S/N-MOS requirements in the field (amount of delay is unknown)

# Lab-to-lab variation

Lab-to-lab reproducibility for wideband was reviewed in S4-180586 [6] (different labs and different rooms) and for the SWB/FB case reported in S4-180708 [4] (different rooms in the same lab).

# Requirements and performance objectives

Data for a number of commercially available UE:s have been presented by HEAD acoustics in [2] and [4], together with suggestions for requirements and performance requirements, also reflected in [5]. The source provided suggestions was also given at SA4#98 in [6].

Building on the mentioned documents, and the presentation and discussion in SQ during SA4#99, in the attached spreadsheet, other values are suggested (for the case that model A is used).

It can be noted that the variation of S-MOS scores is smaller than for N-MOS scores. This is reflected in a smaller difference between “shall” and “should”, for S-MOS.

It is the opinion of the source, that

- If there is nothing wrong with the tested terminals, they provide minimum performance or better, there is no reason why not all should pass the requirement, also with a reproducibility margin.

- A good performer should reach the performance objective, this is expected to be the mark for GSMA logotype HD Voice+.

- High N-MOS in hand-held handsfree mode can be counter-productive since a terminal may be designed for several persons gathered around the UE, not only one person talking from a certain nominal (tested) incidence angle. This is reflected in our suggested limits.

# References

[1]	S4-170370, DESUDAPS - Usage of the new SWB/FB P.835 predictor specification from ETSI, Sony Mobile Communications

[2]	S4-180356, Addition of requirements and objectives for SWB and FB terminals, HEAD acoustics

[3]	S4-151365,	Results of the Round Robin Test on Different Background Noise Simulation Techniques for Hand-Held Hands-Free Terminals	HEAD acoustics GmbH, Knowles Inc., ORANGE, Sony Mobile Communications

[4]	S4-180708, Inter-room reproducibility of TS 103 281 predictor, HEAD acoustics GmbH

[5]	S4-180709, Requirements and objectives for SWB and FB terminals, HEAD acoustics GmbH

[6]	S4-180586, On requirements, objectives and test methods for SWB and FB terminals (SPAN), Sony Mobile Communications