Requirements for Performance Management for Converged Networks

A <xxxxxxxx> by ngmn Alliance

Date: 26-Apr-2013

Version 1.16

(Approved

<by gremium (OC/Board)>)

| Publication | <Confidential & Restricted / NGMN CONFIDENTIAL/ NGMN Confidential with Limited Dissemination/ PUBLIC> |
| --- | --- |
| Project | NGcore Phase 2 |
| Editor in Charge |  |
| Editing Team |  |
| Document Status | Draft Version 1.16 |
| Approved by | <Gremium TWG SC, OC, Board, other> |

Abstract: Short introduction and purpose of document

Document History

| Date | Version | Author | Changes |
| --- | --- | --- | --- |
| 25/05/2012 | V 0.1 |  | INITIAL VERSION |
| 22/11/2012 | V 1.3 | Ma Zhi | Adjust the order of sub sections in section 5. put sub section of General Requirements before sub section of High level functional requirements. Add new sub section 4.2 named high level architecture for the scenario. Cut business scenario Workplace as a Service using Cloud services. refine the figures and text of business scenario of Telemedicine. |
| 23/11/2012 | V 1.4 | Massimo Banzi | Reorganized requirements by introducing Generic Requirements ch. Better outlined Generic Requirements Basic explanation on KQI, KPI, PI in the Introduction |
| 25/11/2012 | v 1.5 | Massimo Banzi Ma Zhi | riformulation of requirements and full document revision |
| 7/12/2012 | V1.7 | Ma Zhi | Add third Scenario Walkman |
| 14/12/2012 | v1.8 | Cecilia Corbi Massimo Banzi | Revised overall structure |
| 11/01/2013 | v1.10 | Ma Zhi Zou Lan Niu Weiguo Massimo Banzi | Further revisions and integration to all the chapters of the document |
| 15/01/2013 | V1.11 | Yvonne Doernhofer | Editorial Review |
| 07/03/2013 | V1.13 | MA ZHI NIU WEIGUO MASSIMO BANZI | Implemented feedback from Beijing NGCOR and MultiSDO meetings |
| 11/04/2013 | V1.14 | MA ZHI NIU WEIGUO JILI MASSIMO BANZI | introduction of KPIs (PIs) for the WIRELESS network for scenario Video Communication alignment of requirements to Use Cases in the suitable slot within requirement tables Update of Terminology table Update OF requirements title and description introduction of Web Browsing scenario and deletion of Telemedicine |
| 30/04/2013 | V1.16 | MA ZHI NIU WEIGUO JILI MASSIMO BANZI |  |

Contents

1	Stream General overview	5

2	Introduction	5

2.1	Scope	6

2.2	References	7

2.3	Terminology & Conventions	7

3	Gap Analysis on existing previous work	10

4	Scenarios/Use Cases	10

4.1	Business Scenario (BS)	10

4.1.1	Video Communication (BS1)	11

4.1.1.1  Buffering video KQIs/KPIs	11

4.1.2	Web Browsing (BS2)	15

4.1.3	Walkman (BS3)	18

4.1.3.1 Download sub scenario	19

4.1.3.3 Audio Streaming sub scenario	21

5	Requirements	22

5.1	Generic Requirements	22

5.2	General Requirements	23

5.3	High Level Functional Requirements	25

5.4	Other	30

6	Performance Management DETAILED INFORMATION	30

6.1	Key Quality Indicators (KQIs)	32

6.2	Key performance indicators (KPIs)	33

6.3	Relation among KPIs, KQIs and PIs from specific Technology Network Elements (NEs)	33

6.4	Prioritization (e.g. priorities table, list, etc…)	35

6.5	Vendor Constraints	35

6.6	<Other if/where applicable>	35

7	Conclusions/Recommendations	35

7.1	35

7.2	35

Appendix 1 - STANDARDS Related to Video Communication	36

Appendix 2 - Architectural and Framework Standards: The TMN/FCAPS Model (ITU-T)	37

Appendix 3 - Relationship between Service/Resource instances and KQI/KPI/PI instances	45

Appendix 4 – Sketch of Resource and Service Management Domain	51

# Stream General overview

- In Chapter 2 we give a description of the reason that leads to the document and its scope.

- In Chapter 3 we give a description of Performance Management as from Literature, trying to focus where this work is focused. Key Quality Indicators (KQIs) and Key Performance Indicators (KPIs) and their relation are also described.

- Chapter 4 is the one of scenarios: An entire description of Business Scenarios with sub-scenarios and related Indicators considered in this work is included. Note that in describing indicators we do not mean to be exhaustive, but just to identify some relevant ones that allows us to better focus on our scope.

- Chapter 5 lists the requirements: Generic Requirements, General Ones and High Level Functional Requirements.

- Chapter 6 is intended for analyzing the gap analysis in existing work.

- In Chapter 7 conclusion and recommendations are reported.

Appendices on existing standards lists and theoretical aspects follow.

# Introduction

The interest in Performance Management has significantly increased due to the changes taking place in the newly competitive telecommunications industry which presents several challenges for service providers as follows.

A variety of providers enters the marketplace and competes for customers. New entrants that are striving to gain market share and to establish themselves as viable suppliers can use performance to provide one means of attracting customers.

The rapid evolution of telecommunications-based applications is accelerating the rate of introduction of new services that use emerging networking technologies. Performance can help encourage customers to use these new services and technologies. The critical dependency between a constantly expanding set of essential business activities and the availability of communication networks and information services means that great numbers of customers are demanding increasingly stringent communications and information performance levels to insure business continuity.

The widespread use of smartphones, tablet computers and other web-enabled mobile devices has caused wireless data services to explode. Service providers’ networks see more bandwidth-hungry traffic each day: To survive, service providers need to decrease their cost per bit while simultaneously optimizing their network to handle more traffic.

Several emerging market needs place a new emphasis on specifying performance. These include the requirement to support emergency response personnel, the opening up of the telecom service market to competition, and the deployment of services based on network technologies. Additionally, the number of ebusiness requiring very high levels of service availability from their external networks and services, such as servers, databases, etc., continues to rapidly grow.

The cell/ packet-based technologies raise new performance specification issues not present in services based on traditional circuit switched networks and leased lines. These new performance specifications are related to defining, monitoring and collecting performance data, and to transforming that data into useful service performance reports.

LTE standards address this huge demand for higher bandwidth, lower latency, and advanced communication services. In LTE networks, many devices need to be managed, increasing the potential points of failure or degradation and service operators are working to compensate for network congestion through improved quality of service (QoS).

These evolving market conditions have forced operators to manage overlapping 2G, 3G and 4G technologies and the convergence of mobile and fixed networks.

In turn, it’s more important than ever that Element Management Systems (EMSs) and Network Management Systems (NMSs) properly control network devices to ensure calls go through, video gets viewed, online games perform, and more.

Performance Management provides functions to evaluate and report on the behaviour of telecommunication equipment and the effectiveness of the network or network element. Therefore the constant monitoring of the quality of Services (Service Performance Management) and Networks (Network Performance Management) are becoming more critical.

Service Performance Management mainly includes:

- Service Support Performance

- Service Operability Performance

- Service Accessibility Performance

- Service Retainability Performance

- Service Integrity Performance

- Service Security Performance

In this document we focus mainly on categories 3, 4, and 5. That leads to network performance indicators that include:

- Planning, Provisioning, and Administrative Performance (Resources and Facilities)

- Dependability  with  Availability Performance and related aspects (Grade of Service)

- Transmission Performance with Propagation aspects

This document refers mainly to items 2 and 3.

## Scope

Following objectives have been identified:

- Delivery of relevant services scenarios for Performance Management in an hybrid environment of converged heterogeneous networks.

- Delivery of recommendations and high level requirements on how to define modelled KQI/KPIs for the identified service scenarios.

- In the second part of the stream timeframe (e.g. in January 2013) it will be evaluated to extend the scope of the stream to cover also the following objective:

- Based on the above 2 stream outcomes, a gap analysis of what already exists on Performance Management (mainly in 3GPP & TM Forum, See also [1] and [2] for references) will be provided. Based on this analysis, a set of KQIs and KPIs of interest will be selected and high level requirements will be defined.

- Delivery of the OSS Requirements for PM on the northbound interface, e. g. performance job schedule, create, modify and delete. And performance file collection requirement.

- Solution requirement (requirements and recommendations for performance data template or file format, such as xml schema)

Out of scope

- Performance measurement definition. Performance detail measurements or counters definition is out of PM scope because there are plenty of measurements in the current network to sort out. Our concern is not the NE specific measurements of the KPIs, but the way to manage the Performance data and their exchanges for the operator’s needs. Indeed, the operator shall be able to trigger  from its OSS PM application, performance jobs for dedicated purpose and need (targeted NEs, or targeted Cells, or targeted domain/ zone/ area), and collect related PM measurements within its OSS PM application. All Performance Management related information exchanges between the EMS and OSS PM applications shall comply with standardized specifications.

## References

- “NGMN Informative List of SON Use Cases”, April 17th, 2007 - [OPS06] PERFORMANCE MANAGEMENT IN REAL TIME

- “NGMN Recommendation on SON and O&M Requirements”, December 5th, 2008 -PERFORMANCE MANAGEMENT IN REAL TIME

- "Self-Optimizing Networks - The Benefits of SON in LTE”,  4G Americas, July 2011 - SELF‐TUNING X2 INDEPENDENT ALGORITHMS

- ITU-T Recommendations E.800

- ITU-T Recommendations E.801

- TMF GB 917 SLA Management Handbook Volume 2 Concepts and Principles Release 2.5

- 3GPP TR 32.831 V1.0.0 , “Study on Alignment of 3GPP Performance Management and TM Forum Interface Program (TIP) Performance Management (Release 10)”

- 3GPP TS 32.410 Telecommunication management; Key Performance Indicators (KPI) for UMTS and GSM, Release 10

- 3GPP TS 32.450: Telecommunication management; Key Performance Indicators (KPI) for E-UTRAN:Definitions, Release 10

- 3GPP TS 32.455: Telecommunication management; Key Performance Indicators (KPI) for the Evolved Packet Core (EPC); Definitions, Release 10

- ETSI TS 102 250-2 V2.2.1 2011-4.

- 3GPP TS 32.401 Telecommunication management; Performance Management (PM); Concept and requirements

- 3GPP TS 32.451: Telecommunication management; Key Performance Indicators (KPI) for E-UTRAN; Requirements, Release 10

- TM Forum GB923 Wireless Service Measurements Handbook V3.0

- TM Forum GB922_Addendum_1_Performance_R12-5_v1-10 Performance Business Entities

- TM Forum GB917 SLA Management Handbook Volume 2 Concepts and Principles Release 2.5

- TM Forum GB929 TAM Application_Framework_R4_5__V4.3

- TM Forum TIP_PM_BA_V1_8 Performance Management Business Agreement

## Terminology & Conventions

    - Conventions

    - Definitions

| Term | Definition | Origin |
| --- | --- | --- |
| Network Element (NE) | A facility or equipment used in the provision of a telecommunications service. Such term also includes features, functions, and capabilities that are provided by means of such facility or equipment, including subscriber numbers, databases, signalling systems, and information sufficient for billing and collection or used in the transmission, routing, or other provision of a telecommunications service | Telecommunications Act of 1996, Federal Communication Commission |
| Element Management System (EMS) | Network Element Manager (EM): provides a package of end-user functions for management of a set of closely related types of Network Elements. These functions can be divided into two main categories: • Element Management Functions for management of Network Elements on an individual basis. These are basically the same functions as supported by the corresponding local terminals. • Sub-Network Management Functions that are related to a network model for a set of Network Elements constituting a clearly defined sub-network, which may include relations between the Network Elements. This model enables additional functions on the sub-network level (typically in the areas of network topology presentation, alarm correlation, service impact analysis and circuit provisioning). | 3GPP TS 32.401 V11.0.0 (2012-09) |
| Network Management System (NMS) | Network Manager (NM): provides a package of end-user functions with the responsibility for the management of a network, mainly as supported by the EM(s) but it may also involve direct access to the Network Elements. All communication with the network is based on open and well-standardised interfaces supporting management of multi-vendor and multi-technology Network Elements. | 3GPP TS 32.401 V11.0.0 (2012-09) |
| North Bound Interface (NBI) | It’s the management interfaces between the Element Management System and Network Manage System. | 3GPP TS 32.101 V11.1.0(2012-12) |
| South Bound Interface (SBI) | It’s the management interfaces between the Nework Element and Element Management System. | 3GPP TS 32.101 V11.1.0(2012-12) |
| Service Management System (SMS) | All systems related to the Service Management Domain as explained in Chap 6 in TAM model | TM Forum - TAM Application_Framework_R4_5__V4.3.doc pag 133 |
| Resource Management Systems | All systems related to the Resource Management Domain as explained in Chap 7 in TAM model | TM Forum - TAM Application_Framework_R4_5__V4.3.doc pag 168 |
| Service Performance Management System (PM System) | Service Performance Management Applications monitor, analyze, and report on the end-end service performance. This can include a Near Real Time, end-to-end view to ensure that each service is functioning correctly as well as a historical view. These applications build on the Resource Performance data and active end-end service performance test data to provide a view of a service. These applications provide a key input to determine the Quality of Service. | TM Forum - TAM Application_Framework_R4_5__V4.3.doc pag 158 |
| Service Inventory | Service Inventory Management represents the applications which contain and maintain information about the instances of services in a telecom organization. A Service Inventory application may store and manage any or all of the following entities: • Customer facing service (CFS) instances, and their attributes • Resource facing service (RFS) instances, and their attributes | TM Forum - TAM Application_Framework_R4_5__V4.3.doc pag 135 |
| Resource Inventory | Resource Inventory applications manage information of all resources used to implement services and products. This application area is typically linked to various element management systems (i.e. building inventory for actual server, applications, network and resource assets) and resource inventory database systems which may or may not be combined with Service Inventory Application(s) or database(s). In addition, Resource management applications have a major role to play managing spare parts; passive resources including cable pairs and external plant and passive customer premises equipment. In addition, Resource Inventory applications are used to discover and manage underutilized or ‘stranded’ resources. | TM Forum - TAM Application_Framework_R4_5__V4.3.doc pag 178 |
| Configuration Management System | A set of tools and databases that are used to manage an IT Service Provider's Configuration data. The CMS also includes information about Incidents, Problems, Known Errors, Changes and Releases; and may contain data about employees, Suppliers, locations, Business Units, Customers and Users. The CMS includes tools for collecting, storing, managing, updating, and presenting data about all Configuration Items and their Relationships. The CMS is maintained by Configuration Management and is used by all IT Service Management Processes. See Configuration Management Database, Service Knowledge Management System. | ITIL® V3 Glossary v3.1.24, 11 May 2007 |
| Configuration Database (CMdb) | A database used to store Configuration Records throughout their Lifecycle. The Configuration Management System maintains one or more CMDBs, and each CMDB stores Attributes of CIs, and Relationships with other CIs. | ITIL® V3 Glossary v3.1.24, 11 May 2007 |
| Configuration Item (CI) | Any Component that needs to be managed in order to deliver an IT Service. Information about each CI is recorded in a Configuration Record within the Configuration Management System and is maintained throughout its Lifecycle by Configuration Management. CIs are under the control of Change Management. CIs typically include IT Services, hardware, software, buildings, people, and formal documentation such as Process documentation and SLAs. | ITIL® V3 Glossary v3.1.24, 11 May 2007 |
| Performance Indicator (PI) | Performance related data produced by Network Elements of wireline and wireless network. See lowest level of Figure 4. | TM Forum GB 917 Release 2.5 |
| Key Performance Indicator (KPI ) | KPI provides a measurement of a specific aspect of the performance of a service resource (network or non-network) or group of service resources of the same type. A KPI is restricted to a specific resource type. | TM Forum GB923 V3.0 |
| Key Quality Indicator (KQI) | KQI provides a measurement of a specific aspect of the performance of the product, product components (services) or service elements and draw their data from a number of sources including the KPIs. | TM Forum GB923 V3.0 |
| Service Accessibility Performance | The ability of a service to be obtained, within specified tolerances and other given conditions, when requested by the user. | ITU-T Recommendation E.800 |
| Service Retainability Performance | The ability of a service, once obtained, to continue to be provided under given conditions for a requested duration | ITU-T Recommendation E.800 |
| Service Integrity Performance | The degree to which a service is provided without excessive impairments, once obtained. | ITU-T Recommendation E.800 |
| Near Real Time | The term “Near Real Time” is a time constraint within which the described task/activity should take place; else the task/activity would be considered a failure. Whether the described task/activity could actually take place within the time constraint is dependent on factors such as complexity of task/activity involved, technology involved etc and should be examined on case by case basis. Near Real Time usually refers to a short period of time expressed in seconds or minutes. |  |
| KPI Instance | A KPI Instance is used for defining an executable instance of the KPI template. A prerequisite to running the instance is that all input, including measurement values and constants must be defined. A KPI instance differs from a KPI template in that it is bound to specific attributes or parameters of network element and can be issued. | IBM Integrated Information Core, Version 1.5 |
| KQI Instance | A KQI Instance is used for defining an executable instance of the KQI template. A prerequisite to running the instance is that all input, including KQI values and constants must be defined. A KQI instance differs from a KQI template in that it is bound to specific attributes or parameters of service and can be issued. |  |

    - Abbreviations

# Gap Analysis on existing previous work

Problem statement

We first have to consider that main problems arise not in managing single technologies, but the collection of data coming from several element with different vendors, different technologies, etc.

A gap analysis in this context has to keep into account the existence of this constraint as currently several SDOs have already addressed this problem.

What has to be considered before doing this gap analysis is to define exactly the scenario and use cases that aim to cover our problem statement.

For instance: 3GPP TR 32.831 V1.0.0, “Study on Alignment of 3GPP Performance Management and TM Forum Interface Program (TIP) Performance Management (Release 10)” [7] provides consistent and aligned PM and PM interfaces in 3GPP and TM Forum.

It:

•	Identifies similarities and differences of the PM capabilities in 3GPP and TM Forum TIP PM BA;

•	Proposes enhancements to 3GPP PM solutions for converged networks and to satisfy TM Forum TIP PM BA requirements;

•	Identifies any required changes in the 3GPP specifications;

•	Identifies any required changes in the TM Forum specifications (to be communicated to TM Forum).

Backwards compatibility of the PM IRP should be maintained as much as possible by re-using existing specifications to the maximum extent.

The counter collected by each of the Network Elements (NE) and the definition of those are NE specific and there is no intention to harmonize those.

However KPIs with a network view should be explored.

# Scenarios/Use Cases

## Business Scenario (BS)

According to statistics, web browsing and streaming would take up over 70% of all traffic flow, thus to ensure good experience of these services is of high priority from both operator and vendor’s point of view.

The following scenarios are applicable under the following general condition: As various technologies have developed over time, the monitoring of the performance of services provided over a network has to evolve accordingly. End-to-end performance indicators (PIs, KPIs, KQIs) are getting more and more important for the converged network where coexist 3G, 4G, copper and optical networks.

These business scenarios cover both the wire-line and the wireless domain and future as well as existing technologies. Besides, OSS layers also impact the whole process of the performance indicators.

Table 1: Business Scenario Decomposition

| Business Scenarios | Scope | Additional Constraints/Criticity |
| --- | --- | --- |
| Video communication (BS1) | Data supplied from various network and service sources such as Media Servers, Transmission Network and acquired from either fixed or mobile terminals | Quality Perceived to be considered |
| Web browsing (BS2) | Data supplied from various network and service sources such as web portals | Short access time Integrity of web content |
| Walkman (BS3) | Walkman Service is one of mobile music services which is realized with music experience consumption platform and client music player software installed in the cell phone. This service can help users to listen to music online, download ringtones and songs. | For the download subscenario , the user’s behavior would be browse songs list and select favorite song(s) or ringtone to download. Song(s) or ringtone download failure and slow download speeds of song(s) or ringtone are important customer perception. |

For sake of description we also introduce the following identification codes for KQI categories

Table 2: KQI Category

| KQI ID | KQI Category |
| --- | --- |
| A KQI | Service Accessibility Performance |
| R KQI | Service Retainability Performance |
| I KQI | Service Integrity Performance |

## Video Communication (BS1)

The quality perceived by the customer is relevant more than the “simple” measurement throughput of data.

There are two main class video service scenarios in networks at present:

- buffering video, e.g. YouTube.

- real time video, e.g. video call, video conference.

We focus on buffering video at the first place.

For buffering video, the quality of the streaming (e.g. picture, audio) itself is not a problem, the customer’s perception usually focus on the accessibility, retainablility and integrity related KQIs. From the service usage aspect, the following KQIs should be identified:

| KQI Category | Key Quality Indicator | Definition |
| --- | --- | --- |
| Service Accessibility Performance | Streaming Start Success Ratio | The Streaming Start Success Ratio describes the probability that the streaming can be started to play by the UE when requested by the user. At this point, the initial buffering process has been finished. |
|  | Streaming Start Delay | The Streaming Start Delay describes the duration of a service access from requesting the stream at the portal until the streaming can be played at the UE |
| Service Integrity Performance | Streaming Stall Frequency | The Streaming Stall Frequency describes average times that the stream playing is stalled and the streaming goes into rebuffering mode during the whole streaming process. |
|  | Average Streaming Stall Time | The Average Streaming Stall Time describes the average time that the every durations between a stream going into rebuffering mode and continuation of the stream |
|  | Average Streaming Throughput | The Average Streaming Throughput describes the average download speed of the streaming data during the whole streaming process |

| Key Quality Indicator | Related KPI | Technology | PI |
| --- | --- | --- | --- |
| Streaming Start Success Ratio | [TS 32.450](eUTRAN): E-RAB Accessibility; [TS 32.410] (UTRAN): RAB Establishment Success Rate; RRC Connection Establishment Success Rate; Service Access Success Rate; PDP Context Activation Success Rate; [TS 32.455] (GERAN): EPS Attach Success Rate; Dedicated EPS Bearer Creation Success Rate; Service Request Success Rate; | Wireless network | [TS 32.425]: Attempted RRC connection establishments; Successful RRC connection establishments; Number of initial E-RABs attempted to setup; Number of initial E-RABs successfully established; [TS 32.426]: Attempted EPS attach procedures; Successful EPS attach procedures; [TS 32.405]: Attempted RAB establishments for PS domain; Successful RAB establishments without queuing for PS domain; Successful RAB establishments with queuing for PS domain; [TS 32.406]: Attempted GPRS attach procedures; Successful GPRS attach procedures; |
| Streaming Start Delay | [TS 32.455]: Dedicated Bearer Set-up Time by MME (Mean); | Wireless network | [TS 32.425]: RRC connection setup time; Mean E-RAB Setup time; Maximum E-RAB Setup time; [TS 32.405]: RAB PS connection set-up time (Mean); RAB PS connection set-up time (Maximum); RRC connection set-up time (Mean); RRC connection set-up time (Maximum); PDP Context set-up time; [TS 32.406]: PDP Context set-up time, initiated by MS; PDP Context set-up time, initiated by Network; |
| Streaming Stall Frequency | [TS 32.450]: E-RAB Retainability; [TS 32.410]: RAB Abnormal Release Rate; GERAN Service Abnormal Release Rate; | Wireless network | [TS 32.425]: DL PDCP SDU drop rate; DL PDCP SDU air interface loss rate; [TS 32.405]: Attempted RRC re-establishments; Failed RRC re-establishments; ;m |
|  | PON/ONU Upstream/Downstream Rate | Wireline network | PON/ONU Upstream/Downstream Rate |
| Average Streaming Stall Time | [TS 32.450]: E-RAB Retainability; E-UTRAN IP Throughput; E-UTRAN IP Latency; [TS 32.410]: Percentage of Established RABs, TotalPS; [TS 32.455]: Mean Active Dedicated EPS Bearer Utilization; | Wireless network | [TS 32.425]: Average DL cell PDCP SDU bit-rate; Maximum DL cell PDCP SDU bit-rate; Average DL PDCP SDU delay; IP Throughput in DL; IP Latency in DL; [TS 32.426]: Number of incoming IP data packets on the S1-MME interface from eNodeB to MME; Number of outgoing IP data packets on the S1-MME interface from MME to eNodeB; [TS 32.406]: Number of outgoing GTP data packets on the Iu interface; Number of incoming GTP data packets on the Iu interface;;m |
|  | PON/ONU Upstream/Downstream Rate | Wireline network | PON/ONU Upstream/Downstream Rate |
| Average Streaming Throughput | [TS 32.450]: E-UTRAN IP Throughput; E-UTRAN IP Latency; | Wireless network | [TS 32.425]: IP Throughput in DL; IP Latency in DL; Average DL cell PDCP SDU bit-rate; Maximum DL cell PDCP SDU bit-rate; [TS 32.426]: Number of incoming IP data packets on the S1-MME interface from eNodeB to MME; Number of outgoing IP data packets on the S1-MME interface from MME to eNodeB; [TS 32.405]: Number of incoming IP data packets on the Gi interface; Number of outgoing IP data packets on the Gi interface; [TS 32.406]: Number of outgoing GTP data packets on the Iu interface; Number of incoming GTP data packets on the Iu interface;;m |
|  | PON/ONU Upstream/Downstream Rate | Wireline network | PON/ONU Upstream/Downstream Rate |

Note: 	The above indicators are just some ones and not yet fully represented. What is to be done yet is to find in our services or in the literature some well represented examples of KQI and how they are related to KPI.

- Each service KQI is impacted by more than one network KPI.

- Not every network KPI will impact a service KQI.

- Each service will have its own network KQI mapping.

## Web Browsing (BS2)

In the networks, Web browsing is the one of most popular service and consumes much bandwidth. Http is the most widely used protocol, the following KQI description is based on the http process.

| KQI Category | Key Quality Indicator | Definition |
| --- | --- | --- |
| Service Accessibility Performance | Page Response Success Ratio | The Page Response Success Ratio describes the probability that the web page can be started to display by the UE when requested by the user. |
|  | Page Response Delay | The Page Response Delay describes the duration of a service access from requesting the page at the UE until the page connection have been built. |
| Service Integrity Performance | Page Display Success Ratio | The Page Display Success Ratio describes the probability that the web page can be displayed successfully by the UE when request by the user. |
|  | Page Display Delay | The Page Display Delay describes the duration of the time from the first data package is sent until the all page content is displayed successfully. |
|  | Average Page Download Throughput | The Average Page Download Throughput describes the average download speed of the web page data during the whole web browsing process. |

| Key Quality Indicator | Related KPI | Technology | PI |
| --- | --- | --- | --- |
| Page Response Success Ratio | [TS 32.450](eUTRAN): E-RAB Accessibility; [TS 32.410] (UTRAN): RAB Establishment Success Rate; RRC Connection Establishment Success Rate; Service Access Success Rate; PDP Context Activation Success Rate; [TS 32.455] (GERAN): EPS Attach Success Rate; Dedicated EPS Bearer Creation Success Rate; Service Request Success Rate; | Wireless network | [TS 32.425]: Attempted RRC connection establishments; Successful RRC connection establishments; Number of initial E-RABs attempted to setup; Number of initial E-RABs successfully established; [TS 32.426]: Attempted EPS attach procedures; Successful EPS attach procedures; [TS 32.405]: Attempted RAB establishments for PS domain; Successful RAB establishments without queuing for PS domain; Successful RAB establishments with queuing for PS domain; [TS 32.406]: Attempted GPRS attach procedures; Successful GPRS attach procedures; |
| Page Response Delay | [TS 32.455]: Dedicated Bearer Set-up Time by MME (Mean); | Wireless network | [TS 32.425]: RRC connection setup time; Mean E-RAB Setup time; Maximum E-RAB Setup time; [TS 32.405]: RAB PS connection set-up time (Mean); RAB PS connection set-up time (Maximum); RRC connection set-up time (Mean); RRC connection set-up time (Maximum); PDP Context set-up time; [TS 32.406]: PDP Context set-up time, initiated by MS; PDP Context set-up time, initiated by Network; |
| Page Display Success Ratio | [TS 32.450]: E-RAB Retainability; [TS 32.410]: RAB Abnormal Release Rate; GERAN Service Abnormal Release Rate; | Wireless network | [TS 32.425]: DL PDCP SDU drop rate; DL PDCP SDU air interface loss rate; [TS 32.405]: Attempted RRC re-establishments; Failed RRC re-establishments; ;m |
|  | PON/ONU Upstream/Downstream Rate | Wireline network | PON/ONU Upstream/Downstream Rate |
| Page Display Delay | [TS 32.450]: E-RAB Retainability; E-UTRAN IP Throughput; E-UTRAN IP Latency; [TS 32.410]: Percentage of Established RABs, TotalPS; [TS 32.455]: Mean Active Dedicated EPS Bearer Utilization; | Wireless network | [TS 32.425]: Average DL cell PDCP SDU bit-rate; Maximum DL cell PDCP SDU bit-rate; Average DL PDCP SDU delay; IP Throughput in DL; IP Latency in DL; [TS 32.426]: Number of incoming IP data packets on the S1-MME interface from eNodeB to MME; Number of outgoing IP data packets on the S1-MME interface from MME to eNodeB; [TS 32.406]: Number of outgoing GTP data packets on the Iu interface; Number of incoming GTP data packets on the Iu interface;;m |
|  | PON/ONU Upstream/Downstream Rate | Wireline network | PON/ONU Upstream/Downstream Rate |
| Average Page Download Throughput | [TS 32.450]: E-UTRAN IP Throughput; E-UTRAN IP Latency; | Wireless network | [TS 32.425]: IP Throughput in DL; IP Latency in DL; Average DL cell PDCP SDU bit-rate; Maximum DL cell PDCP SDU bit-rate; [TS 32.426]: Number of incoming IP data packets on the S1-MME interface from eNodeB to MME; Number of outgoing IP data packets on the S1-MME interface from MME to eNodeB; [TS 32.405]: Number of incoming IP data packets on the Gi interface; Number of outgoing IP data packets on the Gi interface; [TS 32.406]: Number of outgoing GTP data packets on the Iu interface; Number of incoming GTP data packets on the Iu interface;;m |
|  | PON/ONU Upstream/Downstream Rate | Wireline network | PON/ONU Upstream/Downstream Rate |

## Walkman (BS3)

Walkman Service is one of mobile music services which is realized with music experience consumption platform and client music player software installed in the cell phone. This service can help users to listen to music online, download ringtones and songs.

The scenario of the service is as following.

Figure 2: Network Topology for Walkman Business Scenario

- Users open Walkman client application installed in mobile phone and login Walkman System through core network and WAP gateway.

- Users browse songs list and select favourite song(s) or ringtone to download. Walkman System inquires of Central Music Platform the service policy and presents the result information of billing policy and prices to client application. Users confirm it, then client application gets ready to download song(s) or ringtone.

- Walkman System requests the download address from File Storage and returns the download address to client application. Client application starts to download song(s) and/or ringtone from File Storage with the download address. The service supports broken-point continuingly-transferring.

- After the completion of song(s) and/or ringtone download, client application will return the success status to Walkman System. Then Walkman System notifies Music Service Platform to send the copyright of songs and/or ringtone to client application then charging begins.

Identified sub scenarios for Walkman use case are:

Table 7: SubScenarios of Walkman

| ID | SubScenario | Service Attribute | Key Quality Indicator |
| --- | --- | --- | --- |
| BS3.2 | Download sub scenario | Download music is success, and not often failure. | Download Success Rate Data Rate |
| BS3.3 | Audio Streaming | Audio stream is integrity, and not lose some tones or paused. | Data Error Rate |

## 4.1.3.1 Download sub scenario

For the download sub scenario, the user’s behaviour would be browse songs list and select favourite song(s) or ringtone to download. Song(s) or ringtone download failure and slow download speeds of song(s) or ringtone are important customer perception. It involves wireless network, core network, WAP gateway, application network, Central Music Platform and Walkman System.

| KQI Category |  | Key Quality Indicator |  |  | Definition |
| --- | --- | --- | --- | --- | --- |
| Service Accessibility Performance |  | File Download Response Success Rate |  |  | The Download Response Success Rate describes the probability that the user can set up the transmission successfully for download when requested by the user. |
|  |  | File Download Response Delay |  |  | The Download Response Delay describes the download response duration from requesting the download at the portal until the transmission is setup successfully. |
| Service Integrity Performance |  | File Download Success Rate |  |  | The Download Success Rate describes the probability that the user can download the whole songs data successfully from the Walkman system. |
|  |  | File Data Rate |  |  | The Data Rate describes the bit-rate of transmission that songs download from the Walkman system. |
|  |  | File Download Delay |  |  | The Download Delay describes the download duration from the first data packet send to the UE until the whole songs data is download to the UE. |
| Key Quality Indicator | Related KPI |  | Technology | PI |  |
| File Download Response Success Rate | [TS 32.450]: E-RAB Accessibility; [TS 32.410]: RAB Establishment Success Rate; RRC Connection Establishment Success Rate; Service Access Success Rate; PDP Context Activation Success Rate; [TS 32.455]: EPS Attach Success Rate; Dedicated EPS Bearer Creation Success Rate; Service Request Success Rate; |  | Wireless network | [TS 32.425]: Attempted RRC connection establishments; Successful RRC connection establishments; Number of initial E-RABs attempted to setup; Number of initial E-RABs successfully established; [TS 32.426]: Attempted EPS attach procedures; Successful EPS attach procedures; [TS 32.405]: Attempted RAB establishments for PS domain; Successful RAB establishments without queuing for PS domain; Successful RAB establishments with queuing for PS domain; [TS 32.406]: Attempted GPRS attach procedures; Successful GPRS attach procedures; |  |
| File Download Response Delay | [TS 32.455]: Dedicated Bearer Set-up Time by MME (Mean); |  | Wireless network | [TS 32.425]: RRC connection setup time; Mean E-RAB Setup time; Maximum E-RAB Setup time; [TS 32.405]: RAB PS connection set-up time (Mean); RAB PS connection set-up time (Maximum); RRC connection set-up time (Mean); RRC connection set-up time (Maximum); PDP Context set-up time; [TS 32.406]: PDP Context set-up time, initiated by MS; PDP Context set-up time, initiated by Network; |  |
| File Download Success Rate | [TS 32.450]: E-RAB Retainability; [TS 32.410]: RAB Abnormal Release Rate; GERAN Service Abnormal Release Rate; |  | Wireless network | [TS 32.425]: DL PDCP SDU drop rate; DL PDCP SDU air interface loss rate; [TS 32.405]: Attempted RRC re-establishments; Failed RRC re-establishments; ;m |  |
|  | PON/ONU Mean Time between Resynchronizations |  | Wireline network | Mean Time between Resynchronizations(MTBR) |  |
| File Download Delay | [TS 32.450]: E-UTRAN IP Throughput; E-UTRAN IP Latency; Percentage of Established RABs, TotalPS; [TS 32.455]: Mean Active Dedicated EPS Bearer Utilization; |  | Wireless network | [TS 32.425]: Average DL cell PDCP SDU bit-rate; Maximum DL cell PDCP SDU bit-rate; Average DL PDCP SDU delay; DL PDCP SDU drop rate; DL PDCP SDU air interface loss rate; IP Throughput in DL; IP Latency in DL; ;m |  |
|  | PON/ONU Upstream/Downstream Rate |  | Wireline network | PON/ONU Upstream/Downstream Rate |  |
| File Data Rate | [TS 32.450]: E-UTRAN IP Throughput; E-UTRAN IP Latency; [TS 32.410]: Percentage of Established RABs, TotalPS; [TS 32.455]: Mean Active Dedicated EPS Bearer Utilization; |  | Wireless network | [TS 32.425]: IP Throughput in DL; IP Latency in DL; Average DL cell PDCP SDU bit-rate; Maximum DL cell PDCP SDU bit-rate; Average DL PDCP SDU delay; DL PDCP SDU drop rate; DL PDCP SDU air interface loss rate; ;m |  |
|  | PON/ONU Upstream/Downstream Rate |  | Wireline network | PON/ONU Upstream/Downstream Rate |  |

## 4.1.3.3 Audio Streaming sub scenario

For the audio streaming sub scenario, it has two main subscenarios: buffering audio (the audio streaming data have been existed in the server) and real time audio streaming. It involves wireless network, core network, WAP gateway, application network, Central Music Platform and Walkman System.

Table 10: KQIs and Related KPIs of Audio Streaming Sub Scenario

| Key Quality Indicator | KQI Category | Related KPI | KPI description | Technology | PI |
| --- | --- | --- | --- | --- | --- |
| Data Error Rate | Integrity | Data error rate per service | This KPI is obtained by network for each user from user end to application server end. | GSM/UMTS/LTE/WLAN | Data error rate per service per user. |
|  |  | Packet Loss Ratio | This KPI is obtained by The interface of mobile boradband Network Packet Loss Ratio | MobileBroadBand | Packet Loss Ratio(%) |
| Delay | Integrity | Average Delay | This KPI is obtained by the interface of mobile boradband Network Average Delay | MobileBroadBand | Avg.Delay(ms) |
|  |  | Minimum Delay | This KPI is obtained by the interface of mobile boradband Network Minimum Delay | MobileBroadBand | Min.Delay(ms) |
|  |  | Maximum Delay | This KPI is obtained by the interface of mobile boradband Network Maximum Delay | MobileBroadBand | Max.Delay(ms) |
| Jitter | Integrity | Average Jitter | This KPI is obtained by the interface of mobile boradband Network Average Jitter | MobileBroadBand | Avg. Jitter (ms) |

# Requirements

## Generic Requirements

The main actor analyzed for the Business scenarios above is the Performance Management System and their elements EMS, NMS, NEs.

Following requirements would like to cover use cases that may cover both the wireline and the wireless domain and future as well as existing technologies. KQI shall be technology and network agnostic.

| Identifier: REQ-PM-1 | Rel. Use case id : | Priority: | Addressed to: SDOs, OSS vendors |  | Context: OSS |
| --- | --- | --- | --- | --- | --- |
| Title : Independent KQI |  |  |  |  |  |
| Description: KQI should be network type, technology, and vendor independent. |  |  |  |  |  |
| Rationale: The service can be provided on heterogeneous networks with fixed devices or mobile devices moving through them. It is necessary to ensure a suitable level of service independently from the context. This requirement relates all scenarios because it would allow to measure the quality when the service is accessed from either fixed network and mobile network. |  |  |  |  |  |

| Identifier: REQ-PM-2 | Rel. Use case id : | Priority: | Addressed to: SDOs, Equipment vendors, OSS vendors |  | Context: EMS, OSS |
| --- | --- | --- | --- | --- | --- |
| Title : Independent KPI |  |  |  |  |  |
| Description: KPI used to build KQI should be as much as possible network type, technology, and vendor independent. |  |  |  |  |  |
| Rationale: The service can be provided on heterogeneous networks with fixed devices or mobile devices moving through them. It is necessary to ensure a suitable level of service independently from the context. This requirement relates all scenarios because it would allow to measure the quality when the service is accessed from either fixed network and mobile network. |  |  |  |  |  |

| Identifier: REQ-PM-3 | Rel. Use case id : | Priority: | Addressed to: SDOs, Equipment vendors |  | Context: EMS |
| --- | --- | --- | --- | --- | --- |
| Title : Homogeneity of Pis/KPIs |  |  |  |  |  |
| Description: PIs/KPIs shall be defined where relevant (applicable to wireline and wireless technology) in similar ways in every technology in order to allow KPIs to be as much as possible technology independent (according to REQ-PM-1) |  |  |  |  |  |
| Rationale: PIs/KPIs are already existing in each technology, but in order to compute derived KPIs it has to be possible to mix similar PIs/KPIs coming from different technologies to produce common KPIs necessary for a global service KQI. In design of new network elements, it needs to take consideration existence of similar PIs in other technologies. This requirement addresses newly introduced NEs either in the fixed and mobile networks. And do not expect changes are performed on existing ones. It also addresses all the above described scenarios. |  |  |  |  |  |

| Identifier: REQ-PM-4 | Rel. Use case id : | Priority: | Addressed to: SDOs, OSS vendors |  | Context: OSS |
| --- | --- | --- | --- | --- | --- |
| Title : KQIs derivation |  |  |  |  |  |
| Description: In order to ensure performance management of services deployed jointly on different technology domains, it shall be possible to produce KQIs based on KPIs in Near Real Time from different technologies. |  |  |  |  |  |
| Rationale: It might be necessary to collect KPI(s) from different technology domains to produce/calculate KPIs and KQIs in Near Real Time in order to detect violation of agreed Service Level. It also addresses all the above described scenarios. |  |  |  |  |  |

## General Requirements

According to ITU-T Recommendation E.800, six Quality of Service categories are defined as follows:

- Service Support Performance: The ability of an organization to provide a service and assist in its utilization.

- Service Operability Performance: The ability of a service to be successfully and easily operated by a user.

- Service Accessibility Performance: The ability of a service to be obtained, within specified tolerances and other given conditions, when requested by the user.

- Service Retainability Performance: The ability of a service, once obtained, to continue to be provided under given conditions for a requested duration.

- Service  Integrity Performance: The degree to which a service is provided without excessive impairments, once obtained.

- Service Security Performance: The protection provided against unauthorized monitoring, fraudulent use, malicious impairment, misuse, human mistake and natural disaster.

According to the above categories, it seems suitable to ensure that related requirements on KQI/ KPI are impacted by the presence of several network technologies (fixed, mobile, new, old tech., ) This suggests to introduce a requirements for each of the above categories that may suffer different impact from the heterogeneous scenarios we are facing.

| Identifier: REQ-PM-5 | Rel. Use case id : | Priority: | Addressed to: SDOs, OSS vendors |  | Context: OSS |
| --- | --- | --- | --- | --- | --- |
| Title : Genericity of Accessibility KQI |  |  |  |  |  |
| Description: All KQIs related to Accessibility shall be Technology, Network type independent. These should also be SDOs , vendors and operators independent. |  |  |  |  |  |
| Rationale: In providing services to end-users, the first step is to get access to the service, independently from the network or technology used by the client. First after access to the service has been performed, the service can be used. If an accessibility measurement is not considered OK, then the network operator cannot investigate which steps that are required to improve the accessibility towards their customers. For example, the current wireless communication technologies contain GSM, UMTS, LTE, WLAN and etc., which have different features. So the performance measurements of accessibility for different wireless networks vary. The same for access from fixed network, For instance, performance measurement of RRC connection establishment and RAB establishment can reflect the accessibility of UMTS network. In case of Services relying on just one technology it is not necessary to have KQI independent from the technology. |  |  |  |  |  |

| Identifier: REQ-PM-6 | Rel. Use case id : | Priority: | Addressed to: SDOs, OSS vendors |  | Context: OSS |
| --- | --- | --- | --- | --- | --- |
| Title : Genericity of Retainability KQI |  |  |  |  |  |
| Description: All KQIs related to Retainability shall be Technology, Network type independent. These should also be SDOs , vendors and operators independent. |  |  |  |  |  |
| Rationale: When a service is used, it is important that it is not interrupted or aborted. If a retainability measurement is not considered OK, then the network operator cannot investigate which steps that are required to improve the retainability towards their customers. This measurement should be used for observing the impact of telecommunications network on end-users service retainability. For example, the current wireless communication technologies contain GSM, UMTS, LTE, WLAN and etc., which have different features. So the performance measurements of retainability for different wireless networks vary. For instance, performance measurement of RAB abnormal release could reflect the retainability of UMTS network directly. In case of Services relying on just one technology it is not necessary to have KQI independent from the technology. |  |  |  |  |  |

| Identifier: REQ-PM-7 | Rel. Use case id : | Priority: | Addressed to: SDOs, OSS vendors |  | Context: OSS |
| --- | --- | --- | --- | --- | --- |
| Title : Genericity of Integrity KQI |  |  |  |  |  |
| Description: All KQIs related to Integrity shall be Technology, Network type independent. These shall also be SDOs , vendors and operators independent. |  |  |  |  |  |
| Rationale: When a service is used, it is important that the quality of the service is acceptable. If an integrity measurement is not considered OK, then the network operator cannot investigate which steps that are required to improve the quality provided to their customers. This measurement should be used for observing the impact of telecommunications network on end-users service integrity. For example the current wireless communication technologies contain GSM, UMTS, LTE, WLAN and etc., which have different features. So the performance measurements of integrity for different wireless networks vary. For instance, performance measurement of E-UTRAN IP throughput could reflect the integrity of LTE network directly. In case of Services relying on just one technology it is not necessary to have KQI independent from the technology. |  |  |  |  |  |

## High Level Functional Requirements

| Identifier: REQ-PM-8 | Rel. Use case id : | Priority: | Addressed to: SDOs |  | Context: Interface |
| --- | --- | --- | --- | --- | --- |
| Title : Unified collection mechanism (standardization of NBI) |  |  |  |  |  |
| Description: The PIs/KPIs shall be collected by NMS via EMS from different types of network elements in a uniform way. |  |  |  |  |  |
| Rationale: The same collecting approach (same processes, same tools, … ) has to be used to collect Performance information from EMS to NMS for all Network Element types to ensure the possibility of collecting information efficiently for the proposed service, independently from the network , vendor, provider. For example in scenarios above, the file name that contains PI/KPI information and the method to get the performance file should be specified in same way for all kinds of NEs from wireline and wireless networks. |  |  |  |  |  |

| Identifier: REQ-PM-9 | Rel. Use case id : | Priority: | Addressed to: SDOs |  | Context: Interface |
| --- | --- | --- | --- | --- | --- |
| Title : Homogeneity of PIs/KPIs administration with respect to network type and technology (standardization of NBI) |  |  |  |  |  |
| Description: Measurement jobs shall be done for collecting performance PI/KPI values in a converged management environment. The parameters of the job (e. g. frequency, timing,) shall be defined taking into account the presence of several technologies and network types. |  |  |  |  |  |
| Rationale: Strictly associated to the need of a Uniform Collection Mechanism ( Req. REQ-PM-8 ) also attributes (same scales for instance ) need homogeneity in spite of the technology, to ensure their mixing in case of services provided contemporaneously on several networks and / or comparison. For example in the scenarios above, measurement job administration including addition, modification and deletion of attributes and jobs should be done in the same method. |  |  |  |  |  |

| Identifier: REQ-PM-10a | Rel. Use case id : | Priority: | Addressed to: SDOs | Context: Architecture, Interface |
| --- | --- | --- | --- | --- |
| Title : Transfer of PIs |  |  |  |  |
| Description: It shall be possible to transfer PIs in the following ways: - from NE to EMS and then from EMS to NMS via North Bound Interface - from NEs to NMS - from NMS to NMS |  |  |  |  |
| Rationale: It is important to ensure the transfer of performance indicators between the various OSS layers: the PIs collected from the Network layer Element managers, should be passed to OSS Performance Management systems, and then reported to newly evolved Service Quality and Customer Experience management systems. In the case of some NEs without EMS, such as WLAN, router and switch etc. PIs produced from them should be transferred directly to NMS. All this should be done with maximum flexibility, allowing efficient collection through all technologies, harmonizing the existent in each single domain. For example in the scenarios above, for NEs with OMC, PIs should be transferred from NE to EMS then from EMS to NMS by NBI. For NEs without OMC like WLAN, PIs should be transferred directly from NEs to NMS. |  |  |  |  |

| Identifier: REQ-PM-10b | Rel. Use case id : | Priority: | Addressed to: SDOs | Context: Architecture, Interface |
| --- | --- | --- | --- | --- |
| Title : Transfer of KPIs NOTE: AP Rudolf and Yuval to review the definition of PI and KPI and align the rest of the document accordingly, including 10a and 10b. |  |  |  |  |
| Description: It shall be possible to transfer KPIs in the following ways: - from EMS to NMS (in case the collected measurement is key indicator) - from NMS to NMS - from NMS to SMS |  |  |  |  |
| Rationale: It is important to ensure the transfer of key performance indicators between the various OSS layers: NMS, SMS. |  |  |  |  |

| Identifier: REQ-PM-11 | Rel. Use case id : | Priority: | Addressed to: SDOs |  | Context: Architecture |
| --- | --- | --- | --- | --- | --- |
| Title : Unified templates for definition of PIs/KPIs /Derived KPIs |  |  |  |  |  |
| Description: A single template shall be defined for definition of collected PIs/KPIs, and another one for derived KPIs, covering different services in converged management environment. The derived KPI template should at least be based on the collected PI/KPI template. If possible, the template should be common for both cases. The template shall include among others the name, formulation, type of measurements |  |  |  |  |  |
| Rationale: Strictly associated to the need of a uniform collection mechanism ( Req. REQ-PM-7 ) here it is stressed the idea that it is not only necessary to have the same tools and processes for the collection of PIs/KPIs across the network but also the availability of the same templates are a critical aspect that needs to be addressed for efficient collection of KPIs and calculation of the related Quality Indicators As far as possible collected PIs/KPIs should be the same among vendors, to allow easier derived KPIs computation. This requirement relates to all scenarios above, because it would allow the availability of the same template when the service or services are accessed from either mobile network or fixed network. KQIs are to be the same for both accesses. |  |  |  |  |  |

| Identifier: REQ-PM-12 | Rel. Use case id : | Priority: | Addressed to: SDOs |  | Context: Interface |
| --- | --- | --- | --- | --- | --- |
| Title : Unified performance data i.e. PI and KPI exchange format used by EMS and NMS |  |  |  |  |  |
| Description: The performance data i.e. PI and KPI shall have one standard encoding format (e.g. output file format- independent from vendors and technology) |  |  |  |  |  |
| Rationale: Note: add a reference from REQ-PM-8 and REQ-PM-10a to REQ-PM-12. . It needs one standard encoding format to output Performance Data to NMS independent from vendor and technology either in fixed networks or mobile networks. |  |  |  |  |  |

| Identifier: REQ-PM-13 | Rel. Use case id : | Priority: | Addressed to: SDOs |  | Context: Interface |
| --- | --- | --- | --- | --- | --- |
| Title : Unified exchange format for KPIs and KQIs at OSS/BSS Level |  |  |  |  |  |
| Description: A common, harmonized and consistent exchange format for KPI (if it is calculated in OSS) or KQI agreed between interworking OSS/ BSS applications/areas for performance and service quality management shall be defined. Service PM system shall be able to keep and share quality information with all applications supporting SLA management independently from the network technology used and vendor. |  |  |  |  |  |
| Rationale: It is close to REQ-PM-12 but related to the exchange of information among OSS /BSS applications. Whether a single format can be defined for REQ-PM-12 and REQ-PM-13 will be addressed when defining the solution. This requirement relates all scenarios above because the quality information should be shared to support SLA management independently from technology and vendor. |  |  |  |  |  |

| Identifier: REQ-PM-14 | Rel. Use case id : | Priority: | Addressed to: OSS vendors |  | Context: OSS |
| --- | --- | --- | --- | --- | --- |
| Title : Multi-technology OSS Performance Management systems |  |  |  |  |  |
| Description: OSS Performance Management system shall be able to manage KQIs/KPIs/PIs for both fixed and mobile networks, whatever network technology is used. |  |  |  |  |  |
| Rationale: Also Performance Management Systems need to be able to manage data coming from several technologies to produce SQA metrics to measure SLA compliancy. This requirement relates all scenarios above because OSS Performance Management system has the ability to manage performance data covering both fixed and mobile networks to support SLA management independently from technology and vendor. |  |  |  |  |  |

| Identifier: REQ-PM-15 | Rel. Use case id : | Priority: | Addressed to: Equipment vendors |  | Context: EMS |
| --- | --- | --- | --- | --- | --- |
| Title : EMS as a collector to provide Near Real Time PIs to NMS |  |  |  |  |  |
| Description: EMS shall be able to provide PIs to NMS in Near Real Time to enable NMS - to calculate KPIs, if necessary, from the PIs - to forward KPIs to Service Management System |  |  |  |  |  |
| Rationale: It is necessary to feed NMS with PIs in Near Real Time so that NMS can calculate KPIs to monitor the network. This requirement relates all scenarios above because it should be ensured that NMS has ability to obtain performance data without delay to monitor network performance and report to Service Management System. |  |  |  |  |  |

| Identifier: REQ-PM-16 | Rel. Use case id : | Priority: | Addressed to: OSS vendors |  | Context: OSS |
| --- | --- | --- | --- | --- | --- |
| Title : NMS as a collector provides Near Real Time KPIs to Service Management System |  |  |  |  |  |
| Description: NMS shall be able to provide KPIs to Service Management System in Near Real Time to enable Service Management System - to calculate KQIs from the KPIs - to enable Service Management System to verify SLA compliance |  |  |  |  |  |
| Rationale: It is necessary to feed SMS with KPIs in Near Real Time so that SMS can check SLA fulfilment. This requirement relates all scenarios above because it should be ensured that Service Management System has ability to obtain quality information without delay to monitor service quality and verify SLA compliance. |  |  |  |  |  |

| Identifier: REQ-PM-17 | Rel. Use case id : | Priority: | Addressed to: SDOs |  | Context: Architecture, Interface |
| --- | --- | --- | --- | --- | --- |
| Title : Relationship between PI values and related resource instances |  |  |  |  |  |
| Description: It shall be possible to keep relation between PI values and the related resource instances for all technologies, vendors, and network types. |  |  |  |  |  |
| Rationale: This is necessary to ensure the exact identification of the resources impacted by possible lack of performance in multi technology service provisioning. This requirement relates all scenarios above because PIs should conduct network performance assessment and perform various aspects of trend analysis against specific resource instance, including error rate and cause analysis and resource degradation. |  |  |  |  |  |

| Identifier: REQ-PM-18 | Rel. Use case id : | Priority: | Addressed to: SDOs |  | Context: Architecture, Interface |
| --- | --- | --- | --- | --- | --- |
| Title : Relationship between KPI values and related resource instances |  |  |  |  |  |
| Description: It shall be possible to keep relation between KPI valuess and the related resource instances for all technologies, vendors, and network types. |  |  |  |  |  |
| Rationale: This is necessary to ensure the exact identification of the Resources impacted by possible lack of performance in multi technology service provisioning. This requirement relates all scenarios above because KPIs should collect, correlate, consolidate, and validate various performance statistics against specific resource instance. |  |  |  |  |  |

| Identifier: REQ-PM-19 | Rel. Use case id : | Priority: | Addressed to: OSS vendors |  | Context: Architecture, OSS |
| --- | --- | --- | --- | --- | --- |
| Title : KPIs values storage |  |  |  |  |  |
| Description: KPIs values have to be stored at OSS level e.g. in a Database at level of the PM System allowing easy extraction and correlation among technologies, network type and vendors data. KPI values have to be kept in the database to a required period according to the O&M requirements and SLA. These KPI are the ones to be considered when designing KQIs and SLA for newly defined services |  |  |  |  |  |
| Rationale: It is necessary to save and retrieve historical data related to the Performance of the Resources Independently from the technology, vendor, and Network Type to be able to measure the historical evolution of the Quality of the Service. This requirement relates all scenarios above because KPIs should collect, correlate, consolidate, validate and evaluate various performance data for the service quality management and SLA compliance. |  |  |  |  |  |

| Identifier: REQ-PM-20 | Rel. Use case id : | Priority: | Addressed to: OSS vendors |  | Context: Architecture, OSS |
| --- | --- | --- | --- | --- | --- |
| Title : Relationship between KQI values and related service instances |  |  |  |  |  |
| Description: A given service has associated some quality indicators (KQI) and the relation has to be kept to be able to associate the service to the indicators, eventually distributed according different levels of quality for the service (Silver, Gold, Platinum) |  |  |  |  |  |
| Rationale: It has to be possible to correlate Service Instances to related Quality indicators to be able to produce SLA fulfilment indicators for each given service instance. This can be obtained through suitable correlation between KQIs and services. This requirement relates all scenarios above because KQIs should collect, correlate, consolidate, and validate various quality statistics against specific service instance. |  |  |  |  |  |

| Identifier: REQ-PM-21 | Rel. Use case id : | Priority: | Addressed to: OSS vendors |  | Context: Architecture, OSS |
| --- | --- | --- | --- | --- | --- |
| Title : KQI values storage |  |  |  |  |  |
| Description: Quality indicators for a service are defined within the Service Design process and stored in a database at OSS level.. These KQI are the ones to be measured using the converged KPIs . |  |  |  |  |  |
| Rationale: This requirement relates all scenarios above because KQIs should be ensured to output the accurate value without time and location effects even supporting several applications. |  |  |  |  |  |

| Identifier: REQ-PM-22 | Rel. Use case id : | Priority: | Addressed to: OSS vendors |  | Context: Architecture, OSS |
| --- | --- | --- | --- | --- | --- |
| Title : Relationship among Service Instances and Resource Instances |  |  |  |  |  |
| Description: OSS Inventory management system has to keep relations among all Service Instances and related resource instances. Or Instantiated service to a specific customer uses a specific set of resources that have to be known when ensuring SLA fulfillment. The relation has to be kept in PM system (not in the Inventory? To be checked with Pekka) . |  |  |  |  |  |
| Rationale: This requirement relates all scenarios above. KQIs provide a measure of specific aspect of a service and are based on a number of sources including the KPIs. KPIs provide a measurement of a specific aspect of the performance of a service resource or group of service resources of the same type. These are usually computed on the basis of relationship among service instances and resource instances. |  |  |  |  |  |

| Identifier: REQ-PM-23 | Rel. Use case id : | Priority: | Addressed to: SDOs |  | Context: Architecture, Interface |
| --- | --- | --- | --- | --- | --- |
| Title: KPIs have to be made available for the adjustment of the Network configuration (e.g. SON functionalities) |  |  |  |  |  |
| Description: The above defined Exchange format <reference> shall be used to make KPIs available to OSS configuration management system (eg SON functionalities) in a Near Real Time mode, to re-configure the NEs if necessary to ensure SLA fulfillment |  |  |  |  |  |
| Rationale: Performance Management provides constant monitoring of the performance of networks to evaluate and report on the behaviour of telecommunication equipment and the effectiveness of the network, which helps to support the opening up and the deployment or reconfiguration of services with high levels of service availability for customers. This requirement relates to Walkman scenario. When a service is used it is important that it is not interrupted or aborted. If a retainability measurement is not considered OK, then it is required to improve the retainability towards customers. This measurement should be shared to OSS configuration management system for observing the impact of telecommunications network on end-users service retainability and to re-configure the NEs if necessary. For instance, performance measurement of RAB abnormal release could reflect the retainability of UMTS network directly. |  |  |  |  |  |

| Identifier: REQ-PM-24 | Rel. Use case id : | Priority: | Addressed to: SDOs |  | Context: OSS |
| --- | --- | --- | --- | --- | --- |
| Title: the KQI definition should cover the most used typical services in the networks. |  |  |  |  |  |
| Description: The KQI definition shall cover the most used typical services in the networks, e.g. video, web browsing, file download, email. |  |  |  |  |  |
| Rationale: All the application scenarios are composed by several typical services, if the KQIs of the services which are used in the scenario are defined, the KQIs of the application scenario can be derived from the basic typical services KQIs. |  |  |  |  |  |

## Other

<For each requirement introduce a table like the following one clearly identifying each entry to allow cross traceability with Use Cases >

| Identifier: | Rel. Use case id : | Priority: |
| --- | --- | --- |
| Title : |  |  |
| Description: |  |  |
| Rationale: |  |  |

# Performance Management DETAILED INFORMATION

The figure below ITU-T Recommendation E.800 is a framework intended to provide a general guide to the factors which contribute collectively to the overall quality of service as perceived by the user of a telecommunication service. The terms in the diagram can be thought of as generally applying either to the quality of service levels actually achieved in practice, to objectives which represent quality of service goals, or to requirements which reflect design specifications.

The figure is structured to show that one quality of service factor can depend on a number of others. It is important to note – although it is not explicitly stated in any of the definitions contained in this document – that the value of a characteristic measure of a particular factor may depend directly on corresponding values of other factors which contribute to it. This necessitates, whenever the value of a measure is given, that all of the conditions having an impact on that value be clearly stated.

The essential aspect of the global evaluation of a service is the opinion of the users of the service. The result of this evaluation expresses the users’ degrees of satisfaction. This Recommendation provides a framework for:

- The quality of service concept;

- Relating quality of service and network performance;

- A set of performance measures.

It is obvious that a service can be used only if it is provided, and it is desirable that the provider has a detailed knowledge about the quality of the offered service. From the provider’s viewpoint, network performance is a concept by which network characteristics can be defined, measured and controlled to achieve a satisfactory level of service quality.

It is up to the Service Provider to combine different network performance parameters in such a way that the economic requirements of the Service Provider as well as the satisfaction of the User are both fulfilled.

Figure 3: Performance Concepts from ITU-T E800 doc

NOTES

- Each concept may affect the one above collectively or individually.

- For the sake of clarity not all relationships are indicated, though they may be implied on the figure.

## Key Quality Indicators (KQIs)

KQIs provide a measurement of a specific aspect of the performance of the product, product components (services) or service elements and draw their data from a number of sources including the KPIs. There are two main types of KQI.  < specify the origin of the definition in the text and check if it is possible to quote exactly the original definition>

- The product KQI, which is required to monitor the quality of product offered to the end-user.

- The service KQI, which is focused on monitoring the performance of individual product components (services).

Product KQIs will derive some of their data from the Service KQIs, which are obtained by aggregating multiple KPIs. A product consists of a number of services which may be either network or non-network based. Services such as Voice, SMS and WAP are network derived, while itemized billing, customer care and insurance have little dependence on network resource. Correspondingly, KQIs can be sorted to network and non-network KQIs. In this document, service KQIs are only studied in their network related aspects. The KQIs that we want to consider in the present document are the severability ones as described in ITU-T Recommendation E.800 i. e. the ones that provide the ability of a service to be obtained – within specified tolerances and other given conditions – when requested by the user and continue to be provided without excessive impairment for a requested duration.

- Accessibility: The ability of a service to be obtained, within specified tolerances and other given conditions, when requested by the user.

- Retainability: The ability of a service, once obtained, to continue to be provided under given conditions for a requested duration.

- Integrity: The degree to which a service is provided without excessive impairments, once obtained.

## Key performance indicators (KPIs)

- KPIs provide a measurement of a specific aspect of the performance of a service resource (network or non-network) or group of service resources of the same type. A KPI is restricted to a specific resource type. In this document, we are focusing on KPIs related to the above mentioned KQIs, which are the dependability ones as described in ITU-T Recommendation E.800 except for Maintenance Support Performance ones i. e. the ones that provide the ability of an item to be in a state to perform a required function at a given instant of time or at any instant of time within a given time interval, assuming that the external resources, if required, are provided. In addition, we focus Mobility as well

- Reliability: The ability of an item to perform a required function under given conditions for a given time interval.

- Maintainability: The ability of an item under stated conditions of use, to be retained in, or restored to, a state in which it can perform a required function, when maintenance is performed under given conditions and using stated procedures and resources.

- Mobility: it contains the Handover related KPIs (3GPP TS 32.410)

## Relation among KPIs, KQIs and PIs from specific Technology Network Elements (NEs)

From the NEs, the network performance data is aggregated to provide KPIs that provide an indication of service resource performance. The KPIs are then used to produce the Service KQIs that are the key indicators of the service element performance. Service KQIs are then used as the primary input for management of internal or supplier/ partner SLAs that calculate actual service delivery quality against design targets or in the case of supplier/ partner, contractual agreements. Service KQIs provide the main source of data for the Product KQIs that are required to manage product quality and support the contractual SLAs with the customer. The key indicator hierarchy is illustrated in following picture.

Different KPIs are defined for different technologies, which causes the need to make these indicators homogeneous for producing quality indicators for services provided through several technologies at the same time or in alternative.

Figure 4: Key Indicator Hierarchy (see in TM Forum GB917)

NOTES

- Performance Related data in the Figure is referred as Performance Indicators (PIs) in other part of the document..

Service and network performance are mainly indicated by KQIs and KPIs. KQIs provide a measure of specific aspect of a service and are based on a number of sources including the KPIs. KPIs provide a measurement of a specific aspect of the performance of a service resource (network or non-network) or group of service resources of the same type. These are usually computed on the basis of Performance Indicators (PIs) , which are indicated by Performance Related data in the Figure above, collected from Network Elements impacted by specific service provisioning.

Service Performance KQIs are user-oriented while Network Performance KPIs are network provider and technology-oriented. Network Performance KPIs together with Service Performance KQIs need to be identified to capture a more accurate picture of the customer’s perception. Computation of service performance is based on network-related KPIs and non-network related KQIs.

Network performance KPIs are usually defined in terms of the number and/ or intensity of network performance event occurrences during a specified time interval. The values of these KPIs can be computed from event information (e. g. counters) and reported to the customer service.

Criteria to define Network and Service performance KQIs and KPIs:

- Provide concrete repeatable measurements in well-defined quantities without subjective interpretation.

- Be useful to users and service providers in understanding the performance they experience or provide.

- Not exhibit a bias for services supported by one network technology as opposed to another technology.

- Be measurable via a process acceptable to service providers, their customers and, in some cases, outside testing agencies while avoiding including artificial performance goals.

- Be useful as a specification in contractual documents to help customers with high performance requirements purchase the level of service they need.

- Be useful for publication in datasheets.

- Be capable of measuring one provider independent of all others.

- Support a diagnostic mode that can be used to sectionalize or identify the degradation contribution in a long multi-hop, multi-provider path.

The big challenge in this study is not in the identification of the KPIs necessary to derive suitable KQI, since a great deal of work has already been performed by several SDOs, but in understanding how different technology PIs can be mixed together to provide multi-technology/ convergent KPIs and thus derive KQIs

## Prioritization (e.g. priorities table, list, etc…)

<TbD>

## Vendor Constraints

<TbD>

## <Other if/where applicable>

<TbD>

# Conclusions/Recommendations

<TbD>

# Appendix 1 - STANDARDS Related to Video Communication

There are following existing standards which are related to video vommunication:

- 4Q ‘11: HDVC group defined two specifications for IMS-based point-to-point and conferencing video communication

  - UNI specification: support for several access networks: xDSL, Ethernet, LTE. Using IR.92 (VoLTE) and IR.94 (video over LTE) as a base.

  - NNI specification: built by profiling 3GPP TS 29.165 for video service.

- 3GPP is specifying the use cases and requirements for IMS-Based telepresence conference as extensions of existing IMS multi-media conference services and as applicable for different kinds of devices (mobile, fixed, etc.).

- OTTs, with WebRTC aim at deploying voice and video services through web coding. All the necessary for web conversational services will be natively integrated in web browsers, without the need for any other native application or plug-ins.

- TM Forum: GB938_Video_Over_IP_Application, describe how to define a standard way of implementing SLA for Video over IP services.

- 3GPP SA4: TR26.944 End-to-end multimedia services performance metrics.

- ETSI: TS102.250 (PartI~PartVII) Speech and multimedia Transmission Quality (STQ); QoS aspects for popular services in mobile networks.

- ITU: J.144：Objective perceptual video quality measurement techniques for digital cable television in the presence of a full reference;

J.241：Extended video procedures and control signals for ITU-T H.300-series terminals

P.MSW：Parametric non-intrusive assessment of audiovisual media streaming quality - lower resolution application area

# Appendix 2 - Architectural and Framework Standards: The TMN/FCAPS Model (ITU-T)

From Ralf Wolter, “Network Management: Accounting and Performance Strategies”, Benoit Claise, CCIE® No. 2868

The ITU-T introduced the term Telecommunications Management Network (TMN) to describe a separate network that has interfaces to the telecommunication network (or production network). TMN defines interconnection points between the two networks and specifies management functionalities. The best description of how to operate a TMN is defined by the ITU-T recommendations M.3010, M.3400, and X.700.

The purpose of a framework is to describe the big picture, illustrate different functional areas, and identify how they interoperate. The focus of ITU-T M.3400 and X.700 is the specification and classification of management functionalities only. For example, it does not define whether syslog or trap messages are mandatory for event notifications. Neither does it define specific formats for storing accounting records. These details are defined in the lower-level specifications standards. In addition to the framework of M.3400, another recommendation (M.3010) defines the principles for a TMN. It includes details of a Data Communications Network (DCN) as a transport vehicle between the management applications and network elements. A DCN is also known as out-of-band-management, which separates user traffic from management traffic. Figure 3-1 illustrates the relationship between the telecommunications network, also called the service infrastructure, and the TMN.

##### Figure 3-1. TMN and Telecommunications Networks

Another relevant aspect of M.3010 is the concept of layers. Network management tasks are grouped into functional areas such as FCAPS. In addition, a logical layered architecture (LLA) consists of five management layers:

- Network Element Layer (NEL) defines interfaces for the network elements, instantiating functions for device instrumentation, ideally covering all FCAPS areas.

- Element Management Layer (EML) provides management functions for network elements on an individual or group basis. It also supports an abstraction of the functions provided by the network element layer. Examples include determining equipment errors, measuring device temperatures, collecting statistical data for accounting purposes, and logging event notifications and performance statistics.

- Network Management Layer (NML) offers a holistic view of the network, between multiple pieces of equipment and independent of device types and vendors. It manages a network as supported by the element management layer. Examples include end-to-end network utilization reports, root cause analysis, and traffic engineering.

- Service Management Layer (SML) is concerned with, and responsible for, the contractual aspects of services that are being provided to customers. The main functions of this layer are service creation, order handling, service implementation, service monitoring, complaint handling, and invoicing. Examples include QoS management (delay, loss, jitter), accounting per service (VPN), and SLA monitoring and notification.

- Business Management Layer (BML) is responsible for the total enterprise. Business management can be considered a goal-setting approach: "What are the objectives, and how can the network (and network management specifically) help achieve them?"

Figure 3-2 shows the relationship between the different layers as well as the relationship with the FCAPS model. Each management layer is responsible for providing the appropriate FCAPS functionality according to the layer definition. Each layer communicates with the layers above and below it.

##### Figure 3-2. ITU-T M.3010: TMN Logical Layer Architecture

Now that the different layers of the TMN model have been identified, the functionality of each FCAPS area is described next. The TMN architecture has a strong relationship to Open Systems Interconnection (OSI) standards and frameworks. ISO 7498-4 defines the framework, concepts, and terminology of the OSI Management standards: "Information Processing Systems - OSI - Basic Reference Model - Part 4: Management Framework."

The ITU-T M.3400 recommendation is one document within the M series. It specifies five management functional areas (FCAPS):

- Fault management— Detect, isolate, notify, and correct faults encountered in the network.

- Configuration management— Configure aspects of network devices, such as configuration file management, inventory management, and software management.

- Accounting management— Collect usage information of network resources.

- Performance management— Monitor and measure various aspects of performance so that overall performance can be maintained at a defined level.

- Security management— Secure access to network devices, network resources, and services to authorized individuals.

The following section provides more details about each area. This chapter covers the FCAPS model extensively, because it sets the foundation for network management in general and provides a good understanding of accounting and performance management and their potential relationship to each other. Therefore, not only the accounting and performance parts of FCAPS are addressed, but the full model. Afterwards, other standards are discussed solely based on their specific relationship to accounting and performance management. Also, note that there is not an exact match between the brief FCAPS summary in Chapter 1 and the extended details for accounting and performance in this chapter. Chapter 1 provides the authors' definitions from a network element perspective, and this chapter covers FCAPS functionality from all layers of the TMN model.

#### Fault Management

Fault management is a set of functions that enable the detection, isolation, and correction of abnormal operation of the telecommunication network. The quality assurance measurements for fault management include component measurements for Reliability, Availability, and Survivability (RAS). Fault management consists of the following functions:

- RAS quality assurance establishes the reliability criteria that guide the design policy for redundant equipment (a responsibility of configuration management) and the policies of the other function groups in this area.

- Alarm surveillance describes the capability to monitor network element failures in Near Real Time.

- Fault localization describes where the initial failure information is insufficient for fault localization. It has to be augmented with information obtained by additional failure localization routines at the application level.

- Fault correction transfers data concerning the repair of a fault and the control of procedures that use redundant resources to replace equipment or facilities that have failed.

- Testing can be carried out in two ways. In one case, a network element analyzes equipment functions, where processing is executed entirely within the network element. Another method is active testing of external device components, such as circuits, links, and neighbor devices.

- Trouble administration transfers trouble reports originated by customers and trouble tickets originated by proactive failure-detection checks. It supports action to investigate and clear the problem and provides access to the status of services and the progress in clearing each problem.

#### Configuration Management

Configuration management provides functions to identify, collect configuration data from, exercise control over, and provide configuration data to network elements. Configuration management supports the following functions:

- Installing the physical equipment and logical configurations.

- Service planning and negotiation, which addresses planning for the introduction of new services, changing deployed service features, and disconnecting existing services.

- Provisioning, which consists of necessary procedures to bring equipment into service but does not include installation. As soon as the unit is ready for service, the supporting programs are initialized via the TMN. The state of the unit (in service, out of service, standby, or reserved) and selected parameters may also be controlled by provisioning functions.

- Status and control where the TMN provides the capability to monitor and control certain aspects of the network element (NE) on demand. Examples include checking or changing an NE's service state (in service, out of service, or standby) or the state of one of its subparts and initiating diagnostic tests within the NE. Normally, a status check is provided in conjunction with each control function to verify that the resulting action has taken place. When associated with failure conditions, these functions are corrective in nature (such as service restoration).

- Network planning and engineering deals with functions associated with determining the need for growth in capacity and the introduction of new technologies. Planning and engineering are examples of functions across multiple areas, because they relate to the performance section from a monitoring perspective and to the configuration section from an enforcement perspective.

#### Accounting Management

Accounting management lets you measure the use of network services and determine costs to the service provider and charges to the customer for such use. It also supports the determination of charges for services. Accounting management includes the following functions:

- Usage measurement— Consists of the following subfunctions:

- Planning and management of the usage measurement process

- Network and service usage aggregation, correlation, and validation

- Usage distribution

- Usage surveillance

- Usage testing and error correction

- Measurement rules identification

- Usage short-term and long-term storage

- Usage accumulation and validation

- Administration of usage data collection

- Usage generation

- Tariffing and pricing— A tariff is used to determine the amount of payment for services usage.

- Collections and finance— Functionality for administering customer accounts, informing customers of balances and payment dates, and receiving payments.

- Enterprise control— This group supports the enterprise's financial responsibilities, such as budgeting, auditing, and profitability analysis.

#### Performance Management

Performance management provides functions to evaluate and report on the behavior of telecommunication equipment and the effectiveness of the network or network element. Its role is to gather and analyze statistical data for the purpose of monitoring and correcting the behavior and effectiveness of the network, network elements, or other equipment, and to aid in planning, provisioning, maintenance, and quality measurement. Performance management includes the following functions:

- Performance quality assurance— Includes quality measurements, such as performance goals and assessment functions.

- Performance monitoring— This component involves the continuous collection of data concerning the performance of the network element. Acute fault conditions are detected by alarm surveillance methods. Very low rate or intermittent error conditions in multiple equipment units may interact, resulting in poor service quality, and may not be detected by alarm surveillance. Performance monitoring is designed to measure the overall quality, using monitored parameters to detect such degradation. It may also be designed to detect characteristic patterns of impairment before the quality has dropped below an acceptable level. Performance monitoring includes the following functions:

- Performance monitoring policy

- Network performance monitoring event correlation and filtering

- Data aggregation and trending

- Circuit-specific data collection

- Traffic status

- Threshold crossing alert processing

- Trend analysis

- Performance monitoring data accumulation

- Detection, counting, storage, and reporting

- Performance management control— This group includes the setting of thresholds and data analysis algorithms and the collection of performance data. It has no direct effect on the managed network. For network traffic management and engineering, this includes functions that affect the routing and processing of traffic.

- Performance analysis— The collected performance records may require additional processing and analysis to evaluate the entity's performance level. Therefore, performance analysis includes the following functions:

- Recommendations for performance improvement

- Exception threshold policy

- Traffic forecasting (trending)

- Performance summaries (per network and service, and traffic-specific)

- Exception analysis (per network and service, and traffic-specific)

- Capacity analysis (per network and service, and traffic-specific)

- Performance characterization

#### Security Management

Security is required for all functional areas. Security management consists of two main functions:

- Security services for communications provide authentication, access control, data confidentiality, data integrity, and nonrepudiation. These may be exercised in the course of any communications between systems and between users or customers and systems. In addition, a set of pervasive security mechanisms are defined that are applicable to any communication, such as event detection, security audit-trail management, and security recovery.

- Security event detection and reporting reports activities that may be construed as a security violation (unauthorized user, physical tampering with equipment) on higher layers of security applications.

Security management includes the following functions:

- Prevention

- Detection

- Containment and recovery

- Security administration

#### The TMN Framework

The framework shown in Figure 3-3 brings it all together. The different logical layers sit on top of each other; each layer is responsible for implementing the FCAPS functionality and passes the collected information to the next layer. From a customer's applicability perspective, after identifying and prioritizing the requirements, you can map various network management products to this matrix and identify what is required to meet your needs.

##### Figure 3-3. TMN Management Layers and FCAPS

From the perspective of this book, relevant layers of the TMN architecture are the Network Element Layer (NEL); the Element Management Layer (EML), related to similar device types; the Network Management Layer (NML), related to mediation; and the Service Management Layer (SML), related to service monitoring and accounting. The Business Management Layer (BML) is outside the scope of this book. If you're interested, you're encouraged to study the ITU-T M series for more details.

Note that the FCAPS model describes the conceptual model of functional areas; it does not define accounting and performance standards for data collection. Therefore, a second level of standards is required for data collection. At the network element layer, it can be SNMP or IPFIX. IPDR is appropriate at the Element Management Layer.

# Appendix 3 - Relationship between Service/Resource instances and KQI/KPI/PI instances

Relationship between Service/Resource instances and KQI/KPI/PI instances can be reflected by following 2 figures.

And

TM Forum SID also defined the data model of Service/Resource and their Performance generally in GB922_Addendum_1_Performance_R12-5_v1-10.

Figure PF.01 – SID Framework Performance Entities

The Service Performance ABE collects, correlates, consolidates, and validates various performance statistics and other operational characteristics of customer and resource facing service entities. It provides a set of entities that can monitor and report on performance. Each of these entities also conducts network performance assessment against planned goals, performs various aspects of trend analysis, including error rate and cause analysis and Service degradation.  Entities in this ABE also manage the traffic generated by a Service, as well as traffic trend analysis. This is important for newer technologies that separate data, control and management functions for a given Service.

The Resource Performance ABE collects, correlates, consolidates, and validates various performance statistics and other operational characteristics of Resource entities. It provides a set of entities that can monitor and report on performance. The entities in this ABE provide physical, logical, and performance information. Each of these entities also conducts network performance assessment against planned goals, performs various aspects of trend analysis, including error rate and cause analysis and Resource degradation. Entities in this ABE also manage traffic in a Resource. This includes statistics defining Resource loading, and traffic trend analysis.

Service, Resource, and Performance

The Performance ABE is an addition to the SID model’s Common Business Entities domain.  It represents an generalization of attributes and relationships that are common to both the SID Service Performance and Resource Performance ABEs’ entities, such as performance indicators, objectives, and consequences.  Resource performance entities is collected hop by hop (between two network elements), while service performance is an aggregation of multiple resource performance instances that constitute the path a service takes through a network.

Two key entities that comprise the Performance ABE are PerformanceSpecification and Performance.  They represent an application of the SID EntitySpecification/Entity pattern.  A PerformanceSpecification and its related specification entities provide the definition of the performance attributes, such as KPIs, dimensions, and performance objectives.  The Performance entity and its related entities provide the values associated with the manner in which a service or resource is performing or has performed, such as the loss of 5K packets between two network elements.

Figure PF.02 – Service, Resource, and Performance

Figure PF.02 shows the associations that performance entities have with existing service and resource entities, as well as the association that the Performance entity has with the Customer entity.  Note that the dark-shaded entities, CustomerFacingServiceSpecification and CustomerFacingService are not within the scope of the model as they related the Service domain with the Product domain, which is out of scope at this point in time.  Also note that the ResouceSpecification is related directly to the PerformanceSpecification entity and Resource is related to the Performance entity, since both Service and Resource performance deal with Resources.

The associations with the specification entities provide part of the definition of the performance specification entities.  For example, the specification of a GGSN and SGSN define the types of network elements whose performance is measured.

The cardinality of the association between ResourceSpecification and PerformanceSpecification is 0,2 because at most two types of resources’ performance is defined by a PerformanceSpecification.  Similarly, the association between Resource and Performance also has a cardinality of 0,2.  These Resource domain entities are directly related to the respective Performance ABE entities and not to ResourcePerformanceSpec and ResourcePerformance.  The reason is that Resources are used to measure both Service-related and Resource-related performance.

For the ServiceSpecification entity the cardinality is 1 because a given instance of a ServicePerformanceSpec entity measures the performance of a single ServiceSpecification entity, such as a mobile voice call.  Similarly, the cardinality of the association between a Service and a Service performance is 1.

The cardinality between a Customer and Performance is 0,1 because a instance of Performance may or may not be a measure of performance for an instance of Customer.

A PM measurement job is the administrative entity defined by a client application to perform a periodic activity related to PM data. The PM activities may be production of measurements, collection of measurements or PM objective monitoring (scheduled evaluation of thresholds). The PM Measurement Job is a sub class of the PM Management Job (Management Job ABE under TIP Common ABE). The Management Job defines the attributes that may apply to any kind of a job while the Measurement Job adds the attributes that are specific to the Performance Management context.

The PM measurement job includes the following:

- A set of requested performance indicators

- A network/service scope defining a subset of the network or a subset of the service catalog

- Scheduling parameters

- Specific activity parameters, related to collection, production or objective monitoring

A measurement job must define a scope of monitored objects using the monitored objects criteria. The monitored objects criteria support two kinds of monitored objects specification, represented as two sub-entities:

- Monitored Instances criteria - Specifies a list of individual monitored instances (typed as object names).

- Monitored Class criteria - Specifies a monitored object class (a string) in conjunction with a filter object. The filter is composed of name value pairs that can be used to focus the PM collection.

The Measurement Job is shown in figure PF.15 below.

Figure PF.15 – Measurement Job

A Measurement Collection Job is used to control the periodic collection of performance indicators, implemented as a sub-entity of the PM Measurement Job. As the Measurement Collection Job inherits from the Measurement Job, it includes all attributes and associations defined for the base measurement job entity. The Measurement Collection Job and its attributes are shown in figure PF.16 below.

Similarly, a Measurement Production Job is defined as a sub-entity of the Measurement Job. It is used to define a job for generating indicators. A Production Job may have a Collection Job associated to it.  The Production Job is described is also shown in PF.16 below.

Figure PF.16 – Performance Monitoring, Measurement Production &Collection Jobs

# Appendix 4 – Sketch of Resource and Service Management Domain

From  TM Forum – TAM Application_Framework_R4_5__V4.3.doc