# Multi-SDO Project on Converged Management PM Interface definitions

# Document number:	S5eCPM0135

# Source:	Vinay Devadatta, WIPRO

# Title:		Update of PM2"Metadefinitions for Measurements and KPIs"

# Meeting date/time:	Thursday 15th  January  2015

Introduction

This contribution provides an update for collection method types based on version 0.12 of PM2.

Fixed Mobile Convergence (FMC)

Converged Performance Management

PM2: Metadefinitions for Measurements and KPIs

Version 0.13 (draft)

Source: M-SDO Converged PM project

Editor: Christian Toche (Huawei Technologies)

Table of Contents

1	Scope	4

2	References	4

3	Definitions and abbreviations	4

3.1	Definitions	4

3.2	Abbreviations	5

4	Requirements	6

5	Definition of Performance Measurement Metadata	6

Annex A (Informative): Counters/KPIs properties	11

A1	3GPP Performance Metadata	11

A2	TM Forum Performance Metadata	11

A3	Proposed JWG Measurement Profile	12

# 1	Scope

This document defines the metadata for Performance Management (PM) performance measurements (KPIs/counters). The definition would enforce consistency in description/definition of all standardized performance measurements (counters/KPIs) by various SDOs and organizations (or Multi-SDO) involved in the Management of Converged Networks [1,2]. This consistency would facilitate encoding and decoding of performance measurements (counters/KPIs) exchanged (via management protocols) and/or stored in (database or files).

The metadata related to PM operations/notifications is not part of this PM measurement metadata.

The metadata would be published as part of the Model Repertoire [3]. TBD.

# 2	References

- 3GPP TR 32.833 Study on Management of Converged Networks

- TM Forum TR 166 Information Model Federation Concepts and Principles (http://collab.tmforum.org/sf/go/doc13634?nav=1)

- Fixed Mobile Convergence (FMC) Model Repertoire V5.x

- IETF RFC 2578 Structure of Management Information Version 2 (SMIv2)

- 3GPP TS 32.401 V11.0.0 (2012-09) Performance Management (PM); Concept and requirements

- PM8:Description of CollectionMethodTypes, Measurement DataTypes and List of Formulas

# 3	Definitions and abbreviations

For the purposes of this document, the following definitions, symbols and abbreviations apply. For definitions, symbols and abbreviations not found here. See also Fixed Mobile Convergence (FMC) Federated Network Information Model (FNIM) [3].

## 3.1	Definitions

Indicator/Measurement: tba

Granularity period: tba

Recording Interval : tba

Reporting Period: tba

## 3.2	Abbreviations

ATM	Asynchronous Transfer Mode

ATML	?

ATT	?

CC	Cumulative Counter

DER	Discrete Event Register

FNIM	Federated Network Information Model

FMC	Fixed Mobile Convergence

G	?

GSM	Global System for Mobile

HHO	?

HO	?

ID	Identifier

IETF	Internet Engineering Task Force

IP	Internet Protocol

JWG	Joint Working Group

KPI	Key Performance Indicator

MIB	Management Information Base

MM	?

OS	?

PM	Performance Management

RAB	?

RL	?

RLM	?

RRC	?

SAEB	?

SDO	Standards Development Organisations (e.g., 3GPP, TM Forum)

Sgsn	?

SI		Status Inspection

SMS	?

TBA	To Be Added

TBC	To Be Checked

TBD	To Be Defined

U	?

UMTS	Universal Mobile Telecommunications System

UTRAN	UMTS Terrestrial Radio Access Network

VS	?

# 4	Requirements

Metadata is defined as data providing information about aspects of other data. Metadata can be exchanged in protocols, and can be stored and managed in a database, often called a Metadata registry or Metadata repository. Metadata can be defined in machine-readable syntax.

The PM performance measurement metadata is the properties of the performance measurements (counters/KPIs) including their associations to other performance measurements (counters/KPIs). The number of properties and the level of the details in the property definition depend on the metadata intended usage.

The purpose for defining the PM performance measurement metadata is to enforce a consistency in description/definition of all standardized counters/KPIs. This consistency facilitates encoding, decoding, interpreting and in general, processing of counters/KPIs defined by Multi-SDOs involved in the Management of Converged Networks [1, 2].

# 5	Definition of Performance Measurement Metadata

The following table provides the PM performance measurement metadata; i.e., the definition of the properties.

When defining a new measurement and when this new measurement is similar to a measurement already defined in another standard, then this standard should be referenced and only the missing properties should be defined in addition.

| Converged Property name | Description | Legal values | Qualifier |
| --- | --- | --- | --- |
| name e.g, ErroredSeconds | Upper Camel Case. | String | Mandatory |
| GEN::documentation | This property contains a textual description of this modelled element. Should refer (to enable traceability) to a specific requirement. | String | Mandatory |
| Counter::collectionMethodType | This property indicates the form in which this measurement data is obtained. COUNTER This literal defines a Collection Method using a variable whose value can only be incremented and which is reset at the beginning of each granularity period to well defined value (usually “0”). In 3GPP 32.401 [5] it is called Cumulative counter (CC). Refer to [6] for description and further details. CUMULATIVE This literal defines a Collection Method using a variable whose value can only be incremented and which wraps around on reaching maximum possible value that the variable can attain In IETF RFC 2578 [4] it is called Counter32/Counter64. ). Refer to [6] for description and further details. GAUGE This literal defines a Collection Method using a dynamic variable whose value can be incremented or decremented, and which may be reset at the beginning of a granularity period or recording interval ( multiple consecutive granularity periods) See 3GPP 32.401 [5]. ). Refer to [6] for description and further details. DISCRETE_EVENT This literal defines a collection method (using a set of appropriate data containers) where data related to a particular event type is captured and every nth event is registered, where n ≥ 1. The captured information is reset at the beginning of each granularity period. In 3GPP 32.401 [5] it is called Discrete Event Registration (DER). ). Refer to [6] for description and further details. STATUS_INSPECTION This literal defines a Collection Method that uses a mechanism for high frequency samplings of internal counters at predefined rates and captured using appropriate data type. The captured information is reset at the beginning of each granularity period. Please see 3GPP 32.401 [5]. Refer to [6] for description and further details. The differences between theses Collection Methods and guidelines for usage of the same is explained in [6] | COUNTER, CUMULATIVE, GAUGE, DISCRETE_EVENT, STATUS_INSPECTION | Mandatory |
| unit | This property defines the unit by which the measurement is measured. For example: seconds, Celsius, Kelvin, Ampere, Volt, Watt, dB, kBit/s, rate per second, etc. | String | Mandatory |
| condition | This property indicates the condition which causes the measurement result data to be updated. This will be defined for example by identifying protocol related trigger events for starting and stopping measurement processes, or updating the current measurement result value. | String | Optional |
| sourceStandard | This property indentifies the referenced standard. All the other properties (e.g., collectionMethodType) of this measurement must refer to the referenced standard if defined there. In case the definition of this measurement conflicts with the referenced definition, the referenced definition takes precedence. | String | Mandatory |
| measurementFamily | This property identifies the measurement family (e.g. HHO, RAB, SMS). Note that this family may also be used for measurement administration purpose. Note: when defining a new family name, consult the family names defined by other SDOs so that all the names are unique. It is not allowed to define a new family with the same semantics of an already existing family. | String | Mandatory |
| measurementName | This property captures the SDO agnostic measurement name (same name among several SDOs). The name does not carry the measurement family name. | String | Optional |
| sdoAliasNameList | This property captures the list of SDO specific measurement names. The measurement name convention is SDO specific. | String | Optional |
| measurementDataType | This property identifies the type of the value of the measurement, such as Integer, Float, String, or derived value types like a fixed array of values,(Vector) etc., | Integer, Float, String, Also please refer to [6] for details of possible derived value types, | Mandatory |
| measurementDataRange (O) | This property specifies the allowed range of values for a measurement |  | Optional |
| monitoredObjectClassList | This property indicates the list of <<InformationObjectClass>> names to which this measurement is related. | Any defined <<InformationObjectClass>>, e.g. UtranCell, SgsnFunction. | Optional |
| KPI::formulaDescription | This property contains a textual description of the KPI::formulaSpecification. | String | Mandatory |
| KPI::formulaSpecification | This property contains the formula. | String | Mandatory |
| Counter::collectionFormula | This property indicates the formula that may be applied to whole or part of information collected (and optionally processed) using any of the specified Collection Method Types (Counter::collectionMethodType). In practice this is applicable when the collected information consists of multiple values. These formulas are standard statistical functions which reduce a set of values to single indicative value based on type of function applied. Refer to [6] for description of the formulas | SUM, COUNT, MIN, MAX, AVERAGE, FIRST, LAST, PRODUCT, STDEV_SAMPLE, STDEV_POPULATION, VARIANCE_SAMPLE, VARIANCE_POPULATION, MEDIAN, MODE.SNGL, LARGE, SMALL, PERCENTILE.INC, QUARTILE.INC, PERCENTILE.EXC, QUARTILE.EXC OTHER | Optional |
| Counter::collectionFormulaParameter | This property indicates the parameter in case of LARGE, SMALL, PERCENTILE formulae (Counter::Formula). This parameter will indicate different type of value based on formula type. Refer to [6] for details of its usage in different formulas. | Integer | Conditional Mandatory |
| Counter::otherCollectionFormula | This property indicates the formula of a measurement/Counter where the value of “OTHER”. This property is only relevant if Counter::collectionFormula has the value “OTHER”. | String | Conditional Mandatory |
| Counter::samplingInterval | This property indicates the sampling interval of a “collection method type in the case where the collection is carried out by sampling the target at fixed intervals during a granularity period. This is valid for SI and in some cases Gauge Collection Methods. | Integer | Optional |
| Counter:samplingRate | This property indicates the sampling of every Nth event of a specified event type to be observed of those occurring in a granularity period. | Integer; Used in case of DER Collection Method | Conditional Mandatory |
| Counter:processingProperty | This property describes the processing to be carried out on the data collected using a Collection Method. This is valid for DER and SI Collection Methods.In case it is not specified no processing needs to be carried out and appropriate formula ( Counter:collectionFormula) might be applied | String | Optional |

# Annex A (Informative): Counters/KPIs properties

This annex lists the 3GPP currently defined properties for its performance measurements and the TM Forum defined properties for its “indicator” (model for both counters and KPIs).

## A1	3GPP Performance Metadata

3GPP TS 32.40X series is using the following properties for the Measurement Definition Template to describe the counters that are passed over the northbound interface:

- Measurement Name

- Description

- Collection Method

- Condition (when/how is it generated)

- Measurement Value + Unit

- Measurement Type (short name)

- Measurement Object Instance

- Switching Technology

- Radio Technology (?)

The following properties are used for KPIs:

- Long Name

- Description

- Logical Formula Definition

- Physical Formula Definition

- Measurement names used for the KPI

- KPI Object

- KPI category

- Unit of the KPI

- Type of the KPI (Mean, Ratio, Cum)

- Remark

## A2	TM Forum Performance Metadata

TM Forum defines a single entity “Indicator” for modelling both counters and KPIs. Following are the “Indicator Specification” attributes (A SID Metadata entity):

- ID

- Name

- Description

- Perspective

- IndicatorCategory

- Indicator Type

- derivarionAlgorithm

- derivationMethod

- CollectionType

- IndicatorUnit

An Indicator Specification can be attached to:

- A monitored object

- A counter group

- A Counter category

- A Production/Collection job

TIP PM Interface is exposing this data over the interface. A PM consuming application can request the information using the following operations:

- getSupportedObjectClasses

- getSupportedObjects

- getSupportedIndicatorGroups

- getSupportedIndicators

- getSupportedTransport

- getSupportedFileFormat

- getCommonIndicatorGroupsGranularities

- getCommonIndicatorsGranularities

## A3	Proposed JWG Measurement Profile

The converged metadata defined in the previous section are defined in a UML profile to use them as templates for defining measurement instances.

Figure 1: JWG Measurement Profile

The profile is associated to the Class metaclass on a case by case basis. This allows a model designer to define individual measurement instances. See an example below:

Figure 2: Example JWG Measurement Definition

3GPP is a trade mark of ETSI