# Multi-SDO Project on Converged Management PM Interface Definitions

# Document number:	S5eCPM0142 (revision of S5eCPM0140)

# Source:	Vinay Devadatta, Wipro (TM Forum)

# Title:		Performance Information Augmentation Metadata

# Meeting date:	2015-05-12

# Objective

# During the discussions on 2015-01-15, the need to augment performance data with additional information was felt. This augmenting information is not performance measurement, but is used to enhance its usability.  This document proposes a metadata for this type of information which may be incorporated in the PM2 document. This contribution provides an update for definition of “Counter:augmentationInformation” property type based on discussions

# References

# Following is a list of references:

- S5eCPM0128 M-SDO String values of Counters

- Fixed Mobile Convergence (FMC) Converged Performance Management PM2: Metadefinitions for Measurements and KPIs Version 0.14

# Detailed Proposal

# The following entry is proposed in meta data definition table in  Section 5 “Definition of Performance Measurement Metadata” of PM2  document.

| Converged Property name | Description | Legal values | Mandatory(M)/Optional(O)/Canditionaly Mandatory(CM) |
| --- | --- | --- | --- |
| Counter:augmentationInformation | This property captures measurement information which is not performance information but can be used to augment the performance Measurement data. This property may be specified at the counter level or at group of counters level | This property can have different types depending on the information content relevant in a specification.( example – Integer, Real, String, set of Name Value pairs,enumeration etc. ). The specification of this property must include the exact type of this information. Examples Integer : Number of IoT respondents Real: Temperature String : Geolocation Enmeration Type: Operation Status | CM This property must be specified along with the specific information type to be used legally - if this information is necessary for understanding of the related performance information and/or - if it disambiguates the related performance information. |

# Recommendation

form change history:

v1.13.1: minor changes resulting from discussions at CT#41 & SA#41

v1.13.0: mods to enforce linkage amongst stages 1, 2, 3

draft mods Scarrone-Meredith 2008-07 ff

v1.12.1: removes revision marks following approval at SP-29
v1.12.0: includes provision for Study Items (SP-29)

v1.11.0: includes those changes from v1.8.0 agreed at SP-25.

v1.10.0: full circle

v1.9.0: a clean sheet

v1.8.0: includes comments from SA#24

v1.7.0: includes comments from RAN, CN and T #24; also includes “early implementation” data

v1.6.0: includes comments made during review period prior to TSGs#24

v1.5.0: includes comments made at TSGs#23 (Phoenix)

v1.4.0: offered to SA#23 for approval

v1.3.0: offered to CN#23, RAN#23 and T#23 for comments

DRAFT4 v1.3.0: 2004-03-09: Incorporation of comments from Leaders list

DRAFT3 v1.3.0: 2004-02-19: Incorporation of comments from MCC members

DRAFT2 v1.3.0: 2004-01-29: Complete redraft:

v1.2.0: 2002-07-04: "USIM" box changed to "UICC apps"

2003-05-28: spelling of “rapporteur” corrected

2002-07-04: "USIM" box changed to "UICC apps"