3GPP TSG-SA5 (Telecom Management)	S5eHNB---

3GPPSA5-Virtual Meeting on HeNB

- Source:	Verizon

- Title:	High level parameter categories and justification for HeNB

- Document for:	Discussion and Approval

Agenda Item:

# 1	Decision/action requested

The group is asked to discuss the proposed high level parameter categories, and introduce the agreed part into the draft TS 32.592.

# 2	References

[1] 3GPP TS 32.582

[2] 3GPP TS 32.592

[3] TR-196

# 3	Rationale

The Home ENodeB is expected to be a complex entity when compared to a typical home router. The Home eNodeB being an entity in a subscriber’s home is expected to provide atleast the same level of reliability and service availability as a device typically found in a networked home. (E.g. a Broadband home router, DSL modem etc). Thus, from a service provider’s point of view, the Home eNodeB entity deployment requires the remote management of the device beyond the typical configuration and black box treatment. In other words, a service provider expects the remote management of a Home eNodeB be comparable to currently deployed managed networked devices in homes.

The typical Home eNodeB is expected to have multiple sub-systems that work together. This document proposes that the level of detail in the parameter categories include a generic system and sub-system level information that can be used by service providers.

Typical FemtoCells have multiple HW/SW subsystems and SW stacks on the HeNodeB. Service providers are particularly interested in information at a sub-system level of home networked devices. Hence the parameters in the info model should provide the ability for an insight into the system and subsystems of a Home eNodeB.

The standardization of these parameters is important to service providers from a multi-vendor interoperability perspective.

The high level parameter categories are illustrated in the diagram below

# 4	High level parameter categories

The following high level parameter categories are proposed

End of Section