| 3GPP - TM Forum Model Alignment Study |  | Document: S5vTMFa211 |
| --- | --- | --- |
| Meeting date: 30 Nov - 2 Dec 2011 |  |  |
| Author: | Nigel Davis, Ciena. |  |
| Title: | Comments on Umbrella (202) |  |

Introduction

This document provides comments on “S5vTMFa202 FNM Umbrella V1.5”. This document is an update of and supersedes “S5vTMFa194”. The document accounts for all comments made during the face to face meeting in Nanjing (17-19 October 2011) and includes comments and mark-ups that were not yet agreed. There are also additional comments on the version 1.5 Umbrella.

Specific Comments

The following sections are headed with the title or aspect of S5vTMFa202 and provide a brief description of the intent and explanation in italics followed by necessary adjustment in normal type. This pattern of intent in italics followed by detail adjustment is repeated through each section as appropriate.

Margin comments have been used to provide direction to the editor of S5vTMFa202 on changes required to that document.

Note that some of the changes identified in this document are dependent on a decision on the concept of “Inventory”.

It is also proposed that during the Darmstadt meeting the agreements be captured in a UML model that can be shared by the participating parties.

Comments

Extract of existing text with comments etc

4.	Umbrella

The wording “Umbrella classes are partitioned.  Classes in “Partition operational” are used to capture run-time behaviour of managed resources.  Classes in “Partition inventory” are used to capture static behaviour of managed resources.” is not accepted. There is no reason to partition the classes in this way. Indeed such a partition will be likely to lead to unnecessary churn. It is proposed that the attributes of more coherent classes be marked with their expected lifecycle behavior with respect to the viewpoint of the interface concerned and those that are read only simply be marked as such. It is reasonable for any organization to rearrange the classes as they see fit to suit their local mechanism. See new contribution S5vTMFa191-CommentsOnInventory.

It is proposed that the wording be changed to:

“Converged classes are arranged in the model based upon the domain of concern and the degree of abstraction (principles, patterns, architectures, purpose specific views). A class may have attributes that are read only and read/write. The lifecycle of each attribute will be stated (including the change rationale from immutable through complex state machines to free form).”

4.1.1	Class diagram

Clearly the entities on the class diagram need to be aligned with the agreements on the text sections. In addition:

- The SubNetworkConnection is not required in the Use Case and can hence be removed.

- The TerminationPoint should be renamed as TerminationPointEncapsulation and should be shown contained in ManagedElement.

- The class “LayerTermination” should be added (as per discussion below) and should be contained in TerminationPointEncapsulation.

- ManagedElement, ManagementSystem and TopologicalLink should be able to stand alone not being contained in a SubNetwork as an option.

Note that the term “concrete” has been removed here due to the implication of the following (from Wikipedia) “An abstract class … is a class that cannot be instantiated. Such a class is only meaningful if the language supports inheritance.” as it has been agreed that inheritance is not the (only) adoption mechanism. Indeed there is no reason why a body could not adopt a class and use it directly. A class may be adopted as concrete or abstract. Whether the class is concrete or abstract is up to the user. It is accepted however that areas such as naming may cause the need for local refinement of naming rules but even here mechanisms other than basic inheritance may be chosen. For example JOSIF have chosen an injection approach.

The editor’s notes in this section do not convey sufficiently the concern raised in Nanjing that there may be a perception that the classes identified are all that is needed. It is suggested that a stronger text be constructed prior to delivery.

The editor’s note is as follows:

“

Editor Notes:

- UML classes above are not sufficient for FNM NM purposes.  Their class names may not be appropriate.

- These Umbrella classes are for inheritance only.

- Other Umbrella classes may be used for relation other than inheritance.

“

It is suggested that the note be refined to the following:

“

Editor Notes:

- The Umbrella model described in this document provides the set of classes etc that have been agreed for convergence to strengthen consistency of representation in the fixed and mobile environment. For full management of an FNM solution many other classes will be required in addition to those in the Umbrella.

- The Umbrella model cannot be used directly for implementation. Implementation classes must be derived from the Umbrella by Inheritance or some other appropriate mechanism.

- The model for the fixed environment and the model for the mobile environment must use different names for classes etc from those described in the Umbrella.

- Where an implementation class is essentially identical to the abstract class described in the Umbrella the name of the implementation class should be the same as that of the Umbrella class minus the underscore, e.g. the Umbrella class “Function_” would become “Function”.

“

4.1.2.1	SubNetwork_

The alternative name “NetworkDomain” is proposed.

##### 4.1.2.1.1	Definition

It is noted that “a logical grouping or partition of managed resources” is NOT a network unless they are in some way interrelated. There may be a need for an additional class to perform this function. The NetworkDomain is potentially NOT necessary for the Use Case (depending upon the containment decision above).

##### 4.1.2.1.2	Attributes

Notes against the attribute table at the end of this document identify specific concerns.

4.1.2.2	ManagedElement_

##### 4.1.2.2.1	Definition

May want to explore the concept of “ManagementScope” rather than stick with the traditional ManagedElement. This may be too challenging for the short timeframe we have left so perhaps we simply will mark ManagedElement as “likelyToChange”. Note especially that ManagedElements are often distributed and hence do NOT have a single location. It is the equipment that they provide access to that has the location.

It is proposed that the following note be added:

“Note that this model reflects the traditional concept of ManagedElement. Advancements in network technology that lead to both greater integration and extensive virtualization are likely to lead to a need to revisit this model in the near future.”

##### 4.1.2.2.2	Attributes

Notes against the attribute table at the end of this document identify specific concerns.

4.1.2.5	TopologicalLink_

This is a key class for convergence as part of the assessed case in S5vTMFa169 3GPP/TMF Concrete Model Relationships and Use Cases, V1.0 and considered partly in S5vTMFa175 modeling client/server layer objects for transport network CPE .

Need to review the definition especially the terms “communication link”.

4.1.2.5.1	Definition

It is proposed that the definition be changed from “This represents a communication link between two network entities. This does not indicate whether the represented communication link is a physical or logical entity.” To:

“The TopologicalLink represents a communication relationship (forwarding relationship) between two network entities and indicates that traffic is intended to flow “directly” between those two network entities. The layer (or protocol) of the intended flow is provided via an attribute of the TopologicalLink. The TopologicalLink always represented logical relationship.”

Note that the TL is always logical as even when in the lowest layer of concern no physical parameters are provided.

It is proposed that the comments “This is defined for sub-classing purposes” should be removed as this is as true for all classes and should be covered by a general statement.

It is proposed that the following note be added:

“Note: There are various scenarios beyond the initial use case where multiple entities are involved in a single multi-pointed forwarding relationship. This is for further study. It is possible that a multi-pointed solution will supersede this basic solution.”

##### 4.1.2.5.2	Attributes

Notes against the attribute table at the end of this document identify specific concerns.

It is proposed that the following attributes be added.

| Attribute Name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| encapsulatedResilience | M | M | - |
| encapsulatedStructure | M | M | M |
| layerRate | M | M | - |

4.1.2.6	TerminationPoint_

This is the key class for the convergence as part of the assessed case in S5vTMFa169 3GPP/TMF Concrete Model Relationships and Use Cases, V1.0 and considered partly in S5vTMFa175 modeling client/server layer objects for transport network CPE.

Propose the following name change and definition:

Proposal for the Umbrella

Name: TerminationPointEncapsulation (TPE)

4.1.2.6.1	Definition

The following definition is proposed.

“A TerminationPointEncapsulation (TPE) represents one or more functions that terminate/originate a signal, that adapt a signal for use and that enable a signal to propagate. Hence a TPE can represent the end point of a signal flow or the intermediate point of a signal flow.

A TPE is capable of encapsulating multiple transport functions (G.805 termination functions, adapters, points etc) at many different layers where the encapsulated transport functions are all related to the same signal flow. There are specific rules that guide encapsulation (see usage guide). The encapsulated layers may be exposed via LayerTerminations.

The TPE deals equivalently with unidirectional and bidirectional flows (where pairs of unidirectional flows have some shared fate or are considered as related in some way) such that all entities associated with the whole bidirectional flow will be encapsulated in one TPE. Where a bidirectional flow is encapsulated it is possible to connect to only one of the two directions of flow and this can be represented through parameters of the TPE.”

4.1.2.6.2	Attributes

| Attribute Name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| direction | M | M | - |
| tpeType | M | M | - |
| parameters | O | M | O |

It is proposed that further sections be added. The text in these sections was extracted from text provided in the previous document version under the heading “description” and agreed as beyond description.

“4.1.2.6.3	Rationale

The encapsulation of the TPE is performed to both reduce the instances of objects required to represent a given transport assembly and to also simplify the translation from traditional environments where layering is not fully represented. The encapsulation may be opaque, not exposing the layering, or semi-transparent, exposing the explicit layering but compacted into a single instance. Where the encapsulation is semi-transparent the management system client and/or management system server can potentially expand the model back to fully layered as desired. In the semi-transparent case the TerminationPointEncapsulation is composed of many “Layer Terminations”.

4.1.2.6.4	Usage

The TerminatioPointEncapsulation provides a place against which to raise alarms, display parameters and set attributes associated with the signal flow. Where the encapsulation is opaque the definition of the parameters etc will need to be such as to distinguish the encapsulated protocols/layers. The TerminationPointEncapsulation can carry generalized parameters such as name and userLabel.

The TerminationPointEncapsulation can be related:

- Directly to one or more physical ports (i.e. that the signal is associated directly with an externally visible connector)

  - Note that a physical port could also be related to more than one TerminationPointEncapsulation

- To a logical functions that anchor the signal flow (i.e. it is floating between flexible functions in the equipment with no externally visible connector).

- To another TerminationPointEncapsulation to represent a client signal of the supporting TerminationPointEncapsulation where there may be many instances of client.

  - Note that there may be many instances of server TerminationPointEncapsulation that feed a single client (e,g, in the case of VCAT)

The intention is that this class be used directly or with minimal sub-classing, i.e. NOT per port type, however it is recommended that an attribute that represents the port type specification is filled out with a specification value.

For further background see SD1-18 Functional Modelling Concepts and naming refer to SD1-25 Object Naming.”

Additional classes

It is proposed that the following classes and associated text be added to the model.

4.1.2.x	LayerTermination_

4.1.2.x.1	Definition

The LayerTermination encapsulates the functions and points associated with one instance of a layer (ITU-T xxx). The functions include the adapter functions , the termination functions and the connection points of that layer. In this case the term layer is essentially synonymous with the term protocol as use by other standards. All functions encapsulated have the same signal granularity, closely associated characteristic type and essential rate. A specific LayerTermination may be equipped with a subset of capabilities. Where the TPE is semi-transparent the layers encapsulated by a TPE are exposed by the LayerTermination set that it contains.

4.1.2.x.2	Attributes

| Attribute Name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| layer | M | M | - |
| direction | M | M | - |
| ltType | M | M | - |
| terminationState | M | M | O |
| connectionState | M | M | - |
| Index | O | M | - |
| Parameters | O | M | O |

4.1.2.x.3	Rationale

The LayerTermination provides the relevant layer parameters for the semi-transparent TPE cases. Exposure of LayerTerminations is optional.

4.1.2.x.4	Usage

The LayerTermination allows for detailed layer description of a TerminationPointEncapsulation (potentially representing a port) and for precise association of the TerminationPointEncapsulation with a TopologicalLink (or other representatives of forwarding relationship).

4.3	Class attribute definitions

The following section includes the original table with word comments highlighting concerns. A list of new attributes added in classes above is provided after the commented table.

4.3.1	Definitions and legal values

| Attribute Name | Definition | Legal Values |
| --- | --- | --- |
| aEnd | The value of this attribute shall be the Distinguished Name of the alphabetically first instance in the Link subclass name to which this link/relation is associated (i.e., pointing to the instance of <X> as described in the definition of Link IOC in the present document). As an example, with Link_As_Slf, aEnd would contain the Distinguished Name of the AsFunction instance, and the zEnd would contain the Distinguished Name of SlfFunction instance. | Values to be conformant with TS 32.300 [3] |
| dnPrefix | It carries the DN Prefix information as defined in Annex C of 32.300 [2] or no information. |  |
| Id | An attribute whose class name and value can be used as an RDN when naming an instance of the object class. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. | Values to be conformant with TS 32.300 [2] |
| managedElementType | The type of managed element. It is a multi-valued attribute with one or more unique elements. Thus, it may represent one ME functionality or a combination of more than one functionality. The actual syntax and encoding of this attribute is Solution Set specific. | The legal values of this attribute are the names of the IOC(s) that are (a) derived/subclassed from ManagedFunction and (b) directly name-contained by ManagedElement IOC (on the first level below ManagedElement), but with the string “Function” excluded. If a ManagedElement contains multiple instances of a ManagedFunction this attribute will not contain repeated values. The capitalisation (usage of upper/lower case) of characters in this attribute is insignificant. Thus, the IRPManager should be case insensitive when reading these values. Two examples of legal values are: - NodeB; - HLR,VLR. |
| linkType | This attribute defines the type of the link. | Signalling, Bearer, OAM&P, Other or multiple combinations of the above types. |
| locationName | The physical location of this entity (e.g. an address). |  |
| managedElements | Models the role Manager. This attribute contains a list of the DN(s) of the related ManagedElement instance(s). |  |
| managedBy | Models the role subordinate. This attribute contains a list of the DN(s) of the related ManagementNode instance(s). |  |
| protocolName | Name(s) and additional descriptive information for the protocol(s) used for the associated communication link. Syntax and semantic is not specified. |  |
| protocolVersion | Versions(s) and additional descriptive information for the protocol(s) used for the associated communication link. Syntax and semantic is not specified. |  |
| userDefinedNetworkType | Textual information regarding the type of network, e.g. UTRAN. |  |
| userLabel | A user-friendly (and user assignable) name of this object. |  |
| zEnd | The value of this attribute shall be the Distinguished Name of the alphabetically second instance in the Link subclass name to which this link/relation is associated (i.e., pointing to the instance of <Y> as described in the definition of Link IOC in the present document). As an example, with Link_As_Slf, aEnd would contain the Distinguished Name of the AsFunction instance, and the zEnd would contain the Distinguished Name of SlfFunction instance. | Values to be conformant with TS 32.300 [3] |

Additional attributes proposed.

| Attribute Name | Definition | Legal Values |
| --- | --- | --- |
| direction | Client-Server, Server-Client, Bidirectional |  |
| ltType | Name of specification of construction of the LT (CP only…) |  |
| tpeType | Name of specification of construction of the TPE |  |
| terminationState | Indicates whether the Termination function is bound to the connection point or not in c-s, s-c or bi |  |
| connectionState | Indicates whether the LayerTermination is sink, source or bi connected |  |
| index | Provides any relevant indexing of the TP (channel number etc) |  |
| parameters | A generalized list of attribute opportunities |  |
| encapsulatedResilience | An abstraction of the underlying protection of the TopologicalLink |  |
| encapsulatedStructure | Essentially the spec of the TopologicalLink |  |

End of specific comments

END OF DOCUMENT

There will be NO partition of static and dynamic attributes. Properties will be reflected by attribute decoration.