Fixed Mobile Convergence (FMC)

Federated Network Information Model (FNIM)

Umbrella Information Model (UIM)

Version 2.2

Source: Joint 3GPP/TM Forum Model Alignment project

Editor: Edwin Tse (Ericsson)

Disclaimer: Final agreement on this document cannot occur until “Governance / Terms of Reference / Working Procedures” agreements between the involved organizations (including 3GPP & TMF) have been established.

Table of Contents

1	Scope	4

2	References	4

3	Definitions and abbreviations	5

3.1	Definitions	5

3.2	Symbols and abbreviations	5

4	UIM – Partition operational	5

4.1	Class diagram	6

4.2	Class definition	7

4.2.1	Domain_	7

4.2.2	ManagedElement_	7

4.2.3	Function_	8

4.2.4	ManagementSystem_	8

4.2.5	TopologicalLink_	8

4.2.6	TerminationPointEncapsulation_	9

4.2.7	LayerTermination_	9

5	UIM – Partition inventory	10

6	UIM – Class attribute definitions	11

6.1	Definitions and legal values	11

Annex A (informative): Extract from 3GPP TS 32.622 Generic network resources IRP: NRM [5]	13

Annex B (informative): Extract from MTOSI documents	14

Annex C (informative): Rationale and Usage of TPE/LT	19

Figures

Figure 1: UML classes	6

Figure 2: Containment tree of 3GPP TS 32.622 Generic network resources IRP: NRM [4]	13

Figure 3: MTOSI/MTNM Containment	14

Figure 4: MTOSI/MTNM Connection-oriented Class Diagram	15

Figure 5: MTOSI/MTNM Connectionless Class Diagram	16

Figure 6: MTOSI/MTNM Termination Point Model	17

Figure 7: MTOSI/MTNM Inheritance Class Diagram	17

Figure 8: MTOSI/MTNM Inheritance Class Diagram with Attributes	18

Figure 9: UIM related to TM Forum model and ITU-T concepts	20

# Scope

As a result of the analysis of the requirements for the harmonization of the 3GPP and TMF Information Models in the context of FMC basic use cases were developed [6], [14]. These use cases led to the recognition that it would be necessary to define common model elements applicable for wire-line and wireless networks. This document defines these common model elements.

To be noted:

- The Umbrella Information Model (UIM) described in this document provides the set of classes etc. that have been agreed for convergence to strengthen consistency of representation in the fixed and mobile environments. For full management of an FNM solution many other classes will be required in addition to those in the UIM.

- The UIM is necessary but not sufficient for implementation.

- The UIM cannot be used directly for implementation. Implementation classes must be derived from those in the UIM by Inheritance or some other appropriate mechanism.

- Classes derived from those in the UIM (e.g. for the fixed environment) must use different names from those used in the UIM.

- Where an implementation class is essentially identical to the abstract class defined in UIM the name of the implementation class should be the same as that of the UIM minus the underscore, e.g. the UIM class “Function_” would become “Function”.

# References

- ATM Forum, Technical Committee, Network Management, M4 Network View CMIP MIB Specification, CMIP Specification for the M4 Interface, Sep, 1995

- 3GPP TS 32.xyz series on NRM

- 3GPP TS 32.300 Telecommunication management; Configuration Management; Name convention for Managed Objects

- 3GPP S5-102610 S5vTMFa033 E NSN Proposed enhancement of Generic NRM IOCs v3

- 3GPP TS 32.622 Generic network resources IRP: NRM

- 3GPP TR 32.833 Study on Management of Converged Networks (3GPP SA5)

- TM Forum GB922, Information Framework (SID) Suite, Release 9.0 (http://www.tmforum.org/browse.aspx?catID=9285&artf=artf2048)

- TM Forum MTOSI 2.0 (http://www.tmforum.org/MTOSIRelease20/MTOSISolutionSuite/35252/article.html)

- TM Forum SD1-44_ConnectionlessTechnologyManagement.pdf available as part of [8] (Especially Appendix III Mapping MEF – MTNMETH)

- TM Forum SD1-7_DSLOverview.pdf available as part of [8]

- TM Forum SD1-18_layers.pdf available as part of [8] (Especially 4.2.7 ATM and SDH capable STM-4)

- TM Forum Connectionless, Connection Oriented Convergence and TP Modelling (http://tmforum.org/FeatureDescription/ConnectionlessConnection/41718/article.html)

- TM Forum TR 146 Lifecycle Compatibility Release 1.0 (http://www.tmforum.org/TechnicalReports/TR146LifecycleCompatibility/36664/article.html)

- TM Forum TR 166 Information Model Federation Concepts and Principles (http://collab.tmforum.org/sf/go/doc13634?nav=1)

- Fixed Mobile Convergence (FMC) Federated Network Information Model (FNIM)

- TM Forum MTOSI SD1-25_objectNaming.doc

- ITU-T X.200 (07/1994) Information technology – Open Systems Interconnection – Basic Reference Model: The basic model

# Definitions and abbreviations

For the purposes of this document, the following definitions, symbols and abbreviations apply. For definitions, symbols and abbreviations not found here. See also Fixed Mobile Convergence (FMC) Federated Network Information Model (FNIM) [15].

## Definitions

None

## Symbols and abbreviations

FNIM	Federated Network Information Model

FNM	Federated Network Model

FMC	Fixed Mobile Convergent

LT	Layer Termination

ME	Managed Element

NM	Network Management

NW	Network

TPE	Termination Point Encapsulation

# UIM – Partition operational

This section introduces a number of classes that form the UIM. These classes are represented in UML and are implementation neutral views in that they only capture the semantics of the model from both a purpose neutral and purpose specific perspective.  They do not:

a) Include syntax or representation of the information in a system or on-the-wire between systems

b) Relate to the protocol used to create/delete/read/write/modify the NM information.

Various SDOs and organizations will use the UIM classes for definition of Domain/Technology-specific model classes. This procedure will maximize the probability of the domain/technology specific concrete classes (from various SDOs) being semantically consistent, a necessary characteristic for FMC NM purposes.

Currently, 3GPP SA5 defines its abstract classes in its Generic NRM IRP [5] and defines concrete classes in other NRM IRPs such as EUTRAN NRM IRP.  3GPP/SA5 have agreement and will consider changes in classes in its Generic NRM IRP such that they are harmonized (if not identical) to those (to-be-defined) as UIM classes. Classes from other IRPs that are relevant for consideration in the UIM are likely to be reflected in some way in the Generic NRM IRP for use in the UIM.

## Class diagram

The criteria for choosing these classes is their relevance to (e.g. can be used by) Domain/Technology-specific model classes (e.g. 3GPP/SA5 network resource model [2], BBF ATM network management model [1], TMF MTNM, DMTF CIM)”

Note that this set of classes is basic in that their definitions and usage are necessary for the harmonization of various Domain/Technology-specific model classes, forming the so-called FNIM.

These classes are abstract.  Other classes are for further study.

- Domain_ (similar to SubNetwork of 3GPP [5] and MultiLayerSubNetwork of SID/MTOSI [7])

- ManagedElement_ (similar to ManagedElement of 3GPP [5] and SID/MTOSI [7])

- Function_ (similar to ManagedFunction of 3GPP [5] and LogicalResource of SID/MTOSI [7])

- ManagementSystem_ (similar to ManagementNode of 3GPP [5] and OperationsSystem of SID/MTOSI [7])

- TopologicalLink_ (similar to Link [5] of 3GPP and TopologicalLink of SID/MTOSI [7])

- TerminationPointEncapsulation_ (similar to TerminationPoint of SID/MTOSI [7])

- LayerTermination_ (similar to a single layer in the layerParameterList_T structure of SID/MTOSI [7])

Editor’s note: How to tie the naming tree (below left) with the main naming tree is FFS.

Figure 1: UML classes

## Class definition

### Domain_

#### Definition

This groups managed entities:

- Such that the group represents a topological structure which describes the potential for connectivity;

- Subject to common administration;

- With common characteristics.

A domain is a partition of instances of managed entites.

#### 4.2.1.2	Attributes

| Attribute Name | Support Qualifier | Read Qualifier | Write Qualifier | isInvariant Qualifier |
| --- | --- | --- | --- | --- |
| id | M | M, T | - | M, T |
| dnPrefix | M | M, T | - | M, F |
| userLabel | M | M, T | M, T | M, F |
| userDefinedNetworkType | M | M, T | - | M, F |

### ManagedElement_

#### 4.2.2.1	Definition

This (and its contained Function_(s)) represents telecommunications resources (e.g. equipment) within the telecommunications network. This group performs Managed Element (ME) functions, e.g., provides support and/or service to the subscriber.

This can also provide access to a grouping of equipments for management purposes.

An ME communicates with a manager (directly or indirectly) for the purpose of being monitored and/or controlled. MEs may or may not additionally perform element management functionality.

An ME (and its contained Function_(s)) may or may not be geographically distributed. An ME (and its contained Function_(s)) is often referred to as a "Network Element".

#### Attributes

| Attribute Name | Support Qualifier | Read Qualifier | Write Qualifier | isInvariant Qualifier |
| --- | --- | --- | --- | --- |
| id | M | M, T | - | M, T |
| dnPrefix | M | M. T | - | M, F |
| managedElementType | O | M, T | - | M, T |
| userLabel | M | M, T | M, T | M, F |
| locationName | M | M, T | - | M, T |
| Attribute related to role |  |  |  |  |
| managedBy | O | M, T | - | M, F |

### Function_

#### 4.2.3.1	Definition

This represents a process, task, transformation or a relation between inputs and outputs.

#### 4.2.3.2	Attributes

| Attribute Name | Support Qualifier | Read Qualifier | Write Qualifier | isInvariant Qualifier |
| --- | --- | --- | --- | --- |
| id | M | M, T | - | M, T |
| userLabel | M | M, T | M, T | M, F |

### ManagementSystem_

#### 4.2.4.1	Definition

This represents a telecommunications management system (EM) that contains functionality for managing a number of MEs. The management system communicates with the MEs directly or indirectly over one or more interfaces for the purpose of monitoring and/or controlling these MEs.

This class has similar characteristics as the ManagedElement_. The main difference between these two classes is that the ManagementSystem_ has a special association to the MEs that it is responsible for managing.

#### 4.2.4.2	Attributes

| Attribute Name | Support Qualifier | Read Qualifier | Write Qualifier | isInvariant Qualifier |
| --- | --- | --- | --- | --- |
| id | M | M, T | - | M, T |
| userLabel | M | M, T | M, T | M, F |
| Attribute related to role |  |  |  |  |
| managedElements | O | M, T | - | M, F |

### TopologicalLink_

#### 4.2.5.1	Definition

The TopologicalLink_ represents a communication relationship between network entities and indicates that information is intended to flow between those network entities. The TopologicalLink_ always represents a logical relationship.

Editor’s note: This class is essentially a Trail with respect to the position in the network, that the solution needs to be multi-ended and that the Trail in ITU-T is not multi-ended.

#### 4.2.5.2	Attributes

| Attribute Name | Support Qualifier | Read Qualifier | Write Qualifier | isInvariant Qualifier |
| --- | --- | --- | --- | --- |
| id | M | M, T | - | M, T |
| userLabel | M | M, T | M, T | M, F |
| aEnd | M | M, T | - | M, F |
| zEnd | M | M, T | - | M, F |
| layerProtocolNameList | O | M, T | - | M, F |

### TerminationPointEncapsulation_

#### 4.2.6.1	Definition

The TerminationPointEncapsulation_ (TPE) represents one or more functions that terminate/originate a signal that adapt a signal for use, and that enable a signal to propagate. Hence a TPE can represent the end point of a signal flow (see Annex C (Informative): Rationale and Usage of TPE/LT for information on structure).

The TPE can also represent the intermediate point of a signal flow. See Annex C (Informative): Rationale and Usage of TPE/LT for information on TPE structure.

A TPE is capable of encapsulating multiple transport functions (G.805 termination functions, adapters, points etc.) at many different layers where the encapsulated transport functions are all related to the same signal flow. There are specific rules that guide encapsulation (see Annex C (Informative): Rationale and Usage of TPE/LT for information on usage). The encapsulated layers may be exposed (of its details of the transport assembly) via usage of instances of LayerTermination_ (LT).

The TPE deals equivalently with unidirectional and bidirectional flows.

#### 4.2.6.2	Attributes

| Attribute Name | Support Qualifier | Read Qualifier | Write Qualifier | isInvariant Qualifier |
| --- | --- | --- | --- | --- |
| id | M | M, T | - | M, T |
| tpeType | CM | M, T | - | M, F |

#### Attribute constraints

| Name | Definition |
| --- | --- |
| tpeType Support Qualifier | The condition is "The subject class instance name-contains one or more LayerTermination_ instances ". |

### LayerTermination_

#### 4.2.7.1	Definition

The LayerTermination_ (LT) encapsulates the functions and points associated with one instance of a layer [17]. The functions include the adapter functions, the termination functions and the connection points of that layer. In this case the term layer is essentially synonymous with the term protocol as use by other standards. All functions encapsulated have the same signal granularity, closely associated characteristic type and essential rate. A specific LayerTermination may be equipped with a subset of capabilities. Where the TPE is semi-transparent the layers encapsulated by a TPE are exposed by the LayerTermination_ set that it contains.

The LayerTermination_ provides the relevant layer parameters for the semi-transparent TPE cases.

The LayerTermination_ allows for detailed layer description of a TPE (potentially representing a port) and for precise association of the TPE with a TopologicalLink_ (or other representatives of forwarding relationship).

#### 4.2.7.2	Attributes

| Attribute Name | Support Qualifier | Read Qualifier | Write Qualifier | isInvariant Qualifier |
| --- | --- | --- | --- | --- |
| id | M | M, T | - | M, T |
| layerProtocolName | M | M, T | - | M, F |
| direction | M | M, T | - | M, F |
| ltType | M | M, T | - | M, F |
| index | CM | M, T | - | M, F |

#### 4.2.7.3	Attribute constraints

| Name | Definition |
| --- | --- |
| index Support Qualifier | The condition is “More than one LayerTermination_ instances (named-contained by the same TerminationPointEncapsulation_ instance) are associated with the same signal flow.” |

# UIM – Partition inventory

TBD

# UIM – Class attribute definitions

## Definitions and legal values

| Attribute Name |  | Definition | Legal Values |
| --- | --- | --- | --- |
| aEnd |  | The value of this attribute shall be the Distinguished Name of the alphabetically first instance in the Link subclass name to which this link/relation is associated (i.e., pointing to the instance of <X> as described in the definition of Link IOC in the present document). As an example, with Link_As_Slf, aEnd would contain the Distinguished Name of the AsFunction instance, and the zEnd would contain the Distinguished Name of SlfFunction instance. | For instance whose class is defined by 3GPP, the legal values would be in conformant with TS 32.300 [3]. For instance whose class is defined by TM Forum, the legal values would be in conformant with TM Forum MTOSI SD1-25_objectNaming.doc [16]. |
| direction |  | Represents the flow of traffic within the LT. | The values are: - Client-Server: Signal flows down the LT, e.g. traffic is taken from a number of low rate clients and multiplexed into a higher rate server. - Server-Client: Signal flows up the LT. - Bidirectional; Signal flow is both Client-Server and Server-Client. |
| dnPrefix |  | It carries the DN Prefix information or no information. See Annex C of 32.300 [2] for one usage of this attribute. |  |
| id |  | An attribute whose class name and value can be used as an RDN when naming an instance of the object class. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. | Values to be conformant with TS 32.300 [2] Agent set this value during object creation time. |
| index |  | Provides any relevant indexing of the LT (channel number, e.g. ‘3’) |  |
| layerProtocolName |  | Name and additional descriptive information for the protocol/layer used for the LT. Syntax and semantic is not specified. |  |
| layerProtocolNameList |  | Name(s) and additional descriptive information such as version number for the protocol(s)/layer(s) used for the associated communication link. Syntax and semantic is not specified. | Examples: “X2”, “LR Optical Channel” |
| locationName |  | The physical location (e.g. an address) of an entity represented by a (derivative of) ManagedElement_. It may contain no information to support the case where the derivative of ManagedElement_ needs to represent a distributed multi-location NE. | Agent set this value during object creation time. |
| ltType |  | The name of the specification that describes the internal construction of the LT, indicating for example that it possesses a G.805 CP but no G.805 TCP (see [11]). |  |
| managedElementType |  | The type of managed element. It is a multi-valued attribute with one or more unique elements. Thus, it may represent one ME functionality or a combination of more than one functionality. The actual syntax and encoding of this attribute is Solution Set specific. | The legal values of this attribute are the names of the IOC(s) that are (a) derived/subclassed from ManagedFunction and (b) directly name-contained by ManagedElement IOC (on the first level below ManagedElement), but with the string “Function” excluded. If a ManagedElement contains multiple instances of a ManagedFunction this attribute will not contain repeated values. The capitalisation (usage of upper/lower case) of characters in this attribute is insignificant. Thus, the IRPManager should be case insensitive when reading these values. Two examples of legal values are: - NodeB; - HLR,VLR. Agent set this value during object creation time. |
| theManagedElement |  | This is a role-attribute that contains a list of the DN(s) of the related derivatives of ManagedElement_ instance(s). |  |
| theManagementSystem |  | This is a role-attribute that contains a list of the DN(s) of the related derivatives of ManagementSystem_ instance(s). |  |
| tpeType |  | The name of the specification that describes the construction of the TPE emphasising for example the access to the TPE and whether it is associated with a physical port directly or not (see [11]). |  |
| userDefinedNetworkType |  | Textual information indicating network type, e.g. “UTRAN”. It may contain no information if there is no appropriate network type can be used. |  |
| userLabel |  | A user-friendly (and user assignable) name of this object. |  |
| zEnd |  | The value of this attribute shall be the Distinguished Name of the alphabetically second instance in the Link subclass name to which this link/relation is associated (i.e., pointing to the instance of <Y> as described in the definition of Link IOC in the present document). As an example, with Link_As_Slf, aEnd would contain the Distinguished Name of the AsFunction instance, and the zEnd would contain the Distinguished Name of SlfFunction instance. | Same as those defined for aEnd. |

# Annex A (Informative): Extract from 3GPP TS 32.622 Generic network resources IRP: NRM [5]

This appendix is an extract from [5].  One can note the similarities between the UIM class name-containment tree (see 4.1) and that defined in [5].

Figure 2: Containment tree of 3GPP TS 32.622 Generic network resources IRP: NRM [4]

# Annex B (Informative): Extract from MTOSI documents

This appendix provides the collection of SID/MTOSI currently defined classes as candidates for inclusion in UIM.

Figure 3: MTOSI/MTNM Containment

Figure 4: MTOSI/MTNM Connection-oriented Class Diagram

Figure 5: MTOSI/MTNM Connectionless Class Diagram

Figure 6: MTOSI/MTNM Termination Point Model

Figure 7: MTOSI/MTNM Inheritance Class Diagram

Figure 8: MTOSI/MTNM Inheritance Class Diagram with Attributes

# Annex C (Informative): Rationale and Usage of TPE/LT

This Annex provides the rationale and the usage of LT in conjunction with TPE.

Rational

A TPE is capable of encapsulating multiple transport functions (G.805 termination functions, adapters, points etc.) at many different layers where the encapsulated transport functions are all related to the same signal flow. See Figure 9: UIM related to TM Forum model and ITU-T concepts.

The TPE is used to both reduce the instances of objects required to represent a given transport assembly and to also simplify the translation from traditional environments where layering is not fully represented.

The encapsulation may be opaque, i.e. not exposing the layering, or semi-transparent, exposing the explicit layering but compacted into a single TPE instance. In the former case, TPE instance does not need to name-contain any LT. In the latter case, TPE instance needs to name-contain instances of LT.

The TPE deals equivalently with unidirectional and bidirectional flows. A bidirectional flow is where pairings of unidirectional flows have some shared fate or are considered as related in some way such that all entities associated with the whole bidirectional flow will be encapsulated in one TPE. Where a bidirectional flow is encapsulated it is possible to connect to only one of the two directions of flow and this can be represented through parameters of the TPE.

Usage

The TPE provides a place against which to raise alarms, display parameters and set attributes associated with the signal flow.

The TPE can be related:

- Directly to one or more physical ports (i.e. that the signal is associated directly with an externally visible connector)

  - Note that a physical port could also be related to more than one TPE;

- To logical functions that anchor the signal flow (i.e. it is floating between flexible functions in the equipment with no externally visible connector);

- To another supporting TPE to represent a client signal of the supporting TPE where there may be many instances of client;

  - Note that there may be many instances of server TPE that feed a single client (e.g., in the case of VCAT)

For background see SD1-18 Functional Modelling Concepts [11] and naming refer to SD1-25 Object Naming [16].

Figure 9: UIM related to TM Forum model and ITU-T concepts

END OF DOCUMENT

3GPP is a trade mark of ETSI