See text with changes bars for proposed updates from TM Forum TIP RSA team.

Preface

This document provides a running view of the status of the discussions on the Recommendation for Fault Management Harmonization in the 3GPP SA5 / TM Forum NGWW Joint Workgroup.

Document History

| Version | Doc Id | Comment |
| --- | --- | --- |
| 1 | S5eTMF0103 | S5eTMF0095 (HP contribution) reformatted; introductory sections tentatively adapted; status indicators added; no further changes to text in sections 2, 3 and 4 |
| 2 | S5eTMF0110 | output of 7 & 8 April 2011 Budapest Workshop |
| 3 | S5eTMF0114 | output of 14 April FMH call |
| - | S5eTMF0116 | Editor’s proposal for FMH Living List, input to 22 April FMH call (section 0 extended; process related remarks moved to notes) |
| 4 | S5eTMF0117 | output of 22 April FMH call |
| - | S5eTMF0125 | Editor’s proposal for FMH Living List, version 5 (added texts based on S5eTMF0120) |
| - | S5eTMF0130 | Output of 16 June FMH call (includes agreed changes to 4.1.4 - 4.1.9) |
| 5 | S5eTMF0135 | Living List version 5, output of 30 June FMH call (includes agreed changes to 4.2.1 - 4.3.9) |
| 6 | S5eTMF0139 | Living List version 6, output of Montreal meeting, 7 & 8 July (changes: status definitions refined; added 2.6 - 2.8, 4.1.x; changed 2.1 - 2.3, 2.5, 3.1.4, 3.3.1.1, 3.3.1.2, 3.3.4, 4.1.4 - 4.1.9, 4.2.1, 4.2.2, 3 references) |
| 7 | S5eTMF0144 | Living List version 7, output of 5 August meeting (changes: changed 3.3.3, 3.3.5, 3.3.6) |
| - | S5eTMF0145 | Living List version 7 - with editors notes and remarks, indentifying all items that need further consideration - input to 22 September FMH call |
| 8 | S5eTMF0147 | Living List version 8, output of 22 September FMH call (changes: all open items identified and owners for action items assigned) |

Table of Contents

0	Introduction	4

0.1	Document purpose	4

0.2	Underlying assumptions	4

0.3	Document structure	4

0.4	Subsection structure	4

1.	Commonalities	5

2.	Structural Changes	6

2.1	OSS-to-OSS System Context	6

2.2	Alarm Id	6

2.3	Notification Id	6

2.4	Threshold Information	7

2.5	Alarm Correlation	7

2.6	Distinction of correlation and root cause	8

2.7	Correlating alarm with event	8

2.8	Handling of non-active alarms	8

3	Functional Changes	10

3.1	Object Model	10

3.1.1	Probable Cause	10

3.1.2	Acknowledgement information	10

3.1.3	Void	11

3.1.4	Settable alarm fields	11

3.1.5	User Id	11

3.2	Notifications	11

3.2.1	AlarmListRebuilt	11

3.2.2	NotificationType	11

3.3	Operations	12

3.3.1	Get  directives	12

3.3.1.1	getResourceAlarm	12

3.3.1.2	getResourceAlarms	12

3.3.1.3	Attribute selector	12

3.3.2	Set directives	13

3.3.3	Create directive	13

3.3.4	Async mode	13

3.3.5	Output of directives	14

3.3.6	Idempotency	14

4	Minor Changes	15

4.1	Object Model	15

4.1.1	Enumeration Convention	15

4.1.2	History of Actions on Alarms in Alarm List	15

4.1.3	Difference in State Diagrams	15

4.1.4	Alarm Reporting Time	16

4.1.5	State Change Definition	16

4.1.6	Monitored Attributes	16

4.1.7	Service User Identification	17

4.1.x	Service Provider Identification	17

4.1.8	Alarm Detector Identifier	17

4.1.9	Mandatory Comments	17

4.2	Notifications	18

4.2.1	notifyComments	18

4.2.2	Object Class/Instance in AlarmListRebuilt	18

4.3	Operations	18

4.3.1	Group/Ungroup directives	18

4.3.2	Input alarm id	19

4.3.3	Input alarm id + severity for acknowledge directive	19

4.3.4	Tracking Info as input	19

4.3.5	Specific format for bad alarm info for acknowledge directive	19

4.3.6	Output status parameter	20

4.3.7	Input comment information	20

4.3.8	Filter type	20

4.3.9	Exceptions	20

Appendix A.  References	22

Temporary appendix : Status of items	23

# 0	Introduction

## 0.1	Document purpose

This document provides a running view of the status of the discussions on the Recommendations for Fault Management Harmonization in the 3GPP SA5 / TM Forum NGWW Joint FMH Workgroup.

When the status (see subsection 0.4) for all items is either A or D, the discussions are completed and the document will be sent to 3GPP and TM Forum as the final deliverable of the workgroup.

## 0.2	Underlying assumptions

This document assumes that both 3GPP SA5 and TM Forum are willing to make changes to their interfaces, based on the Recommendations in this document. This is a mandatory condition for success.

## 0.3	Document structure

The document is organized by category of change

- Structural Changes, that can have multiple impacts

- Functional Changes

- Minor Changes

Within each category, object model, notification and operation differences are covered.

## 0.4	Subsection structure

Each subsection is composed of the following parts:

- a description of the addressed item, identifying relevant differences between the 3GPP and the TM Forum solutions or approaches;

- one or more notes (in italics), describing the status of the discussion and agreed next steps towards resolution of the item;

- a Recommendation, describing the way in which harmonization can be obtained, this Recommendation is agreed by the joint team and offered for further consideration to 3GPP and TM Forum;

- a status indicator, indicating the status of the Recommendation:

| Status | Meaning |
| --- | --- |
| U | Under discussion |
| P | Recommendation is Provisionally agreed; changes to the Recommendation can still be made, but require written contribution. |
| A | Agreed Recommendation on how to reach harmonization |
| D | agreement on a Recommendation on how to reach harmonization could not be reached |

Color coding

yellow: pure editorial issues still to be resolved

pink: underlined/crossed out text: indicates changes proposed by the editor which have not yet been accepted by the workgroup

green: editor’s notes and remarks to help progress the document

# 1.	Commonalities

The 2 interfaces are sharing the same objects, alarm, monitored entity and comments. Even if some objects like alarm list are not explicitly modeled in TIP_RAM_IA, these objects are support objects in SA5_FM_IS and not explicitly used in generated interface. In summary, we could say that the 2 interfaces have about 90% commonality.

The alarm object has 30 attributes that are common to both interfaces, out of which only 1 (threshold information) has a different syntax in the 2 interfaces. SA5_FM_IS has 3 additional attributes (notification id, correlatedNotifications and StateChangeDefinition) compared to TIP_RAM_IA and TIP_RAM_IA has 8 additional attributes, mainly coming from the additional requirements present in TIP_RAM_BA.

In terms of notifications, both interfaces share the same notifications, except for the notifyComments that is merged in the notifyChangedAlarm in TIP_RAM_IA.

In terms of operations, the 6 operations present in SA5_FM_IS are also present in TIP_RAM_IA, even if the latter has 8 additional operations, 3 getAlarm operations, 2 setAlarm operations, 1 createAlarm, group/ungroupAlarm directives and getTrackingRecord. Apart from the get operations, the other ones are related to the additional requirements present in TIP_RAM_IA.

Note that differences in the syntax carried on the wire are outside of the scope of this document. Focus is only on the semantics.

# 2.	Structural Changes

## 2.1	OSS-to-OSS System Context

Description:

This point refers to section 1.1 of TR167.

[TIP_RAM_IA] defines an OSS-to-OSS business scenario [reference to TAM requested]. This scenario drives a number of new attributes (serviceAffecting, potentialRootCauseIndication) and new directives (set, create).

While some attributes like escalation and directives like create are quite specific to OSS-to-OSS (interfaces 3 or 4 in TR32.101, figure 1), most of the others can also be applied to Itf-N system context.

[TIP_RAM_IA] now includes a separate profile, named “Enhanced profile”, covering the OSS-OSS scenario. A supporting document ([SD_RAM_01_RAM_Profiles],) describes the various RAM profiles.

Note that for the rest of this document the Standard profile of TIP_RAM will be used as reference for comparison.

Notes: Description for TIP_RAM_IA agreed. Description for 3GPP to be added; maybe from 32.829 Recommendations in clause 6. Action for SA5 (Edwin)

Recommendation:  Note: Action by team required

Status: U

## 2.2	Alarm Id

Description:

Both SA5_FM_IS and TIP_RAM_IA define an alarmId that uniquely identifies an alarm record within an active alarm list. However there is some difference in the definition.

3GPP guarantees that the scope of uniqueness is the active alarm list. 3GPP does not specify when an alarmId can be reused. This implies that the alarmId can be reused as soon as the alarm is removed from the alarm list.

From RAM, the identifier is considered as the unique field for identifying alarms. The use of the identifier (alarmId plus context) allows consistency of identification when the RAM interfaces is used at various levels, i.e. between EMS and NMS and between NMSs. TIP RAM specifies that, when an alarm-owning system needs to assign an AlarmId, an AlarmId once used in an alarm list should never be reused.

Clients might keep reference to alarmIds and if their lifecycle is not synchronized and if alarmIds are reused, then some operations (ack or grouping) might end up being done on the wrong alarm. Alarm Ids are also used for correlating alarms and uniqueness is also required for this purpose.

TIP_RAM is considering that the use of 2 identifiers, alarm id and notification id  for identifying the same information, i.e. the alarm record, is making the job of the NMS (IRP Manager) more complex, as it has to maintain both as indexes and use one or the other depending on the context.

Notes: Description agreed.  SA5 will consider in which way alarmIds can be made sufficiently unique to properly support these use cases. Action for SA5 (Edwin) Action: Marc and Edwin and Joerg to further discuss and refine the problem statement, and the motivation for the uniqueness.

Recommendation: Note: Action by team required

Status: U

## 2.3	Notification Id

Description:

SA5_FM_IS uses notificationId as defined in the notification header. 3GPP uses notification to carry alarm information (e.g. notifyNewAlarm) and other information such as network configuration change information (e.g. notifyObjectCreation).  Notification has two parts, i.e. Header and Body.  Header has a parameter called notificationID.  Body can be alarm information or other information such as configuration management information.  The usage of notification ID is identical to that used in ITU-T TMN to uniquely identify an instance of notification from other instances.

In 3GPP, the usage of notificationId is the one way to guarantee unique alarm identifications for correlation purposes. However, the usage of notificationId is the only way to guarantee uniqueness to support correlation of alarms and events.

TIP_RAM does not use notificationId. NotificationId is not defined in TIP BaseNotification as there is no event logging facility today in TIP. TIP_RAM uses an identifier (alarmId plus context) to uniquely identify alarms for alarm correlation and for directives.

Note: Description agreed.

Recommendation: In TIP, notificationId is out of scope for RAM, so harmonization of this item is concluded not to be possible within the current scope of the Fault Management Harmonization team.

Note: This issue has been escalated to the Steering Team, it is waiting for feedback.

Status: D (110708)

## 2.4	Threshold Information

Description:

This point refers to the section 2.3 of TR167.

SA5_FM_IS thresholdInformation is identical to ITU-T X.733 thresholdInformation and IETF SNMP [add reference] thresholdInformation.

TIP_RAM_IA thresholdInformation has been defined in cooperation with the RSA PM team as part of the internal TM Forum harmonization. Differences with 3GPP are:

- TIP RAM includes thresholdId and thresholdCrossingDescription;

- TIP RAM supports thresholdInformation related to multiple monitored attributes.
For this case, the trigger condition, i.e. the specific combination of monitored attribute values which generates a threshold crossing alarm, is vendor specific;

- TIP RAM thresholdInformation can carry historical  threshold crossing information;

- TIP RAM does not provide armtime, and low and and high values, for single attribute thresholds.

It is noted that this subject, although closely related to Fault Management, is really inside Performance Management and cannot be resolved within the scope of the FMH project.

Notes: Description agreed. This issue has been escalated to the Steering Team, it is waiting for feedback.

Recommendation:

Status: U

## 2.5	Alarm Correlation

Description:

3GGP SA5 has completed TR 32.832, a study on Alarm Correlation (consisting of AC1 and AC2 functions) and Alarm Root Cause Analysis (RCA). 3GPP has completed the work on the AC1 function and on RCA and will start working on AC2 in the near future.

TIP RAM supports the AC1 function and the RCA function but does not (yet) support the AC2 function and it does not support correlation involving non-alarm-type events.

Recommendation: TIP RAM should start to study AC2, in order to harmonize its solution in the study and design phases with the solution being designed in 3GPP.

Status: A (110708)

## 2.6	Distinction of correlation and root cause

Description:

3GPP specification makes a distinction between alarm correlation and (alarm) root cause analysis.  The specification provides capability for clients to be aware of the distinction.

Alarm correlation identifies a set of related alarms/events whose root cause highly probably is the same, but which root cause is not identified.

Root cause analysis identifies a root cause of a set of alarms/events, together with the set of related alarms/events.

It is common for root-cause-analysis ‘engine/algorithm/function’ to know the correlation (of a set of alarms) but still not be able to identify its cause.  In such case, the 3GPP specified capability allows such knowledge (i.e. correlation only) be made known to client, aiding the client to determine the root cause.  Without such capability, the correlation result is lost.

In the modeling of these capabilities, 3GPP follow ITU-T specifications.

TIP_RAM is using similar concept of alarm grouping and root cause analysis:

- Alarm grouping, using the parent-underlying relationship between alarms, allowing building groups of alarms related to a parent. It indicates a correlation between alarms, but does not indicate that the selected parent is a potential root cause.

- Potential root cause indication: this flag indicates that a given alarm at a given point of the analysis is a potential root cause. It can be set independently of the grouping or might be done simultaneously.

Action on Marc: clarify text of TIP description

Recommendation:

It is concluded that both solutions provide the same capabilities: alarm grouping/correlation and root cause identification. Alarm grouping/correlation has been modeled in different ways. For the time being this difference is accepted. Further study may lead to changes, but none are envisioned now.

Note: further analysis by the team and additional text needed

Status: U

## 2.7	Correlating alarm with event

Description:

3GPP supports a capability allowing indication of correlation result that includes events (not just alarms).  The motivation is to recognize the fact that many managed resources emit events (not alarms) that are related to alarms or caused by or causing emission of alarms. Availability of this capability is becoming more urgent in the mobile network management when SON functions will be implemented.

Correlation of alarms with events is considered as out of scope for TIP_RAM Release 1.0. A standard event logging facility and interface would be a prerequisite for offering generalized access to events.

Notes: Description agreed

Recommendation: Note: team action required

Status: U

## 2.8	Handling of non-active alarms

Description:

3GPP supports a capability, the Notification Log IRP, allowing a client to query historical alarms (i.e. those that are no longer in AlarmList). It is used to aid post processing, e.g. study of trends and is essential for preventive maintenance.

In TIP_RAM, an alarm can be removed from the alarm list as soon as it is cleared (or cleared and acknowledged) but the decision to remove it from the alarm list is considered implementation-specific. Non-active alarms are by default not visible across the interface unless when using a get by id, however, an active alarm can be grouped to a non-active alarm as long as it is in the alarm list.

Long-term access to alarms in datawarehouse or for reporting or long term trending is considered as out of scope of the RAM interface. A generic event logging facility is not specified today in TIP.

Notes: Description agreed

Recommendation: Note: team action required

Status: U

# 3	Functional Changes

## 3.1	Object Model

### 3.1.1	Probable Cause

Description:

SA5_FM_IS defines the probable cause as an enumeration, while TIP_RAM_IA defines it as a string with qualified text. The possible values will be defined outside of the RAM specification for simpler extensibility.

It would be good to understand better how extensibility of the probable cause is achieved in 3GPP. It seems to be linked with a new release of the specification.

Definition of probable causes in TM Forum as a separate document can be a joint work item between 3GPP and TM Forum.

[Note:

3GPP current approach is:

- Its stage-2 (“i.e. platform independent definition”) specification defines X number of probable causes and does not allow vendor-extension;

- Its stage-3 (“i.e. platform specific definition”) protocol allows X legal values.

- If group agreed to have two more PC, a new release Stage-2 will define X+2 PCs and its corresponding released stage-3 protocol would allows X+2 legal values.

TIP_RAM approach is:

- define X number of probable causes and does not allow vendor-extension; this definition is in separate document (called Y) than the “protocol specification” document.

- protocol allows any values.

- If group agreed to have two more PCs, update (a new release) document Y.

Notes: description to be reformatted (= Action for Editor), further changes needed?

Recommendation:  Note: Action by team required

Status: U

### 3.1.2	Acknowledgement information

Description:

Both interfaces have an ackState with similar values. The same is true for ackTime and ackUserId.

These attributes are optional for TIP_RAM_IA because specifying them as mandatory was considered to make the interface more complex than required.

3GPP made support of acknowledgement mandatory because without it a raised and subsequently cleared alarm can be removed from the active alarm list without the IRPmanager being aware.

After reconsideration by the TIP RAM team of the reasons and effects of making the Acknowledge directive mandatory, it was confirmed that the Acknowledge directive stays optional in TIP RAM.

Recommendation: Both teams believe they have reasons to keep their solution, so no agreement for harmonization could be reached.

Status: D

### 3.1.3	Void

### 3.1.4	Settable alarm fields

Description:

The TIP RAM interface was exposing 9 alarm attributes as also settable across the interface by the client: perceived severity, specific problem, proposed repair actions, additionalText, backup status + object, alarm escalation, service affecting, potential root cause indication.

The TIP RAM team considers that an alarm is a combination of information coming from the element (perceived severity, specific problem, proposed repair actions, additionalText, backup status + object, alarm escalation) and management information (service affecting, potential root cause indication). While it can be agreed that the information coming from the element should not be settable by the client, the management information should be settable by the client.

The TIP RAM allows now for the Standard profile to set the management information, i.e. the 2 attributes: service affecting and potential root cause indication, by the client. For the Enhanced profile, all 9 attributes will be settable by the client.

Note: Edwin to propose description for SA5

Recommendation: Note: Action by team required

Status: U

### 3.1.5	User Id

Description:

For SA5_FM_IS the user id attributes (ack, clear) and corresponding notification attributes and operation parameters are seen as mandatory.

For TIP_RAM_IA, the user id attributes were optional. These attributes and the corresponding notification attributes are now mandatory for the standard alarm profile.

Hence 3GPP and TIP are aligned.

Recommendation: no changes needed

Status: A (110408)

## 3.2	Notifications

### 3.2.1	AlarmListRebuilt

Description:

The notification notifyAlarmListRebuilt is mandatory in SA5_FM_IS.

It was optional in the standard profile of TIP_RAM_IA, and it was not supported in the Simple Alarm Reporting profile in TIP RAM. In TIP RAM it is now mandatory in the standard profile and it remains not supported in the Simple Alarm Reporting profile of TIP RAM.

Recommendation: After the change in TIP_RAM_IA, there is semantical alignment; no further changes needed.

Status: A (110707)

### 3.2.2	NotificationType

Description:

SA5_FM_IS and TIP RAM IA have both a solution for a sender to indicate the type of nootification. There is no difference in semantics; but on xml level there is a syntactical difference.The encoding of this in 3GPP Solution Set [include reference] for xml may have duplicated information to indicate “type of notification”.

Recommendation:  no change

Status:  A (110310)

- 3.3	Operations

### 3.3.1	Get  directives

### 3.3.1.1	getResourceAlarm

Description:

TIP_RAM_IA was defining two different directives: getResourceAlarm and getResourceAlarmIds. The Use Case for getResourceAlarm concerned the case that the OSS does not have the full alarm information, but only the alarmId. The Use Case for getResourceAlarmIds related to out-of-sync situations and subsequent performance issues.

It was noted during the FMH meetings that the getIds can be obtained by using the standard get directive with attribute selector = identifier. For this reason, the TIP RAM team agreed not to support an explicit getResourceAlarmIds. This directive (getResourceAlarmIds) has been removed and documentation updated accordingly.

SA5_FM_IS does not have the getResourceAlarm directive.

Notes: Description for TIP RAM agreed; description for SA5 to be added. Action SA5 (Edwin)

Recommendation:  Note: Action by team required

Status: U

### 3.3.1.2	getResourceAlarms

Description:

TIP_RAM_IA was providing two getResourceAlarms directives: getAllActiveAlarms and getResourceAlarms. The getAllActiveAlarms directive was used for getting all active alarms in the Simple Alarm Reporting profile.

The TIP RAM team agreed to add the alarmAckState parameter to the getResourceAlarms directive, with the same set of values as SA5_FM_IS. This change impacts the Simple Alarm Reporting profile, but getting all active alarms can still be done without defining a filter, simply by setting the enum to the right value. The return will still be a list of alarms by iterator. The directive getAllActiveAlarms has been removed in all profiles.

SA5_FM_IS has a single getAlarmList directive with input parameters for getting alarms in a given state or matching a given object, in addition to the filter.

Recommendation:  Note: Action by team required

Status: U

### 3.3.1.3	Attribute selector

Description:

TIP_RAM_IA offers the capability in the getResourceAlarms directive to select this list of alarm attributes that will be returned with each alarm matching the criteria.. The optional  input parameter “attribute selector” can be filled with a set of alarm field names. In this case, only the listed fields will be returned for each alarm matching the criteria. If the attribute selector is empty or not present, all the fields of each alarm are returned. This parameter is available in all profiles.

3GPP_FM_IS does not support this capability.

Notes: Descriptive text is still preliminary. Action for TIP (Marc) to propose new or additional text. Support mandatory or optional? In which profile(s)?

Recommendation:  Note: Action by team required

Status: U

### 3.3.2	Set directives

Description:

TIP_RAM_IA defines 2 set directives, which can set attribute values on records currently present in the AlarmList:

- setResourceAlarm, for setting a single object by Id;

- setResourceAlarms, for setting multiple objects by filter.

These directives are only available in the Standard (equivalent to Itf-N) and Enhanced profiles.

The settable alarm fields are described in the document SD_RAM_01_RAM_profiles.xls.

For the Standard profile, the following fields are settable:

- aliasNames

- extensionInfo

- serviceAffecting

- potentialRootCauseIndication

SA5_FM_IS does not have this capability. 3GPP considers this capability for all attributes to be related to OSS-OSS interfaces only; this 3GPP position is documented in TR 32.829 [add reference].

Notes: Description agreed. TIP RAM team to revalidate the underlying assumptions/use cases for these directives at the Itf-N level. Action for TIP (Marc) to adapt the description to the current situation in TIP

Recommendation:  Note: Action by team required

Status: U

### 3.3.3	Create directive

Description:

TIP_RAM_IA defines a create directive for the OSS-to-OSS scenario in the Enhanced profile that is not part of the scope of SA5_FM_IS. This is related to the point on the OSS-to-OSS scenario.<add bookmark to section 2.1??>

SA5_FM_IS …

Notes: Description partially agreed. 
3GPP to propose text. Action SA5 (Edwin)

Recommendation: Note: Action by team required

Status: U

### 3.3.4	Async mode

Description:

SA5_FM_IS is defining 2 modes for the getAlarmList directive: synchronous and asynchronous using events. Asynchronous mode is only supported by CMIP SS, which has been retired since Release 7. SA5 is working on a CR to remove the asynchronous mode from SA5_FM_IS.

TIP_RAM does not support asynchronous directive response.

Recommendation: After the change has been implemented by SA5, no further changes are needed.

Status: A (110707)

### 3.3.5	Output of directives

Description:

For unack, clear and comment directives, SA5_FM_IS returns a structure containing alarmId indicating the alarms that are not successfully unacked or cleared. Also a failure code is returned to indicate why the unack or clear was not successful.

TIP RAM changed the the output parameter to return failed objects with a failure code as string.

Recommendation:  No further changes required.

Status: A

### 3.3.6	Idempotency

Description:

TIP_RAM_IA was using idempotent operations for ack, unack and clear. TIP RAM removed idempotency for these 3 directives. If executed a second time, a failed id with matching failure code will be returned

SA5_FM_IS has a strict set of pre-conditions for those operations, making them not idempotent. . E.g. client 1 and client 2 issue ack on alarmId 6. The server processes client 1 request first and state of alarmId 6 is changed to acked. The server will record client 1 as the issuer of the ack. Subsequently, server processes client 2 request and rejects the request, while indicating that alarmId 6 was already acked.

In both cases, no change is made alarmId 6 as a result of client 2 request.

Recommendation: No further changes required.

Status: A

# 4	Minor Changes

## 4.1	Object Model

### 4.1.1	Enumeration Convention

Description:

As described in section 1.4.1 of TR167, different conventions are used by each team.

The convention used by 3GPP cannot be directly implemented as no variable can have blanks. TIP_RAM_IA is using the java constant convention that is suited for coding.

This only a matter of syntax.

Recommendation: If 3GPP has other reasons to revise its solution set in a non-backward compatible way, this change should be included.

Status: A (110414)

### 4.1.2	History of Actions on Alarms in Alarm List

Description:

SA5_FM_IS is using specific fields in the alarm record where information about actions is stored, i.e. alarm raised time, alarm clear time, alarm acknowledgment time. Only the most recent changes will be recorded, previous changes will not be recorded in the alarm record. Previous changes can be obtained by querying the Notification Log, providing as input the alarmId and the notificationId.

In TIP_RAM_IA the same fields are present. In addition, TIP_RAM_IA is using Tracking Records to provide an history of actions on alarms. A tracking record contains: time of the action, userid, systemid, description of the action, (reference to) alarmId. The access method is getTrackingRecords with input parameter alarmId; the result is a list of tracking records. Tracking records are not part of an alarm.

Notes: Description agreed.

Recommendation: Note: Action by team required

Status: U

### 4.1.3	Difference in State Diagrams

Description:

In SA5_FM_IS, if the perceived severity of an acknowledged alarm is changed, then this alarm become unacknowledged. The reason is that an alarm, whose severity has changed, should be brought again to the attention of the operator. This is especially important when the severity is changed from a lower to a higher value.

Currently TIP_RAM does not change the acknowledgment state when the severity is changed. The TIP RAM team discussed this point. The argument of 3GPP is understood, but acknowledgement has a given meaning and it might not be up to the EMS to reset it. So no change in TIP RAM state diagram is proposed

Notes: description to be reviewed and validated by the team.

Recommendation: Note: Action by team required

Status: U

### 4.1.4	Alarm Reporting Time

Description:

TIP_RAM_IA uses this attribute to indicate the time (as a date + time) at which the alarm was reported. It might be different from the alarmRaisedTime. For instance, if the alarm list is maintained by an EMS, the alarmRaisedtime would be the time the alarm was detected by the NE, while the alarmReportingTime would be the time this alarm was reported by the EMS. alarmReportingTime is supported for compatibility with MTOSI RTM

SA5_FM_IS uses notification, an emission from Agent to Manager to report alarm information such as alarm raised, alarm cleared.  The event time attribute of the notifyNewAlarm notification carries the alarm raised time.

Notes: Description agreed.

Recommendation: Note: Action by team required

Status: U

### 4.1.5	State Change Definition

Description:

The StateChangeDefinition attribute does not exist in TIP_RAM_IA. The reasons for this choict are the following:

- he TIP RAM team thought the attribute was not really needed, as the NE have quite often states that are slightly different from the values defined in X.731. Adding this attribute therefore is of limited interest.

- this attribute relates to state information and the definition of the state information in SID is different from the definition in ITU-T X.731, so if exposed, this field would have a structure different from the one used in 3GPP Alarm IRP.

Sofor the reasons above, the choice was made not to expose this attribute in TIP RAM.

SA5_FM_IS alarm record can carry an attribute called StateChangeDefinition.  This purpose of this attribute is specified in ITU-T X.733 as “This parameter, when present, is used to indicate a state transition associated with the alarm.” The state change definition is defined in ITU-T Rec. X.731 as “8.2.2.3 State change definition. This parameter set consists of a set of sequences of the three parameters: Attribute identifier, Old attribute value and New attribute value…”.

Notes: Description for SA5 agreed, Marc to propose improved text for TIP; Action TIP (Marc)

Recommendation:  Note: Action by team required

Status: U

### 4.1.6	Monitored Attributes

Description:

Both interfaces have this attribute and it is defined in both as a set of attribute value pairs.

For TIP_RAM_IA, it includes two fields: name and value, both as strings. The datatype used by TIP_RAM_IA is a shared one whose change might impact other models. The TIP RAM team view is that the type of the attribute is known by the attribute name and introducing the type might lead to inconsistencies.

SA5_FM_IS alarm record can carry an attribute called monitoredAttributes. it contains three fields; attribute name, attribute value and attribute type. Supported types are string, integer, unsignedInt, boolean, dateTime, base64Binary. The attribute value has the type indicated in the attribute type.

Recommendation: The solutions differ only in syntax, not in semantics, hence no changes are needed.

Status: A (110707)

### 4.1.7	Service User Identification

Description:

TIP_RAM_IA alarm record can carry an attribute securityServiceUser. Its semantics is “It identifies the service-user whose request for service led to the generation of the security alarm”  Note that the attribute name has a prefix to indicate they are specific to security alarms.

SA5_FM_IS alarm record can carry a serviceUser attribute.  The semantics of it is “It identifies the service-user whose request for service provided by the serviceProvider led to the generation of the security alarm”.  Note that this attribute is not prefixed with ‘security;.

Recommendation: The solutions differ only in name, not in semantics, hence no changes are needed.

Status: A (110707)

### 4.1.x	Service Provider Identification

Description:

TIP_RAM_IA alarm record can carry an attribute securityServiceProvider. Its semantics is “It identifies the service-provider whose service request provokes the generation of the security alarm”  Note that the attribute name has a prefix to indicate it is specific to security alarms.

SA5_FM_IS alarm record can carry a serviceProvider attribute.  The semantics of it is “It identifies the service-provider whose service is requested by the serviceUser and the service request provokes the generation of the security alarm.”  Note that this attribute is not prefixed with ‘security’.

Recommendation: The solutions differ only in name, not in semantics, hence no changes are needed.

Status: A (110707)

### 4.1.8	Alarm Detector Identifier

Description:

SA5_FM_IS alarm record can carry an attribute securityAlarmDetector.  It carries the identity of the detector of a security alarm

TIP_RAM_IA alarm record can carry an attribute alarmDetector.  Its semantics is “It provides the identity of the detector of an alarm”.  It has been derived from securityAlarmDetector, but it is not prefixed with ‘security’ as the alarm detector can detect  non security alarms.

Recommendation: For security alarms there is no difference in semantics. For non-security alarms, there is a difference. An agreed solution to resolve this difference could not be identified.

Status: D (110707)

### 4.1.9	Mandatory Comments

Description:

An SA5_FM_IS alarm record can carry an attribute called comments.  Agent support of this attribute is mandatory if the set comment directive is supported.  Note that IRPManager (TMF termed OSS) sets/makes comments on specific alarm in active AlarmList.  If IRPManager has set/made a comment on a particular alarm, the alarm record must track/carry the comments; else the alarm record is empty.

In TIP_RAM_IA, comments is an optional field as it is not part of the Simple Alarm Reporting profile.

If the commentResourceAlarms directive is supported by a FM server, then this attribute shall also be supported and comments entered should be visible in the alarm.

Notes: Description for SA5 agreed. Marc to clarify/enhance text for TIP_RAM_IA. Action TIP (Marc)

Recommendation:  Note: Action by team required

Status: U

## 4.2	Notifications

### 4.2.1	notifyComments

Description:

SA5_FM_IS emits a notification to all notification subscribers indicating that an IRPManager have set/made comment on a particular active alarm in AlarmList.  The notification header has an attribute called notificationType that indicates this type of notifications (as opposed to other types such as one that carry cleared alarm information).

TIP_RAM_IA considers comment as a datatype and therefore uses the changedAlarm notification to carry the “comment” information.

Recommendation: The solutions differ only in syntax, not in semantics, hence no changes are needed.

Status: A (110707)

### 4.2.2	Object Class/Instance in AlarmListRebuilt

Description:

SA5_FM_IS has a mandatory capability for IRPAgent to report when its alarmList has been rebuilt.  The representation of the entities (e.g. network element) under management is organized/named in hierarchy.  Any of these entities can be in alarm state.

Situation one: When Agent loses confidence of the alarm information of the whole hierarchy, it indicates such situation by placing the value of systemDN in the Object Class/ Object Instance fields

Situation two: When Agent loses confidence of the alarm information of part of the hierarchy, it indicates such situation by placing the value of DN of the top managed entity of that hierarchy part in the objectClass/objectInstance field.

TIP_RAM_IA has identical capability.  It handles the Situation two (described above) in the same way as in SA5_FM_IS.  However, in Situation one, its objectClass/Instance attribute is absent.

TIP_RAM_IA considers its approach cleaner.

Recommendation: The solutions are semantically identical. It was noted that both solutions do not support the identification of multiple parts of the hierarchy. Both SA5 and TIP may consider adding this capability. At this moment no changes are proposed.

Status: A (110707)

## 4.3	Operations

### 4.3.1	Group/Ungroup directives

Description:

These 2 directives are defined in TIP_RAM_IA to manipulate parent/underlying alarm association. They are present in the standard profile and in the enhanced profile.

Notes: TIP_RAM_IA description agreed; Edwin to contribute description  from SA5 point of view;  Action SA5 (Edwin)

Recommendation:  Note: Action by team required

Status: U

### 4.3.2	Input alarm id

Description:

SA5_FM_IS uses a set of alarm ids as input for unack, clear and comment directives.

TIP_RAM_IA uses a set of identifiers, which are the equivalent form, but using the TIP identifier.

Recommendation: Semantics are identical, hence no changes are needed.

Status: A

### 4.3.3	Input alarm id + severity for acknowledge directive

Description:

SA5_FM_IS uses for the ack directive a set of (alarm  id + severity) while for the other directives, like unack, clear and comment, it uses only a set of alarm id. The reason is that …

TIP_RAM_IA uses for the ack directive only a set of alarm id.

Notes: TIP_RAM_IA description agreed; Edwin to propose modified & enhanced description for  SA5

Recommendation:  Note: Action by team required

Status: U

### 4.3.4	Tracking Info as input

Description:

TIP_RAM_IA used as input argument for the ack/ unack and clear directives a Tracking Record to pass user id and system id.

SA5_FM_IS uses separate attributes.

The TIP RAM team uses now separate parameters for user id and system id for the ack, unack and clear directives.

Recommendation: After the change in TIP_RAM_IA, there is semantical alignment; no further changes needed.

Status: A

### 4.3.5	Specific format for bad alarm info for acknowledge directive

Description:

SA5_FM_IS uses as output for the unack, clear and comment directives a set of alarm id and failure reason as string. However for the ack directive, there are 2 failure reasons: one as an enum and an additional one as string.

TIP_RAM_IA uses for ack also a set of alarm id and failure reason as string.

Recommendation: The difference is at the syntax level; semantically SA5_FM_IS and TIP_RAM_IA are aligned, so no changes are needed.

Status: A

### 4.3.6	Output status parameter

Description:

SA5_FM_IS has a parameter in all responses/confirmations indicating whether the operation was successful or not.

TIP_RAM_IA does not have such parameter. It handles all not succesful operations by means of exceptions.

Notes: Edwin to propose additional text to the description explaining SA5 solution.

Recommendation:  Note: Action by team required

Status: U

Description:

### 4.3.7	Input comment information

In the SetComment operation, SA5_FM_IS uses separate fields for text, user id and system id.

TIP_RAM_IA used a comment datatype as input.

The TIP RAM team agreed to use separate input fields for text, user id and system id.

Recommendation: After the change made by TIP_RAM_IA, no further changes are needed.

Status: A

### 4.3.8	Filter type

Description:

SA5_FM_IS uses a filter in directives; this filter seems to be based on xpath for.

In TIP_RAM_IA, the filter for common directives (get and set) is a choice of template or query filter. Template filter is based on combination of attribute values of the objects. Query filter is based on xpath.

For the count directives, TIP_RAM_IA only uses a query (xpath) filter.

The use of template in RAM common directives is bringing some more ease of use, so we think it should be kept. For the query part, the use of xpath seems common.

Notes:  The description should be adapted to describe the filter capabilities, which filter language is used is a secondary issue. Edwin to provide a description of SA5_FM_IS filter capabilities.

Recommendation:  Note: Action by team required

Status: U

### 4.3.9	Exceptions

Description:

SA5_FM_IS has only a single exception, OperationFailed, applicable to all operations.

TIP_RAM_IA is using a larger set of exceptions with 6 pre-defined exceptions that are applicable to all directives

The pre-defined exceptions defined in TIP are:

- AccessDenied

- CommunicationLoss

- InternalError

- InvalidInput

- NotImplemented

- UnableToComply

Notes: Description to be revisited.  Action by TIP (Marc) to list the 6 exceptions; action by Edwin to extend/clarify the SA5 text

Recommendation: Note: Action by team required

Status: U

# Appendix A.  References

This appendix contains references to, or copies of, documents

| Reference | Description |
| --- | --- |
| SA5_FM_IS | 3GPP Telecommunication management Fault Management Part 2: Alarm Integration Reference Point (IRP) Information Service (IS) (Release 10) |
| TIP_RAM_IA | TM Forum TIP Resource Alarm Management Information Agreement |
| TIP_RAM_BA | TM Forum TMF524 Resource Alarm Management Business Agreement Rel 1.1, Version 1.5 S5eTMF0086 |
| TR167 | TM Forum TR167 Fault Management Interface Comparison Version 1.0 S5eTMF0090 |
| TR32.101 | 3GPP Telecommunication management Fault Management Principles and high level requirements |
| SD_RAM_01_RAM_Profiles | TM Forum Resource Alarm Management Supporting Document Definition of RAM Profiles S5eTMF0121 |
| ITU-T X.733 | ITU-T Recommendation X.733 Information technology – Open Systems Interconnection – Systems Management Alarm reporting function |

# Temporary appendix : Status of items

| Title | Description | Recommendation | Status |
| --- | --- | --- | --- |
| 2. Structural changes |  |  |  |
| 2.1 OSS-to-OSS System Context | partially agreed |  | U |
| 2.2 Alarm Id | agreed |  | U |
| 2.3 Notification Id | agreed | to be escalated to Steering Team | D |
| 2.4 Threshold Information | agreed | escalated to Steering Team | U |
| 2.5 Alarm Correlation | agreed | available | A |
| 2.6 Distinction of correlation and root cause |  |  | U |
| 2.7 Correlating alarm with event | agreed |  | U |
| 2.8 Handling of non-active alarms | agreed |  | U |
| 3 Functional Changes |  |  |  |
| 3.1 Object Model |  |  |  |
| 3.1.1 Probable Cause |  |  | U |
| 3.1.2 Acknowledgement information | agreed | none available | D |
| 3.1.3 Void | - | - | - |
| 3.1.4 Settable fields |  |  | U |
| 3.1.5 User Id | agreed | available | A |
| 3.2 Notifications |  |  |  |
| 3.2.1 AlarmListRebuilt | agreed | available | A |
| 3.2.2 NotificationType | agreed | available | A |
| 3.3 Operations |  |  |  |
| 3.3.1 Get directives |  |  |  |
| 3.3.1.1 getResourceAlarm | partially agreed |  | U |
| 3.3.1.2 getResourceAlarms |  |  | U |
| 3.3.1.3 Attribute selector |  |  | U |
| 3.3.2 Set directives | agreed |  | U |
| 3.3.3 Create directive | partially agreed |  | U |
| 3.3.4 Async mode | agreed | available | A |
| 3.3.5 Output of directives | agreed | available | A |
| 3.3.6 Idempotency | agreed | available | A |
| 4 Minor Changes |  |  |  |
| 4.1 Object Model |  |  |  |
| 4.1.1 Enumeration Convention | agreed | available | A |
| 4.1.2 History of Actions on Alarms in Alarm List | agreed |  | U |
| 4.1.3 Difference in State Diagrams | agreed |  | U |
| 4.1.4 Alarm Reporting Time | agreed |  | U |
| 4.1.5 State Change Definition | partially agreed |  | U |
| 4.1.6 Monitored Attributes | agreed | available | A |
| 4.1.7 Service User & Provider | agreed | available | A |
| 4.1.x Service Provider Identification | agreed | available | A |
| 4.1.8 Alarm Detector Identifier | agreed | available | D |
| 4.1.9 Mandatory Comments | partially agreed |  | U |
| 4.2 Notifications |  |  |  |
| 4.2.1 notifyComments | agreed | available | A |
| 4.2.2 Object Class/Instance in AlarmListRebuilt | agreed | available | A |
| 4.3 Operations |  |  |  |
| 4.3.1 Group/Ungroup directives | partially agreed |  | U |
| 4.3.2 Input alarm id | agreed | available | A |
| 4.3.3 Input alarm id + severity for acknowledge directive | partially agreed |  | U |
| 4.3.4 Tracking Info as input | agreed | available | A |
| 4.3.5 Specific format for bad alarm info for acknowledge directive | agreed | available | A |
| 4.3.6 Output status parameter | partially agreed |  | U |
| 4.3.7 Input comment information | agreed | no changes | A |
| 4.3.8 Filter type |  |  | U |
| 4.3.9 Exceptions |  |  | U |