TSG-RAN Working Group 3, O&M adhoc Meeting #3	TSGR3#5(99) 607

Münster, GERMANY, 29-30 June 1999

Agenda Item:	4

Source: 		Motorola

Title: 		Cell Availability / Unavailability logical O&M NBAP Procedures

Document for:	Decision

# Introduction

In 25.433 NBAP Specification, the Cell Setup (section 8.1.5.1) and Cell Delete (section 8.1.5.2) procedures appear to span two domains:

- Network management domain – here the cell is defined (i.e., cell parameters set) by the management system.

- Logical O&M domain – here the CRNC and Node B either share cell configuration information to make a cell available for carrying traffic or remove a cell context known between the network elements.

By exceeding the logical O&M domain, the procedures appear to increase the difficulties in creating a multi-vendor UTRAN environment.

The intent of this contribution is to limit the procedures defined in NBAP specification to the logical O&M domain.  Creation and deletion of cell parameters remains within the network management domain.  The logical O&M domain is restricted to synchronization between the CRNC and the Node B.  This synchronization consists of an agreement of which cells exist, as well as a sharing of cell parameters owned by one network element and required by the other network element.

A new terminology has been employed in an attempt to emphasize the logical O&M domain limits of the NBAP procedures.  The procedure scope is confined to the logical O&M perspective of making a cell available or unavailable for carrying traffic.

# Procedure Definition

## Cell Availability Procedure

The Cell Availability procedure is used for cell existence synchronization between the CRNC and the Node B to indicate that the specified cell is available to carry traffic. The CRNC becomes aware of cells via the CRNC management system.  The Node B becomes aware of cells via the Node B management system.  The CRNC tells the Node B of the cells it is aware.  The Node B responds to the CRNC with cells that it is also aware. The Node B stores (remembers) the logical cells of which the Node B is not aware and notifies the CRNC as it becomes aware of cells from the CRNC list.

CELL AVAILABILITY REQUEST

CRNC

Node B

CELL AVAILABILITY RESPONSE

CELL AVAILABILITY REQUEST message contains:

- Cell identity

- Logical cell parameters owned by the CRNC and required by the Node B

If the Node B management system has configured the implementation specific and logical cell parameters at the Node B for the cell identity listed in the request, the Node B indicates success in the response.

CELL AVAILABILITY RESPONSE message contains:

- Cell identity

- Logical cell parameters owned by the Node B and required by the CRNC

If the Node B is not aware of the cell identity specified in the request or has not been configured with its required cell parameters, a negative acknowledge (NACK) is sent in the response (failure case).  After sending the CELL AVAILABILITY RESPONSE (NACK), once the Node B receives and applies from the Node B management system the necessary cell parameters (both implementation specific and logical), for any of the cells that the Node B NACK’ed, the Node B sends an indication of the cell availability.

CRNC

Node B

CELL AVAILABILITY REQUEST

CELL AVAILABILITY RESPONSE (with NACK)

Node B received (from Node B management system) necessary cell parameters and applied them

CELL AVAILABILITY INDICATION

CELL AVAILABILITY ACKNOWLEDGE

CELL AVAILABILITY INDICATION message contains:

- Cell identity

- Logical cell parameters owned by the Node B and required by the CRNC

CELL AVAILABILITY ACKNOWLEDGE message contains:

- Cell identity

- Logical cell parameters owned by the CRNC and required by the Node B

The four messages shown above (CELL AVAILABILITY request / response / indication / acknowledge) should handle the synchronization issue and also relieve the CRNC of tracking Node B cells of which the CRNC has no knowledge.

Note that cell parameter changes from the CRNC should not be allowed for the cell that the Node B sends a NACK in the CELL AVAILABILITY RESPONSE.

## Cell Unavailability Procedure

The cell unavailability procedure is used to remove the agreement of a cell’s existence between a CRNC and Node B.  The following two messages correspond to when the CRNC initiates the procedure.

CRNC

Node B

CELL UNAVAILABILITY INDICATION

CELL UNAVAILABILITY ACKNOWLEDGE

CELL UNAVAILABILITY REQUEST message contains:

- Cell identity

CELL UNAVAILABILITY RESPONSE message contains:

- Cell identity

- ACK (success) or NACK (error case)

The following two messages are employed when the Node B initiates the procedure.  Note that the Node B only initiates the procedure when agreement on a cell existence has previously occurred.  The Node B does not initiate this procedure if the Node B is aware of a logical cell of which the CRNC is not aware.

CRNC

Node B

CELL UNAVAILABILITY INDICATION

CELL UNAVAILABILITY ACKNOWLEDGE

CELL UNAVAILABILITY INDICATION message contains:

- Cell identity

CELL UNAVAILABILITY ACKNOWLEDGE message contains:

- Cell identity

- ACK (success) or NACK (error case)

# Presumptions

A Node B control port is available for communication between the RNC and Node B, for the procedures to be executed successfully.

# Proposal:

- To replace the Cell Setup procedure found in 25.433 (v1.0.2) section 8.1.5.1 with the Cell Available procedure.

- To replace the Cell Delete procedure found in 25.433 (v1.0.2) section 8.1.5.2 with the Cell Unavailable procedure.