| 3GPP TSG-SA5 (Telecom Management) Meeting #8 Bonn, Germany, 07-10 December, 1999 |  | Tdoc S5-99288 |
| --- | --- | --- |

| Title: | IRP Information Model & IRP Solution Sets in compliance condition |
| --- | --- |
| Source: | Ericsson (Håkan Andersson, e-mail hakan.andersson@epl.ericsson.se) |
| Agenda item: | 9.1 |
| Document for: | decision |
| Category: | Contribution to 32.101 |
| Document Summary: | This document proposes the inclusion of ‘IRP Information Models’ & ‘IRP Solution Sets’ in the compliance conditions |
| Specification(s) involved: | 32.101 |
| Other information: | - |

## 5.10	Compliance Conditions

For a UMTS entity (Management System or NE) to be compliant to a given UMTS Management Interface, all the following conditions must be satisfied:

- it implements the management functionality following the IRP Information Model specified by the relevant 3GPP UMTS Management Interface Specifications applicable to that interface

- it provides at least one of the valid IRP Solution Sets specified by 3GPP UMTS for that interface

- it provides at least one of the valid Application Protocols specified by 3GPP UMTS Application Protocols for that interface

- it provides at least one standard networking protocol

- in case the entity does not offer the management interface on its own, a Q-Adapter must be provided. This Q-Adapter must be provided independently of any other UMTS NE and/or UMTS Management System.