# 3GPP TSG SA WG5 (Telecom Management) Meeting #100	S5-152148

# 13-17 April 2015 Dubrovnik, Croatia	revision of S5-140abc

# Source:	Alcatel-Lucent

# Title:	pCR 32.422 Definition and abbreviations related to MBSFN MDT

# Document for:	Discussion and approval

Agenda Item:	6.4.4

# 1	Decision/action requested

Discussion and approval

# 2	References

[1] 3GPP TS 36.300 Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); overall description; Stage 2

[2] 3GPP TS 37.320: "Universal Terrestrial Radio Access (UTRA) and Evolved Universal Terrestrial Radio Access (E-UTRA); Radio measurement collection for Minimization of Drive Tests (MDT); Overall description, Stage 2".

# 3	Rationale

Introduce definition and abbreviations related to MBSFN MDT in TS 32.422.

# 4	Detailed proposal

First modification

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [4] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [4].

Area based MDT: MDT data is collected from UEs in a specified area. The area is defined as a list of cells (UTRAN or E-UTRAN) or as a list of tracking/routing/location areas. The area based MDT is an enhancement of the management based trace functionality. Area based MDT can be either a logged MDT or Immediate MDT.

Immediate MDT: Collection of UE measurements in connected mode.

Logged MDT: Collection of UE measurements in idle mode.

Logged MBSFN MDT: Collection of MBSFN measurements in idle and connected mode. Applicable only for eUTRAN.

Signalling based MDT:MDT data is collected from one specific UE. The UE that is participating in the MDT data collection is specified as IMEI(SV) or as IMSI. The signalling based MDT is an enhancement of the signalling based subscriber and equipment trace. A signalling based MDT can be either a logged MDT or Immediate MDT.

MBSFN Area: See 3GPP TS 36.300 [37]

MBSFN Area Reserved Cell: See 3GPP TS 36.300 [37]

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [4], 3GPP TS 32.101 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [4].

AS	Application Server

BGCF	Breakout Gateway Control Function

CSCF	Call Session Control Function

I-CSCF	Interrogating-CSCF

IM CN SS	IP Multimedia Core Network Subsystem

IMEI-TAC	IMEI Type Allocation Code

MBSFN			MBMS over a Single Frequency Network

MBMS			Multimedia Broadcast Multicast Services

MDT	Minimization of Drive Tests

MRFC	Multimedia Resource Function Controller

P-CSCF	Proxy - CSCF

RCEF	RRC Connection Establishment Failure

RLF	Radio Link Failure

S-CSCF	Serving-CSCF

TAU	Tracking Area Update