# 3GPP TSG SA WG5 (Telecom Management) Meeting #100	S5-152333

# 13-17 April 2015 Dubrovnik, Croatia	revision of S5-140abc

# Source:	Alcatel-Lucent

# Title:	pCR 32.422 Changes related to ananymization for MBSFN MDT

# Document for:	Discussion and approval

Agenda Item:	6.4.4

# 1	Decision/action requested

Discussion and approval

# 2	References

[1] 3GPP TS 36.300 Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); overall description; Stage 2

[2] 3GPP TS 37.320: "Universal Terrestrial Radio Access (UTRA) and Evolved Universal Terrestrial Radio Access (E-UTRA); Radio measurement collection for Minimization of Drive Tests (MDT); Overall description, Stage 2".

# 3	Rationale

Introduce MBSFN MDT related changes in TS 32.422.

# 4	Detailed proposal

First modification

## 4.7	Anonymization of MDT data for area based MDT

If the job type is either Immediate MDT or Logged MDT or Logged MBSFN MDT or combined Immediate MDT and trace, the anonymization requirements are applicable as described in this clause.

In case of UTRAN/E-UTRAN the anonymization of MDT data depends on the configuration parameter received at the MDT configuration. There are two levels of anonymization:

-	Using IMEI-TAC.

-	Not sending any identity to the TCE.

If the anonymization parameter value is not known or the anonymization indicates that no identity should be sent to the TCE:

-	For E-UTRAN the eNB should not send the CELL TRAFFIC TRACE message to the MME.

-	For UTRAN the RNC should not send the UPLINK INFORMATION EXCHANGE REQUEST message to the SGSN/MSC Server.

If the anonymization indicates that the IMEI-TAC  is required:

-	For E-UTRAN eNB should send the CELL TRAFFIC TRACE message to the MME and shall put the following in the privacy indicator IE based on job type:

- “immediate MDT” if the job type is “Immediate MDT only”  and “Immediate MDT and Trace”

-	 “logged MDT” if the job type is “Logged MDT Only”

-	“logged MDT” if the job type is “Logged MBSFN MDT”

MME shall look up of the IMEI-TAC and send to the TCE if the privacy indicator indicates logged MDT or Immediate MDT

-	For UTRAN the RNC shall send the UPLINK INFORMATION EXCHANGE REQUEST message to the SGSN/MSC Server with IMSI information. SGSN/MSC Server shall find the corresponding IMEI and send the IMEI-TAC to the TCE.