# 3GPP TSG SA WG5 (Telecom Management) Meeting #102	S5-154184

# 24-28 August 2015, Beijing (China)	revision of S5-15xxxx

# Source:	NTT DOCOMO

# Title:	pCR TR 32.842 Addressing editor’s note in FM UC clause 5.1.5

# Document for:	Approval

# Agenda Item:	6.5.3 Study on Network Management of Virtualized Networks

# 1	Decision/action requested

Discuss and approve on the text proposal.

# 2	References

[REF-A]	TR 32.842 “Telecommunication management; Study on network management of Virtualized Networks”.

# 3	Rationale

Editor’s note in clause 5.1.5.1 requests clarifying the statement about “Refer to clause 5.1.4 in the present document for another option”.

The purpose of the UC “Failure management when the alarm is generated by VNFC” is to highlight the need of reporting about failures that are particularly relevant to the deployment of the VNF and constituent VNFCs on the virtualized infrastructure, and not to the specific VNF application faults. Therefore, it should not be stated as an option to UC in clause 5.1.4, but rather as complementary.

# 4	Detailed proposal

It is proposed to make the following changes to TR 32.842 v121 [REF-A].

1st proposed change

### 5.1.5	Failure management when the alarm is generated by VNFC

#### 5.1.5.1	Introduction

A VNF can be composed of a single or multiple VNF component(s) (VNFC). A VNFC is an internal component of a VNF providing a sub-set of that VNF's functionality defined by a VNF Provider, with the main characteristic that a single instance of this component maps one-to-one to a single virtualisation container [7]. A virtualisation container is a partition of a compute node that provides an isolated virtualized computation environment. An example of a virtualization container is a virtual machine (VM).

A single VNFC can fail due to diverse reasons. Possible failures may be caused by VNF software (e.g., drivers, the operating system supporting the VNFC-application specific software) or by the own VNFC deployment (e.g., runtime communication of a VNFC with another VNFC). Such failures can affect the overall VNF and need to be reported to trigger the automatic or on-demand healing of the VNF.

This use case exemplifies the failure management when the alarm is generated by the VNFC and such alarm is in the first instance sent to the VNFM (refer to clause 11.2 in [8] and step 4.2 in clause B.6 in [2] for examples).

#### 5.1.5.2	Actor

ETSI MANO VNFM.

#### 5.1.5.3	Pre-condition

The NFV management and orchestration (ETSI MANO) operation is active.

The 3GPP management operation is active.

The VNF is instantiated, configured (both deployment and application) and running.

The VNF may have detected an application failure and be already reported to EM.

#### 5.1.5.4	Description

The following steps are executed:

1.	The VNFC detects a failure concerning its own context (e.g. drivers, operating system, software, etc.) or the own VNF deployment (e.g. runtime communication of a VNFC with another VNFC).

2.	The VNF notifies the VNFM about the VNFC failure.

3.	The VNFM can correlate the VNFC failure with other virtualised resources failures received [2].

4.	In the case that the corrective action needs to be performed by the VNFM and the auto-healing is enabled, the VNFM can execute automatically the healing process notifying to the EM about the start and completion of the healing operation [2]. If the auto-healing is not enabled or the healing operation is not successful, the VNFM sends a notification of VNF failure to the EM.

5.	Same steps as stated in 3~7 in clause 5.1.2.4.

#### 5.1.5.5	Post-condition

If the VNF has self-healed, the network operation works normally, the operator knows the recovery from the VNF failure. If not, the operator knows about the VNF failure.

End