3GPP TSG-SA5 Meeting #104 	S5-156086

# 16-20 November 2015, Anaheim (USA)												revision of S5-15xabc

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.623 | CR | 0007 | rev | - | Current version: | 11.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add missing id attribute |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | OAM11 |  |  |  |  |  |  | Date: |  | 2015-10-28 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-11 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | - The id attribute, which is inherited from IOC Top_ in TS 28.620 via IOC ManagedFunction, is missing in the mapping table and IDL definitions for ManagedFunction. - In all IOCs except ManagedFunction and EP_RP, the id attribute is mapped to “xxxId” which is the old convention; the current convention is that it should be both named as “id” in IS and mapped to “id” in SS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the missing id attribute in the mapping tables and IDL definitions for ManagedFunction. - Follow the current convention that “id” in each IOC should be mapped to “id”. - Editorial changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The TS would have faults which potentially could lead to implementation errors and interoperability issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1,A.2.2.1, A.2.2.2, A.2.2.3, A.2.2.4, A.2.2.5, A.2.2.6, A.2.2.7, A.2.2.9, A.3.3. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

# 1	Scope

The TS 28.62x-series (Generic Network Resources IRP) define an Integration Reference Point (IRP) through which an "IRPAgent" (typically an Element Manager or Network Element) can communicate Network Management related information to one or several  "IRPManagers" (typically Network Managers).

This TS-family specifies a generic Network Resource Model, NRM (also referred to as a Management Information Model - MIM) with definitions of Information Object Classes (IOCs) and Managed Object Classes (MOCs).

# The present document specifies the Solution Set definition for the Generic NRM IRP.

# The Solution Set definition is related to 3GPP TS 28.622 V11.2.X [4].

Next Modified Section

## A.2.2	Information Object Class (IOC) mapping

This Solution Set supports reference attributes for relations other than containment relations between objects. Reference attributes are therefore introduced in each MOC where needed.

### A.2.2.1	IOC SubNetwork

Mapping from NRM IOC SubNetwork attributes to SS equivalent MOC SubNetwork attributes

| IS Attributes | SS Attributes | SS Type |
| --- | --- | --- |
| id | id | string |
| dnPrefix | dnPrefix | string |
| userLabel | userLabel | string |
| userDefinedNetworkType | userDefinedNetworkType | string |
| setOfMcc | setOfMcc | GenericNetworkResourcesIRPSystem::AttributeTypes::StringSet |

### A.2.2.2	IOC ManagedElement

Mapping from NRM IOC ManagedElement attributes and association roles to SS equivalent MOC ManagedElement attributes

| IS Attributes | SS Attributes | SS Type |
| --- | --- | --- |
| id | id | string |
| dnPrefix | dnPrefix | string |
| userLabel | userLabel | string |
| locationName | locationName | string |
| vendorName | vendorName | string |
| userDefinedState | userDefinedState | string |
| managedElementType | managedElementType | GenericNetworkResourcesIRPSystem::AttributeTypes::StringSet |
| managedBy | managedBy | GenericNetworkResourcesIRPSystem::AttributeTypes::MOReferenceSet |
| swVersion | swVersion | string |

### A.2.2.3	IOC MeContext

# Mapping from NRM IOC MeContext attributes to SS equivalent MOC MeContext attributes

| IS Attributes | SS Attributes | SS Type |
| --- | --- | --- |
| id | id | string |
| dnPrefix | dnPrefix | string |

### A.2.2.4	IOC ManagementNode

Mapping from NRM IOC ManagementNode attributes and association roles to SS equivalent MOC ManagementNode attributes

| IS Attributes | SS Attributes | SS Type |
| --- | --- | --- |
| id | id | string |
| userLabel | userLabel | string |
| locationName | locationName | string |
| vendorName | vendorName | string |
| userDefinedState | userDefinedState | string |
| managedElements | managedElements | GenericNetworkResourcesIRPSystem::AttributeTypes::MOReferenceSet |
| swVersion | swVersion | string |

### A.2.2.5	IOC VsDataContainer

Mapping from NRM IOC VsDataContainer attributes and association roles to SS equivalent MOC VsDataContainer attributes

| IS Attributes | SS Attributes | SS Type |
| --- | --- | --- |
| id | id | string |
| vsDataType | vsDataType | string |
| vsData | vsData | any |
| vsDataFormatVersion | vsDataFormatVersion | string |

### A.2.2.6	IOC ManagedFunction

Mapping from NRM IOC ManagedFunction attributes and association roles to SS equivalent MOC ManagedFunction attributes

| IS Attributes | SS Attributes | SS Type |
| --- | --- | --- |
| id | id | string |
| userLabel | userLabel | string |

### A.2.2.7	IOC IRPAgent

# Mapping from NRM IOC IRPAgent attributes to SS equivalent MOC IRPAgent attributes

| IS Attributes | SS Attributes | SS Type |
| --- | --- | --- |
| id | id | string |
| systemDN | systemDN | string |

### A.2.2.8	IOC Top

# Mapping from NRM IOC Top attributes to SS equivalent attributes in all MOCs

| IS Attributes | SS Attributes | SS Type |
| --- | --- | --- |
| objectClass | CLASS | string |
| objectInstance | No direct mapping |  |

### A.2.2.9	IOC Link

# Mapping from NRM IOC Link attributes to SS equivalent MOC IRPAgent attributes

| IS Attributes | SS Attributes | SS Type |
| --- | --- | --- |
| id | id | string |
| userLabel | userLabel | string |
| aEnd | aEnd | GenericNetworkResourcesIRPSystem::AttributeTypes::MOReference |
| zEnd | zEnd | GenericNetworkResourcesIRPSystem::AttributeTypes::MOReference |
| linkType | linkType | LinkTypeType |
| protocolName | protocolName | string |
| protocolVersion | protocolVersion | string |

### A.2.2.10	IOC EP_RP

# Mapping from NRM IOC EP_RP attributes to SS equivalent MOC EP_RP attributes

| IS Attributes | SS Attributes | SS Type |
| --- | --- | --- |
| id | id | string |
| userLabel | userLabel | string |
| farEndEntity | farEndEntity | GenericNetworkResourcesIRPSystem::AttributeTypes::MOReference |

Next Modified Section

## A.3.3	IDL specification "GenericNetworkResourcesNRMDefs.idl"

//File: GenericNetworkResourcesNRMDefs.idl

#ifndef _GENERIC_NETWORK_RESOURCES_NRM_DEFS_IDL_

#define _GENERIC_NETWORK_RESOURCES_NRM_DEFS_IDL_

// This statement must appear after all include statements

#pragma prefix "3gppsa5.org"

/**

* This module defines constants for each MO class name and

* the attribute names for each defined MO class.

*/

module GenericNetworkResourcesNRMDefs

{

/**

*  Definitions for MO class Top

*/

interface Top

{

// Attribute Names

//

const string CLASS = "Top";

};

/**

*  Definitions for MO class SubNetwork

*/

interface SubNetwork : Top

{

const string CLASS = "SubNetwork";

// Attribute Names

//

const string id = "id";

const string dnPrefix = "dnPrefix";

const string userLabel = "userLabel";

const string userDefinedNetworkType = "userDefinedNetworkType";

const string setOfMcc = "setOfMcc";

};

/**

*  Definitions for MO class ManagedElement

*/

interface ManagedElement : Top

{

const string CLASS = "ManagedElement";

// Attribute Names

//

const string id = "id";

const string dnPrefix = "dnPrefix";

const string managedElementType = "managedElementType";

const string userLabel = "userLabel";

const string vendorName = "vendorName";

const string userDefinedState ="userDefinedState";

const string locationName ="locationName";

const string managedBy = "managedBy";

const string swVersion = "swVersion";

};

/**

*  Definitions for MO class MeContext

*/

interface MeContext : Top

{

const string CLASS = "MeContext";

// Attribute Names

//

const string id = "id";

const string dnPrefix = "dnPrefix";

};

/**

*  Definitions for MO class ManagementNode

*/

interface ManagementNode : Top

{

const string CLASS = "ManagementNode";

// Attribute Names

//

const string id = "id";

const string userLabel = "userLabel";

const string vendorName = "vendorName";

const string userDefinedState = "userDefinedState";

const string locationName = "locationName";

const string managedElements = "managedElements";

const string swVersion = "swVersion";

};

/**

*  Definitions for abstract MO class ManagedFunction

*

*/

interface ManagedFunction : Top

{

const string CLASS = "ManagedFunction";

// Attribute Names

//

const string id = "id";

const string userLabel = "userLabel";

};

/**

*  Definitions for MO class IRPAgent

*/

interface IRPAgent : Top

{

const string CLASS = "IRPAgent";

// Attribute Names

//

const string id = "id";

const string systemDN = "systemDN";

};

/**

*  Definitions for abstract MO class Link

*  This inherits from ManagedFunction

*  The attributes aEnd and zEnd are populated with the DNs
       *  of the entities associated via the link class.
       *  The aEnd takes the DN of the 1st entity in alphabetical order,

*  the zEnd takes the 2nd entity in alphabetical order of the class

*  names.

*/

interface Link : ManagedFunction

{

const string CLASS = "Link";

// Attribute Names

//

const string aEnd = "aEnd";

const string zEnd = "zEnd";

const string linkType = "linkType";

const string protocolName = "protocolName";

const string protocolVersion = "protocolVersion";

};

/**

*  Definitions for MO class VsDataContainer

*/

interface VsDataContainer : Top

{

const string CLASS = "VsDataContainer";

// Attribute Names

//

const string id = "id";

const string vsDataType = "vsDataType";

const string vsData = "vsData";

const string vsDataFormatVersion = "vsDataFormatVersion";

};

/**

*  Definitions for abstract MO class EP_RP

*/

interface EP_RP : Top

{

const string CLASS = "EP_RP";

// Attribute Names

//

const string farEndEntity = "farEndEntity";

const string id = "id";

const string userLabel = "userLabel";

};      /**

*  This module adds datatypes definitions for the Link Class

*  These attributes are not the basic datatypes already defined

*/

module LinkAttributeTypes

{

enum LinkType

{

SIGNALLING,

BEARER,

OAM_AND_P,

OTHER

};

typedef sequence <LinkType> LinkTypeType;

};

};

#endif // _GENERIC_NETWORK_RESOURCES_NRM_DEFS_IDL_

End of modifications