3GPP TS 28.633 V13.1.0 (2016-03)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Telecommunication management;

Inventory Management (IM) Network Resource Model (NRM) Integration Reference Point (IRP);

Solution Set (SS) definitions

(Release 13)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. 
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.	 
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

NRM, IRP, Converged Management, Inventory Management

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2016, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	4

1	Scope	5

2	References	5

3	Definitions and abbreviations	5

3.1	Definitions	5

3.2	Abbreviations	6

4	Solution Set definitions	6

Annex A (normative):	XML definitions	7

A.0	General	7

A.1	Architectural features	7

A.1.0	Introduction	7

A.1.1	Syntax for Distinguished Names	7

A.2	Mapping	7

A.3	Solution Set definitions	7

A.3.1	XML definition structure	7

A.3.2	XML schema "inventoryNrm.xsd"	8

A.3.3	XML schema "inventoryNrmAlt2.xsd"	10

Annex B (informative):	Change history	13

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

The present document is part of a TS-family covering the 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Telecommunication management; as identified below:

32.690:	Inventory Management (IM); Requirements

28.631:	Inventory Management (IM) Network Resource Model (NRM) Integration Reference Point (IRP); Requirements

28.632:	Inventory Management (IM) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)

28.633:	Inventory Management (IM) Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions

Inventory Management (IM), in general, provides the operator with the ability to assure correct and effective operation of the 3G network as it evolves. IM actions have the objective to monitor the actual configuration on the Network Elements (NEs) and Network Resources (NRs), and they may be initiated by the operator or by functions in the Operations Systems (OSs) or NEs. The final goal of IM is the establishment of an accurate and timely model of the actual inventory in the NEs or NRs.

The present document covers the Inventory Management (IM) Network Resource Model (NRM) Integration Reference Point (IRP): Solution Set (SS) definitions.

# 1	Scope

The present document provides the NRM-specific part related to the Inventory Management NRM IRP IS in 3GPP TS 28.632 [1] of solution set definitions.

This Solution Set definitions specification is related to 3GPP TS 28.632 V 12.0.X.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 28.632: " Telecommunication management; Inventory Management (IM) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[2]	3GPP TS 32.612: "Telecommunication management; Configuration Management (CM); Bulk CM Integration Reference Point (IRP): Information Service (IS)".

[3]	3GPP TS 32.616: "Telecommunication management; Configuration Management (CM); Bulk CM Integration Reference Point (IRP): Solution Set (SS) definitions".

[4]	W3C REC-xml11-20060816: "Extensible Markup Language (XML) 1.1 (Second Edition)".

[5]	Void

[6]	W3C XML Schema Definition Language (XSD) 1.1 Part 1: Structures.

[7]	W3C XML Schema Definition Language (XSD) 1.1 Part 2: Datatypes.

[8]	W3C REC-xml-names-20060816: "Namespaces in XML 1.1 (Second Edition)".

[9]	3GPP TS 32.300: "Telecommunication management; Configuration Management (CM); Name convention for Managed Objects".

[10]	3GPP TS 32.342: "Telecommunication management; File Transfer (FT) Integration Reference Point (IRP): Information Service (IS)".

[11]	3GPP TS 28.623: “Generic network resources Integration Reference Point (IRP); Solution Set (SS) definition”.

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the following terms and definitions apply:

XML file: See definition of [11].

XML document: See definition of [11].

XML declaration: See definition of [11].

XML element: See definition of [11].

empty XML element: See definition of [11].

XML content (of an XML element): See definition of [13].

XML start-tag: See definition of [11].

XML end-tag: See definition of [11].

XML empty-element tag: See definition of [11].

XML attribute specification: See definition of [11].

DTD: See definition of [11].

XML schema: See definition of [11].

XML namespace: See definition of [11].

XML complex type: See definition of [11].

XML element type: See definition of [11].

## 3.2	Abbreviations

For the purposes of the present document, the following abbreviations apply:

CORBA	Common Object Request Broker Architecture

DTD	Document Type Definition

IM	Inventory Management

IRP	Integration Reference Point

IS	Information Service

NRM	Network Resource Model

XML	eXtensible Markup Language

XSD	XML Schema Definition

# 4	Solution Set definitions

This specification defines the following 3GPP Inventory Management (IM) Network Resource Model (NRM) IRP Solution Set (SS) definitions:

-	3GPP Inventory Management (IM) Network Resource Model (NRM) IRP XML definitions (Annex A)

CORBA Solution Set is not present in the current version of this specification.

Annex A (normative):
XML definitions

# A.0	General

This annex provides the NRM-specific part related to the Inventory Management NRM IRP IS in 3GPP TS 28.632 [1] of the XML file format definition for the Bulk Configuration Management IRP IS in 3GPP TS 32.612 [2] as well as for use with File Tranfer IRP [10].

The XML file formats are based on XML [4], XML Schema [6] [7] and XML Namespace [8] standards.

# A.1	Architectural features

## A.1.0	Introduction

The overall architectural feature of Inventory Management NRM IRP IS is specified in 3GPP TS 28.632 [1].

This clause specifies features that are specific to the XML Schema definitions.

## A.1.1	Syntax for Distinguished Names

The syntax of a Distinguished Name is defined in 3GPP TS 32.300 [9].

# A.2	Mapping

The mapping is not present in the current version of this specification.

# A.3	Solution Set definitions

## A.3.1	XML definition structure

The overall description of the file format of inventory data XML files is provided by 3GPP TS 32.616 [3].

A.3.2 defines the NRM-specific XML schema inventoryNrm.xsd for the Inventory Management NRM IRP IS alternative 1 defined in 3GPP TS 28.632 [1].

A.3.3 defines the NRM-specific XML schema inventoryNrmAlt2.xsd for the Inventory Management NRM IRP IS alternative 2 as defined in 3GPP TS 28.632 [1].

XML schema inventoryNrmAlt1.xsd and inventoryNrmAlt2.xsd explicitly declare NRM-specific XML element types for the related NRM.

The definition of those NRM-specific XML element types complies with the generic mapping rules defined in 3GPP TS 32.616 [3].

## A.3.2	XML schema "inventoryNrm.xsd"

The following XML schema inventoryNrm.xsd is the NRM-specific schema for the Inventory Management NRM IRP IS alternative 1 defined in 3GPP TS 28.632 [1].

<?xml version="1.1" encoding="UTF-8"?>

<!--
  3GPP TS 28.633 Inventory Management NRM IRP
  Inventory data file NRM-specific XML schema
  inventoryNrm.xsd
-->

<schema
  targetNamespace=
"http://www.3gpp.org/ftp/specs/archive/28_series/28.633#inventoryNrm"
  elementFormDefault="qualified"
  xmlns="http://www.w3.org/2001/XMLSchema"
  xmlns:xn=
"http://www.3gpp.org/ftp/specs/archive/28_series/28.623#genericNrm"
  xmlns:in=
"http://www.3gpp.org/ftp/specs/archive/28_series/28.633#inventoryNrm"
>

  <import
    namespace=
"http://www.3gpp.org/ftp/specs/archive/28_series/28.623#genericNrm"
  />

  <!-- Inventory Management Alternative 1 NRM IRP NRM class associated XML elements -->
  <simpleType name="eightOctetsType">

<restriction base="hexBinary">

<length value="8"/>

</restriction>

</simpleType>

<simpleType name="fourOctetsType">

<restriction base="hexBinary">

<length value="4"/>

</restriction>

</simpleType>

<simpleType name="angleValueType">

<restriction base="short">

<minInclusive value="0"/>

<maxInclusive value="3600"/>

</restriction>

</simpleType>

  <element
    name="InventoryUnit"
    substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass"
  >
    <complexType>
      <complexContent>
        <extension base="xn:NrmClass">
          <sequence>
            <element
             name="attributes"
             minOccurs="0">
              <complexType>
                <all>
                  <element
                    name="inventoryUnitType"
                    type="string"
                  />
                  <element
                    name="vendorUnitFamilyType"
                    type="string"
                    minOccurs="0"
                  />
                  <element
                    name="vendorUnitTypeNumber"
                    type="string"
                    minOccurs="0"
                  />
                  <element
                    name="vendorName"
                    type="string"/>
                  <element
                    name="serialNumber"
                    type="string"
                    minOccurs="0"/>
                  <element
                    name="dateOfManufacture"
                    type="date"
                    minOccurs="0"/>
                  <element 
                    name="dateOfLastService"
                    type="date"
                    minOccurs="0"/>
                  <element
                    name="unitPosition"
                    type="string"
                    minOccurs="0"/>
                  <element
                    name="manufacturerData"
                    type="string"
                    minOccurs="0"/>
                  <element
                    name="versionNumber"
                    type="string"
                    minOccurs="0"/>
                  <element name="relatedFunction" type="xn:dn" minOccurs="0"/>

</all>
              </complexType>
            </element>
            <choice minOccurs="0" maxOccurs="unbounded">
              <element ref="in:InventoryUnit"/>
              <element ref="xn:VsDataContainer"/>
              <element ref="in:TmaInventoryUnit"/>

<element ref="in:AntennaInventoryUnit"/>
            </choice>
          </sequence>
        </extension>
      </complexContent>
    </complexType>
  </element>
  <element name="TmaInventoryUnit" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from InventoryUnit-->

<element name="inventoryUnitType" type="string"/>

<element name="vendorUnitFamilyType" type="string" minOccurs="0"/>

<element name="vendorUnitTypeNumber" type="string" minOccurs="0"/>

<element name="vendorName" type="string"/>

<element name="serialNumber" type="string" minOccurs="0"/>

<element name="dateOfManufacture" type="date" minOccurs="0"/>

<element name="dateOfLastService" type="date" minOccurs="0"/>

<element name="unitPosition" type="string" minOccurs="0"/>

<element name="manufacturerData" type="string" minOccurs="0"/>

<element name="versionNumber" type="string" minOccurs="0"/>

<element name="relatedFunction" type="xn:dn" minOccurs="0"/>

<!-- End of inherited attributes from InventoryUnit -->

<element name="tmaNumberOfNonLinearGainValues" type="short" minOccurs="0"/>

<element name="tmaNonLinearGainValue" type="short" minOccurs="0"/>

<element name="tmaAdditionalDataFieldNumber" type="short" minOccurs="0"/>

<element name="tmaAntennaModelNumber" type="string" minOccurs="0"/>

<element name="tmaAntennaOperatingBands" type="short" minOccurs="0"/>

<element name="tmaBeamwidthForEachOpBandInBandOrder" type="in:eightOctetsType" minOccurs="0"/>

<element name="tmaGainForEachOpBandInBandOrder" type="in:fourOctetsType" minOccurs="0"/>

<element name="tmaInstallationDate" type="string" minOccurs="0"/>

<element name="tmaInstallersId" type="string" minOccurs="0"/>

<element name="tmaMaxSupportedGain" type="short" minOccurs="0"/>

<element name="tmaMinSupportedGain" type="short" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="in:InventoryUnit"/>

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="AntennaInventoryUnit" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from InventoryUnit-->

<element name="inventoryUnitType" type="string"/>

<element name="vendorUnitFamilyType" type="string" minOccurs="0"/>

<element name="vendorUnitTypeNumber" type="string" minOccurs="0"/>

<element name="vendorName" type="string"/>

<element name="serialNumber" type="string" minOccurs="0"/>

<element name="dateOfManufacture" type="date" minOccurs="0"/>

<element name="dateOfLastService" type="date" minOccurs="0"/>

<element name="unitPosition" type="string" minOccurs="0"/>

<element name="manufacturerData" type="string" minOccurs="0"/>

<element name="versionNumber" type="string" minOccurs="0"/>

<element name="relatedFunction" type="xn:dn" minOccurs="0"/>

<!-- End of inherited attributes from InventoryUnit-->

<element name="maxTiltValue" type="in:angleValueType" minOccurs="0"/>

<element name="minTiltValue" type="in:angleValueType" minOccurs="0"/>

<element name="mechanicalOffset" type="in:angleValueType" minOccurs="0"/>

<element name="baseElevation" type="integer" minOccurs="0"/>

<element name="latitude" type="decimal" minOccurs="0"/>

<element name="longitude" type="decimal" minOccurs="0"/>

<element name="patternLabel" type="string" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="in:InventoryUnit"/>

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>
</schema>

## A.3.3	XML schema "inventoryNrmAlt2.xsd"

The following XML schema inventoryNrmAlt2.xsd is the NRM-specific schema for the Inventory Management NRM IRP IS alternative 2 defined in 3GPP TS 28.632 [1].

<?xml version="1.1" encoding="UTF-8"?>

<!--

3GPP TS 28.633 Inventory Management NRM IRP

Inventory data file NRM-specific XML schema

inventoryNrmAlt2.xsd

-->

<schema xmlns="http://www.w3.org/2001/XMLSchema" xmlns:xn="http://www.3gpp.org/ftp/specs/archive/28_series/28.623#genericNrm" xmlns:in="http://www.3gpp.org/ftp/specs/archive/28_series/28.633#inventoryNrmAlt2" targetNamespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.633#inventoryNrmAlt2" elementFormDefault="qualified">

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.623#genericNrm"/>

<!-- Inventory Management  Alternative 2 NRM IRP NRM class associated XML elements -->

<element name="InventoryUnitNE" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<element name="neId" type="string"/>

<element name="customerIdentifier" type="string" minOccurs="0"/>

<element name="productName" type="string"/>

<element name="vendorName" type="string"/>

<element name="productType" type="string" minOccurs="0"/>

<element name="salesUniqueId" type="string" minOccurs="0"/>											<element name="operatorUniqueName" type="string" minOccurs="0"/>

<element name="siteId" type="integer" minOccurs="0"/>

<element name="additionalInformation" type="string" minOccurs="0"/>

<element name="hWList" type="xn:dnList" minOccurs="0"/>

<element name="sWList" type="xn:dnList" minOccurs="0"/>

<element name="lICList" type="xn:dnList" minOccurs="0"/>

<element name="mFunction" type="xn:dn" minOccurs="0"/>

</all>

</complexType>

</element>

<element ref="in:InventoryUnitNE" minOccurs="0" maxOccurs="unbounded"/>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="InventoryUnitHw" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<element name="hwId" type="string"/>

<element name="hwType" type="string"/>

<element name="hwName" type="string" minOccurs="0"/>

<element name="vendorName" type="string" minOccurs="0"/>

<element name="hwVersion" type="string"/>

<element name="salesUniqueId" type="string" minOccurs="0"/>

<element name="hwUnitLocation" type="string"/>

<element name="model" type="string" minOccurs="0"/>

<element name="hwCapability" type="string" minOccurs="0"/>

<element name="modificationDate" type="string" minOccurs="0"/>

<element name="manualDataEntry" type="string" minOccurs="0"/>

<element name="additionalInformation" type="string" minOccurs="0"/>

<element name="nEList" type="xn:dnList" minOccurs="0"/>

<element name="sWList" type="xn:dnList" minOccurs="0"/>

<element name="lICList" type="xn:dnList" minOccurs="0"/>

<element name="mFunction" type="xn:dn" minOccurs="0"/>

</all>

</complexType>

</element>

<element ref="in:InventoryUnitHw" minOccurs="0" maxOccurs="unbounded"/>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="InventoryUnitSw" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<element name="swId" type="string"/>

<element name="swName" type="string" minOccurs="0"/>

<element name="vendorName" type="string" minOccurs="0"/>

<element name="swVersion" type="string" minOccurs="0"/>

<element name="salesUniqueId" type="string" minOccurs="0"/>

<element name="classification" type="string"/>

<element name="swInstallationTime" type="dateTime" minOccurs="0"/>

<element name="swActivationTime" type="dateTime" minOccurs="0"/>

<element name="swStatus" type="string" minOccurs="0"/>

<element name="additionalInformation" type="string" minOccurs="0"/>

<element name="nEList" type="xn:dnList" minOccurs="0"/>

<element name="hWList" type="xn:dnList" minOccurs="0"/>

<element name="lICList" type="xn:dnList" minOccurs="0"/>

<element name="mFunction" type="xn:dn" minOccurs="0"/>

</all>

</complexType>

</element>

<element ref="in:InventoryUnitSw" minOccurs="0" maxOccurs="unbounded"/>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="InventoryUnitLic" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<element name="licId" type="string"/>

<element name="licType" type="string" minOccurs="0"/>

<element name="vendorName" type="string" minOccurs="0"/>

<element name="validity" type="string" minOccurs="0"/>

<element name="key" type="string" minOccurs="0"/>

<element name="licActivationTime" type="dateTime" minOccurs="0"/>

<element name="licStatus" type="string" minOccurs="0"/>

<element name="salesUniqueId" type="string" minOccurs="0"/>

<element name="additionalInformation" type="string" minOccurs="0"/>

<element name="nEList" type="xn:dnList" minOccurs="0"/>

<element name="hWList" type="xn:dnList" minOccurs="0"/>

<element name="sWList" type="xn:dnList" minOccurs="0"/>

<element name="mFunction" type="xn:dn" minOccurs="0"/>

</all>

</complexType>

</element>

<element ref="in:InventoryUnitLic" minOccurs="0" maxOccurs="unbounded"/>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

</schema>

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| 2014-06 | SA#64 | SP-140332 | 001 | - | upgrade XSD | 11.0.0 | 11.1.0 |
|  |  | SP-140358 | 002 | - | remove the feature support statements |  |  |
| 2014-09 | SA#65 | SP-140560 | 003 | - | Update the link from Solution Set to Information Service due to the end of Release 12 | 11.1.0 | 12.0.0 |
| 2016-01 | SA#70 |  |  |  | Upgrade to Rel-13 (MCC) | 12.0.0 | 13.0.0 |
| 2016-03 | SA#71 | SP-160031 | 006 | - | Make the XML schema well formed | 13.0.0 | 13.1.0 |