# 3GPP TSG SA WG5 (Telecom Management) Meeting #105	S5-161312

# 25-29 January 2016, Bratislava (Slovakia)	revision of S5-161363

# Source:	ZTE

# Title:	Add high level use case of enabling/disabling the auto-scaling of a VNF instance(s)

# Document for:	Approval

Agenda Item:	6.5.1.5

1	Decision/action requested

Agree to add the proposed use case.

2	References

[1]		3GPP TS 28. 525 “Telecommunication management; Life Cycle Management (LCM) for mobile networks that include virtualized network functions; Requirements“

3	Rationale

This contribution proposes the high level use case of enaling/disabling the auto-scaling of a VNF instance(s).

In some cases, for example, when the operator prepares the emergency response plan for an event, some VNFs may be expanded (scaled up or out), and the operator usually wants the capacities of these VNFs to be kept until the end of the event. So if these VNFs have auto-scaling functions, the operator may need to stop these auto-scaling functions temporary, and start them again after the event.

4	Detailed proposal

1st modified section

5.1	Requirements

REQ-MAMO-LCM-FUN-X EM shall have a capability allowing NM to enable/disable the auto-scaling of the corresponding VNF instance(s) for an NE.

REQ-MAMO-LCM-FUN-Y VNFM shall provide a capability allowing EM to enable/disable the auto-scaling of a specific VNF instance(s).

Note 1:	Disabling auto-scaling means VNFM is not allowed to start new auto-scaling operation to scale the corresponding VNF instance(s).

Note 2:	Enabling auto-scaling means VNFM is allowed to start new auto-scaling operation to scale the corresponding VNF instance(s).

2nd modified section

5.4	High-level use cases

5.4.x	Use case of enabling/disabling the auto-scaling of the corresponding VNF instance(s) for an NE

| Use Case | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | Enable/Disable the auto-scaling of a VNF instance(s). |  |
| Actors and Roles | NM requests the enabling/disabling the auto-scaling of the corresponding VNF instance(s) for an NE. |  |
| Telecom resources | EM, NM, VNFM |  |
| Assumptions | The auto-scaling, which means automatic procedure controlled and executed by the VNFM to scale the capacity of a VNF instance, is supported by VNFM. |  |
| Pre-conditions |  |  |
| Begins when | The operator decides to enable/disable the auto-scaling of the corresponding VNF instance(s) for an NE. |  |
| Step 1 (M) | NM requests EM to enable/disable the auto-scaling of the corresponding VNF instance(s) for the subject NE. |  |
| Step 2 (M) | EM requests VNFM to enable/disable the auto-scaling of the specific VNF instance(s). |  |
| Ends when | Ends when all steps identified above are completed or when an exception occurs. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The auto-scaling of the corresponding VNF instance(s) for an NE is enabled/disabled. |  |
| Traceability | REQ-MAMO-LCM-FUN-X, REQ-MAMO-LCM-FUN-Y |  |

End of modified sections