3GPP TR 32.817 V14.0.0 (2016-09)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Telecommunication management;

Study on management of measurement collection in network sharing context

(Release 14)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners’ Publications Offices.

Keywords

management, network sharing, measurement

3GPP

Postal address

3GPP support office address

650 Route des Lucioles – Sophia Antipolis

Valbonne – FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2016, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	4

1	Scope	6

2	References	6

3	Definitions and abbreviations	6

3.1	Definitions	6

3.2	Abbreviations	7

4	Use Cases	7

4.1	Business level use case	7

4.2	Service level use case	7

5	Potential requirements	7

5.1	Potential business level requirements	7

5.2	Potential specification level requirements	8

6	Potential solutions	8

6.1	Additions to Measurement type definitions	8

6.2	Use of PM IRP	9

7	Conclusions	10

8	Recommendations	10

Annex A:	Change history	11

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

Y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

Data volume measurements for RAN sharing are specified in TS 36.314[2]:

"4.1.9 	Data Volume for Shared Networks

The objective of this measurement is to measure the data volume transmitted or received by the eNB in a configured measurement period, for the different PLMN identities in a shared network.  The measurement is performed per configured PLMN ID per configured QoS profile criteria.

The QoS profile criteria may include one or more of the following criteria: one QCI indicator, one GBR Indicator, one ARP Indicator, where

-	a QCI Indicator identifies one specific QCI value. QCI values range from 1 to 256. If the indicator is not set then all QCI values should be taken into account.

-	a GBR Indicator identifies one GBR range value defined by OAM. If the indicator is not set then all GBR ranges should be taken into account.

-	an ARP Indicator identifies one ARP priority value. ARP values range from 1 to 15. If the indicator is not set then all ARP priority values should be taken into account.

The QoS profile criteria are satisfied when all the configured indicators are matched. It is possible that none of the QoS parameters are configured in which case the measurement is collected per configured PLMN ID.

4.1.9.1	Data volume for shared networks in DL

Protocol Layer: PDCP

| Definition | Data Volume for shared networks in DL. Amount of PDCP SDU bits in downlink delivered from PDCP layer to RLC layer in a measurement period. The measurement is performed per configured PLMN ID per configured DL QoS profile criteria. The unit is Mbit. |
| --- | --- |

4.1.9.2	Data Volume for shared networks in UL

Protocol Layer: PDCP

| Definition | Data Volume for shared networks in UL. Amount of PDCP SDU bits successfully received by the eNB in uplink in a measurement period. The measurement is performed per configured PLMN ID per configured UL QoS profile criteria. The unit is Mbit. |
| --- | --- |

"

TS 36.:300 [9] specified the following for Resource useage reporting for shared networks:

"23.12	Resource usage reporting for shared networks

The eNB may be configured to report the resource usage as data volume for the different PLMN identities of the operators in a shared network. The measurements, which are defined in TS 36.314 [63], are based on the QoS parameter values exchanged at E-RAB setup/modification.

OAM configures data volume reporting criteria. The configured data volume criteria define the PLMN IDs and the QoS profile criteria for the collection and reporting. The data volume reports are per configured PLMN ID and per configured QoS profile per DL/UL.

The QoS profile configuration shall be the same for all configured PLMN IDs and for all eNBs. The configured QoS profile criteria can differ in UL and DL in terms of GBR ranges. It shall be possible to collect and report up to 200 counter types and a maximum of 200 counter instances.

Data volumes for the DL/UL direction of a GBR bearer shall be collected within a given DL/UL GBR range if the DL/UL E-RAB guaranteed bit rate is within a configured DL/UL range delimited by a minimum and maximum value. The GBR ranges shall be five in number for DL and five in number for UL. GBR ranges shall be non-overlapping and configurable. The configured set of GBR ranges applies to the whole RAN and to all PLMN IDs in the RAN. "

This means that the PM mechanism needs to be changed to support limitation of the number of measurements. It is further necessary to ensure that the report format supports to describe how these measurements are defined or limited.

The solutions might be beneficial also for other measurements.

# 1	Scope

The study is to investigate:

-	Solutions for how to manage measurement collection in network sharing context over Itf-N.

-	Solutions for how to indicate high reliability to different E-UTRAN measurements.

-	Whether the PM file format would need to be changed and possible solutions.

The solution is intended to be valid for data volume measurements, but a more generically applicable solution should be considered, if feasible. The solution can be valid also for non-shared networks.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 36.314: "Evolved Universal Terrestrial Radio Access (E-UTRA); Layer 2 – Measurements".

[3]	 Void

[4]	3GPP TS 32.401: "Performance Management (PM); Concept and requirements".

[5]	3GPP TS 32.130: "Telecommunication management; Network sharing; Concepts and requirements".

[6]	3GPP TS 32.425: "Performance Management (PM); Performance measurements Evolved Universal Terrestrial Radio Access Network (E-UTRAN)".

[7]	3GPP TS 32.412: "Performance Management (PM) Integration Reference Point (IRP): Information Service (IS)".

[8]	3GPP TS 32.416: "Performance Management (PM) Integration Reference Point (IRP); Solution Set (SS) definitions".

[9]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

EUR	Euro

# 4	Use Cases

## 4.1	Business level use case

The operator has made an agreement to act as a Master Operator, MOP, for another operator (Participating Operator, POP) for RAN sharing, ref 3GPP TS 32.130 [5]. An example of an agreement is regulated in anstates the following:

The QoS profile criteria ARP 1 to ARP  10 and GBR-5 for QCI- 4 and QCI-8 are supported. The DL QCI-4, GBR-5, and ARP-1  to ARP 10 are charged x EUR per Mbit, while UL QCI-4, GBR-5, and ARP-1  to ARP 10 are charged y EUR per Mbit. QCI-8 is charged z EUR per Mbit regardless of QoS. 
For cross operator accounting purpose, the network needs to reliably provide data volume measurements for the used QoS profile criteria to the MOP.

## 4.2	Service level use case

The network manager requests a measurement job for data volume measurements for shared network (ref. 3GPP TS 32.130 [5]) for charging purposes with high reliability, for a subset of data volume counters per shared PLMN, per UL/DL traffic direction and per QoS profile criteria. The QoS profile criteria may include one or more of the following criteria: one QCI Indicator, one GBR Indicator, one ARP Indicator, where:

-	a QCI Indicator identifies one specific QCI value. QCI values range from 0 to 255. If the indicator is not set then all QCI values should be taken into account.

-	a GBR Indicator identifies one GBR range value defined by the management system. If the indicator is not set then all GBR ranges should be taken into account.

-	an ARP Indicator identifies one ARP value. ARP values range from 1 to 15. If the indicator is not set then all ARP values should be taken into account.

An overall maximum number of 200 counter instances (measurement instances) can be recorded per granularity period.

The measurement type is specified by sub-clause 4.1.9 in 3GPP TS 36.314 [2].
All the counters instances in the measurement job should be subject to the highest reliability as it is for cross operator accounting purpose. The levels of reliability and their definitions are vendor specific.

The network provides the statistic counters each granularity period.

# 5	Potential requirements

## 5.1	Potential business level requirements

REQ-NETSHAR_PM-CON-1		The MOP shall be able to charge the POPs for the data volume used by POP’s users per selected QCI criteria via statistical measurements defined for shared networks.

-	The QoS profile criteria may include QCI Indicator, GBR Indicator, ARP Indicator.

-	It shall be possible to differentiate between DL and UL.

REQ-NETSHAR_PM-CON-2		The MOP shall be able to set the level of reliability for the selected measurements defined for cross operator accounting purpose in shared networks.

REQ-NETSHAR_PM-CON-3		A maximum number of 200 counter instances (for measurements defined for cross operator accounting purpose in shared networks) can be recorded per granularity period.

## 5.2	Potential specification level requirements

REQ-NETSHAR_PM-FUN-1		The IRPAgent shall have the capability to support subscription of UL and DL data volume measurements per QoS profile criteria for cross operator accounting purpose from the IRPManager.
The QoS profile criteria may include one or more of the following criteria: one QCI indicator, one GBR Indicator, one ARP Indicator, where:

-	a QCI Indicator identifies one specific QCI value. QCI values range from 0 to 255. If the indicator is not set then all QCI values should be taken into account.

-	 a GBR Indicator identifies one GBR range value defined by the management system. If the indicator is not set then all GBR ranges should be taken into account.

-	 an ARP Indicator identifies one ARP value. ARP values range from 1 to 15. If the indicator is not set then all ARP values should be taken into account.

REQ-NETSHAR_PM-FUN-2		The IRPAgent shall be able to support that the IRPManager can set levels of reliability for the counter instances in a measurement job. The levels of reliability and their definitions are vendor specific.

REQ-NETSHAR_PM-FUN-3		The IRPAgent shall be able to support up maximum number of 200 counter instances to be recorded per granularity period of data volume measurements for cross operator accounting purpose.

REQ-NETSHAR_PM-FUN-4		The IRPAgent shall be able to support the IRPManager to access a file containing data volume measurements for cross operator accounting purposes every granularity period.

# 6	Potential solutions

## 6.1	Additions to	Measurement type definitions

(Ref: 3GPP TS 32.425 clause 4.4)

DL cell PDCP SDU Data Volume

a)	This measurement provides the Data Volume (amount of PDCP SDU bits) in the downlink delivered from PDCP layer to RLC layer.  The measurement is calculated per PLMN ID and per E-RAB QoS profile (QCI, ARP and GBR). 
The unit is Mbit.

b)	CC

c)	This measurement is obtained by counting the number of bits entering the eNodeB/RN. The measurement is performed at the PDCP SDU level. The measurement is performed per configured PLMN ID per configured DL QoS profile criteria. (See 3GPP TS 36.314, clause 4.1.9).

d)	Each measurement is an integer value representing the number of bits measured in Mbits. The number of measurements is equal to the number of PLMNs multiplied by the number of QoS profiles.

e)	The measurement name has the form DRB.PdcpSduVolumeDl_Filter 
Filter: PLMNID (1-6), QCImax (1-255), QCImin (1-255), ARPmax (1-15), ARPmin (1-15), GBR (1-5) 
[Total no. of measurement instances] x [no. of filter values for all  measurements] (DL and UL) ≤ 200. 
Example: A measurement with filter PLMN=2, QCImax=4, QCImin=2, ARPmax=10, ARPmin=1 and GBR=3 results in a measurement name: DRB.PdcpSduVolumeDl_Plmn2Qci2-4Arp1-10Gbr3.

f)	EUtranCellFDD
EUtranCellTDD

g)	Valid for packet switched traffic

h)	EPS

i)	Can be used by MOP for cross-operator accounting in shared networks.

UL cell PDCP SDU Data Volume

a)  This measurement provides the Data Volume (amount of PDCP SDU bits) in the uplink delivered from PDCP layer to RLC layer.  The measurement is calculated per PLMN ID and per E-RAB QoS profile (QCI, ARP and GBR). 
The unit is Mbit.

b)	CC

c)	This measurement is obtained by counting the number of bits entering the eNodeB/RN. The measurement is performed at the PDCP SDU level. The measurement is performed per configured PLMN ID per configured UL QoS profile criteria. (See 3GPP TS 36.314, clause 4.1.9).

d)	Each measurement is an integer value representing the number of bits measured in Mbits. The number of measurements is equal to the number of PLMNs multiplied by the number of QoS profiles.

e)	The measurement name has the form DRB.PdcpSduVolumeUl_Filter 
Filter: PLMNID (1-6), QCImax (1-255), QCImin (1-255), ARPmax (1-15), ARPmin (1-15), GBR (1-5) 
[Total no. of measurement instances] x [no. of filter values for all measurements] (DL and UL) ≤ 200. 
Example: A measurement with filter PLMN=2, QCImax=4, QCImin=2, ARPmax=10, ARPmin=1 and GBR=3 results in a measurement name DRB.PdcpSduVolumeUl_Plmn2Qci2-4Arp1-10Gbr3.f)	EUtranCellFDD
EUtranCellTDD

g)	Valid for packet switched traffic

h)	EPS

i)	Can be used by MOP for cross-operator accounting in shared networks.

## 6.2	Use of PM IRP

Ref: 3GPP TS 32.412

Proposed additions to 6.5.1	Definition and legal values

| measurementSubFilter | It specifies a sub part of a filter that determines the conditions for stepping a specific measurement. | Specified for all measurements that have the suffix "filter" in item e) of the measurement definition (see 3GPP TS 32.425) |
| --- | --- | --- |

Additions to 7.3.1	Operation createMeasurementJob

7.3.1.2	Input parameters

| measurementFilter | O | List of <MeasurementType.measurementSubFilter> | See clause 6.5.1 (definition and legal values). |
| --- | --- | --- | --- |

| Reliability | O | JobMeasurementJob.jobReliability | Vendor specific. Specifies reliability for a measurement job. |
| --- | --- | --- | --- |

# 7	Conclusions

The proposed solutions are fulfilling the request to facilitate cross operator accounting for shared networks. The solutions can also be used for other new measurements.

# 8	Recommendations

The solutions are proposed to be implemented in 3GPP TS 32.425 [6], 3GPP TS 32.412 [7] and 3GPP TS 32.416 [8). The requirements are to be specified in 3GPP TS 32.130 [5].

Annex A:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | Tdoc | CR | Rev | Cat | Subject/Comment | New version |
| 2016-09 | SA#73 | SP-160619 |  |  |  | Presented for information and approval | 1.0.0 |
| 2016-09 | SA#73 |  |  |  |  | Upgrade to change control version | 14.0.0 |
| 2016-12 | SA#74 | SP-160856 | 0001 | 1 | F | Replacing the reference to LS with reference to TS 36.300 | 14.1.0 |