# 3GPP TSG SA WG5 (Telecom Management) Meeting #112	S5-171729

# 27-31 March 2017, Guilin (China)	revision of S5A-17xabc

# Source:	3GPP CT1

# Title:	Reply LS to the progress of QoE Measurement Collection for Streaming to RAN3, SA4, SA5 and CT1

# Document for:	Information, Discussion

Agenda Item:	6.1

# 3GPP TSG CT WG 1 Meeting 101bis	TDoc C1-170510

Spokane (WS), USA, 16-20 January 2017

Title:	Reply LS to the progress of QoE Measurement Collection for Streaming to RAN3, SA4, SA5 and CT1

Response to:	LS C1-170291/R2-168917on LS on the progress of QoE Measurement Collection for Streaming to RAN3, SA4, SA5 and CT1

Release:	Rel-14

Work Item:	UMTS_QMC_Streaming

Source:	3GPP CT WG1

To:	3GPP RAN WG2, 3GPP SA WG4

Cc:	3GPP RAN WG3, 3GPP SA WG5

Contact person:	Christian Herrero-Veron

Christian dot Herrero at Huawei dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

# 1	Overall description

CT1 thank RAN2 for their LS in C1-170291/R2-168917 and the information provided.

CT1 have discussed the content of the LS and would like to provide the following information:

- The NAS protocol used for UTRAN (Iu mode) has no means to transport the reporting container of a maximum size of 60,000 bytes;

- substantial and extensive changes would be required to be able to transport the reporting container and this would require changes in both the UE and the SGSN. The specifications impacted would be TS 24.008, and 24.007; and

- it may be possible to use an AT command to deliver the reporting container data to the access stratum but due to the size of the container this would require detail analysis from CT1. This would require at least changes to TS 27.007.

Since in CT1 understanding the reporting container is transparent to NAS as it is used by application layer, CT1 wonder:

- Whether this container can be transported by means of application layer protocol rather than NAS protocol, e.g. TCP.

- Whether the usage of NAS protocol for sending the required container with its maximum size (60,000 bytes) can be accommodated by signalling radio bearers in UTRAN.

# 2	Actions

To 3GPP RAN WG2

ACTION: 	CT1 kindly ask RAN2 to consider the information provided by this LS and answers the questions (1) and (2) above.

To 3GPP SA WG4

ACTION: 	CT1 kindly ask SA4 to consider the information provided by this LS and answer the question (1) above.

# Dates of next TSG CT WG1 meetings

TSG CT WG1 Meeting 102	13 -17 February 2017	Dubrovnik, Croatia, EU

TSG CT WG1 Meeting 103	3- 7 April 2017	Spokane (WS), USA