# 3GPP TSG SA WG5 (Telecom Management) Meeting #115	S5-176240

# 27 November-1 December 2017, Reno, Nevada, USA	revision of S5A-17xabc

# Source:	Ericsson

# Title:	Overview of use cases and requirements for management of network slicing

# Document for:	Discussion

Agenda Item:	6.4.3

# 1	Decision/action requested

The group is asked to endorse this contribution

# 2	References

[1]	3GPP TR 28.801 Study on anagement and orchestration of network slicing for next generation network

[2]	3GPP TS 28.530 v0.3.0

[3]	S5-175224 pCR 28.530 Overview of use cases and requirements for management of network slicing

# 3	Rationale

The study for management and orchestration of network slicing for next generation network [1] has identified management functions that are used for the creation and management of network slice instances. Here the needed concept UC Network Slices are discussed.

At the meeting #115 in Busan, Ericsson presented discussion paper [3] on message exchange between consumer and producer using asynchronous message transfer methods. The 3GPP IRPs however support synchronous message transfer method. This discussion paper has been updated to reflect synchronous mode and therefore enabling reuse of the 3GPP IRPs.

## 3.1	Functional view

The management functions are the CSMF (Communication Service Management Function), the NSMF (Network Slice Management Function) and, the NSSMF (Network Slice Subnet Management Function), as shown in Figure 1.

Figure 1: The management functions for network slice management

Each function has a defined role as described in [1] and each function supports business level use cases to enable an operator (network operator or service provider) to manage a network slice instance during its lifecycle.

## 3.2	Use cases

The operator or the CSMF of the operator interacts with the NSMF. The following use cases describe this interaction:

1) Create a network slice instance

2) Activate a network slice instance

3) Modify a network slice instance

4) De-activate a network slice instance

5) Terminate a network slice instance

To satisfy these use cases, the NSMF interacts with an NSSMF that is responsible for the network slice subnet Instance.  The following use cases describe this interaction:

1) Create a network slice subnet instance

2) Activate a network slice subnet instance

3) Modify a network slice subnet instance

4) De-activate a network slice subnet instance

5) Terminate a network slice subnet instance

The use cases for network slicing as discussed in [1] have an underlying pattern this is best described via a consumer – producer model, where the consumer sends a request to the producer the producer responds/replies to the request. Since the producer (NSMF) relies on other producers (NSSMFs) the completion of a request and subsequent reply depends on many factors.

## 3.3	Use case pattern

The use case patterns are based on a consumer-producer paradigm. For the use cases for network slice management the consumer can be the CSMF or a person in the Operators’ organization. The producer can be the NSMF, the NSMF manages all network slices that fall under the responsibility of the Operators’ organization. For the use cases for network slice subnet management the consumer is the NSMF (the same NSMF that is the producer for the network slice management) and the producer is the NSSMF.

The basic pattern identified for the use cases: create, activate, modify, de-activate, and terminate are shown in figure 2, 3, and 4.

Figure 2: Create and terminate object instance pattern

Create object instance

Consumer requests creation of an object instance by supplying the object instance information (see interaction 1). Producer replies indicating the object instance is created, the creation failed or the request was rejected (see interaction 2). In case the object instance is created the producer provides the identified of the created object instance. In case the request failed or was rejected the Producer provides failure or reject reason..

Terminate object instance

Consumer requests termination of an object instance by supplying the object instance id information (see interaction 3). Producer replies indicating the request is accepted, failed or rejected (see interaction 4). Producer provides the failure or reject reason..

Figure 3: Modify object instance pattern

Modify object instance

Consumer requests modification of an object instance by supplying the object instance id and modification (see interaction 1). Producer replies indicating the request is accepted, failed or rejected (see interaction 2). Producer provides the reject or failure reason.

Figure 4: Activate and de-activate object instance pattern

Activate object instance

Consumer requests activation of an object instance by supplying the object instance id (see interaction 1). Producer replies indicating the request is accepted, failed or rejected (see interaction 2). Producer provides the failure or reject reason..

De-activate object instance

Consumer requests de-activation of an object instance by supplying the object instance id (see interaction 3). Producer replies indicating the request is accepted, failed or rejected (see interaction 4). Producer provides the failure or reject reason.

.

# 4	Detailed proposal

The use cases and requirements are introduced by individual pCR’s for both network slice and network slice subnet, to be included in TS 28.530 [2]:

pCR’s for the network slice use cases and requirements:

Use case and requirement for create a network slice instance (update of existing UC in [2].)

Use case and requirement for activate a network slice instance

Use case and requirement for deactivate a network slice instance

Use case and requirement for modify a network slice instance

Use case and requirement for terminate a network slice instance

pCR’s for the network slice subnet use cases and requirements:

Use case and requirement for create a network slice subnet instance (update of existing UC in [2].)

Use case and requirement for activate a network slice subnet instance

Use case and requirement for deactivate a network slice subnet instance

Use case and requirement for modify a network slice subnet instance

Use case and requirement for terminate a network slice subnet instance