# 3GPP TSG SA WG5 (Telecom Management) Meeting #117	S5-181379

# 29 January - 2 February 2018, Rome, Italy	revision of S5-18133

# Source:	ETRI

# Title:	Proposed addition of Business level general requirements

# Document for:	Approval

Agenda Item:	6.4.1

1	Decision/action requested

The group is asked to discuss and approve the contribution.

2	References

[a]	TS 28.530 v0.3.0: “Management of network slicing in mobile networks; Concepts, use cases and requirements”.

3	Rationale

This pCR proposes to add additional Business level general requirements.

4	Detailed proposal

It is proposed to make the following changes to TR 28.530.

1st modified section

5	Business level requirements

5.1	Requirements

5.1.1	General requirements

REQ-5GNS-CON-01 The network slicing management architecture shall allow any deployment options within the Network Operator’s domain.

REQ-5GNS-CON-02	 The set of network slicing management functions shall be generic to all kinds of network function and network function provider.

REQ-5GNS-CON-03	 The interfaces in between the network slicing management function(s) composing the network slicing management function set shall be standardized.

Editor’s note: the set of network slicing management functions identified in TR 28.801 are: CSMF, NSMF, NSSMF.

REQ-5GNS-CON-04 The interfaces between the network slicing management function set and the network function, or its network function related management function, shall be standardized.

Editor’s note: the set of network slicing management functions identified in TR 28.801 are: CSMF, NSMF, NSSMF.

REQ-5GNS-CON-05 The network slicing management architecture shall provide capabilities to manage the total view of all created slice instances.

REQ-5GNS-CON-06 The network slicing management architecture should provide management capabilities that are dedicated to each slice instance. The instance management dedicated to a slice is required to work independently from the instance management dedicated to another slice.

REQ-5GNS-CON-07 The network slicing management architecture shall allow managing multiple slices (as slice instances) simultaneously or independently along with their lifecycle.

End of modified sections