# 3GPP TSG SA WG5 (Telecom Management) Meeting #118	S5-182248

# 9 - 13 April 2018, Beijing, China	revision of S5-182abc

# Source:	Huawei

# Title:	pCR 28.531 Add UC of network slice subnet configuration of slice constituents

# Document for:	Discussion and approval

Agenda Item:	6.4.2

1	Decision/action requested

It is asked to discuss and approve this contribution.

2	References

[1]	Draft 3GPP TS 28.530: "Management of network slicing in mobile networks Concepts, use cases and requirements".

[3]	Draft 3GPP TS 28.531: "Management of network slicing in mobile networks Concepts, use cases and requirements".

[3]	Draft 3GPP TS 23.501: “System Architecture for the 5G System".

[4]	Draft 3GPP TS 23.502: “Procedures for the 5G System Stage 2".

3	Rationale

In 3GPP TS 28.530 [1], the NSIs should be instantiated and configured in order to create a slice.

----------------------------------------------- Extract from 3GPP TS 28.530 [1] ---------------------------------------------

4.3.3	Commissioning

NSI provisioning in the commissioning phase includes creation of the NSI. During NSI creation all needed resources are allocated and configured to satisfy the network slice requirements. The creation of an NSI can include creation and/or modification of the NSI constituents.

------------------------------------------------ -----   End of the extract ---------------------------------------------------------------

In the use case of Network slice subnet instance creation in 3GPP TS 28.531 [2], configurations to the NSSI constituents is performed by NSSMF

----------------------------------------------- Extract from 3GPP TS 28.531 [2] ---------------------------------------------

  - 5.1.2	Network slice subnet instance creation

| Step 7 (M) | NSSMF configures the NSSI constituents. In case of RAN NSSI, the configuration contains RRM policy information for individual Radio cells. In the cells shared by multiple NSSIs such policy includes guidance for split of Radio resources between the NSSIs. Editor’s NOTE: other aspects of how NSSMF configures the NSSI constituents are FFS. |  |
| --- | --- | --- |

------------------------------------------------ -----   End of the extract --------------------------------------------------------------

Besides basic configuration management performed by O&AM, configurations on network and network slice operation are also configured locally at NFs by O&AM shown in 23.501 and 23.502 [3,4].

----------------------------------------------- Extract from 3GPP TS 23.501 [3] ---------------------------------------------

The NRF may be configured by OAM with information on the available UPF(s) or the UPF may register itself onto the NRF.

------------------------------------------------ -----   End of the extract --------------------------------------------------------------

----------------------------------------------- Extract from 3GPP TS 23.502 [4] ---------------------------------------------

NOTE 1:	The NF profile of NF service consumer interact with NRF are configured by OAM system.

------------------------------------------------ -----   End of the extract --------------------------------------------------------------

----------------------------------------------- Extract from 3GPP TS 23.502 [4] ---------------------------------------------

NOTE:	The updated NF profile of NF instance are configured by OAM system.

------------------------------------------------ -----   End of the extract --------------------------------------------------------------

Thus, to configure the NSSI that used to support the requested NSI, the configuration management (CM) service should be provided by 3GPP management system (e.g., NSSMF). Moreover, according the network and network slicing operation information, the CM service provider of a NSSI should be able to derive and decompose configurations (operation information) from the service comsumer into configuration requests to corresponding CM service providers (e.g., CM of other NSSI, CM of NFs).

----------------------------------------------- Extract from 3GPP TS 28.531 [2] ---------------------------------------------

  - 4.2	Configuration information for the constituents of an NSI

To use network slice to support communication service or deliver a network slice as a service, the 3GPP defined constituents of the NSI should be configured by 3GPP management system according to the types and requirements of the network slice so that the NSI can be operated and maintained.

The configuration information of these components may include:

- Slice differentiation information: Required information to differentiate NSIs if the constituent is shared by multiple NSIs (e.g., a set of different configurations for NG-RAN for different network slices);

- Network function selection information: Information on the selection of required functionalities of NFs (e.g., AMF) according to the requirements of this NSI;

- Connection information: The logical link information for the NFs (e.g., UPF) to communicate with appropriate NFs and NSSIs (e.g. CN, RAN subnets) belonging to the NSI;

- Operation information: The required information for the operation of the NF (e.g., UPF topology for the SMF);

NOTE 1: the list of information above is not exhaustive.

NOTE 2: the list of information above is not all necessary for an NSI

For non-3GPP domain of the NSI, 3GPP management system sends the configuration requirements to corresponding non-3GPP management system.
------------------------------------------------ -----   End of the extract -----------------------------------------------------------------

In this contribution, a use case to describe the CM service of a NSSI is proposed to TS 28.531 [2].

4	Detailed proposal

It is proposed to make the following changes to TS 28.531 [2].

Start First Change

5.2.x	Network slice subnet configuration of slice constituents

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To provide service for slice-specific (re)configuration of NSSI constituents. |  |
| Actors and Roles | NSS CM service consumer (e.g., the operator or NSMF) |  |
| Telecom resources | NSS CM service provider (e.g., NSSMF) Network slice subnet instance NF(s) |  |
| Assumptions | NSS CM service consumer is authorized by the NSS CMservice provider to provide slice operation information (see 4.2 in 28 531 [2]) for (re-)configuring NSSI constituents |  |
| Pre-conditions | NSSI is created or is in the creation phase. |  |
| Begins when | NSS CM service consumer wants to (re-)configure the constituents of a NSSI (e.g., adding new network slice information to NSSF and UPF information to NRF or SMF). |  |
| Step 1 (O) | NSS management service consumer obtains Network slice subnet instance information | 28.531 5.1.5 |
| Step 2 (M) | NSS management service consumer sends requests to NSS management service provider with slice operation information for (re-)configuring a network slice subnet consitituent. NOTE: The requests may affect (one or a group of) specific (common) constituent (s) (RAN, dedicated NFs, shared NFsTNs). How to provide the operation information of TNs is FFS. NOTE: The needs of a standardized format (e.g., a specification) of this slice operation information is FFS. |  |
| Step 3 (M) | NSS management service provider (derives and) decomposes the recieved slice operation information, and then makes them as separate CM requests for each constituent if necessary and applicable. These (decomposed) requests may require services from other CM service providers (e.g., other NSS service providers, CM of NFs). NOTE: The configurations should not conflict with other configurations, related UCs are FFS. These requests may include two types: - Configuration of dedicated NFs: The NSS management service consumer requests to (re-)configure to a specific constituent of the NSS (e.g., configure the SMF with the information of new instantiated UPFs, see 6.3.2, 6.3.3 in [3]) - Configuration of shared NFs :The NSS management service consumer requests to (re-)configure a common (shared) constituent (e.g., see 6.3.3 for NRF in [3]) of the NSS with slice operation information (see 4.2 in 28.531 [2]) so that this information can be accessed by other constituents of the NSS (e.g., NSSF, AMF, SMF) NOTE: The needs of a standardized format (e.g., a specification) of this slice operation information in NRF or other applicable NFs is FFS. |  |
| Step 4 (M) | NSS management service provider processes the requests. If other CM service providers are involved, CM requests are sent to corresponding CM service providers |  |
| Step 5 (O) | NSS management service provider receives the processing result from other CM service providers. |  |
| Step 6 (M) | NSS management service provider sends the processing result to NSS management service consumers. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The required (re)configuration is configured at the corresponding constituent(s). |  |
| Traceability | REQ-NSM_NSSMSP-FUN-x |  |

Start Second Change

5.2.3	Requirements for interface provided by NSSMF

REQ-NSM_NSSMSP-FUN-x	The NSS management service provider shall have the capability allowing its consumer to provide slice specific operation information for the (re)configuration to a NSS constituent.

End of changes