# 3GPP TSG SA WG5 (Telecom Management) Meeting #119	S5-183207

# 14-18 May 2018, La Jolla (US)

# Source:	China Mobile, Huawei

# Title:	pCR 28.552 Add PRB Usage measurements

# Document for:	Approval

Agenda Item:	6.4.5

1	Decision/action requested

It is asked to discuss and approve this contribution.

2	References

[1]	3GPP TS 28.552 Management and orchestration of networks and network slicing; NR and NG-RAN performance measurements and assurance v0.2.0

[2]	3GPP TS 32.404 Performance Management (PM); Performance measurements; Definitions and template v14.1.0

[3]	3GPP TS 32.425 Performance Management (PM); Performance measurements Evolved Universal Terrestrial Radio Access Network v15.0.0

[4]           3GPP TS 36.314 Evolved Universal Terrestrial Radio Access (E-UTRA); Layer 2 - Measurements v15.0.0

3	Rationale

The proposed radio resource utilization related measurements (Total PRB Usage) can enable the operator to monitor the resource utilization status of the radio network.

4	Detailed proposal

Start of Modified Section

5	Performance measurements and assurance data for gNB

5.2	Performance measurements and assurance data for gNB-DU

5.2.x	Radio resource utilization related measurements

5.2.x.y	DL Total PRB Usage

- This measurement provides the total usage (in percentage) of physical resource blocks (PRBs) on the downlink for any purpose.

- SI

- This measurement is obtained as: , where is the DL total PRB usage, which is percentage of PRBs used, averaged during time period  with value range: 0-100%; is a count of full physical resource blocks and all PRBs used for DL traffic transmission shall be included; is total number of PRBs available for DL traffic transmission during time period ; and is the time period during which the measurement is performed.

- A single integer value from 0 to 100.

- RRU.PrbTotDl, which indicates the DL PRB Usage for all traffic

- GNBDUFunction. NRCellDU (ENGNBDUFunction.NRCellDU for NR option 3)

- Valid for packet switched traffic

- NR and NG-RAN

5.2.x.z	UL Total PRB Usage

- This measurement provides the total usage (in percentage) of physical resource blocks (PRBs) on the uplink for any purpose.

- SI

- This measurement is obtained as: , where is the UL total PRB usage, which is percentage of PRBs used, averaged during time period  with value range: 0-100%; is a count of full physical resource blocks and all PRBs used for UL traffic transmission shall be included; is total number of PRBs available for UL traffic transmission during time period ; and is the time period during which the measurement is performed

- A single integer value from 0 to 100.

- RRU.PrbTotUl, which indicates the UL PRB Usage for all traffic

- GNBDUFunction. NRCellDU (ENGNBDUFunction.NRCellDU for NR option 3)

- Valid for packet switched traffic

- NR and NG-RAN

End of Modified Section