3GPP TSG-SA5 Meeting #122	S5-187055

# Spokane,US, 12-16 November 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0013 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add use case and definitions of PDCP data volume measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_SLICE_ePA |  |  |  |  |  | Date: |  |  | 2018/11/2 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the dual connectivity scenarios, the monitoring of the flow split is needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Definitions of PDCP PDU Volume are respectively added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The monitoring of the flow split in DC is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.X (new), 5.1.1.X.1 (new), 5.1.1.X.2 (new), A.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st modified section

5.1.1.X  PDCP PDU data volume Measurement

5.1.1.X.1 DL cell PDCP PDU Data Volume

- This measurement provides the Data Volume (amount of PDCP PDU bits) in the downlink delivered from GNBCUUPFunction to GNBDUFunction. The measurement is split into subcounters bear type.

b)	CC

c)	This measurement is obtained by counting the number of DL PDCP PDU bits on GNBCUUPFunction sent to GNBDUFunction throughput F1-U.

d)	Each measurement is an integer value representing the number of bits measured in Mbits (1MBits=1000*1000 bits).

e)	The measurement name has the form DRB.PDCPPduVolume.X

where X is SCGSplit,MCGSplit,MCG,SCGcorresponsing to MR-DC or EN-DC configuration.

f)	NRCellCU

g)	Valid for packet switched traffic

h)	5GS

5.1.1.X.2 UL cell PDCP PDU Data Volume

a)  This measurement provides the Data Volume (amount of PDCP PDU bits) in the uplink GNBCUUPFunction received from GNBDUFunction. The measurement is split into subcounters bear type for MR-DC or EN-DC.
The unit is Mbit (1MBits=1000*1000 bits).

b)	CC

c)	This measurement is obtained by counting the number of bits entering the GNBCUUPFunction. The measurement is performed at the PDCP PDU level.

d)	Each measurement is an integer value representing the number of bits measured in Mbits.

e)	The measurement name has the form DRB.RLCSduVolumeUl.X

where X is SCGSplit,MCGSplit,MCG,SCGcorresponsing to MR-DC or EN-DC configuration.

f)	NRCellCU

g)	Valid for packet switched traffic

h)	5GS

Next modified section

A.x Monitoring of cell PDCP PDU Data Volume at MR-DC or EN-DC

Monitoring of the PDCP PDU data volume combined with RLC SDU data volume of GNBCellDU measurement  at MR-DC or EN-DC scenario is valuable information for operators to monitor the potential problem for DC.

End of modifications