3GPP TSG-SA5 Meeting #122	S5-187056

# Spokane,US, 12-16 November 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0014 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add use case and definitions of PDCP throughput measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_SLICE_ePA |  |  |  |  |  | Date: |  |  | 2018/11/2 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Monitoring of PDCP layer fully buffered throughput is helpful to evaluate the NR RAN congestion and network expanding, and is also useful to analyse the performance of flow controlling features. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Definitions of PDCP throughput are respectively added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The monitoring of PDCP layer throughput is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.X (new), A.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st modified section

5.1.1.X  Average DL UE fully buffered Throughput

- This measurement provides the average down link fully buffered UE throughput of NR UE on GNBCUUPFuntion. This measurement is intended for UE throughput when PDCP buffer size is more the desired buffer size for DU’s   .This measurement only consider  PDCP buffering excluding the volume and time of the not fully buffered  based on  DDDS feedback [TS 38.425]. The fully buffered state is judged by PDCP based on “Desired buffer size for the data radio bearer” from DU. The UE data volume refers to the total volume scheduled for each UE regardless if using primary- or supplemental aggregated carriers. The measurement is optionally split into subcounters per bear Type or per QoS level (5QI or QCI in NR option 3).

- DER(N=1)

This measurement is obtained according to the following formula based on the “ThroughputVolume”, “ThroughputTime”. It is optionally split into subcounters for each bear Type or each QoS level.

Figure 1

Table 5.1.1.X-1

| T0 | T0 : PDCP SDU volume size to be delivered is more than desired buffer size for the data radio bearer of RLC entities (for split bear considering first & secondary) based on desired buffer size per UE per bear through F1 interface message DL DATA DELIVERY STATUS [TS 38.425] . |
| --- | --- |
| T1 | T1 : PDCP SDU volume size to be delivered is less than desired buffer size for the data radio bearer of RLC entities (for split bear considering first & secondary) based on desired buffer size per UE per bear through F1 interface message DL DATA DELIVERY STATUS after T0. |
| T2 | T2 : PDCP SDU volume size to be delivered begins more than buffer size for the data radio bearer of first RLC entities based on the DL DATA DELIVERY STATUS . |
| T3 | T3 : PDCP SDU volume size to be delivered is more than buffer size for the data radio bearer of first RLC entities based on the last second DL DATA DELIVERY STATUS before T4. |
| T4 | T4 : PDCP SDU volume size to be delivered is less than buffer size for the data radio bearer of secondary RLC entities based on the DL DATA DELIVERY STATUS after T2. |
| T5 | T5 : PDCP SDU volume size to be delivered begins more than buffer size for the data radio bearer of secondary RLC entities based on the DL DATA DELIVERY STATUS . |
| T6 | T6 : PDCP SDU volume size to be delivered is more than buffer size for the data radio bearer of secondary RLC entities based on the last second DL DATA DELIVERY STATUS before T4. |
| T7 | T7 : PDCP SDU volume size to be delivered is less than buffer size for the data radio bearer of secondary RLC entities based on the DL DATA DELIVERY STATUS after T2. |
| Tfullybufferedleg1 | T3-T2 |
| Tfullybufferedleg2 | T6-T5 |
| TfullybufferedSplit | T1-T0 |
| ThroughputTime | If split bear ,ThroughputTime=TfullybufferedSplit Otherwise ThroughputTime= Tfullybufferedleg1+Tfullybufferedleg2 NOTE: ThroughputTime.MCG,ThroughputTime. SCG equals the fully buffered time of actually used leg. |
| ThrVolfullybuffereSplit | The data PDCP SDU volume for the first RLC entity per UE per bear, in kbits successfully transmitted (acknowledged by) in DL when PDCP SDU volume size to be delivered is more than buffer size for the data radio bearer of first and secondary RLC entities. |
| ThrVolfullybufferedleg1 | The data PDCP SDU volume for the first RLC entity per UE per bear, in kbits successfully transmitted (acknowledged by) in DL when PDCP SDU volume size to be delivered is more than buffer size for the data radio bearer of first RLC entity. |
| ThrVolfullybufferedleg2 | The data PDCP SDU volume for the secondary RLC entity per UE per bear, in kbits successfully transmitted (acknowledged by ) in DL when PDCP SDU volume size to be delivered is more than buffer size for the data radio bearer of secondary RLC entity. |
| ThroughputVolume | If split bear ,ThroughputVolume=ThrVolfullybuffereSplit Otherwise ThrVolfullybufferedleg1+ThrVolfullybufferedleg2 NOTE: ThroughputVolume.MCG,ThroughputVolume.SCG equals the transmitted volume in fully buffered time of actually used leg. |

- Each measurement is a real value representing the throughput in kbits per second. The number of measurements is equal to one. If the optional QoS level measurement is performed, the number of measurements is equal to the number of 5QIs.

- The measurement name has the form 
DRB.ThroughputVolume, DRB.ThroughputTime or optionally DRB.ThroughputVolume.SCG, DRB.ThroughputTime.SCG, DRB.ThroughputVolume.MCG, DRB.ThroughputTime.MCG, DRB.ThroughputVolume.Split, DRB.ThroughputTime.Split,DRB.ThroughputVolume.QOS,  DRB.ThroughputTime.QOS, where QOS identifies the target quality of service class.

- NRCellCU for non split bear,  NRRelation for split bear.

- Valid for packet switched traffic

- 5GS

- One usage of this measurement is for performance assurance within integrity area (user plane connection quality).

Next modified section

A.x Use case monitoring of DL PDCP fully buffered Throughput

To monitor DL PDCP fully buffered throughput is essential, to ensure end user satisfaction and well functioning and well configured cells. DL PDCP fully buffered throughput is to help the evaluation of the RAN congestion and network expanding, and is also useful to analyse the performance of flow controlling features.

End of modifications