3GPP TSG-SA5 Meeting #122	S5-187194

# Spokane, US, 12-16 November 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.541 | CR | 0037 | rev | - | Current version: | 15.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Change NRM IOCs for network slice priority support stage 3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2018-10-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The network slice priority as a feature has not been addressed. The relevant change to NRM IOCs for stage 3 that include network slice priority needs to be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add NRM IOCs for network slice priority support stage 3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 4.3 and 4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of change

I.4	Solution Set definitions

I.4.1	XML definition structure

The overall description of the file format of configuration data XML files is provided by 3GPP TS 32.616 [33].

This annex defines the NRM-specific XML schema sliceNrm.xsd for the network slice Information Model defined in clause 6.

XML schema sliceNrm.xsd explicitly declares NRM-specific XML element types for the related NRM.

The definition of those NRM-specific XML element types complies with the generic mapping rules defined in 3GPP TS 32.616 [33].

I.4.2	Graphical representation

The graphical representation is not present in the current version of the present document.

I.4.3	XML schema "sliceNrm.xsd"

<?xml version="1.0" encoding="UTF-8"?>

<!--

3GPP TS 28.541 network slice Network Resource Model

XML schema definition

sliceNrm.xsd

-->

<schema xmlns="http://www.w3.org/2001/XMLSchema"

xmlns:xn="http://www.3gpp.org/ftp/specs/archive/28_series/28.623#genericNrm"

xmlns:sl="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#sliceNrm"

xmlns:nn="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#nrNrm"

xmlns:ngc="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#ngcNrm"

xmlns:en="http://www.3gpp.org/ftp/specs/archive/28_series/28.659#eutranNrm"

xmlns:sm="http://www.3gpp.org/ftp/specs/archive/28_series/28.626#stateManagementIRP"

targetNamespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#sliceNrm" elementFormDefault="qualified">

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.623#genericNrm"/>

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#nrNrm"/>

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#ngcNrm"/>

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.659#eutranNrm"/>

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.626#stateManagementIRP"/>

<simpleType name="MobilityLevel">

<restriction base="string">

<enumeration value="STATIONARY"/>

<enumeration value="NOMADIC"/>

<enumeration value="RESTRICTED MOBILITY"/>

<enumeration value="FULLY MOBILITY"/>

</restriction>

</simpleType>

<simpleType name="SharingLevel">

<restriction base="string">

<enumeration value="SHARED"/>

<enumeration value="NON-SHARED"/>

</restriction>

</simpleType>

<complexType name="PerfReq">

<!-- Refer to definitions in TS 22.261-->

<sequence>

<choice minOccurs="0" maxOccurs="1">

<element name="perfReqEMBB" type="sl:PerfReqEmbb"/>

<element name="perfReqUrllc" type="sl:PerfReqUrllc"/>

</choice>

</sequence>

</complexType>

<complexType name="PerfReqEmbb">

<sequence>

<element name="expDataRateDL" type="integer" minOccurs="0"/>

<element name="expDataRateUL" type="integer" minOccurs="0"/>

<element name="areaTrafficCapDL" type="integer" minOccurs="0"/>

<element name="areaTrafficCapUL" type="integer" minOccurs="0"/>

<element name="userDensity" type="integer" minOccurs="0"/>

<element name="activityFactor" type="integer " minOccurs="0"/>

<element name="uESpeed" type="integer" minOccurs="0"/>

<element name="coverage" type="string" minOccurs="0"/>

</sequence>

</complexType>

<complexType name="PerfReqUrllc">

<sequence>

<element name="e2eKatency" type="integer" minOccurs="0"/>

<element name="jitter" type="integer" minOccurs="0"/>

<element name="survivalTime" type="integer" minOccurs="0"/>

<element name="cSAvailability" type="float" minOccurs="0"/>

<element name="reliability" type="float" minOccurs="0"/>

<element name="expDataRate" type="integer " minOccurs="0"/>

<element name="payloadSize" type="integer" minOccurs="0"/>

<element name="trafficDensity" type="string" minOccurs="0"/>

<element name="connDensity" type="float" minOccurs="0"/>

<element name="serviceDimension" type="string " minOccurs="0"/>

</sequence>

</complexType>

<complexType name="ServiceProfile">

<sequence>

<element name="serviceProfileId" type="string"/>

<element name="nSSAI" type="ngc:NssaiList"/>

<element name="pLMNId" type="en:PLMNId"/>

<element name="perfReq" type="sl:PerfReq"/>

<element name="maxNumberofUEs" type="long" minOccurs="0"/>

<element name="coverageAreaTAList" type="ngc:NgTACList" minOccurs="0"/>

<element name="latency" type="integer" minOccurs="0"/>

<element name="uEMobilityLevel" type="integer" minOccurs="0"/>

<element name="resourceSharingLevel" type="integer" minOccurs="0"/>

<element name="priority" type="string"/>

</sequence>

</complexType>

<complexType name="ServiceProfileList">

<sequence>

<element name="serviceProfile" type="sl:ServiceProfile"/>

</sequence>

</complexType>

<complexType name="SliceProfile">

<sequence>

<element name="sliceProfileId" type="string"/>

<element name="nSSAI" type=" ngc:NssaiList"/>

<element name="pLMNId" type="en:PLMNId"/>

<element name="perfReq" type="sl:PerfReq"/>

<element name="maxNumberofUEs" type="long" minOccurs="0"/>

<element name="coverageAreaTAList" type="ngc:NgTACList" minOccurs="0"/>

<element name="latency" type="integer" minOccurs="0"/>

<element name="uEMobilityLevel" type="sl: MobilityLevel" minOccurs="0"/>

<element name="resourceSharingLevel" type="integer" minOccurs="0"/>

<element name="priority" type="string"/>

</sequence>

</complexType>

<complexType name="SliceProfileList">

<sequence>

<element name="sliceProfile" type="sl:SliceProfile"/>

</sequence>

</complexType>

<complexType name="NsInfo">

<!-- Refer to definitions in subclause 8.3.3.2.2 of ETSI NFV IFA013 -->

<sequence>

<element name="nsInstanceId" type="string"/>

<element name="nsName" type="string"/>

<element name="description" type="string"/>

</sequence>

</complexType>

<element name="NetworkSlice" substitutionGroup="xn:SubNetworkOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<!-- Inherited attributes from SubNetwork -->

<element name="dnPrefix" type="string" minOccurs="0"/>

<element name="userLabel" type="string"/>

<element name="userDefinedNetworkType" type="string"/>

<element name="setOfMcc" type="string" minOccurs="0"/>

<!-- End of inherited attributes from SubNetwork -->

<element name="nSSIId" type="xn:dn"/>

<element name="operationalState" type="sm:operationalStateType"/>

<element name="administrativeState" type="sm:administrativeStateType"/>

<element name="serviceProfileList" type="sl:ServiceProfileList"/>

<element name="sst" type="ngc:Sst"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="sl:NetworkSliceSubnet"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="NetworkSliceSubnet" substitutionGroup="xn:SubNetworkOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<!-- Inherited attributes from SubNetwork -->

<element name="dnPrefix" type="string" minOccurs="0"/>

<element name="userLabel" type="string"/>

<element name="userDefinedNetworkType" type="string"/>

<element name="setOfMcc" type="string" minOccurs="0"/>

<!-- End of inherited attributes from SubNetwork -->

<element name="mFIdList" type="xn:dnList"/>

<element name="ConstituentNSSIIdList" type="xn:dnList"/>

<element name="State" type="sm:operationalStateType"/>

<element name="nsInfo" type="sl:NsInfo" minOccurs="0"/>

<element name="sliceProfileList" type="sl:SliceProfileList"/>

<element name="sst" type="ngc:Sst"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="sl:NetworkSliceSubnet"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

</schema>

Annex J (normative):
JSON definitions for network slice

J.1	General

This annex contains the JSON definitions for the network slice NRM, in accordance with network slice NRM Information Model definitions specified in clause 6.

J.2	Architectural features

The overall architectural feature of network slice information model is specified in clause 6, this clause specifies features that are specific to the Schema definitions.

J.3	Mapping

J.3.1	IOC mapping

Mapping from the IOCs defined in the information model to SS equivalent definitions are listed in the following table.

| IOC Name | JSON definitions' Name | JSON Type |
| --- | --- | --- |
| NetworkSlice | networkSlice | object |
| NetworkSliceSubnet | networkSliceSubnet | object |
| ServiceProfile | serviceProfile | object |
| SliceProfile | sliceProfile | object |

J.3.2	Attributes mapping

Mapping from the attributes of IOCs defined in the information model to SS equivalent definitions are listed in the following table.

| Object Attributes | JSON definitions' Name | JSON Type |
| --- | --- | --- |
| nSSIId | nSSIId | genericNrm:dn |
| serviceProfileId | serviceProfileId | string |
| sliceProfileId | sliceProfileId | string |
| constituentNSSIIdList | constituentNSSIIdList | genericNrm:dnList |
| mFList | mFList | genericNrm:dnList |
| operationalState | operationalState | stateManagementNrm: operationalStateType |
| administrativeState | administrativeState | stateManagementNrm: administrativeStateType |
| nsInfo | nsInfo | NsInfo |
| pLMNId | pLMNId | eutranNrm:PLMNId |
| perfReq | perfReq | PerfReq |
| nSSAI | nSSAI | nrNrm:Nssai |
| maxNumberofUEs | maxNumberofUEs | number |
| coverageAreaTAList | coverageAreaTAList | ngcNrm:TACList |
| latency | latency | number |
| uEMobilityLevel | uEMobilityLevel | number |
| resourceSharingLevel | resourceSharingLevel | number |
| serviceProfileList | serviceProfileList | object |
| sliceProfileList | sliceProfileList | object |
| sST | sST | ngcNrm:SST |
| priority | priority | string |

J.4	Solution Set definitions

J.4.1	JSON definition structure

JSON is used as resource representations format carried in the HTTP request and HTTP response message bodies. The properties (key-value pairs) on an object are defined using the properties keyword.

J.4.2	Graphical representation

The graphical representation is not present in the current version of the present document.

J.4.3	JSON schema "sliceNrm.json"

{

"$schema": "http://json-schema.org/draft-04/schema#"

"id": "http://3gpp.org/28541/sliceNrm.json",

"description": "JSON based solution set definitions for network slice and network slice subnet",

"_referenced_schema":"http://3gpp.org/28623/genericNrm.json",

"_referenced_schema":"http://3gpp.org/28626/stateManagementNrm.json",

"_referenced_schema":"http://3gpp.org/28541/nrNrm.json",

"_referenced_schema":"http://3gpp.org/28541/ngcNrm.json",

"definitions": {

"networkSlice": {

"type": "object",

"properties": {

"dnPrefix": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"userDefinedNetworkType": {

"type": "string"

},

"setOfMcc": {

"type": "string"

},

"nSSIId": {

"$ref": "http://3gpp.org/28623/genericNrm.json#definitions/dn"

},

"operationalState": {

"$ref": "http://3gpp.org/28659/stateManagementNrm.json#definitions/operationalState"

},

"administrativeState": {

"$ref": "http://3gpp.org/28659/stateManagementNrm.json#definitions/administrativeState"

},

"serviceProfileList": {

"$ref": "#/definitions/ServiceProfileList"

},

"sst": {

"$ref": "http://3gpp.org/28541/ngcNrm.json#definitions/Sst"

}

},

"required": [

"userLabel",

"userDefinedNetworkType",

"nSSIId",

"operationalState",

"administrativeState",

"serviceProfileList",

"sst"

]

}

"networkSliceSubnet": {

"type": "object",

"properties": {

"dnPrefix": {

"type": "string"

},

"userLabel": {

"type": "string"

},

"userDefinedNetworkType": {

"type": "string"

},

"setOfMcc": {

"type": "string"

},

"mFIdList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#definitions/dnlist"

},

"constituentNSSIIdList": {

"$ref": "http://3gpp.org/28623/genericNrm.json#definitions/dnlist"

},

"state": {

"$ref": "http://3gpp.org/28659/stateManagementNrm.json#definitions/operationalState"

},

"nsInfo": {

"$ref": "#/definitions/NsInfo"

},

"sliceProfileList": {

"$ref": "#/definitions/ServiceProfileList"

},

"sst": {

"$ref": "http://3gpp.org/28541/ngcNrm.json#definitions/Sst"

}

},

"required": [

"userLabel",

"pLMNIdList",

"mFIdList",

"constituentNSSIIdList",

"state"

"sliceProfileList",

"sst"

]

},

"MobilityLevel": {

"type": "string",

"enum": [

"STATIONARY",

"NOMADIC",

"RESTRICTED MOBILITY",

"FULLY MOBILITY",

]

},

"SharingLevel": {

"type": "string",

"enum": [

"SHARED",

"NON-SHARED"

]

},

"PrefReq": {

"type": "object",

"properties": {

"prefReqEmbb": {

"$ref": "#/definitions/PrefReqEmbb"

},

"prefReqUrllc": {

"$ref": "#/definitions/PrefReqUrllc"

}

}

},

"PerfReqEmbb":{

"type": "object",

"properties": {

"expDataRateDL": {

"type": "number"

},

"expDataRateUL": {

"type": "number"

},

"areaTrafficCapDL": {

"type": "number"

},

"areaTrafficCapUL": {

"type": "number"

},

"userDensity": {

"type": "number"

},

"activityFactor": {

"type": "number"

},

"uESpeed": {

"type": "number"

},

"coverage": {

"type": "string"

}

}

},

"PerfReqUrrlc":{

"type": "object",

"properties": {

"e2eLatency": {

"type": "number"

},

"jitter": {

"type": "number"

},

"survivalTime": {

"type": "number"

},

"areaTrafficCapUL": {

"type": "number"

},

"cSAvailability": {

"type": "number"

},

"reliability": {

"type": "number"

},

"expDataRate": {

"type": "number"

},

"payloadSize": {

"type": "number"

}

"trafficDensity": {

"type": "string"

},

"connDensity": {

"type": "number"

},

"serviceDimension": {

"type": "string"

}

}

},

"ServiceProfile": {

"type": "object",

"properties": {

"serviceProfileId": {

"type": "string"

},

"nSSAI": {

"$ref": "http://3gpp.org/28541/nrNrm.json#definitions/Nssai"

},

"pLMNId": {

"$ref": "http://3gpp.org/28659/eutranNrm.json#definitions/PLMNId"

},

"perfReq": {

"$ref": "#/definitions/PrefReq"

},

"maxNumberofUEs": {

"type": "number"

},

"coverageAreaTAList": {

"$ref": "http://3gpp.org/28541/ngcNrm.json#definitions/TACList"

},

"latency": {

"type": "number"

},

"uEMobilityLevel": {

"type": "number "

},

"resourceSharingLevel": {

"type": "integer "

}

"priority": {

"type": "string "

}

}

},

"ServiceProfileList": {

"type": "array",

"item": {

"$ref": "#/definitions/ServiceProfile"

}

}

"SliceProfile": {

"type": "object",

"properties": {

"sliceProfileId": {

"type": "string"

},

"nSSAI": {

"$ref": "http://3gpp.org/28541/nrNrm.json#definitions/Nssai"

},

"pLMNId": {

"$ref": "http://3gpp.org/28659/eutranNrm.json#definitions/PLMNId"

},

"perfReq": {

"$ref": "#/definitions/PrefReq"

},

"maxNumberofUEs": {

"type": "number"

},

"coverageAreaTAList": {

"$ref": "http://3gpp.org/28541/ngcNrm.json#definitions/TACList"

},

"latency": {

"type": "number"

},

"uEMobilityLevel": {

"type": "number "

},

"resourceSharingLevel": {

"type": "integer "

}

"priority": {

"type": "string"

}

}

},

"SliceProfileList": {

"type": "array",

"item": {

"$ref": "#/definitions/SliceProfile"

}

}

"NsInfo": {

"type": "object",

"properties": {

"nsInstanceId": {

"type": "string"

},

"nsName": {

"type": "string"

},

"description ": {

"type": "string"

}

}

}

}

}

Annex K (normative):
YANG definitions for Network Slice NRM

K.1	General

This annex contains the YANG definitions for the Network Slice NRM, in accordance with Network Slicing information model definitions specified in clause 6.

K.2	Architectural features

K.3	Mapping

K.3.1	IOC mapping

Mapping from the IOCs defined in the information model to SS equivalent definitions are listed in the following table.

| IOC Name | YANG definitions' Name | YANG Type |
| --- | --- | --- |
| NetworkSlice | NetworkSlice | Contianer in module |
| NetworkSliceSubnet | NetworkSliceSubnet | Contianer in module |
| ServiceProfile | serviceProfile | Reusable Container in Module |
| SliceProfile | SliceProfile | Reusable Container in Module |

K.3.2	Attributes mapping

Mapping from the attributes of IOCs defined in the information model to SS equivalent definitions are listed in the following table.

| Object Attributes | YANG definitions' Name | YANG Type |
| --- | --- | --- |
| Simple Type Attributes | Same to the Name in Stage 2 | leaf |
| Complex Type Attribute |  | Reusable Container |
| Simple Type List | Same to the Name in Stage 2 with removing List/s at the end of the attribute | leaf-list |
| Complex Type List | Same to the Name in Stage 2 with removing List/s at the end of the attribute | list |

K.4	Solution Set definitions

K.4.1	YANG definition structure

K.4.2	Graphical representation

module: NetworkSlice

+--rw NetworkSlice

+--ro objectClass               string

+--ro objectInstance            nrm-type:t_dn

+--ro id                        nrm-type:t_dn

+--ro dnPrefix                  string

+--rw userLabel                 string

+--rw userDefinedNetworkType    string

+--ro setOfMcc*                 nrm-type:t_mcc

+--ro nSSIId                    nrm-type:t_dn

+--ro operationalState          nrm-type:t_operationalState

+--rw administrativeState       nrm-type:t_administrativeState

+--rw serviceProfile* [serviceProfileId]

|  +--rw serviceProfileId        nrm-type:t_dn

|  +--rw s-NSSAI*                nrm-type:t_s-NSSAI

|  +--rw pLMNId

|  |  +--rw MCC   t_mcc

|  |  +--rw MNC   t_mnc

|  +--rw perfReq

|  |  +--rw (SST)

|  |     +--:(eMBB)

|  |     |  +--rw expDataRateDL          uint16

|  |     |  +--rw expDataRateU           uint16

|  |     |  +--rw areaTrafficCapDL       uint16

|  |     |  +--rw areaTrafficCapUL       uint16

|  |     |  +--rw userDensity            uint16

|  |     |  +--rw activityFactor         uint16

|  |     |  +--rw uESpeed                uint16

|  |     |  +--rw coverage               string

|  |     +--:(uRLLC)

|  |        +--rw e2eLatency             uint16

|  |        +--rw jitter                 uint16

|  |        +--rw survivalTime           uint16

|  |        +--rw cSAvailability         decimal64

|  |        +--rw reliability            decimal64

|  |        +--rw expDataRate            uint16

|  |        +--rw payloadSize            string

|  |        +--rw trafficDensity         uint16

|  |        +--rw connDensity            uint16

|  |        +--rw serviceAreaDimension   string

|  +--rw maxNumberofUEs         uint64

|  +--rw coverageAreaTA*         nrm-type:t_tAC

|  +--rw latency                 uint16

|  +--rw uEMobilityLevel        nrm-type:t_uEMobilityLevel

|  +--rw resourceSharingLevel   nrm-type:t_resourceSharingLevel

+--rw sST                       nrm-type:t_SST

module: NetworkSliceSubnet

+--rw NetworkSliceSubnet

+--ro objectClass               string

+--ro objectInstance            nrm-type:t_dn

+--ro id                        nrm-type:t_dn

+--ro dnPrefix                  string

+--rw userLabel                 string

+--rw userDefinedNetworkType    string

+--ro setOfMcc*                 nrm-type:t_mcc

+--ro mFId*                     nrm-type:t_dn

+--ro constituentNSSIIdList*    nrm-type:t_dn

+--rw State                     string

+--ro nsInfo                   nrm-type:t_dn

+--rw sliceProfile* [sliceProfileId]

|  +--rw sliceProfileId          nrm-type:t_dn

|  +--rw s-NSSAI*                nrm-type:t_s-NSSAI

|  +--rw pLMNId

|  |  +--rw MCC   t_mcc

|  |  +--rw MNC   t_mnc

|  +--rw perfReq

|  |  +--rw (SST)

|  |     +--:(eMBB)

|  |     |  +--rw expDataRateDL          uint16

|  |     |  +--rw expDataRateU           uint16

|  |     |  +--rw areaTrafficCapDL       uint16

|  |     |  +--rw areaTrafficCapUL       uint16

|  |     |  +--rw userDensity            uint16

|  |     |  +--rw activityFactor         uint16

|  |     |  +--rw uESpeed                uint16

|  |     |  +--rw coverage               string

|  |     +--:(uRLLC)

|  |        +--rw e2eLatency             uint16

|  |        +--rw jitter                 uint16

|  |        +--rw survivalTime           uint16

|  |        +--rw cSAvailability         decimal64

|  |        +--rw reliability            decimal64

|  |        +--rw expDataRate            uint16

|  |        +--rw payloadSize            string

|  |        +--rw trafficDensity         uint16

|  |        +--rw connDensity            uint16

|  |        +--rw serviceAreaDimension   string

|  +--rw maxNumberofUEs         uint64

|  +--rw coverageAreaTA*         nrm-type:t_tAC

|  +--rw latency                 uint16

|  +--rw uEMobilityLevel        nrm-type:t_uEMobilityLevel

|  +--rw resourceSharingLevel   nrm-type:t_resourceSharingLevel

+--rw sST                       nrm-type:t_SST

K.4.3	YANG schema

K.4.3.1	General type definition "nrm-types-3gpp.yang", refer to Annex G

K.4.3.2	Basic IOC definition which can be reused by specific ManagedFuncion. "Top.yang", "ManagedElement.yang", "ManagedFunction.yang", "EP_RP.yang", "SubNetwork", refer to Annex G

K.4.3.3	NetworkSlice Module, "NetworkSlice.yang"

module NetworkSlice {

namespace "urn:3gpp:tsg:sa5:nrm:NetworkSlice";

prefix "ns";

import SubNetwork { prefix subnet; revision-date "2018-07-31"; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

include ServiceProfile;

description "";

revision 2018-08-09 {

description "Initial revision";

}

container NetworkSlice {

uses subnet:SubNetwork;

leaf nSSIId {

mandatory "true";

config "false";

description "";

type nrm-type:t_dn;

}

leaf operationalState {

mandatory "true";

config "false";

description "";

type nrm-type:t_operationalState;

}

leaf administrativeState {

mandatory "true";

config "true";

description "";

type nrm-type:t_administrativeState;

}

list serviceProfile {

min-elements "1";

config "true";

description "";

key serviceProfileId;

uses ServiceProfile;

}

leaf sST {

mandatory "true";

config "true";

description "";

type nrm-type:t_SST;

}

}

}

K.4.3.4	ServiceProfile Submodule, "ServiceProfile.yang"

submodule ServiceProfile {

belongs-to NetworkSlice { prefix ns; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

import perfReq { prefix perf; revision-date "2018-08-10"; }

revision 2018-08-09 {

description "Initial revision";

}

grouping ServiceProfile {

leaf serviceProfileId {

mandatory "true";

description "";

type nrm-type:t_dn;

}

leaf-list s-NSSAI  {

min-elements "1";

config "true";

description "";

type nrm-type:t_s-NSSAI;

}

container pLMNId {

presence "true";

config "true";

description "";

uses nrm-type:pLMNId;

}

container perfReq {

presence "true";

config "true";

description "";

uses perf:perfReq;

}

leaf maxNumberofUEs {

mandatory "false";

config "true";

description "";

type uint64;

}

leaf-list coverageAreaTA {

min-elements "1";

config "true";

description "";

type nrm-type:t_tAC;

}

leaf latency {

mandatory "true";

config "true";

description "";

type uint16;

}

leaf uEMobilityLevel {

mandatory "false";

config "true";

description "";

type nrm-type:t_uEMobilityLevel;

}

leaf resourceSharingLevel {

mandatory "false";

config "true";

description "";

type nrm-type:t_resourceSharingLevel;

}

}

leaf priority {

mandatory "false";

config "true";

description "";

type nrm-type:t_priority;

}

}

}

K.4.3.5	NetworkSlice Module, "NetworkSliceSubnet.yang"

module NetworkSliceSubnet {

namespace "urn:3gpp:tsg:sa5:nrm:NetworkSliceSubnet";

prefix "nss";

import SubNetwork { prefix subnet; revision-date "2018-07-31"; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

include SliceProfile;

description "";

revision 2018-08-09 {

description "Initial revision";

}

container NetworkSliceSubnet {

uses subnet:SubNetwork;

leaf-list mFId {

min-elements  "0";

config "false";

description "";

type nrm-type:t_dn;

}

leaf-list constituentNSSIIdList {

min-elements  "0";

config "false";

description "";

type nrm-type:t_dn;

}

leaf State {

mandatory "true";

config "true";

description "FFS";

type string;

}

leaf nsInfo {

mandatory "false";

config "false";

description "";

type nrm-type:t_dn;

}

list sliceProfile {

min-elements  "1";

config "true";

description "";

key sliceProfileId;

uses SliceProfile;

}

leaf sST {

mandatory "true";

config "true";

description "";

type nrm-type:t_SST;

}

}

}

K.4.3.6	SliceProfile Submodule, "SliceProfile.yang"

submodule SliceProfile {

belongs-to NetworkSliceSubnet { prefix nss; }

import nrm-types-3gpp { prefix nrm-type; revision-date "2018-07-31"; }

import perfReq { prefix perf; revision-date "2018-08-10"; }

revision 2018-08-09 {

description "Initial revision";

}

grouping SliceProfile {

leaf sliceProfileId {

mandatory "true";

description "";

type nrm-type:t_dn;

}

leaf-list s-NSSAI  {

min-elements "1";

config "true";

description "";

type nrm-type:t_s-NSSAI;

}

container pLMNId {

presence "true";

config "true";

description "";

uses nrm-type:pLMNId;

}

container perfReq {

presence "true";

config "true";

description "";

uses perf:perfReq;

}

leaf maxNumberofUEs {

mandatory "false";

config "true";

description "";

type uint64;

}

leaf-list coverageAreaTA {

min-elements "1";

config "true";

description "";

type nrm-type:t_tAC;

}

leaf latency {

mandatory "true";

config "true";

description "";

type uint16;

}

leaf uEMobilityLevel {

mandatory "false";

config "true";

description "";

type nrm-type:t_uEMobilityLevel;

}

leaf resourceSharingLevel {

mandatory "false";

config "true";

description "";

type nrm-type:t_resourceSharingLevel;

}

leaf priority {

mandatory "false";

description "";

type nrm-type:t_priority;

}

}

}

K.4.3.7	perfReq Module, "perfReq.yang"

module perfReq {

namespace "urn:3gpp:tsg:sa5:nrm:perfReq";

prefix "perf";

revision 2018-08-10 {

description "Initial revision";

}

grouping perfReq {

choice SST {

case eMBB {

leaf expDataRateDL { type uint16; }

leaf expDataRateU {type uint16; }

leaf areaTrafficCapDL { type uint16; }

leaf areaTrafficCapUL { type uint16; }

leaf userDensity { type uint16; }

leaf activityFactor { type uint16; }

leaf uESpeed { type uint16; }

leaf coverage { type string; }

}

case uRLLC {

leaf e2eLatency { type uint16; }

leaf jitter {type uint16; }

leaf survivalTime { type uint16; }

leaf cSAvailability {

type decimal64 {

fraction-digits 4;

range 1..99.9999;

}

}

leaf reliability {

type decimal64 {

fraction-digits 4;

range 1..99.9999;

}

}

leaf expDataRate { type uint16; }

leaf payloadSize { type string; }

leaf trafficDensity { type uint16; }

leaf connDensity { type uint16; }

leaf serviceAreaDimension { type string; }

}

}

}

End of change

}