3GPP TSG-SA5 Meeting #122	                                                                                     S5-187339

Spokane, US, 12 - 16 Nov 2018

Title:	Reply LS on QoS monitoring

Response to:	S5-187039/S2-1811558

Release:	Release 16

Work Item:	FS_5G_URLLC

Source:	SA5

To:	SA2, RAN2, RAN3

Cc:

Contact Person:

Name:	Zou Lan, Yizhi Yao

E-mail Address:	zoulan@huawei.com,yizhi.yao@intel.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

SA5 thanks SA2 for the LS in S5-187039/S2-1811558. SA5 is currently working on the “End-to-end latency of 5G network” KPI and related measurements definition under the Rel-16 Work item “Enhancement of performance assurance for 5G networks including network slicing“. SA5 captured the “End-to-end latency” KPI definition in TS 28.554.

SA5 has discussed the two questions from SA2 and would like to provide the following answers:

Q1: to provide feedback on the study of the QoS Monitoring for URLLC services in SA2.

SA5 understands that “RTT3 “ and “One way delay 3” measurements described in SA2 TR 23.725 subclause 6.8 represent per UE per QoS Flow measurements. The KPI and measurements on end to end latency being defined by SA5 are not for per UE per QoS flow, SA5 latency measurements is defined as average latency per cell (for all UEs) for gNB-DU, further average delay measurements are also defined for all packets in CU-UP/F1-U/GNB-DU).The SA5 KPI and measurements may utilize the end to end latency measurements per UE and per QoS flow (to be) defined by SA2, RAN2 and RAN3.

Q2: whether QoS monitoring for end-to-end packet delay per UE per QoS Flow can be deduced from the mechanisms defined already in SA5 or they are under the remit of ongoing SA5 work and whether (if needed) these mechanisms can be further extended based on new requirements from SA2 and SA1 (defined in TS 22.261).

So far, SA5 has no mechanism defined to support the monitoring of End-to-end packet delay per UE per QoS Flow. SA5 has specified the MDT (Minimization of Drive Test) mechanism to collect the radio measurements per UE for LTE network. SA5 needs to work further for the mechanism to collect the end to end packet delay per UE per QoS flow for 5G.SA5 is happy to cooperate with SA2, RAN2 and RAN3 and will update the progress on this topic.

2. Actions:

To SA2, RAN2, RAN3

ACTION: 	SA5 kindly asks SA2, RAN2 and RAN3 to take the above information into account for their further work and keep SA5 updated about the progress on end to end QoS monitoring.

3. Date of Next TSG-SA WG5 Meetings:

3GPPSA5#123		21 Jan - 25 Jan 2019			Montreal

3GPPSA5#124		25 Feb - 1 Mar 2019			Taipei