3GPP TSG-SA5 Meeting #123	S5-191279

# Montreal,CA, 21-25 January 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0008 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add use case and definitions of flow setup related measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ETRI, KT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_SLICE_ePA |  |  |  |  |  | Date: |  |  | 2019/1/21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In order to provide performance assurance of 5G services, it is important to measure user traffic as granular as possible. The current UPF measurement over N3 is at the granularity of data volume of entire N3. Flow level measurement of user plane traffic on N3 will provide more detailed information of tthe service performance assurance. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the following measurements: - number of incoming GTP data packets per flow on the N3 interface, from (R)AN to UPF - number of outgoing GTP data packets per flow on the N3 interface, from UPF to (R)AN - number of octets of incoming GTP data packets per flow on the N3 interface, from (R)AN to UPF - number of octets of outgoing GTP data packets per flow on the N3 interface, from UPF to (R)AN |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Granular level UP traffic measurement on N3 cannot be made. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.X (new), A.X (change) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st modified section

5.4.1.x	Number of incoming GTP data packets per flow on the N3 interface, from (R)AN to UPF

a)	This measurement provides the number of GTP data packets per flow on the N3 interface which have been accepted and processed by the GTP-U protocol entity on the N3 interface.

b)	CC

c)	Reception by the UPF of a GTP-U data PDU on the N3 interface from the (R)AN. See TS 23.501 [4].

d)	A single integer value.

e)	GTP.InDataPktFlowN3UPF

f)	EP_N3

g)	Valid for packet switching.

h)	5GS

5.4.1.x	Number of outgoing GTP data packets per flow on the N3 interface, from UPF to (R)AN

a)	This measurement provides the number of GTP data packets per flow on the N3 interface which have been generated by the GTP-U protocol entity on the N3 interface.

b)	CC

c)	Transmission by the UPF of a GTP-U data PDU of on the N3 interface to the (R)AN. See TS 23.501 [4].

d)	A single integer value.

e)	GTP.OutDataPktFlowN3UPF

f)	EP_N3

g)	Valid for packet switching.

h)	5GS

5.4.1.x	Number of octets of incoming GTP data packets per flow on the N3 interface, from (R)AN to UPF

a)	This measurement provides the number of octets of incoming GTP data packets per flow on the N3 interface which have been generated by the GTP-U protocol entity on the N3 interface.

b)	CC

c)	Reception by the UPF of a GTP-U data PDU on the N3 interface from (R)AN. See TS 23.501 [4].

d)	A single integer value

e)	GTP.InDataOctFlowN3UPF

f)	EP_N3

g)	Valid for packet switching

h)	5GS

5.4.1.x	Number of octets of outgoing GTP data packets per flow on the N3 interface, from UPF to (R)AN

a)	This measurement provides the number of octets of outgoing GTP data packets per flow on the N3 interface which have been generated by the GTP-U protocol entity on the N3 interface.

b)	CC

c)	Transmission by the UPF of a GTP-U data PDU on the N3 interface to the(R)AN, .See TS 23.501 [4].

d)	A single integer value

e)	GTP.OutDataOctFlowN3UPF

f)	EP_N3

g)	Valid for packet switching

h)	5GS

Next modified section

A.11	N3 data volume related measurements

N3 related measurements are used to measure data volume on N3 interface including incoming and outgoing of GTP data packets in total and per flow without counting the mandatory part of the GTP-U header

It is useful to analyse transport bandwidth usage of N3 interface in total and flow granularity. If the transport bandwidth usage is too high, more bandwidth should be deployed, or load balance should be considered according to core network dimension if there are multiple UPFs connected to multiple gNodeBs.  Decision on the additional bandwidth provisioning or load balancing can be made in more fine grained level due to the flow level measurement.

So it is necessary to define N3 related measurements.

End of modifications