3GPP TSG-SA5 Meeting #124 	S5-192251

# Taipei,Taiwan 25 February -1 March 2019										Revision of S5-19xxxx

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.541 | CR | 0078 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add datatype definition for PerfReq |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NETSLICE-5GNRM |  |  |  |  |  | Date: |  |  | 2019-02-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to the new methodology the datatype have the definitions documented in separate clauses. Currently there are no datatype definitions other than descriptive text in the attribute definition table. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New sub-clause for PerfReq <<choice>> is added, new sub-clause for eMBBPerReq <<dataType>>, uRLLCPerfReq <<dataType>> and mIoTPerfReq <<dataType>> are added. The table for uRLLC perfroamce requirements is not in 22.261 any longer and therefore no source is available and the attribute definition table is eMBBPerReq only. The attributes for uRLLC and mIoT are addressed via a NOTE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is no definition for datatype PerfReq. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3, 6.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First modified section

# 6	Information Model definitions for network slice NRM

## 6.1	Imported information entities and local labels

| Label reference | Local label |
| --- | --- |
| TS 28.622 [30], IOC, Top | Top |
| TS 28.622 [30], IOC, SubNetwork | SubNetwork |
| TS 28.622 [30], IOC, ManagedFunction | ManagedFunction |
| TS 28.658 [19], attribute, pLmnIdList | pLMNIDList |

Second modified section

#### 6.3.4.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| sliceProfileId | M | T | F | T | T |
| sNSSAIList | M | T | T | F | T |
| pLMNIdList | M | T | T | F | T |
| perfReq | M | T | T | F | T |
| maxNumberofUEs | O | T | T | F | T |
| coverageAreaTAList | O | T | T | F | T |
| latency | O | T | T | F | T |
| uEMobilityLevel | O | T | T | F | T |
| resourceSharingLevel | O | T | T | F | T |

#### 6.3.4.3	Attribute constraints

None.

#### 6.3.4.4	Notifications

Not applicable

## 6.3.x1	PerfReq <<choice>>

### 6.3.x1.1	Definition

This parameter specifies the choice of a set of requirements to the NSI in terms supported slice type.

Depending on the sST value, the choice of perfReq will be one of these:

- list of eMBB performance requirements (eMBBPerfReq),

- list of uRLLC performance requirements (uRLLCPerfReq),

- list of mIoT performance requirements (mIoTPerfReq)

#### 6.3.x1.2	Attributes

None

#### 6.3.x1.3	Attribute constraints

None

#### 6.3.x1.4               Notifications

The common notifications defined in subclause 6.5 are valid for this data type, without exceptions or additions.

### 6.3.x2	eMBBPerfReq <<dataType>>

#### 6.3.x2.1	Definition

#### This data type represents eMBBPerfReq which is a list of entries where an entry identifies a performance requirement to the NSI in terms of the scenarios defined in the Table 7.1-1 of TS 22.261 [28].

#### 6.3.x2.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| expDataRateDL | M | T | T | F | T |
| expDataRateUL | M | T | T | F | T |
| areaTrafficCapDL | M | T | T | F | T |
| areaTrafficCapUL | M | T | T | F | T |
| userDensity | M | T | T | F | T |
| activityFactor | M | T | T | F | T |
| uESpeed | M | T | T | F | T |
| coverage | M | T | T | F | T |

#### 6.3.x2.2	Attribute constraints

None

#### 6.3.x2.4               Notifications

The common notifications defined in subclause 6.5 are valid for this data type, without exceptions or additions.

### 6.3.x3	uRLLCPerfReq <<dataType>>

NOTE: the list of uRLLCPerfReq attributes is not addressed in the present document.

### 6.3.x4	mIoTPerfReq <<dataType>>

NOTE: the list of mIoTPerfReq attributes is not addressed in the present document.

Third modified section

## 6.4	Attribute definition

### 6.4.1	Attribute properties

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| nSSIId | An attribute whose "name+value" can be used as an RDN when naming an instance of NetworkSliceSubnet IOC. This RDN uniquely identifies the NetworkSliceSubnet IOC instance within the scope of its containing (parent) object instance. | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| serviceProfileId | A unique identifier of property of network slice related requirement should be supported by the network slice instance. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| sliceProfileId | A unique identifier of the property of network slice subnet related requirement should be supported by the network slice subnet instance. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| constituentNSSIList | It is a list of DN of MOI(s) for the constituent NSSI associated with the network slice subnet instance. | type: DN multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| mFList | It is a list of DN of the MOI(s) for the NF instances associated with the network slice subnet instance. | type: DN multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| operationalState | It indicates the operational state of the NSI. It describes whether or not the resource is physically installed and working. allowedValues: "ENABLED", "DISABLED". The meaning of these values is as defined in 3GPP TS 28.625 [17] and ITU-T X.731 [18]. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| administrativeState | It indicates the administrative state of the NSI. It describes the permission to use or prohibition against using the NSl imposed through the OAM services. allowedValues: "Locked", "Unlocked" or "Shuttingdown" The meaning of these values is as defined in 3GPP TS 28.625 [17] and ITU-T X.731 [18]. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| nsInfo | This attribute contains the NsInfo of the NS instance corresponding to the network slice subnet instance. The NsInfo is described in clause 8.3.3.2.2 of ETSI GS NFV-IFA 013 [29]. | type: String multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: No default value isNullable: True |
| nNSIId | An attribute uniquely identifies the network slice instance. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| perfReq | This parameter specifies the choice of requirements to the NSI in terms of the scenarios defined in the TS 22.261 [28]. Depending on the sST value, the list of performance requirements will be - list of eMBB performance requirements (eMBBPerfReq), - list of uRLLC performance requirements (uRLLCPerfReq), - list of mIoT performance requirements (mIoTPerfReq) NOTE: the list of uRLLCPerfReq is not addressed in the present document. NOTE: the list of mIoTPerfReq is not addressed in the present document. Allowed value: eMBBPerfReq, uRLLCPerfReq, mIoTPerfReq NOTE: Limitation on attribute values in instances of ServiceProfile is not addressed in the present document | type: PerfReq multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| sNSSAIList | This parameter specifies the S-NSSAI list to be supported by the new NSI to be created or the existing NSI to be re-used. sNSSAList is defined in subclause 4.4.1 |  |
| maxNumberofUEs | An attribute specifies the maximum number of UEs may simultaneously access the network slice instance. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| coverageAreaTAList | An attribute specifies a list of <TrackingArea> where the NSI can be selected. | type: <<dataType>> multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| latency | An attribute specifies the packet transmission latency (millisecond) through the RAN, CN, and TN part of 5G network and is used to evaluate utilization performance of the end-to-end network slice instance. See clause 6.3.1 of 28.554 [27]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| uEMobilityLevel | An attribute specifies the mobility level of UE accessing the network slice instance. See 6.2.1 of TS 22.261 [28]. allowedValues: stationary, nomadic, restricted mobility, fully mobility. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: True |
| resourceSharingLevel | An attribute specifies whether the resources to be allocated to the network slice instance may be shared with another network slice instance(s). allowedValues: shared, non-shared. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: Yes isNullable: True |
| serviceProfileList | An attribute specifies a list of ServiceProfile (see clause 6.3.3) supported by the network slice instance | type: << dataType >> multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| sliceProfileList | An attribute specifies a list of SliceProfile (see clause 6.3.4) supported by the network slice subnet instance | type: << dataType >> multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| sST | This parameter specifies the slice/service type of the network slice. See clause 5.15.2 of 3GPP TS 23.501 [2]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| expDataRateDL | This attribute specifes the performance requirement for Experienced data rate (DL) to the NSI in terms of the scenarios defined in the Table 7.1-1 of TS 22.261 [28]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| expDataRateUL | This attribute specifes the performance requirement for Experienced data rate (UL) to the NSI in terms of the scenarios defined in the Table 7.1-1 of TS 22.261 [28]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| areaTrafficCapDL | This attribute specifes the performance requirement for Area traffic capacity (DL)to the NSI in terms of the scenarios defined in the Table 7.1-1 of TS 22.261 [28]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| areaTrafficCapUL | This attributes specifes the performance requirement for Area traffic capacity (UL) to the NSI in terms of the scenarios defined in the Table 7.1-1 of TS 22.261 [28]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| userDensity | This attribute specifes the performance requirement for Overall user density to the NSI in terms of the scenarios defined in the Table 7.1-1 of TS 22.261 [28]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| activityFactor | This attribute specifes the performance requirement for Activity factor to the NSI in terms of the scenarios defined in the Table 7.1-1 of TS 22.261 [28]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| uESpeed | This attribute specifes the performance requirement for UE speed to the NSI in terms of the scenarios defined in the Table 7.1-1 of TS 22.261 [28]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| coverage | This attribute specifes the performance requirement for Coverage to the NSI in terms of the scenarios defined in the Table 7.1-1 of TS 22.261 [28]. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |

## 6.5	Common notifications

This subclause presents a list of notifications, defined in [35], that provisioning management service consumer can receive. The notification parameter objectClass/objectInstance, defined in [26], would capture the DN of an instance of an IOC defined in the present document.

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyMOIAttributeValueChanges | O |  |
| notifyMOICreation | O |  |
| notifyMOIDeletion | O |  |

End of changes