3GPP TSG-SA5 Meeting #124 	S5-192154

# Taipei, Taiwan 25 February -1 March 2019										Revision of S5-19xxxx

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0069 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add use case and definitions of PDCP Split Volume measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_SLICE_ePA |  |  |  |  |  | Date: |  |  | 2019/02/2 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the E-UTRA-NR Dual Connectivity scenario, the split bear will transmit the traffic to LTE and NR. The PDCP split bear data volume needs to be monitored. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Definitions of PDCP split bear data volume are respectively added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.3.6.x (new), 5.1.3.6.y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st modified section

5.1.3.6.x 	DL PDCP PDU Split to LTE Data Volume

- This measurement provides the Data Volume (amount of PDCP PDU bits) in the downlink delivered from PDCP entity to LTE RLC in the E-UTRA-NR Dual Connectivity scenario (in option 3x).

b)	CC.

c)	This measurement is obtained by counting the number of DL PDCP PDU bits sent by GNB-CU to LTE RLC entity in the E-UTRA-NR Dual Connectivity scenario (in option 3x).

d)	Each measurement is an integer value representing the number of bits measured in Mbits (1MBits=1000*1000 bits).

e)	The measurement name has the form QosFlow.PdcpPduVolumeDL.3xSplitToLTE

f)	GNBCUUPFunction.

NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality). NRCellCU measurement applies only for 2-split deployment.

5.1.3.6.y	UL PDCP PDU Split to LTE Data Volume

a)  This measurement provides the Data Volume (amount of PDCP PDU bits) in the uplink delievered from LTE RLC entity to PDCP entity in the E-UTRA-NR Dual Connectivity scenario (in option 3x).

b)	CC

c)	This measurement is obtained by counting the number of bits entering the GNB-CU. The measurement is performed at the PDCP PDU level.

d)	Each measurement is an integer value representing the number of bits measured in Mbits.

e)	The measurement name has the form QosFlow.PdcpSduVolumeUl.3xSplitFromLTE.

f)	GNBCUUPFunction.

NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality). NRCellCU measurement applies only for 2-split deployment.

End of modifications