3GPP TSG-SA5 Meeting #124 	S5-192178

# Taipei,Taiwan 25 February -1 March 2019										Revision of S5-19xxxx

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0070 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add radio resource utilization of network slice instance measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_SLICE_ePA |  |  |  |  |  | Date: |  |  | 2019-02-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Network slicing is an important feature in 5GS, monitoring physical radio resource utilization of network slice instance is helpful for opeators to be aware of the load information of network slice instance. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add radio resource utilizaion of network slice intstance related measurements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.2.X(new), 5.1.1.2.Y(new), A.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start change

5.1.1.2.X  DL PRB Usage of Network Slice Instance

a)	This measurement provides the total usage (in percentage) of physical resource blocks (PRBs) on the downlink per S-NSSAI for slice performance measurements purposes.

b)	CC.

c)	Each measurement sample is obtained as:  MT= Ms(T)Ps(T)*100, where M(T) is the DL total PRB usage per network slice instance, which is percentage of PRBs used, averaged during time period T with value range: 0-100%; Ms(T) is a count of all PRBs used for transmission by the network slice instance during time period T, and Ps(T) is the total number of PRBs associated with the slice instance during time period T. T is the time period during which the measurement is performed.

d)	A set of integers, value from 0 to 100. Each integer represents the PRB usage of one network slice instance.

e)	RRU.PrbTotDl.SNSSAI, which indicates the DL PRB Usage for all traffic per network slice instance.

f)	NRCellDU

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for monitoring the load of the radio physical layer per network slice instance.

5.1.1.2.Y  UL PRB Usage of Network Slice Instance

a)	This measurement provides the total usage (in percentage) of physical resource blocks (PRBs) on the uplink per S-NSSAI for slice performance measurements purposes.

b)	CC.

c)	Each measurement sample is obtained as:  MT= Ms(T)Ps(T)*100, where M(T) is the UL total PRB usage per network slice instance, which is percentage of PRBs used, averaged during time period T with value range: 0-100%; Ms(T) is a count of all PRBs used for transmission by the network slice instance during time period T, and Ps(T) is the total number of PRBs associated with the slice instance during time period T. T is the time period during which the measurement is performed.

d)	A set of integers, value from 0 to 100. Each integer represents the PRB usage of one network slice instance.

e)	RRU.PrbTotUl.SNSSAI, which indicates the UL PRB Usage for all traffic per network slice instance.

f)	NRCellDU

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for monitoring the load of the radio physical layer per network slice instance.

Second of changes

A.6		Monitoring of physical radio resource utilization

The physical radio resource utilization measurements could provide operators the load information of the radio network during the measurement time period. The physical radio resource utilization measurements should reflect the average usage and the usage distribution of the radio resource of the physical layer. The measurements can make the operator to be aware of whether a cell has ever experienced high load or not in the monitoring period, and is a key input to network capacity planning and load balancing.

Network slicing is an important feature in 5GS, monitoring physical radio resource utilization of network slice instance is helpful for opeators to be aware of the load information of network slice instance.

End of changes