3GPP TSG-SA5 Meeting #125AH 	S5- 194190

# Sapporo, JAPAN, 25 - 28 June 2019					                  Revision of S5-19xxxx

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.541 | CR | 0113 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | YANG solution |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NETSLICE-5GNRM |  |  |  |  |  | Date: |  |  | 2019-06-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Align stage 3 YANG solution with stage 2 definition |  |  |  |  |  |  |  |  |
| Summary of change: |  | Provide the stage 3 YANG solution that aligns with stage 2 definitions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The stage 3 solutions is not aligned with the stage 2 definitions. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, Appendix E, Appendix H, Appendix K. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Aligned with conditionally-aggreed CRs: S5-193529, S5-193420, S5-193447, S5-193438, S5-193440, S5-193406 |  |  |  |  |  |  |  |  |

First Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System".

[3]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[4]	3GPP TS 38.401: "NG-RAN; Architecture description".

[5]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[6]	3GPP TS 38.420: "NG-RAN; Xn general aspects and principles".

[7]	3GPP TS 38.470: "NG-RAN; F1 general aspects and principles".

[8]	3GPP TS 38.473: "NG-RAN; F1 application protocol (F1AP)".

[9]	3GPP TS 37.340: "NR; Multi-connectivity; Overall description; Stage 2".

[10]	3GPP TS 28.540: "Management and orchestration; 5G Network Resource Model (NRM);Stage 1".

[11]	3GPP TS 28.662: "Telecommunication management; Generic Radio Access Network (RAN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS) ".

[12]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception".

[13]	3GPP TS 23.003: "Numbering, Addressing and Identification".

[14]	3GPP TS 36.410: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 general aspects and principles".

[15]	3GPP TS 36.423: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 application protocol".

[16]	3GPP TS 36.425: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 interface user plane protocol".

[17]	3GPP TS 28.625: "State Management Data Definition Integration Reference Point (IRP); Information Service (IS)".

[18]	ITU-T Recommendation X.731: "Information technology - Open Systems Interconnection - Systems Management: State management function".

[19]	3GPP TS 28.658: "Telecommunications management; Evolved Universal Terrestrial Radio Access Network (E-UTRAN) Network Resource Model (NRM) Integration Reference Point (IRP): Information Service (IS)".

[20]	3GPP TS 28.702: "Core Network (CN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[21]	3GPP TS 28.708: "Telecommunication management; Evolved Packet Core (EPC) Network Resource Model (NRM) Integration Reference Point (IRP): Information Service (IS)".

[22]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)".

[23]	3GPP TS 29.510: "5G system; Network Function Repository Services; Stage 3".

[24]	3GPP TS 29.531: "5G System; Network Slice Selection Services Stage 3".

[25]	Void.

[26]	3GPP TS 28.531: "Management and orchestration; Provisioning".

[27]	3GPP TS 28.554: "Management and orchestration; 5G End to end Key Performance Indicators (KPI)".

[28]	3GPP TS 22.261: "Service requirements for next generation new services and markets".

[29]	ETSI GS NFV-IFA 013 V2.4.1 (2018-02) "Network Function Virtualisation (NFV); Management and Orchestration; Os-Ma-nfvo Reference Point - Interface and Information Model Specification".

[30]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[31]	Void.

[32]	3GPP TS 38.211: "NR; Physical channels and modulation".

[33]	3GPP TS 32.616: "Telecommunication management; Configuration Management (CM); Bulk CM Integration Reference Point (IRP); Solution Set (SS) definitions".

[34]	3GPP TS 28.623: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions".

[35]	3GPP TS 28.532: "Management and orchestration; Management services".

[36]	Void.

[37]	IETF RFC 791: "Internet Protocol".

[38]	IETF RFC 2373: "IP Version 6 Addressing Architecture".

[39]	IEEE 802.1Q: "Media Access Control Bridges and Virtual Bridged Local Area Networks".

[40]	ETSI GS NFV-IFA 015 (V2.4.1): "Network Function Virtualisation (NFV); Management and Orchestration; Report on NFV Information Model".

[41]	3GPP TS 38.213: "NR; Physical layer procedures for control".

[42]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[43]	3GPP TS 32.156: "Telecommunication management; Fixed Mobile Convergence (FMC) model repertoire".

[x]	IETF RFC 8340: "YANG Tree Diagrams".

Second Change

Annex E (normative):
YANG definitions for NR NRM

# E.1	General

This annex contains the YANG definitions for the NR and NG-RAN NRM, in accordance with NR and NG-RAN NRM information model definitions specified in clause 4.

# E.2	Modules

This is the list of YANG modules for NR and NG-RAN NRM. See zipped YANG module file.

_3gpp-ep-rp.yang

_3gpp-managed-element.yang

_3gpp-managed-function.yang

_3gpp-measurements.yang

_3gpp-nr-nrm-bwp.yang

_3gpp-nr-nrm-ep.yang

_3gpp-nr-nrm-eutrancellrelation.yang

_3gpp-nr-nrm-eutranetwork.yang

_3gpp-nr-nrm-eutranfreqrelation.yang

_3gpp-nr-nrm-eutranfrequency.yang

_3gpp-nr-nrm-externalamffunction.yang

_3gpp-nr-nrm-externalenbfunction.yang

_3gpp-nr-nrm-externaleutrancell.yang

_3gpp-nr-nrm-externalgnbcucpfunction.yang

_3gpp-nr-nrm-externalgnbcuupfunction.yang

_3gpp-nr-nrm-externalgnbdufunction.yang

_3gpp-nr-nrm-externalnrcellcu.yang

_3gpp-nr-nrm-externalservinggwfunction.yang

_3gpp-nr-nrm-externalupffunction.yang

_3gpp-nr-nrm-gnbcucpfunction.yang

_3gpp-nr-nrm-gnbcuupfunction.yang

_3gpp-nr-nrm-gnbdufunction.yang

_3gpp-nr-nrm-nrcellcu.yang

_3gpp-nr-nrm-nrcelldu.yang

_3gpp-nr-nrm-nrcellrelation.yang

_3gpp-nr-nrm-nrfreqrelation.yang

_3gpp-nr-nrm-nrfrequency.yang

_3gpp-nr-nrm-nrnetwork.yang

_3gpp-nr-nrm-nrsectorcarrier.yang

_3gpp-subnetwork.yang

_3gpp-top.yang

_3gpp-yang-extensions.yang

# _3gpp-yang-types.yang

## E.3	Graphical representation

See K.3.

Next Change

Annex H (normative):
YANG definitions for 5GC

# H.1	General

This annex contains the YANG definitions for the 5GC NRM, in accordance with 5GC information model definitions specified in clause 4.

# H.2	Modules

This is the list of YANG modules for 5GC NRM. See zipped YANG module file.

_3gpp-5gc-nrm-affunction.yang

_3gpp-5gc-nrm-amffunction.yang

_3gpp-5gc-nrm-amfregion.yang

_3gpp-5gc-nrm-amfset.yang

_3gpp-5gc-nrm-ausffunction.yang

_3gpp-5gc-nrm-dnfunction.yang

_3gpp-5gc-nrm-ep.yang

_3gpp-5gc-nrm-externalamffunction.yang

_3gpp-5gc-nrm-externalnrffunction.yang

_3gpp-5gc-nrm-externalnssffunction.yang

_3gpp-5gc-nrm-lmffunction.yang

_3gpp-5gc-nrm-n3iwffunction.yang

_3gpp-5gc-nrm-nfprofile.yang

_3gpp-5gc-nrm-nfservice.yang

_3gpp-5gc-nrm-ngeirfunction.yang

_3gpp-5gc-nrm-nrffunction.yang

_3gpp-5gc-nrm-nssffunction.yang

_3gpp-5gc-nrm-nwdaffunction.yang

_3gpp-5gc-nrm-pcffunction.yang

_3gpp-5gc-nrm-seppfunction.yang

_3gpp-5gc-nrm-smffunction.yang

_3gpp-5gc-nrm-smsffunction.yang

_3gpp-5gc-nrm-udmfunction.yang

_3gpp-5gc-nrm-udrfunction.yang

_3gpp-5gc-nrm-udsffunction.yang

_3gpp-5gc-nrm-upffunction.yang

_3gpp-ep-rp.yang

_3gpp-managed-element.yang

_3gpp-managed-function.yang

_3gpp-measurements.yang

_3gpp-subnetwork.yang

_3gpp-top.yang

_3gpp-yang-extensions.yang

_3gpp-yang-types.yang

# H.3	Graphical representation

See K.3.

Next Change

Annex K (normative):
YANG definitions for Network Slice NRM

# K.1	General

This annex contains the YANG definitions for the Network Slice NRM, in accordance with Network Slicing information model definitions specified in clause 6.

# K.2	Modules

This is the list of YANG modules for Network Slice NRM. See zipped YANG module file.

_3gpp-ep-rp.yang

_3gpp-managed-element.yang

_3gpp-managed-function.yang

_3gpp-measurements.yang

_3gpp-ns-nrm-networkslice.yang

_3gpp-ns-nrm-networkslicesubnet.yang

_3gpp-ns-nrm-perfreq.yang

_3gpp-ns-nrm-serviceprofile.yang

_3gpp-ns-nrm-sliceprofile.yang

_3gpp-subnetwork.yang

_3gpp-top.yang

_3gpp-yang-extensions.yang

_3gpp-yang-types.yang

# K.3	Model tree diagram

This is the YANG module tree diagram whose syntax is defined in [x].

module: _3gpp-managed-element

+--rw ManagedElement* [id]

+--rw id                             string

+--rw attributes

|  +--rw dnPrefix?                   types3gpp:DistinguishedName

|  +--rw userLabel?                  string

|  +--ro locationName?               string

|  +--ro managedBy*                  types3gpp:DistinguishedName

|  +--ro managedElementTypeList*     string

|  +--ro supportedMeasurementsGPs* [measurementType] {MeasurementsUnderManagedElement}?

|  |  +--ro measurementType    string

|  |  +--ro supportedGPs*      uint32

|  +--ro vendorName?                 string

|  +--rw userDefinedState?           string

|  +--ro swVersion?                  string

|  +--rw priorityLabel               uint32

+--rw MeasurementControl* [id] {MeasurementsUnderManagedElement}?

|  +--rw id                   string

|  +--rw attributes

|  |  +--rw pMAdministrativeState?        types3gpp:AdministrativeState

|  |  +--ro pMOperationalState            types3gpp:OperationalState

|  |  +--rw defaultFileLocation?          string

|  |  +--rw defaultFileBasedGP            uint32

|  |  +--rw defaultFileReportingPeriod    uint32

|  |  +--rw defaultStreamTarget?          string {StreamingSupported}?

|  |  +--rw defaultStreamBasedGP          uint32 {StreamingSupported}?

|  +--rw MeasurementReader* [id]

|     +--rw id            string

|     +--rw attributes

|        +--rw measurementTypes*        -> ../../../../attributes/supportedMeasurementsGPs/measurementType

|        +--rw managedObjectDNsBasic*   types3gpp:DistinguishedName

|        +--rw managedObjectDNs*        types3gpp:DistinguishedName

|        +--rw fileLocation?            string

|        +--rw fileBasedGP              -> ../../../../attributes/supportedMeasurementsGPs/supportedGPs

|        +--rw fileReportingPeriod      uint32

|        +--rw streamTarget?            string

|        +--rw streamBasedGP            -> ../../../../attributes/supportedMeasurementsGPs/supportedGPs

+--rw gnbdu3gpp:GNBDUFunction* [id]

|  +--rw gnbdu3gpp:id                      string

|  +--rw gnbdu3gpp:attributes

|  |  +--rw gnbdu3gpp:userLabel?           string

|  |  +--rw gnbdu3gpp:vnfParametersList!

|  |  |  +--rw gnbdu3gpp:vnfInstanceId    string

|  |  |  +--rw gnbdu3gpp:vnfdId?          string

|  |  |  +--rw gnbdu3gpp:flavourId?       string

|  |  |  +--rw gnbdu3gpp:autoScalable     boolean

|  |  +--rw gnbdu3gpp:peeParametersList!

|  |  |  +--rw gnbdu3gpp:siteIdentification    string

|  |  |  +--rw gnbdu3gpp:siteLatitude?         decimal64

|  |  |  +--rw gnbdu3gpp:siteLongitude?        decimal64

|  |  |  +--rw gnbdu3gpp:siteDescription       string

|  |  |  +--rw gnbdu3gpp:equipmentType         string

|  |  |  +--rw gnbdu3gpp:environmentType       string

|  |  |  +--rw gnbdu3gpp:powerInterface        string

|  |  +--rw gnbdu3gpp:priorityLabel        uint32

|  |  +--rw gnbdu3gpp:gNBId                int64

|  |  +--rw gnbdu3gpp:gNBIdLength          int32

|  |  +--rw gnbdu3gpp:gNBDUId              int64

|  |  +--rw gnbdu3gpp:gNBDUName            string

|  |  +--rw gnbdu3gpp:pLMNId* [mcc mnc]

|  |     +--rw gnbdu3gpp:mcc    Mcc

|  |     +--rw gnbdu3gpp:mnc    Mnc

|  +--rw nrsectcarr3gpp:NRSectorCarrier* [id]

|  |  +--rw nrsectcarr3gpp:id            string

|  |  +--rw nrsectcarr3gpp:attributes

|  |     +--rw nrsectcarr3gpp:userLabel?                    string

|  |     +--rw nrsectcarr3gpp:vnfParametersList!

|  |     |  +--rw nrsectcarr3gpp:vnfInstanceId    string

|  |     |  +--rw nrsectcarr3gpp:vnfdId?          string

|  |     |  +--rw nrsectcarr3gpp:flavourId?       string

|  |     |  +--rw nrsectcarr3gpp:autoScalable     boolean

|  |     +--rw nrsectcarr3gpp:peeParametersList!

|  |     |  +--rw nrsectcarr3gpp:siteIdentification    string

|  |     |  +--rw nrsectcarr3gpp:siteLatitude?         decimal64

|  |     |  +--rw nrsectcarr3gpp:siteLongitude?        decimal64

|  |     |  +--rw nrsectcarr3gpp:siteDescription       string

|  |     |  +--rw nrsectcarr3gpp:equipmentType         string

|  |     |  +--rw nrsectcarr3gpp:environmentType       string

|  |     |  +--rw nrsectcarr3gpp:powerInterface        string

|  |     +--rw nrsectcarr3gpp:priorityLabel                 uint32

|  |     +--rw nrsectcarr3gpp:txDirection                   types3gpp:TxDirection

|  |     +--rw nrsectcarr3gpp:configuredMaxTxPower          int32

|  |     +--rw nrsectcarr3gpp:arfcnDL                       int32

|  |     +--rw nrsectcarr3gpp:arfcnUL                       int32

|  |     +--rw nrsectcarr3gpp:bSChannelBwDL                 int32

|  |     +--rw nrsectcarr3gpp:bSChannelBwUL                 int32

|  |     +--rw nrsectcarr3gpp:sectorEquipmentFunctionRef    types3gpp:DistinguishedName

|  +--rw bwp3gpp:BWP* [id]

|  |  +--rw bwp3gpp:id            string

|  |  +--rw bwp3gpp:attributes

|  |     +--rw bwp3gpp:userLabel?           string

|  |     +--rw bwp3gpp:vnfParametersList!

|  |     |  +--rw bwp3gpp:vnfInstanceId    string

|  |     |  +--rw bwp3gpp:vnfdId?          string

|  |     |  +--rw bwp3gpp:flavourId?       string

|  |     |  +--rw bwp3gpp:autoScalable     boolean

|  |     +--rw bwp3gpp:peeParametersList!

|  |     |  +--rw bwp3gpp:siteIdentification    string

|  |     |  +--rw bwp3gpp:siteLatitude?         decimal64

|  |     |  +--rw bwp3gpp:siteLongitude?        decimal64

|  |     |  +--rw bwp3gpp:siteDescription       string

|  |     |  +--rw bwp3gpp:equipmentType         string

|  |     |  +--rw bwp3gpp:environmentType       string

|  |     |  +--rw bwp3gpp:powerInterface        string

|  |     +--rw bwp3gpp:priorityLabel        uint32

|  |     +--rw bwp3gpp:bwpContext           BwpContext

|  |     +--rw bwp3gpp:isInitialBwp         IsInitialBwp

|  |     +--rw bwp3gpp:subCarrierSpacing    uint32

|  |     +--rw bwp3gpp:cyclicPrefix         CyclicPrefix

|  |     +--rw bwp3gpp:startRB              uint32

|  |     +--rw bwp3gpp:numberOfRBs          uint32

|  +--rw nrcelldu3gpp:NRCellDU* [id]

|  |  +--rw nrcelldu3gpp:id            string

|  |  +--rw nrcelldu3gpp:attributes

|  |     +--rw nrcelldu3gpp:userLabel?              string

|  |     +--rw nrcelldu3gpp:vnfParametersList!

|  |     |  +--rw nrcelldu3gpp:vnfInstanceId    string

|  |     |  +--rw nrcelldu3gpp:vnfdId?          string

|  |     |  +--rw nrcelldu3gpp:flavourId?       string

|  |     |  +--rw nrcelldu3gpp:autoScalable     boolean

|  |     +--rw nrcelldu3gpp:peeParametersList!

|  |     |  +--rw nrcelldu3gpp:siteIdentification    string

|  |     |  +--rw nrcelldu3gpp:siteLatitude?         decimal64

|  |     |  +--rw nrcelldu3gpp:siteLongitude?        decimal64

|  |     |  +--rw nrcelldu3gpp:siteDescription       string

|  |     |  +--rw nrcelldu3gpp:equipmentType         string

|  |     |  +--rw nrcelldu3gpp:environmentType       string

|  |     |  +--rw nrcelldu3gpp:powerInterface        string

|  |     +--rw nrcelldu3gpp:priorityLabel           uint32

|  |     +--rw nrcelldu3gpp:cellLocalId             int32

|  |     +--ro nrcelldu3gpp:operationalState?       types3gpp:OperationalState

|  |     +--rw nrcelldu3gpp:administrativeState     types3gpp:AdministrativeState

|  |     +--ro nrcelldu3gpp:cellState?              types3gpp:CellState

|  |     +--rw nrcelldu3gpp:pLMNIdList* [mcc mnc]

|  |     |  +--rw nrcelldu3gpp:mcc    Mcc

|  |     |  +--rw nrcelldu3gpp:mnc    Mnc

|  |     +--rw nrcelldu3gpp:sNSSAIList*             types3gpp:SNssai

|  |     +--rw nrcelldu3gpp:nRPCI                   int32

|  |     +--rw nrcelldu3gpp:nRTAC                   types3gpp:Tac

|  |     +--rw nrcelldu3gpp:arfcnDL                 int32

|  |     +--rw nrcelldu3gpp:arfcnUL                 int32

|  |     +--rw nrcelldu3gpp:arfcnSUL                int32

|  |     +--rw nrcelldu3gpp:bSChannelBwDL           int32

|  |     +--rw nrcelldu3gpp:bSChannelBwUL?          int32

|  |     +--rw nrcelldu3gpp:bSChannelBwSUL?         int32

|  |     +--rw nrcelldu3gpp:ssbFrequency            int32

|  |     +--rw nrcelldu3gpp:ssbPeriodicity          int32

|  |     +--rw nrcelldu3gpp:ssbSubCarrierSpacing    int32

|  |     +--rw nrcelldu3gpp:ssbOffset               int32

|  |     +--rw nrcelldu3gpp:ssbDuration             int32

|  |     +--rw nrcelldu3gpp:nRSectorCarrierRef*     types3gpp:DistinguishedName

|  |     +--rw nrcelldu3gpp:bWPRef*                 types3gpp:DistinguishedName

|  |     +--rw nrcelldu3gpp:nRFrequencyRef*         types3gpp:DistinguishedName

|  +--rw ep3gpp:EP_F1C* [id]

|  |  +--rw ep3gpp:id            string

|  |  +--rw ep3gpp:attributes

|  |     +--rw ep3gpp:userLabel?                  string

|  |     +--ro ep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro ep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro ep3gpp:measurementType    string

|  |     |  +--ro ep3gpp:supportedGPs*      uint32

|  |     +--rw ep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw ep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw ep3gpp:vlanId       uint16

|  |     +--rw ep3gpp:remoteAddress               inet:ip-address

|  +--rw ep3gpp:EP_F1U* [id]

|     +--rw ep3gpp:id            string

|     +--rw ep3gpp:attributes

|        +--rw ep3gpp:userLabel?                  string

|        +--ro ep3gpp:farEndEntity?               types3gpp:DistinguishedName

|        +--ro ep3gpp:supportedMeasurementsGPs* [measurementType]

|        |  +--ro ep3gpp:measurementType    string

|        |  +--ro ep3gpp:supportedGPs*      uint32

|        +--rw ep3gpp:localAddress* [ipAddress vlanId]

|        |  +--rw ep3gpp:ipAddress    inet:ip-address

|        |  +--rw ep3gpp:vlanId       uint16

|        +--rw ep3gpp:remoteAddress               inet:ip-address

+--rw pcf3gpp:PCFFunction* [id]

|  +--rw pcf3gpp:id            string

|  +--rw pcf3gpp:attributes

|  |  +--rw pcf3gpp:userLabel?           string

|  |  +--rw pcf3gpp:vnfParametersList!

|  |  |  +--rw pcf3gpp:vnfInstanceId    string

|  |  |  +--rw pcf3gpp:vnfdId?          string

|  |  |  +--rw pcf3gpp:flavourId?       string

|  |  |  +--rw pcf3gpp:autoScalable     boolean

|  |  +--rw pcf3gpp:peeParametersList!

|  |  |  +--rw pcf3gpp:siteIdentification    string

|  |  |  +--rw pcf3gpp:siteLatitude?         decimal64

|  |  |  +--rw pcf3gpp:siteLongitude?        decimal64

|  |  |  +--rw pcf3gpp:siteDescription       string

|  |  |  +--rw pcf3gpp:equipmentType         string

|  |  |  +--rw pcf3gpp:environmentType       string

|  |  |  +--rw pcf3gpp:powerInterface        string

|  |  +--rw pcf3gpp:priorityLabel        uint32

|  |  +--rw pcf3gpp:pLMNIdList* [mcc mnc]

|  |  |  +--rw pcf3gpp:mcc    Mcc

|  |  |  +--rw pcf3gpp:mnc    Mnc

|  |  +--rw pcf3gpp:sBIFQDN?             inet:domain-name

|  |  +--ro pcf3gpp:sBIServiceList*      string

|  |  +--rw pcf3gpp:sNSSAIList*          types3gpp:SNssai

|  +--rw cep3gpp:EP_N5* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_N7* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_N15* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_N16* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_Rx* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_SBI_X* [id]

|     +--rw cep3gpp:id            string

|     +--rw cep3gpp:attributes

|        +--rw cep3gpp:userLabel?                  string

|        +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|        +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|        |  +--ro cep3gpp:measurementType    string

|        |  +--ro cep3gpp:supportedGPs*      uint32

|        +--rw cep3gpp:localAddress* [ipAddress vlanId]

|        |  +--rw cep3gpp:ipAddress    inet:ip-address

|        |  +--rw cep3gpp:vlanId       uint16

|        +--rw cep3gpp:remoteAddress               inet:ip-address

|        +--ro cep3gpp:sBIService*                 string

+--rw af3gpp:AFFunction* [id]

|  +--rw af3gpp:id            string

|  +--rw af3gpp:attributes

|  |  +--rw af3gpp:userLabel?           string

|  |  +--rw af3gpp:vnfParametersList!

|  |  |  +--rw af3gpp:vnfInstanceId    string

|  |  |  +--rw af3gpp:vnfdId?          string

|  |  |  +--rw af3gpp:flavourId?       string

|  |  |  +--rw af3gpp:autoScalable     boolean

|  |  +--rw af3gpp:peeParametersList!

|  |  |  +--rw af3gpp:siteIdentification    string

|  |  |  +--rw af3gpp:siteLatitude?         decimal64

|  |  |  +--rw af3gpp:siteLongitude?        decimal64

|  |  |  +--rw af3gpp:siteDescription       string

|  |  |  +--rw af3gpp:equipmentType         string

|  |  |  +--rw af3gpp:environmentType       string

|  |  |  +--rw af3gpp:powerInterface        string

|  |  +--rw af3gpp:priorityLabel        uint32

|  +--rw cep3gpp:EP_N6* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_Rx* [id]

|     +--rw cep3gpp:id            string

|     +--rw cep3gpp:attributes

|        +--rw cep3gpp:userLabel?                  string

|        +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|        +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|        |  +--ro cep3gpp:measurementType    string

|        |  +--ro cep3gpp:supportedGPs*      uint32

|        +--rw cep3gpp:localAddress* [ipAddress vlanId]

|        |  +--rw cep3gpp:ipAddress    inet:ip-address

|        |  +--rw cep3gpp:vlanId       uint16

|        +--rw cep3gpp:remoteAddress               inet:ip-address

+--rw udm3gpp:UDMFunction* [id]

|  +--rw udm3gpp:id            string

|  +--rw udm3gpp:attributes

|  |  +--rw udm3gpp:userLabel?           string

|  |  +--rw udm3gpp:vnfParametersList!

|  |  |  +--rw udm3gpp:vnfInstanceId    string

|  |  |  +--rw udm3gpp:vnfdId?          string

|  |  |  +--rw udm3gpp:flavourId?       string

|  |  |  +--rw udm3gpp:autoScalable     boolean

|  |  +--rw udm3gpp:peeParametersList!

|  |  |  +--rw udm3gpp:siteIdentification    string

|  |  |  +--rw udm3gpp:siteLatitude?         decimal64

|  |  |  +--rw udm3gpp:siteLongitude?        decimal64

|  |  |  +--rw udm3gpp:siteDescription       string

|  |  |  +--rw udm3gpp:equipmentType         string

|  |  |  +--rw udm3gpp:environmentType       string

|  |  |  +--rw udm3gpp:powerInterface        string

|  |  +--rw udm3gpp:priorityLabel        uint32

|  |  +--rw udm3gpp:pLMNIdList* [mcc mnc]

|  |  |  +--rw udm3gpp:mcc    Mcc

|  |  |  +--rw udm3gpp:mnc    Mnc

|  |  +--rw udm3gpp:sBIFQDN?             inet:domain-name

|  |  +--ro udm3gpp:sBIServiceList*      string

|  |  +--rw udm3gpp:sNSSAIList*          types3gpp:SNssai

|  +--rw cep3gpp:EP_N8* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_N10* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_N13* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_SBI_X* [id]

|     +--rw cep3gpp:id            string

|     +--rw cep3gpp:attributes

|        +--rw cep3gpp:userLabel?                  string

|        +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|        +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|        |  +--ro cep3gpp:measurementType    string

|        |  +--ro cep3gpp:supportedGPs*      uint32

|        +--rw cep3gpp:localAddress* [ipAddress vlanId]

|        |  +--rw cep3gpp:ipAddress    inet:ip-address

|        |  +--rw cep3gpp:vlanId       uint16

|        +--rw cep3gpp:remoteAddress               inet:ip-address

|        +--ro cep3gpp:sBIService*                 string

+--rw udr3gpp:UDRFunction* [id]

|  +--rw udr3gpp:id            string

|  +--rw udr3gpp:attributes

|  |  +--rw udr3gpp:userLabel?           string

|  |  +--rw udr3gpp:vnfParametersList!

|  |  |  +--rw udr3gpp:vnfInstanceId    string

|  |  |  +--rw udr3gpp:vnfdId?          string

|  |  |  +--rw udr3gpp:flavourId?       string

|  |  |  +--rw udr3gpp:autoScalable     boolean

|  |  +--rw udr3gpp:peeParametersList!

|  |  |  +--rw udr3gpp:siteIdentification    string

|  |  |  +--rw udr3gpp:siteLatitude?         decimal64

|  |  |  +--rw udr3gpp:siteLongitude?        decimal64

|  |  |  +--rw udr3gpp:siteDescription       string

|  |  |  +--rw udr3gpp:equipmentType         string

|  |  |  +--rw udr3gpp:environmentType       string

|  |  |  +--rw udr3gpp:powerInterface        string

|  |  +--rw udr3gpp:priorityLabel        uint32

|  |  +--rw udr3gpp:pLMNIdList* [mcc mnc]

|  |  |  +--rw udr3gpp:mcc    Mcc

|  |  |  +--rw udr3gpp:mnc    Mnc

|  |  +--rw udr3gpp:sBIFQDN?             inet:domain-name

|  |  +--ro udr3gpp:sBIServiceList*      string

|  |  +--rw udr3gpp:sNSSAIList*          types3gpp:SNssai

|  +--rw cep3gpp:EP_SBI_X* [id]

|     +--rw cep3gpp:id            string

|     +--rw cep3gpp:attributes

|        +--rw cep3gpp:userLabel?                  string

|        +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|        +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|        |  +--ro cep3gpp:measurementType    string

|        |  +--ro cep3gpp:supportedGPs*      uint32

|        +--rw cep3gpp:localAddress* [ipAddress vlanId]

|        |  +--rw cep3gpp:ipAddress    inet:ip-address

|        |  +--rw cep3gpp:vlanId       uint16

|        +--rw cep3gpp:remoteAddress               inet:ip-address

|        +--ro cep3gpp:sBIService*                 string

+--rw smf3gpp:SMFFunction* [id]

|  +--rw smf3gpp:id            string

|  +--rw smf3gpp:attributes

|  |  +--rw smf3gpp:userLabel?           string

|  |  +--rw smf3gpp:vnfParametersList!

|  |  |  +--rw smf3gpp:vnfInstanceId    string

|  |  |  +--rw smf3gpp:vnfdId?          string

|  |  |  +--rw smf3gpp:flavourId?       string

|  |  |  +--rw smf3gpp:autoScalable     boolean

|  |  +--rw smf3gpp:peeParametersList!

|  |  |  +--rw smf3gpp:siteIdentification    string

|  |  |  +--rw smf3gpp:siteLatitude?         decimal64

|  |  |  +--rw smf3gpp:siteLongitude?        decimal64

|  |  |  +--rw smf3gpp:siteDescription       string

|  |  |  +--rw smf3gpp:equipmentType         string

|  |  |  +--rw smf3gpp:environmentType       string

|  |  |  +--rw smf3gpp:powerInterface        string

|  |  +--rw smf3gpp:priorityLabel        uint32

|  |  +--rw smf3gpp:pLMNIdList* [mcc mnc]

|  |  |  +--rw smf3gpp:mcc    Mcc

|  |  |  +--rw smf3gpp:mnc    Mnc

|  |  +--ro smf3gpp:nRTACList*           types3gpp:Tac

|  |  +--rw smf3gpp:sBIFQDN?             inet:domain-name

|  |  +--ro smf3gpp:sBIServiceList*      string

|  |  +--rw smf3gpp:sNSSAIList*          types3gpp:SNssai

|  +--rw cep3gpp:EP_N4* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_N7* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_N10* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_N11* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_N16* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_S5C* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_SBI_X* [id]

|     +--rw cep3gpp:id            string

|     +--rw cep3gpp:attributes

|        +--rw cep3gpp:userLabel?                  string

|        +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|        +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|        |  +--ro cep3gpp:measurementType    string

|        |  +--ro cep3gpp:supportedGPs*      uint32

|        +--rw cep3gpp:localAddress* [ipAddress vlanId]

|        |  +--rw cep3gpp:ipAddress    inet:ip-address

|        |  +--rw cep3gpp:vlanId       uint16

|        +--rw cep3gpp:remoteAddress               inet:ip-address

|        +--ro cep3gpp:sBIService*                 string

+--rw ngeir3gpp:NGEIRFunction* [id]

|  +--rw ngeir3gpp:id            string

|  +--rw ngeir3gpp:attributes

|  |  +--rw ngeir3gpp:userLabel?           string

|  |  +--rw ngeir3gpp:vnfParametersList!

|  |  |  +--rw ngeir3gpp:vnfInstanceId    string

|  |  |  +--rw ngeir3gpp:vnfdId?          string

|  |  |  +--rw ngeir3gpp:flavourId?       string

|  |  |  +--rw ngeir3gpp:autoScalable     boolean

|  |  +--rw ngeir3gpp:peeParametersList!

|  |  |  +--rw ngeir3gpp:siteIdentification    string

|  |  |  +--rw ngeir3gpp:siteLatitude?         decimal64

|  |  |  +--rw ngeir3gpp:siteLongitude?        decimal64

|  |  |  +--rw ngeir3gpp:siteDescription       string

|  |  |  +--rw ngeir3gpp:equipmentType         string

|  |  |  +--rw ngeir3gpp:environmentType       string

|  |  |  +--rw ngeir3gpp:powerInterface        string

|  |  +--rw ngeir3gpp:priorityLabel        uint32

|  |  +--rw ngeir3gpp:pLMNIdList* [mcc mnc]

|  |  |  +--rw ngeir3gpp:mcc    Mcc

|  |  |  +--rw ngeir3gpp:mnc    Mnc

|  |  +--rw ngeir3gpp:sBIFQDN?             inet:domain-name

|  |  +--ro ngeir3gpp:sBIServiceList*      string

|  |  +--rw ngeir3gpp:sNSSAIList*          types3gpp:SNssai

|  +--rw cep3gpp:EP_N17* [id]

|     +--rw cep3gpp:id            string

|     +--rw cep3gpp:attributes

|        +--rw cep3gpp:userLabel?                  string

|        +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|        +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|        |  +--ro cep3gpp:measurementType    string

|        |  +--ro cep3gpp:supportedGPs*      uint32

|        +--rw cep3gpp:localAddress* [ipAddress vlanId]

|        |  +--rw cep3gpp:ipAddress    inet:ip-address

|        |  +--rw cep3gpp:vlanId       uint16

|        +--rw cep3gpp:remoteAddress               inet:ip-address

+--rw gnbcucp3gpp:GNBCUCPFunction* [id]

|  +--rw gnbcucp3gpp:id            string

|  +--rw gnbcucp3gpp:attributes

|  |  +--rw gnbcucp3gpp:userLabel?           string

|  |  +--rw gnbcucp3gpp:vnfParametersList!

|  |  |  +--rw gnbcucp3gpp:vnfInstanceId    string

|  |  |  +--rw gnbcucp3gpp:vnfdId?          string

|  |  |  +--rw gnbcucp3gpp:flavourId?       string

|  |  |  +--rw gnbcucp3gpp:autoScalable     boolean

|  |  +--rw gnbcucp3gpp:peeParametersList!

|  |  |  +--rw gnbcucp3gpp:siteIdentification    string

|  |  |  +--rw gnbcucp3gpp:siteLatitude?         decimal64

|  |  |  +--rw gnbcucp3gpp:siteLongitude?        decimal64

|  |  |  +--rw gnbcucp3gpp:siteDescription       string

|  |  |  +--rw gnbcucp3gpp:equipmentType         string

|  |  |  +--rw gnbcucp3gpp:environmentType       string

|  |  |  +--rw gnbcucp3gpp:powerInterface        string

|  |  +--rw gnbcucp3gpp:priorityLabel        uint32

|  |  +--rw gnbcucp3gpp:gNBId                int64

|  |  +--rw gnbcucp3gpp:gNBIdLength          int32

|  |  +--rw gnbcucp3gpp:gNBCUName            string

|  |  +--rw gnbcucp3gpp:pLMNId* [mcc mnc]

|  |     +--rw gnbcucp3gpp:mcc    Mcc

|  |     +--rw gnbcucp3gpp:mnc    Mnc

|  +--rw nrcellcu3gpp:NRCellCU* [id]

|  |  +--rw nrcellcu3gpp:id                         string

|  |  +--rw nrcellcu3gpp:attributes

|  |  |  +--rw nrcellcu3gpp:userLabel?             string

|  |  |  +--rw nrcellcu3gpp:vnfParametersList!

|  |  |  |  +--rw nrcellcu3gpp:vnfInstanceId    string

|  |  |  |  +--rw nrcellcu3gpp:vnfdId?          string

|  |  |  |  +--rw nrcellcu3gpp:flavourId?       string

|  |  |  |  +--rw nrcellcu3gpp:autoScalable     boolean

|  |  |  +--rw nrcellcu3gpp:peeParametersList!

|  |  |  |  +--rw nrcellcu3gpp:siteIdentification    string

|  |  |  |  +--rw nrcellcu3gpp:siteLatitude?         decimal64

|  |  |  |  +--rw nrcellcu3gpp:siteLongitude?        decimal64

|  |  |  |  +--rw nrcellcu3gpp:siteDescription       string

|  |  |  |  +--rw nrcellcu3gpp:equipmentType         string

|  |  |  |  +--rw nrcellcu3gpp:environmentType       string

|  |  |  |  +--rw nrcellcu3gpp:powerInterface        string

|  |  |  +--rw nrcellcu3gpp:priorityLabel          uint32

|  |  |  +--rw nrcellcu3gpp:cellLocalId            int32

|  |  |  +--rw nrcellcu3gpp:pLMNIdList* [mcc mnc]

|  |  |  |  +--rw nrcellcu3gpp:mcc    Mcc

|  |  |  |  +--rw nrcellcu3gpp:mnc    Mnc

|  |  |  +--rw nrcellcu3gpp:sNSSAIList*            types3gpp:SNssai

|  |  |  +--rw nrcellcu3gpp:rRMPolicyType          uint16

|  |  |  +--rw nrcellcu3gpp:rRMPolicy              string

|  |  |  +--rw nrcellcu3gpp:rRMPolicyRatio1List* [groupId]

|  |  |  |  +--rw nrcellcu3gpp:groupId            uint32

|  |  |  |  +--rw nrcellcu3gpp:sNSSAIList*        types3gpp:SNssai

|  |  |  |  +--rw nrcellcu3gpp:rRRMPolicyRatio?   uint8

|  |  |  +--rw nrcellcu3gpp:rRMPolicyRatio2* [groupId]

|  |  |  |  +--rw nrcellcu3gpp:groupId                    uint32

|  |  |  |  +--rw nrcellcu3gpp:sNSSAIList*                types3gpp:SNssai

|  |  |  |  +--rw nrcellcu3gpp:quotaType?                 quotaType

|  |  |  |  +--rw nrcellcu3gpp:rRMPolicyMaxRatio?         uint8

|  |  |  |  +--rw nrcellcu3gpp:rRMPolicyMarginMaxRatio?   uint8

|  |  |  |  +--rw nrcellcu3gpp:rRMPolicyMinRatio?         uint8

|  |  |  |  +--rw nrcellcu3gpp:rRMPolicyMarginMinRatio?   uint8

|  |  |  +--ro nrcellcu3gpp:nRFrequencyRef?        types3gpp:DistinguishedName

|  |  +--rw eutranfreqrel3gpp:EUtranFreqRelation* [id]

|  |  |  +--rw eutranfreqrel3gpp:id            string

|  |  |  +--rw eutranfreqrel3gpp:attributes

|  |  |     +--rw eutranfreqrel3gpp:userLabel?                    string

|  |  |     +--rw eutranfreqrel3gpp:vnfParametersList!

|  |  |     |  +--rw eutranfreqrel3gpp:vnfInstanceId    string

|  |  |     |  +--rw eutranfreqrel3gpp:vnfdId?          string

|  |  |     |  +--rw eutranfreqrel3gpp:flavourId?       string

|  |  |     |  +--rw eutranfreqrel3gpp:autoScalable     boolean

|  |  |     +--rw eutranfreqrel3gpp:peeParametersList!

|  |  |     |  +--rw eutranfreqrel3gpp:siteIdentification    string

|  |  |     |  +--rw eutranfreqrel3gpp:siteLatitude?         decimal64

|  |  |     |  +--rw eutranfreqrel3gpp:siteLongitude?        decimal64

|  |  |     |  +--rw eutranfreqrel3gpp:siteDescription       string

|  |  |     |  +--rw eutranfreqrel3gpp:equipmentType         string

|  |  |     |  +--rw eutranfreqrel3gpp:environmentType       string

|  |  |     |  +--rw eutranfreqrel3gpp:powerInterface        string

|  |  |     +--rw eutranfreqrel3gpp:priorityLabel                 uint32

|  |  |     +--rw eutranfreqrel3gpp:cellIndividualOffset?         types3gpp:QOffsetRange

|  |  |     +--rw eutranfreqrel3gpp:blackListEntry*               uint16

|  |  |     +--rw eutranfreqrel3gpp:blackListEntryIdleMode*       uint16

|  |  |     +--rw eutranfreqrel3gpp:cellReselectionPriority       int32

|  |  |     +--rw eutranfreqrel3gpp:cellReselectionSubPriority?   uint8

|  |  |     +--rw eutranfreqrel3gpp:pMax                          int32

|  |  |     +--rw eutranfreqrel3gpp:qOffsetFreq?                  int32

|  |  |     +--rw eutranfreqrel3gpp:qQualMin                      int32

|  |  |     +--rw eutranfreqrel3gpp:qRxLevMin                     int32

|  |  |     +--rw eutranfreqrel3gpp:threshXHighP                  int32

|  |  |     +--rw eutranfreqrel3gpp:threshXHighQ                  int32

|  |  |     +--rw eutranfreqrel3gpp:threshXLowP                   int32

|  |  |     +--rw eutranfreqrel3gpp:threshXLowQ?                  int32

|  |  |     +--rw eutranfreqrel3gpp:tReselectionEutra             uint8

|  |  |     +--rw eutranfreqrel3gpp:tReselectionEutraSfHigh       uint8

|  |  |     +--rw eutranfreqrel3gpp:tReselectionEutraSfMedium     uint8

|  |  |     +--rw eutranfreqrel3gpp:eUtranFrequencyRef            types3gpp:DistinguishedName

|  |  +--rw eutrancellrel3gpp:EUtranCellRelation* [id]

|  |  |  +--rw eutrancellrel3gpp:id            string

|  |  |  +--rw eutrancellrel3gpp:attributes

|  |  |     +--rw eutrancellrel3gpp:userLabel?                      string

|  |  |     +--rw eutrancellrel3gpp:vnfParametersList!

|  |  |     |  +--rw eutrancellrel3gpp:vnfInstanceId    string

|  |  |     |  +--rw eutrancellrel3gpp:vnfdId?          string

|  |  |     |  +--rw eutrancellrel3gpp:flavourId?       string

|  |  |     |  +--rw eutrancellrel3gpp:autoScalable     boolean

|  |  |     +--rw eutrancellrel3gpp:peeParametersList!

|  |  |     |  +--rw eutrancellrel3gpp:siteIdentification    string

|  |  |     |  +--rw eutrancellrel3gpp:siteLatitude?         decimal64

|  |  |     |  +--rw eutrancellrel3gpp:siteLongitude?        decimal64

|  |  |     |  +--rw eutrancellrel3gpp:siteDescription       string

|  |  |     |  +--rw eutrancellrel3gpp:equipmentType         string

|  |  |     |  +--rw eutrancellrel3gpp:environmentType       string

|  |  |     |  +--rw eutrancellrel3gpp:powerInterface        string

|  |  |     +--rw eutrancellrel3gpp:priorityLabel                   uint32

|  |  |     +--rw eutrancellrel3gpp:tCI                             uint64

|  |  |     +--rw eutrancellrel3gpp:isRemoveAllowed                 ActionAllowed

|  |  |     +--rw eutrancellrel3gpp:isHOAllowed                     ActionAllowed

|  |  |     +--rw eutrancellrel3gpp:isICICInformationSendAllowed    ActionAllowed

|  |  |     +--rw eutrancellrel3gpp:isLBAllowed                     ActionAllowed

|  |  |     +--rw eutrancellrel3gpp:isESCoveredBy                   EnergySavingCoverage

|  |  |     +--rw eutrancellrel3gpp:qOffset                         types3gpp:QOffsetRange

|  |  |     +--ro eutrancellrel3gpp:cellIndividualOffset?           types3gpp:QOffsetRange

|  |  |     +--rw eutrancellrel3gpp:adjacentCell                    types3gpp:DistinguishedName

|  |  +--rw nrcellrel3gpp:NRCellRelation* [id]

|  |  |  +--rw nrcellrel3gpp:id            string

|  |  |  +--rw nrcellrel3gpp:attributes

|  |  |     +--rw nrcellrel3gpp:userLabel?              string

|  |  |     +--rw nrcellrel3gpp:vnfParametersList!

|  |  |     |  +--rw nrcellrel3gpp:vnfInstanceId    string

|  |  |     |  +--rw nrcellrel3gpp:vnfdId?          string

|  |  |     |  +--rw nrcellrel3gpp:flavourId?       string

|  |  |     |  +--rw nrcellrel3gpp:autoScalable     boolean

|  |  |     +--rw nrcellrel3gpp:peeParametersList!

|  |  |     |  +--rw nrcellrel3gpp:siteIdentification    string

|  |  |     |  +--rw nrcellrel3gpp:siteLatitude?         decimal64

|  |  |     |  +--rw nrcellrel3gpp:siteLongitude?        decimal64

|  |  |     |  +--rw nrcellrel3gpp:siteDescription       string

|  |  |     |  +--rw nrcellrel3gpp:equipmentType         string

|  |  |     |  +--rw nrcellrel3gpp:environmentType       string

|  |  |     |  +--rw nrcellrel3gpp:powerInterface        string

|  |  |     +--rw nrcellrel3gpp:priorityLabel           uint32

|  |  |     +--rw nrcellrel3gpp:nRTCI?                  uint64

|  |  |     +--rw nrcellrel3gpp:cellIndividualOffset

|  |  |     |  +--rw nrcellrel3gpp:rsrpOffsetSsb?     types3gpp:QOffsetRange

|  |  |     |  +--rw nrcellrel3gpp:rsrqOffsetSsb?     types3gpp:QOffsetRange

|  |  |     |  +--rw nrcellrel3gpp:sinrOffsetSsb?     types3gpp:QOffsetRange

|  |  |     |  +--rw nrcellrel3gpp:rsrpOffsetCsiRs?   types3gpp:QOffsetRange

|  |  |     |  +--rw nrcellrel3gpp:rsrqOffsetCsiRs?   types3gpp:QOffsetRange

|  |  |     |  +--rw nrcellrel3gpp:sinrOffsetCsiRs?   types3gpp:QOffsetRange

|  |  |     +--rw nrcellrel3gpp:nRFreqRelationRef       types3gpp:DistinguishedName

|  |  |     +--rw nrcellrel3gpp:adjacentNRCellRef       types3gpp:DistinguishedName

|  |  +--rw nrfreqrel3gpp:NRFreqRelation* [id]

|  |     +--rw nrfreqrel3gpp:id            string

|  |     +--rw nrfreqrel3gpp:attributes

|  |        +--rw nrfreqrel3gpp:userLabel?                    string

|  |        +--rw nrfreqrel3gpp:vnfParametersList!

|  |        |  +--rw nrfreqrel3gpp:vnfInstanceId    string

|  |        |  +--rw nrfreqrel3gpp:vnfdId?          string

|  |        |  +--rw nrfreqrel3gpp:flavourId?       string

|  |        |  +--rw nrfreqrel3gpp:autoScalable     boolean

|  |        +--rw nrfreqrel3gpp:peeParametersList!

|  |        |  +--rw nrfreqrel3gpp:siteIdentification    string

|  |        |  +--rw nrfreqrel3gpp:siteLatitude?         decimal64

|  |        |  +--rw nrfreqrel3gpp:siteLongitude?        decimal64

|  |        |  +--rw nrfreqrel3gpp:siteDescription       string

|  |        |  +--rw nrfreqrel3gpp:equipmentType         string

|  |        |  +--rw nrfreqrel3gpp:environmentType       string

|  |        |  +--rw nrfreqrel3gpp:powerInterface        string

|  |        +--rw nrfreqrel3gpp:priorityLabel                 uint32

|  |        +--rw nrfreqrel3gpp:offsetMO

|  |        |  +--rw nrfreqrel3gpp:rsrpOffsetSsb?     types3gpp:QOffsetRange

|  |        |  +--rw nrfreqrel3gpp:rsrqOffsetSsb?     types3gpp:QOffsetRange

|  |        |  +--rw nrfreqrel3gpp:sinrOffsetSsb?     types3gpp:QOffsetRange

|  |        |  +--rw nrfreqrel3gpp:rsrpOffsetCsiRs?   types3gpp:QOffsetRange

|  |        |  +--rw nrfreqrel3gpp:rsrqOffsetCsiRs?   types3gpp:QOffsetRange

|  |        |  +--rw nrfreqrel3gpp:sinrOffsetCsiRs?   types3gpp:QOffsetRange

|  |        +--rw nrfreqrel3gpp:blackListEntry*               uint16

|  |        +--rw nrfreqrel3gpp:blackListEntryIdleMode*       uint16

|  |        +--rw nrfreqrel3gpp:cellReselectionPriority?      uint32

|  |        +--rw nrfreqrel3gpp:cellReselectionSubPriority?   uint8

|  |        +--rw nrfreqrel3gpp:pMax?                         int32

|  |        +--rw nrfreqrel3gpp:qOffsetFreq?                  types3gpp:QOffsetRange

|  |        +--rw nrfreqrel3gpp:qQualMin?                     int32

|  |        +--rw nrfreqrel3gpp:qRxLevMin                     int32

|  |        +--rw nrfreqrel3gpp:threshXHighP                  int32

|  |        +--rw nrfreqrel3gpp:threshXHighQ                  int32

|  |        +--rw nrfreqrel3gpp:threshXLowP                   int32

|  |        +--rw nrfreqrel3gpp:threshXLowQ                   int32

|  |        +--rw nrfreqrel3gpp:tReselectionNR                int32

|  |        +--rw nrfreqrel3gpp:tReselectionNRSfHigh          uint8

|  |        +--rw nrfreqrel3gpp:tReselectionNRSfMedium        uint8

|  |        +--rw nrfreqrel3gpp:nRFrequencyRef                types3gpp:DistinguishedName

|  +--rw ep3gpp:EP_E1* [id]

|  |  +--rw ep3gpp:id            string

|  |  +--rw ep3gpp:attributes

|  |     +--rw ep3gpp:userLabel?                  string

|  |     +--ro ep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro ep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro ep3gpp:measurementType    string

|  |     |  +--ro ep3gpp:supportedGPs*      uint32

|  |     +--rw ep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw ep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw ep3gpp:vlanId       uint16

|  |     +--rw ep3gpp:remoteAddress               inet:ip-address

|  +--rw ep3gpp:EP_F1C* [id]

|  |  +--rw ep3gpp:id            string

|  |  +--rw ep3gpp:attributes

|  |     +--rw ep3gpp:userLabel?                  string

|  |     +--ro ep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro ep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro ep3gpp:measurementType    string

|  |     |  +--ro ep3gpp:supportedGPs*      uint32

|  |     +--rw ep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw ep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw ep3gpp:vlanId       uint16

|  |     +--rw ep3gpp:remoteAddress               inet:ip-address

|  +--rw ep3gpp:EP_NgC* [id]

|  |  +--rw ep3gpp:id            string

|  |  +--rw ep3gpp:attributes

|  |     +--rw ep3gpp:userLabel?                  string

|  |     +--ro ep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro ep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro ep3gpp:measurementType    string

|  |     |  +--ro ep3gpp:supportedGPs*      uint32

|  |     +--rw ep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw ep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw ep3gpp:vlanId       uint16

|  |     +--rw ep3gpp:remoteAddress               inet:ip-address

|  +--rw ep3gpp:EP_XnC* [id]

|  |  +--rw ep3gpp:id            string

|  |  +--rw ep3gpp:attributes

|  |     +--rw ep3gpp:userLabel?                  string

|  |     +--ro ep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro ep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro ep3gpp:measurementType    string

|  |     |  +--ro ep3gpp:supportedGPs*      uint32

|  |     +--rw ep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw ep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw ep3gpp:vlanId       uint16

|  |     +--rw ep3gpp:remoteAddress               inet:ip-address

|  +--rw ep3gpp:EP_X2C* [id]

|     +--rw ep3gpp:id            string

|     +--rw ep3gpp:attributes

|        +--rw ep3gpp:userLabel?                  string

|        +--ro ep3gpp:farEndEntity?               types3gpp:DistinguishedName

|        +--ro ep3gpp:supportedMeasurementsGPs* [measurementType]

|        |  +--ro ep3gpp:measurementType    string

|        |  +--ro ep3gpp:supportedGPs*      uint32

|        +--rw ep3gpp:localAddress* [ipAddress vlanId]

|        |  +--rw ep3gpp:ipAddress    inet:ip-address

|        |  +--rw ep3gpp:vlanId       uint16

|        +--rw ep3gpp:remoteAddress               inet:ip-address

+--rw sepp3gpp:SEPPFunction* [id]

|  +--rw sepp3gpp:id            string

|  +--rw sepp3gpp:attributes

|  |  +--rw sepp3gpp:userLabel?           string

|  |  +--rw sepp3gpp:vnfParametersList!

|  |  |  +--rw sepp3gpp:vnfInstanceId    string

|  |  |  +--rw sepp3gpp:vnfdId?          string

|  |  |  +--rw sepp3gpp:flavourId?       string

|  |  |  +--rw sepp3gpp:autoScalable     boolean

|  |  +--rw sepp3gpp:peeParametersList!

|  |  |  +--rw sepp3gpp:siteIdentification    string

|  |  |  +--rw sepp3gpp:siteLatitude?         decimal64

|  |  |  +--rw sepp3gpp:siteLongitude?        decimal64

|  |  |  +--rw sepp3gpp:siteDescription       string

|  |  |  +--rw sepp3gpp:equipmentType         string

|  |  |  +--rw sepp3gpp:environmentType       string

|  |  |  +--rw sepp3gpp:powerInterface        string

|  |  +--rw sepp3gpp:priorityLabel        uint32

|  |  +--rw sepp3gpp:pLMNIdList* [mcc mnc]

|  |     +--rw sepp3gpp:mcc    Mcc

|  |     +--rw sepp3gpp:mnc    Mnc

|  +--rw cep3gpp:EP_N32* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_SBI_IPX* [id]

|     +--rw cep3gpp:id            string

|     +--rw cep3gpp:attributes

|        +--rw cep3gpp:userLabel?                  string

|        +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|        +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|        |  +--ro cep3gpp:measurementType    string

|        |  +--ro cep3gpp:supportedGPs*      uint32

|        +--rw cep3gpp:localAddress* [ipAddress vlanId]

|        |  +--rw cep3gpp:ipAddress    inet:ip-address

|        |  +--rw cep3gpp:vlanId       uint16

|        +--rw cep3gpp:remoteAddress               inet:ip-address

|        +--ro cep3gpp:sBIService*                 string

+--rw gnbcuup3gpp:GNBCUUPFunction* [id]

|  +--rw gnbcuup3gpp:id            string

|  +--rw gnbcuup3gpp:attributes

|  |  +--rw gnbcuup3gpp:userLabel?           string

|  |  +--rw gnbcuup3gpp:vnfParametersList!

|  |  |  +--rw gnbcuup3gpp:vnfInstanceId    string

|  |  |  +--rw gnbcuup3gpp:vnfdId?          string

|  |  |  +--rw gnbcuup3gpp:flavourId?       string

|  |  |  +--rw gnbcuup3gpp:autoScalable     boolean

|  |  +--rw gnbcuup3gpp:peeParametersList!

|  |  |  +--rw gnbcuup3gpp:siteIdentification    string

|  |  |  +--rw gnbcuup3gpp:siteLatitude?         decimal64

|  |  |  +--rw gnbcuup3gpp:siteLongitude?        decimal64

|  |  |  +--rw gnbcuup3gpp:siteDescription       string

|  |  |  +--rw gnbcuup3gpp:equipmentType         string

|  |  |  +--rw gnbcuup3gpp:environmentType       string

|  |  |  +--rw gnbcuup3gpp:powerInterface        string

|  |  +--rw gnbcuup3gpp:priorityLabel        uint32

|  |  +--rw gnbcuup3gpp:gNBId                int64

|  |  +--rw gnbcuup3gpp:gNBIdLength          int32

|  |  +--rw gnbcuup3gpp:pLMNIdList* [mcc mnc]

|  |     +--rw gnbcuup3gpp:mcc    Mcc

|  |     +--rw gnbcuup3gpp:mnc    Mnc

|  +--rw ep3gpp:EP_E1* [id]

|  |  +--rw ep3gpp:id            string

|  |  +--rw ep3gpp:attributes

|  |     +--rw ep3gpp:userLabel?                  string

|  |     +--ro ep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro ep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro ep3gpp:measurementType    string

|  |     |  +--ro ep3gpp:supportedGPs*      uint32

|  |     +--rw ep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw ep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw ep3gpp:vlanId       uint16

|  |     +--rw ep3gpp:remoteAddress               inet:ip-address

|  +--rw ep3gpp:EP_F1U* [id]

|  |  +--rw ep3gpp:id            string

|  |  +--rw ep3gpp:attributes

|  |     +--rw ep3gpp:userLabel?                  string

|  |     +--ro ep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro ep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro ep3gpp:measurementType    string

|  |     |  +--ro ep3gpp:supportedGPs*      uint32

|  |     +--rw ep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw ep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw ep3gpp:vlanId       uint16

|  |     +--rw ep3gpp:remoteAddress               inet:ip-address

|  +--rw ep3gpp:EP_NgU* [id]

|  |  +--rw ep3gpp:id            string

|  |  +--rw ep3gpp:attributes

|  |     +--rw ep3gpp:userLabel?                  string

|  |     +--ro ep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro ep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro ep3gpp:measurementType    string

|  |     |  +--ro ep3gpp:supportedGPs*      uint32

|  |     +--rw ep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw ep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw ep3gpp:vlanId       uint16

|  |     +--rw ep3gpp:remoteAddress               inet:ip-address

|  +--rw ep3gpp:EP_XnU* [id]

|  |  +--rw ep3gpp:id            string

|  |  +--rw ep3gpp:attributes

|  |     +--rw ep3gpp:userLabel?                  string

|  |     +--ro ep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro ep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro ep3gpp:measurementType    string

|  |     |  +--ro ep3gpp:supportedGPs*      uint32

|  |     +--rw ep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw ep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw ep3gpp:vlanId       uint16

|  |     +--rw ep3gpp:remoteAddress               inet:ip-address

|  +--rw ep3gpp:EP_X2U* [id]

|  |  +--rw ep3gpp:id            string

|  |  +--rw ep3gpp:attributes

|  |     +--rw ep3gpp:userLabel?                  string

|  |     +--ro ep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro ep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro ep3gpp:measurementType    string

|  |     |  +--ro ep3gpp:supportedGPs*      uint32

|  |     +--rw ep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw ep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw ep3gpp:vlanId       uint16

|  |     +--rw ep3gpp:remoteAddress               inet:ip-address

|  +--rw ep3gpp:EP_S1U* [id]

|     +--rw ep3gpp:id            string

|     +--rw ep3gpp:attributes

|        +--rw ep3gpp:userLabel?                  string

|        +--ro ep3gpp:farEndEntity?               types3gpp:DistinguishedName

|        +--ro ep3gpp:supportedMeasurementsGPs* [measurementType]

|        |  +--ro ep3gpp:measurementType    string

|        |  +--ro ep3gpp:supportedGPs*      uint32

|        +--rw ep3gpp:localAddress* [ipAddress vlanId]

|        |  +--rw ep3gpp:ipAddress    inet:ip-address

|        |  +--rw ep3gpp:vlanId       uint16

|        +--rw ep3gpp:remoteAddress               inet:ip-address

+--rw nwdaf3gpp:NWDAFFunction* [id]

|  +--rw nwdaf3gpp:id            string

|  +--rw nwdaf3gpp:attributes

|  |  +--rw nwdaf3gpp:userLabel?           string

|  |  +--rw nwdaf3gpp:vnfParametersList!

|  |  |  +--rw nwdaf3gpp:vnfInstanceId    string

|  |  |  +--rw nwdaf3gpp:vnfdId?          string

|  |  |  +--rw nwdaf3gpp:flavourId?       string

|  |  |  +--rw nwdaf3gpp:autoScalable     boolean

|  |  +--rw nwdaf3gpp:peeParametersList!

|  |  |  +--rw nwdaf3gpp:siteIdentification    string

|  |  |  +--rw nwdaf3gpp:siteLatitude?         decimal64

|  |  |  +--rw nwdaf3gpp:siteLongitude?        decimal64

|  |  |  +--rw nwdaf3gpp:siteDescription       string

|  |  |  +--rw nwdaf3gpp:equipmentType         string

|  |  |  +--rw nwdaf3gpp:environmentType       string

|  |  |  +--rw nwdaf3gpp:powerInterface        string

|  |  +--rw nwdaf3gpp:priorityLabel        uint32

|  |  +--rw nwdaf3gpp:pLMNIdList* [mcc mnc]

|  |  |  +--rw nwdaf3gpp:mcc    Mcc

|  |  |  +--rw nwdaf3gpp:mnc    Mnc

|  |  +--rw nwdaf3gpp:sBIFQDN?             inet:domain-name

|  |  +--ro nwdaf3gpp:sBIServiceList*      string

|  |  +--rw nwdaf3gpp:sNSSAIList*          types3gpp:SNssai

|  +--rw cep3gpp:EP_SBI_X* [id]

|     +--rw cep3gpp:id            string

|     +--rw cep3gpp:attributes

|        +--rw cep3gpp:userLabel?                  string

|        +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|        +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|        |  +--ro cep3gpp:measurementType    string

|        |  +--ro cep3gpp:supportedGPs*      uint32

|        +--rw cep3gpp:localAddress* [ipAddress vlanId]

|        |  +--rw cep3gpp:ipAddress    inet:ip-address

|        |  +--rw cep3gpp:vlanId       uint16

|        +--rw cep3gpp:remoteAddress               inet:ip-address

|        +--ro cep3gpp:sBIService*                 string

+--rw smsf3gpp:SMSFFunction* [id]

|  +--rw smsf3gpp:id            string

|  +--rw smsf3gpp:attributes

|  |  +--rw smsf3gpp:userLabel?           string

|  |  +--rw smsf3gpp:vnfParametersList!

|  |  |  +--rw smsf3gpp:vnfInstanceId    string

|  |  |  +--rw smsf3gpp:vnfdId?          string

|  |  |  +--rw smsf3gpp:flavourId?       string

|  |  |  +--rw smsf3gpp:autoScalable     boolean

|  |  +--rw smsf3gpp:peeParametersList!

|  |  |  +--rw smsf3gpp:siteIdentification    string

|  |  |  +--rw smsf3gpp:siteLatitude?         decimal64

|  |  |  +--rw smsf3gpp:siteLongitude?        decimal64

|  |  |  +--rw smsf3gpp:siteDescription       string

|  |  |  +--rw smsf3gpp:equipmentType         string

|  |  |  +--rw smsf3gpp:environmentType       string

|  |  |  +--rw smsf3gpp:powerInterface        string

|  |  +--rw smsf3gpp:priorityLabel        uint32

|  |  +--rw smsf3gpp:pLMNIdList* [mcc mnc]

|  |  |  +--rw smsf3gpp:mcc    Mcc

|  |  |  +--rw smsf3gpp:mnc    Mnc

|  |  +--rw smsf3gpp:sBIFQDN?             inet:domain-name

|  |  +--ro smsf3gpp:sBIServiceList*      string

|  +--rw cep3gpp:EP_N20* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_N21* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_MAP_SMSC* [id]

|     +--rw cep3gpp:id            string

|     +--rw cep3gpp:attributes

|        +--rw cep3gpp:userLabel?                  string

|        +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|        +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|        |  +--ro cep3gpp:measurementType    string

|        |  +--ro cep3gpp:supportedGPs*      uint32

|        +--rw cep3gpp:localAddress* [ipAddress vlanId]

|        |  +--rw cep3gpp:ipAddress    inet:ip-address

|        |  +--rw cep3gpp:vlanId       uint16

|        +--rw cep3gpp:remoteAddress               inet:ip-address

+--rw udsf3gpp:UDSFFunction* [id]

|  +--rw udsf3gpp:id            string

|  +--rw udsf3gpp:attributes

|  |  +--rw udsf3gpp:userLabel?           string

|  |  +--rw udsf3gpp:vnfParametersList!

|  |  |  +--rw udsf3gpp:vnfInstanceId    string

|  |  |  +--rw udsf3gpp:vnfdId?          string

|  |  |  +--rw udsf3gpp:flavourId?       string

|  |  |  +--rw udsf3gpp:autoScalable     boolean

|  |  +--rw udsf3gpp:peeParametersList!

|  |  |  +--rw udsf3gpp:siteIdentification    string

|  |  |  +--rw udsf3gpp:siteLatitude?         decimal64

|  |  |  +--rw udsf3gpp:siteLongitude?        decimal64

|  |  |  +--rw udsf3gpp:siteDescription       string

|  |  |  +--rw udsf3gpp:equipmentType         string

|  |  |  +--rw udsf3gpp:environmentType       string

|  |  |  +--rw udsf3gpp:powerInterface        string

|  |  +--rw udsf3gpp:priorityLabel        uint32

|  |  +--rw udsf3gpp:pLMNIdList* [mcc mnc]

|  |  |  +--rw udsf3gpp:mcc    Mcc

|  |  |  +--rw udsf3gpp:mnc    Mnc

|  |  +--rw udsf3gpp:sBIFQDN?             inet:domain-name

|  |  +--ro udsf3gpp:sBIServiceList*      string

|  |  +--rw udsf3gpp:sNSSAIList*          types3gpp:SNssai

|  +--rw cep3gpp:EP_SBI_X* [id]

|     +--rw cep3gpp:id            string

|     +--rw cep3gpp:attributes

|        +--rw cep3gpp:userLabel?                  string

|        +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|        +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|        |  +--ro cep3gpp:measurementType    string

|        |  +--ro cep3gpp:supportedGPs*      uint32

|        +--rw cep3gpp:localAddress* [ipAddress vlanId]

|        |  +--rw cep3gpp:ipAddress    inet:ip-address

|        |  +--rw cep3gpp:vlanId       uint16

|        +--rw cep3gpp:remoteAddress               inet:ip-address

|        +--ro cep3gpp:sBIService*                 string

+--rw dn3gpp:DNFunction* [id]

|  +--rw dn3gpp:id            string

|  +--rw dn3gpp:attributes

|  |  +--rw dn3gpp:userLabel?           string

|  |  +--rw dn3gpp:vnfParametersList!

|  |  |  +--rw dn3gpp:vnfInstanceId    string

|  |  |  +--rw dn3gpp:vnfdId?          string

|  |  |  +--rw dn3gpp:flavourId?       string

|  |  |  +--rw dn3gpp:autoScalable     boolean

|  |  +--rw dn3gpp:peeParametersList!

|  |  |  +--rw dn3gpp:siteIdentification    string

|  |  |  +--rw dn3gpp:siteLatitude?         decimal64

|  |  |  +--rw dn3gpp:siteLongitude?        decimal64

|  |  |  +--rw dn3gpp:siteDescription       string

|  |  |  +--rw dn3gpp:equipmentType         string

|  |  |  +--rw dn3gpp:environmentType       string

|  |  |  +--rw dn3gpp:powerInterface        string

|  |  +--rw dn3gpp:priorityLabel        uint32

|  +--rw cep3gpp:EP_N6* [id]

|     +--rw cep3gpp:id            string

|     +--rw cep3gpp:attributes

|        +--rw cep3gpp:userLabel?                  string

|        +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|        +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|        |  +--ro cep3gpp:measurementType    string

|        |  +--ro cep3gpp:supportedGPs*      uint32

|        +--rw cep3gpp:localAddress* [ipAddress vlanId]

|        |  +--rw cep3gpp:ipAddress    inet:ip-address

|        |  +--rw cep3gpp:vlanId       uint16

|        +--rw cep3gpp:remoteAddress               inet:ip-address

+--rw nssf3gpp:NSSFFunction* [id]

|  +--rw nssf3gpp:id            string

|  +--rw nssf3gpp:attributes

|  |  +--rw nssf3gpp:userLabel?           string

|  |  +--rw nssf3gpp:vnfParametersList!

|  |  |  +--rw nssf3gpp:vnfInstanceId    string

|  |  |  +--rw nssf3gpp:vnfdId?          string

|  |  |  +--rw nssf3gpp:flavourId?       string

|  |  |  +--rw nssf3gpp:autoScalable     boolean

|  |  +--rw nssf3gpp:peeParametersList!

|  |  |  +--rw nssf3gpp:siteIdentification    string

|  |  |  +--rw nssf3gpp:siteLatitude?         decimal64

|  |  |  +--rw nssf3gpp:siteLongitude?        decimal64

|  |  |  +--rw nssf3gpp:siteDescription       string

|  |  |  +--rw nssf3gpp:equipmentType         string

|  |  |  +--rw nssf3gpp:environmentType       string

|  |  |  +--rw nssf3gpp:powerInterface        string

|  |  +--rw nssf3gpp:priorityLabel        uint32

|  |  +--rw nssf3gpp:pLMNIdList* [mcc mnc]

|  |  |  +--rw nssf3gpp:mcc    Mcc

|  |  |  +--rw nssf3gpp:mnc    Mnc

|  |  +--rw nssf3gpp:sBIFQDN?             inet:domain-name

|  |  +--rw nssf3gpp:sNSSAIList*          types3gpp:SNssai

|  |  +--rw nssf3gpp:nSIIdListWrap*       types3gpp:NsiId

|  +--rw cep3gpp:EP_N22* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_N31* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_SBI_X* [id]

|     +--rw cep3gpp:id            string

|     +--rw cep3gpp:attributes

|        +--rw cep3gpp:userLabel?                  string

|        +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|        +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|        |  +--ro cep3gpp:measurementType    string

|        |  +--ro cep3gpp:supportedGPs*      uint32

|        +--rw cep3gpp:localAddress* [ipAddress vlanId]

|        |  +--rw cep3gpp:ipAddress    inet:ip-address

|        |  +--rw cep3gpp:vlanId       uint16

|        +--rw cep3gpp:remoteAddress               inet:ip-address

|        +--ro cep3gpp:sBIService*                 string

+--rw ausf3gpp:AUSFFunction* [id]

|  +--rw ausf3gpp:id            string

|  +--rw ausf3gpp:attributes

|  |  +--rw ausf3gpp:userLabel?           string

|  |  +--rw ausf3gpp:vnfParametersList!

|  |  |  +--rw ausf3gpp:vnfInstanceId    string

|  |  |  +--rw ausf3gpp:vnfdId?          string

|  |  |  +--rw ausf3gpp:flavourId?       string

|  |  |  +--rw ausf3gpp:autoScalable     boolean

|  |  +--rw ausf3gpp:peeParametersList!

|  |  |  +--rw ausf3gpp:siteIdentification    string

|  |  |  +--rw ausf3gpp:siteLatitude?         decimal64

|  |  |  +--rw ausf3gpp:siteLongitude?        decimal64

|  |  |  +--rw ausf3gpp:siteDescription       string

|  |  |  +--rw ausf3gpp:equipmentType         string

|  |  |  +--rw ausf3gpp:environmentType       string

|  |  |  +--rw ausf3gpp:powerInterface        string

|  |  +--rw ausf3gpp:priorityLabel        uint32

|  |  +--rw ausf3gpp:pLMNIdList* [mcc mnc]

|  |  |  +--rw ausf3gpp:mcc    Mcc

|  |  |  +--rw ausf3gpp:mnc    Mnc

|  |  +--rw ausf3gpp:sBIFQDN?             inet:domain-name

|  |  +--ro ausf3gpp:sBIServiceList*      string

|  |  +--rw ausf3gpp:sNSSAIList*          types3gpp:SNssai

|  +--rw cep3gpp:EP_N12* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_N13* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_SBI_X* [id]

|     +--rw cep3gpp:id            string

|     +--rw cep3gpp:attributes

|        +--rw cep3gpp:userLabel?                  string

|        +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|        +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|        |  +--ro cep3gpp:measurementType    string

|        |  +--ro cep3gpp:supportedGPs*      uint32

|        +--rw cep3gpp:localAddress* [ipAddress vlanId]

|        |  +--rw cep3gpp:ipAddress    inet:ip-address

|        |  +--rw cep3gpp:vlanId       uint16

|        +--rw cep3gpp:remoteAddress               inet:ip-address

|        +--ro cep3gpp:sBIService*                 string

+--rw upf3gpp:UPFFunction* [id]

|  +--rw upf3gpp:id            string

|  +--rw upf3gpp:attributes

|  |  +--rw upf3gpp:userLabel?           string

|  |  +--rw upf3gpp:vnfParametersList!

|  |  |  +--rw upf3gpp:vnfInstanceId    string

|  |  |  +--rw upf3gpp:vnfdId?          string

|  |  |  +--rw upf3gpp:flavourId?       string

|  |  |  +--rw upf3gpp:autoScalable     boolean

|  |  +--rw upf3gpp:peeParametersList!

|  |  |  +--rw upf3gpp:siteIdentification    string

|  |  |  +--rw upf3gpp:siteLatitude?         decimal64

|  |  |  +--rw upf3gpp:siteLongitude?        decimal64

|  |  |  +--rw upf3gpp:siteDescription       string

|  |  |  +--rw upf3gpp:equipmentType         string

|  |  |  +--rw upf3gpp:environmentType       string

|  |  |  +--rw upf3gpp:powerInterface        string

|  |  +--rw upf3gpp:priorityLabel        uint32

|  |  +--rw upf3gpp:pLMNIdList* [mcc mnc]

|  |  |  +--rw upf3gpp:mcc    Mcc

|  |  |  +--rw upf3gpp:mnc    Mnc

|  |  +--ro upf3gpp:nRTACList*           types3gpp:Tac

|  |  +--rw upf3gpp:sNSSAIList*          types3gpp:SNssai

|  +--rw cep3gpp:EP_N4* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_N3* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_N9* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_S5U* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_SBI_X* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  |     +--ro cep3gpp:sBIService*                 string

|  +--rw cep3gpp:EP_N6* [id]

|     +--rw cep3gpp:id            string

|     +--rw cep3gpp:attributes

|        +--rw cep3gpp:userLabel?                  string

|        +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|        +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|        |  +--ro cep3gpp:measurementType    string

|        |  +--ro cep3gpp:supportedGPs*      uint32

|        +--rw cep3gpp:localAddress* [ipAddress vlanId]

|        |  +--rw cep3gpp:ipAddress    inet:ip-address

|        |  +--rw cep3gpp:vlanId       uint16

|        +--rw cep3gpp:remoteAddress               inet:ip-address

+--rw lmf3gpp:LMFFunction* [id]

|  +--rw lmf3gpp:id            string

|  +--rw lmf3gpp:attributes

|  |  +--rw lmf3gpp:userLabel?           string

|  |  +--rw lmf3gpp:vnfParametersList!

|  |  |  +--rw lmf3gpp:vnfInstanceId    string

|  |  |  +--rw lmf3gpp:vnfdId?          string

|  |  |  +--rw lmf3gpp:flavourId?       string

|  |  |  +--rw lmf3gpp:autoScalable     boolean

|  |  +--rw lmf3gpp:peeParametersList!

|  |  |  +--rw lmf3gpp:siteIdentification    string

|  |  |  +--rw lmf3gpp:siteLatitude?         decimal64

|  |  |  +--rw lmf3gpp:siteLongitude?        decimal64

|  |  |  +--rw lmf3gpp:siteDescription       string

|  |  |  +--rw lmf3gpp:equipmentType         string

|  |  |  +--rw lmf3gpp:environmentType       string

|  |  |  +--rw lmf3gpp:powerInterface        string

|  |  +--rw lmf3gpp:priorityLabel        uint32

|  |  +--rw lmf3gpp:pLMNIdList* [mcc mnc]

|  |     +--rw lmf3gpp:mcc    Mcc

|  |     +--rw lmf3gpp:mnc    Mnc

|  +--rw cep3gpp:EP_NLS* [id]

|     +--rw cep3gpp:id            string

|     +--rw cep3gpp:attributes

|        +--rw cep3gpp:userLabel?                  string

|        +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|        +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|        |  +--ro cep3gpp:measurementType    string

|        |  +--ro cep3gpp:supportedGPs*      uint32

|        +--rw cep3gpp:localAddress* [ipAddress vlanId]

|        |  +--rw cep3gpp:ipAddress    inet:ip-address

|        |  +--rw cep3gpp:vlanId       uint16

|        +--rw cep3gpp:remoteAddress               inet:ip-address

+--rw amf3gpp:AMFFunction* [id]

|  +--rw amf3gpp:id            string

|  +--rw amf3gpp:attributes

|  |  +--rw amf3gpp:userLabel?           string

|  |  +--rw amf3gpp:vnfParametersList!

|  |  |  +--rw amf3gpp:vnfInstanceId    string

|  |  |  +--rw amf3gpp:vnfdId?          string

|  |  |  +--rw amf3gpp:flavourId?       string

|  |  |  +--rw amf3gpp:autoScalable     boolean

|  |  +--rw amf3gpp:peeParametersList!

|  |  |  +--rw amf3gpp:siteIdentification    string

|  |  |  +--rw amf3gpp:siteLatitude?         decimal64

|  |  |  +--rw amf3gpp:siteLongitude?        decimal64

|  |  |  +--rw amf3gpp:siteDescription       string

|  |  |  +--rw amf3gpp:equipmentType         string

|  |  |  +--rw amf3gpp:environmentType       string

|  |  |  +--rw amf3gpp:powerInterface        string

|  |  +--rw amf3gpp:priorityLabel        uint32

|  |  +--rw amf3gpp:pLMNIdList* [mcc mnc]

|  |  |  +--rw amf3gpp:mcc    Mcc

|  |  |  +--rw amf3gpp:mnc    Mnc

|  |  +--rw amf3gpp:aMFIdentifier!

|  |  |  +--rw amf3gpp:amfRegionId?   AmfRegionId

|  |  |  +--rw amf3gpp:amfSetId?      AmfSetId

|  |  |  +--rw amf3gpp:amfPointer?    AmfPointer

|  |  +--rw amf3gpp:sBIFQDN?             inet:domain-name

|  |  +--ro amf3gpp:sBIServiceList*      string

|  |  +--rw amf3gpp:weightFactor?        uint16

|  |  +--rw amf3gpp:sNSSAIList*          types3gpp:SNssai

|  +--rw cep3gpp:EP_N2* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_N8* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_N11* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_N12* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_N14* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_N15* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_N17* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_N20* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_N22* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_N26* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_NLS* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_NLG* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_SBI_X* [id]

|     +--rw cep3gpp:id            string

|     +--rw cep3gpp:attributes

|        +--rw cep3gpp:userLabel?                  string

|        +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|        +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|        |  +--ro cep3gpp:measurementType    string

|        |  +--ro cep3gpp:supportedGPs*      uint32

|        +--rw cep3gpp:localAddress* [ipAddress vlanId]

|        |  +--rw cep3gpp:ipAddress    inet:ip-address

|        |  +--rw cep3gpp:vlanId       uint16

|        +--rw cep3gpp:remoteAddress               inet:ip-address

|        +--ro cep3gpp:sBIService*                 string

+--rw nrf3gpp:NRFFunction* [id]

|  +--rw nrf3gpp:id            string

|  +--rw nrf3gpp:attributes

|  |  +--rw nrf3gpp:userLabel?           string

|  |  +--rw nrf3gpp:vnfParametersList!

|  |  |  +--rw nrf3gpp:vnfInstanceId    string

|  |  |  +--rw nrf3gpp:vnfdId?          string

|  |  |  +--rw nrf3gpp:flavourId?       string

|  |  |  +--rw nrf3gpp:autoScalable     boolean

|  |  +--rw nrf3gpp:peeParametersList!

|  |  |  +--rw nrf3gpp:siteIdentification    string

|  |  |  +--rw nrf3gpp:siteLatitude?         decimal64

|  |  |  +--rw nrf3gpp:siteLongitude?        decimal64

|  |  |  +--rw nrf3gpp:siteDescription       string

|  |  |  +--rw nrf3gpp:equipmentType         string

|  |  |  +--rw nrf3gpp:environmentType       string

|  |  |  +--rw nrf3gpp:powerInterface        string

|  |  +--rw nrf3gpp:priorityLabel        uint32

|  |  +--rw nrf3gpp:pLMNIdList* [mcc mnc]

|  |  |  +--rw nrf3gpp:mcc    Mcc

|  |  |  +--rw nrf3gpp:mnc    Mnc

|  |  +--rw nrf3gpp:sBIFQDN?             inet:domain-name

|  |  +--rw nrf3gpp:nSIIdListWrap*       types3gpp:NsiId

|  |  +--rw nrf3gpp:sNSSAIList*          types3gpp:SNssai

|  |  +--rw nrf3gpp:nFProfileList* [nfInstanceID]

|  |     +--rw nrf3gpp:nfInstanceID                        string

|  |     +--rw nrf3gpp:nfType                              types3gpp:NfType

|  |     +--rw nrf3gpp:nfStatus                            NFStatus

|  |     +--rw nrf3gpp:heartBeatTimer?                     uint16

|  |     +--rw nrf3gpp:plmnList* [mcc mnc]

|  |     |  +--rw nrf3gpp:mcc    Mcc

|  |     |  +--rw nrf3gpp:mnc    Mnc

|  |     +--rw nrf3gpp:sNssais* [sst sd]

|  |     |  +--rw nrf3gpp:sst    uint32

|  |     |  +--rw nrf3gpp:sd     string

|  |     +--rw nrf3gpp:perPlmnSnssaiList* [idx]

|  |     |  +--rw nrf3gpp:idx           uint32

|  |     |  +--rw nrf3gpp:plmnId* [mcc mnc]

|  |     |  |  +--rw nrf3gpp:mcc    Mcc

|  |     |  |  +--rw nrf3gpp:mnc    Mnc

|  |     |  +--rw nrf3gpp:sNssaiList* [sst sd]

|  |     |     +--rw nrf3gpp:sst    uint32

|  |     |     +--rw nrf3gpp:sd     string

|  |     +--rw nrf3gpp:nsiList*                            string

|  |     +--rw nrf3gpp:fqdn?                               inet:domain-name

|  |     +--rw nrf3gpp:interPlmnFqdn?                      inet:domain-name

|  |     +--rw nrf3gpp:ipv4Addresses*                      inet:ipv4-address

|  |     +--rw nrf3gpp:ipv6Addresses*                      inet:ipv6-address

|  |     +--rw nrf3gpp:allowedPlmns* [mcc mnc]

|  |     |  +--rw nrf3gpp:mcc    Mcc

|  |     |  +--rw nrf3gpp:mnc    Mnc

|  |     +--rw nrf3gpp:allowedNfTypes*                     types3gpp:NfType

|  |     +--rw nrf3gpp:allowedNfDomains*                   string

|  |     +--rw nrf3gpp:allowedNssais* [sst sd]

|  |     |  +--rw nrf3gpp:sst    uint32

|  |     |  +--rw nrf3gpp:sd     string

|  |     +--rw nrf3gpp:priority?                           uint16

|  |     +--rw nrf3gpp:capacity?                           uint16

|  |     +--rw nrf3gpp:load?                               types3gpp:Load

|  |     +--rw nrf3gpp:locality?                           string

|  |     +--rw nrf3gpp:nrfInfo* [idx]

|  |     |  +--rw nrf3gpp:idx               uint32

|  |     |  +--rw nrf3gpp:servedUdrInfo* [nfInstanceID]

|  |     |  |  +--rw nrf3gpp:nfInstanceID                      string

|  |     |  |  +--rw nrf3gpp:groupId?                          string

|  |     |  |  +--rw nrf3gpp:supiRanges* [start end pattern]

|  |     |  |  |  +--rw nrf3gpp:start      string

|  |     |  |  |  +--rw nrf3gpp:end        string

|  |     |  |  |  +--rw nrf3gpp:pattern    string

|  |     |  |  +--rw nrf3gpp:gpsiRanges* [start end pattern]

|  |     |  |  |  +--rw nrf3gpp:start      string

|  |     |  |  |  +--rw nrf3gpp:end        string

|  |     |  |  |  +--rw nrf3gpp:pattern    string

|  |     |  |  +--rw nrf3gpp:externalGroupIdentifiersRanges* [start end pattern]

|  |     |  |  |  +--rw nrf3gpp:start      string

|  |     |  |  |  +--rw nrf3gpp:end        string

|  |     |  |  |  +--rw nrf3gpp:pattern    string

|  |     |  |  +--rw nrf3gpp:supportedDataSets*                DataSetId

|  |     |  +--rw nrf3gpp:servedUdmInfo* [nfInstanceID]

|  |     |  |  +--rw nrf3gpp:nfInstanceID                      string

|  |     |  |  +--rw nrf3gpp:groupId?                          string

|  |     |  |  +--rw nrf3gpp:supiRanges* [start end pattern]

|  |     |  |  |  +--rw nrf3gpp:start      string

|  |     |  |  |  +--rw nrf3gpp:end        string

|  |     |  |  |  +--rw nrf3gpp:pattern    string

|  |     |  |  +--rw nrf3gpp:gpsiRanges* [start end pattern]

|  |     |  |  |  +--rw nrf3gpp:start      string

|  |     |  |  |  +--rw nrf3gpp:end        string

|  |     |  |  |  +--rw nrf3gpp:pattern    string

|  |     |  |  +--rw nrf3gpp:externalGroupIdentifiersRanges* [start end pattern]

|  |     |  |  |  +--rw nrf3gpp:start      string

|  |     |  |  |  +--rw nrf3gpp:end        string

|  |     |  |  |  +--rw nrf3gpp:pattern    string

|  |     |  |  +--rw nrf3gpp:routingIndicators*                string

|  |     |  +--rw nrf3gpp:servedAusfInfo* [nfInstanceID]

|  |     |  |  +--rw nrf3gpp:nfInstanceID         string

|  |     |  |  +--rw nrf3gpp:groupId?             string

|  |     |  |  +--rw nrf3gpp:supiRanges* [start end pattern]

|  |     |  |  |  +--rw nrf3gpp:start      string

|  |     |  |  |  +--rw nrf3gpp:end        string

|  |     |  |  |  +--rw nrf3gpp:pattern    string

|  |     |  |  +--rw nrf3gpp:routingIndicators*   string

|  |     |  +--rw nrf3gpp:servedAmfInfo* [nfInstanceID]

|  |     |  |  +--rw nrf3gpp:nfInstanceID            string

|  |     |  |  +--rw nrf3gpp:amfRegionId?            string

|  |     |  |  +--rw nrf3gpp:amfSetId?               string

|  |     |  |  +--rw nrf3gpp:guamiList* [idx]

|  |     |  |  |  +--rw nrf3gpp:idx       uint32

|  |     |  |  |  +--rw nrf3gpp:plmnId* [mcc mnc]

|  |     |  |  |  |  +--rw nrf3gpp:mcc    Mcc

|  |     |  |  |  |  +--rw nrf3gpp:mnc    Mnc

|  |     |  |  |  +--rw nrf3gpp:amfId* [amfRegionId amfSetId amfPointer]

|  |     |  |  |     +--rw nrf3gpp:amfRegionId    AmfRegionId

|  |     |  |  |     +--rw nrf3gpp:amfSetId       AmfSetId

|  |     |  |  |     +--rw nrf3gpp:amfPointer     AmfPointer

|  |     |  |  +--rw nrf3gpp:taiList* [idx]

|  |     |  |  |  +--rw nrf3gpp:idx       uint32

|  |     |  |  |  +--rw nrf3gpp:plmnId* [mcc mnc]

|  |     |  |  |  |  +--rw nrf3gpp:mcc    Mcc

|  |     |  |  |  |  +--rw nrf3gpp:mnc    Mnc

|  |     |  |  |  +--rw nrf3gpp:tac?      types3gpp:Tac

|  |     |  |  +--rw nrf3gpp:taiRangeList* [idx]

|  |     |  |  |  +--rw nrf3gpp:idx             uint32

|  |     |  |  |  +--rw nrf3gpp:plmnId* [mcc mnc]

|  |     |  |  |  |  +--rw nrf3gpp:mcc    Mcc

|  |     |  |  |  |  +--rw nrf3gpp:mnc    Mnc

|  |     |  |  |  +--rw nrf3gpp:tacRangeList* [start end]

|  |     |  |  |     +--rw nrf3gpp:start      string

|  |     |  |  |     +--rw nrf3gpp:end        string

|  |     |  |  |     +--rw nrf3gpp:pattern?   string

|  |     |  |  +--rw nrf3gpp:backupInfoAmfFailure* [idx]

|  |     |  |  |  +--rw nrf3gpp:idx       uint32

|  |     |  |  |  +--rw nrf3gpp:plmnId* [mcc mnc]

|  |     |  |  |  |  +--rw nrf3gpp:mcc    Mcc

|  |     |  |  |  |  +--rw nrf3gpp:mnc    Mnc

|  |     |  |  |  +--rw nrf3gpp:amfId* [amfRegionId amfSetId amfPointer]

|  |     |  |  |     +--rw nrf3gpp:amfRegionId    AmfRegionId

|  |     |  |  |     +--rw nrf3gpp:amfSetId       AmfSetId

|  |     |  |  |     +--rw nrf3gpp:amfPointer     AmfPointer

|  |     |  |  +--rw nrf3gpp:backupInfoAmfRemoval* [idx]

|  |     |  |  |  +--rw nrf3gpp:idx       uint32

|  |     |  |  |  +--rw nrf3gpp:plmnId* [mcc mnc]

|  |     |  |  |  |  +--rw nrf3gpp:mcc    Mcc

|  |     |  |  |  |  +--rw nrf3gpp:mnc    Mnc

|  |     |  |  |  +--rw nrf3gpp:amfId* [amfRegionId amfSetId amfPointer]

|  |     |  |  |     +--rw nrf3gpp:amfRegionId    AmfRegionId

|  |     |  |  |     +--rw nrf3gpp:amfSetId       AmfSetId

|  |     |  |  |     +--rw nrf3gpp:amfPointer     AmfPointer

|  |     |  |  +--rw nrf3gpp:n2InterfaceAmfInfo* [idx]

|  |     |  |     +--rw nrf3gpp:idx                          uint32

|  |     |  |     +--rw (nrf3gpp:address)?

|  |     |  |     |  +--:(nrf3gpp:ipv4EndpointAddress)

|  |     |  |     |  |  +--rw nrf3gpp:ipv4EndpointAddress*   inet:ipv4-address

|  |     |  |     |  +--:(nrf3gpp:ipv6EndpointAddress)

|  |     |  |     |     +--rw nrf3gpp:ipv6EndpointAddress*   inet:ipv6-address

|  |     |  |     +--rw nrf3gpp:amfName?                     string

|  |     |  +--rw nrf3gpp:servedSmfInfo* [nfInstanceID]

|  |     |  |  +--rw nrf3gpp:nfInstanceID         string

|  |     |  |  +--rw nrf3gpp:sNssaiSmfInfoList* [idx]

|  |     |  |  |  +--rw nrf3gpp:idx               uint32

|  |     |  |  |  +--rw nrf3gpp:sNssai* [sst sd]

|  |     |  |  |  |  +--rw nrf3gpp:sst    uint32

|  |     |  |  |  |  +--rw nrf3gpp:sd     string

|  |     |  |  |  +--rw nrf3gpp:dnnSmfInfoList* [dnn]

|  |     |  |  |     +--rw nrf3gpp:dnn    string

|  |     |  |  +--rw nrf3gpp:taiList* [idx]

|  |     |  |  |  +--rw nrf3gpp:idx       uint32

|  |     |  |  |  +--rw nrf3gpp:plmnId* [mcc mnc]

|  |     |  |  |  |  +--rw nrf3gpp:mcc    Mcc

|  |     |  |  |  |  +--rw nrf3gpp:mnc    Mnc

|  |     |  |  |  +--rw nrf3gpp:tac?      types3gpp:Tac

|  |     |  |  +--rw nrf3gpp:taiRangeList* [idx]

|  |     |  |  |  +--rw nrf3gpp:idx             uint32

|  |     |  |  |  +--rw nrf3gpp:plmnId* [mcc mnc]

|  |     |  |  |  |  +--rw nrf3gpp:mcc    Mcc

|  |     |  |  |  |  +--rw nrf3gpp:mnc    Mnc

|  |     |  |  |  +--rw nrf3gpp:tacRangeList* [start end]

|  |     |  |  |     +--rw nrf3gpp:start      string

|  |     |  |  |     +--rw nrf3gpp:end        string

|  |     |  |  |     +--rw nrf3gpp:pattern?   string

|  |     |  |  +--rw nrf3gpp:pgwFqdn?             inet:domain-name

|  |     |  |  +--rw nrf3gpp:accessType*          AccessType

|  |     |  +--rw nrf3gpp:servedUpfInfo* [nfInstanceID]

|  |     |  |  +--rw nrf3gpp:nfInstanceID         string

|  |     |  |  +--rw nrf3gpp:sNssaiUpfInfoList* [idx]

|  |     |  |  |  +--rw nrf3gpp:idx               uint32

|  |     |  |  |  +--rw nrf3gpp:sNssai* [sst sd]

|  |     |  |  |  |  +--rw nrf3gpp:sst    uint32

|  |     |  |  |  |  +--rw nrf3gpp:sd     string

|  |     |  |  |  +--rw nrf3gpp:dnnUpfInfoList* [dnn]

|  |     |  |  |     +--rw nrf3gpp:dnn                string

|  |     |  |  |     +--rw nrf3gpp:dnaiList*          string

|  |     |  |  |     +--rw nrf3gpp:pduSessionTypes*   PduSessionType

|  |     |  |  +--rw nrf3gpp:smfServingArea*      string

|  |     |  |  +--rw nrf3gpp:interfaceUpfInfo* [idx]

|  |     |  |  |  +--rw nrf3gpp:idx                            uint32

|  |     |  |  |  +--rw nrf3gpp:interfaceType                  UPInterfaceType

|  |     |  |  |  +--rw (nrf3gpp:address)?

|  |     |  |  |  |  +--:(nrf3gpp:ipv4EndpointAddresses)

|  |     |  |  |  |  |  +--rw nrf3gpp:ipv4EndpointAddresses*   inet:ipv4-address

|  |     |  |  |  |  +--:(nrf3gpp:ipv6EndpointAddresses)

|  |     |  |  |  |  |  +--rw nrf3gpp:ipv6EndpointAddresses*   inet:ipv6-address

|  |     |  |  |  |  +--:(nrf3gpp:endpointFqdn)

|  |     |  |  |  |     +--rw nrf3gpp:endpointFqdn?            inet:domain-name

|  |     |  |  |  +--rw nrf3gpp:networkInstance?               string

|  |     |  |  +--rw nrf3gpp:iwkEpsInd?           boolean

|  |     |  |  +--rw nrf3gpp:pduSessionTypes*     PduSessionType

|  |     |  +--rw nrf3gpp:servedPcfInfo* [nfInstanceID]

|  |     |  |  +--rw nrf3gpp:nfInstanceID    string

|  |     |  |  +--rw nrf3gpp:dnnList*        string

|  |     |  |  +--rw nrf3gpp:supiRanges* [start end pattern]

|  |     |  |  |  +--rw nrf3gpp:start      string

|  |     |  |  |  +--rw nrf3gpp:end        string

|  |     |  |  |  +--rw nrf3gpp:pattern    string

|  |     |  |  +--rw nrf3gpp:rxDiamHost?     string

|  |     |  |  +--rw nrf3gpp:rxDiamRealm?    string

|  |     |  +--rw nrf3gpp:servedBsfInfo* [nfInstanceID]

|  |     |  |  +--rw nrf3gpp:nfInstanceID         string

|  |     |  |  +--rw nrf3gpp:ipv4AddressRanges* [start end]

|  |     |  |  |  +--rw nrf3gpp:start    inet:ipv4-address

|  |     |  |  |  +--rw nrf3gpp:end      inet:ipv4-address

|  |     |  |  +--rw nrf3gpp:dnnList*             string

|  |     |  |  +--rw nrf3gpp:ipDomainList*        string

|  |     |  |  +--rw nrf3gpp:ipv6PrefixRanges* [start end]

|  |     |  |     +--rw nrf3gpp:start    inet:ipv6-prefix

|  |     |  |     +--rw nrf3gpp:end      inet:ipv6-prefix

|  |     |  +--rw nrf3gpp:servedChfInfo* [nfInstanceID]

|  |     |     +--rw nrf3gpp:nfInstanceID     string

|  |     |     +--rw nrf3gpp:supiRangeList* [start end pattern]

|  |     |     |  +--rw nrf3gpp:start      string

|  |     |     |  +--rw nrf3gpp:end        string

|  |     |     |  +--rw nrf3gpp:pattern    string

|  |     |     +--rw nrf3gpp:gpsiRangeList* [start end pattern]

|  |     |     |  +--rw nrf3gpp:start      string

|  |     |     |  +--rw nrf3gpp:end        string

|  |     |     |  +--rw nrf3gpp:pattern    string

|  |     |     +--rw nrf3gpp:plmnRangeList* [mcc mnc]

|  |     |        +--rw nrf3gpp:mcc    Mcc

|  |     |        +--rw nrf3gpp:mnc    Mnc

|  |     +--rw nrf3gpp:customInfo?                         string

|  |     +--rw nrf3gpp:recoveryTime?                       yang:date-and-time

|  |     +--rw nrf3gpp:nfServicePersistence?               boolean

|  |     +--rw nrf3gpp:nfServices* [serviceInstanceID]

|  |     |  +--rw nrf3gpp:serviceInstanceID                   string

|  |     |  +--rw nrf3gpp:serviceName                         ServiceName

|  |     |  +--rw nrf3gpp:versions* [apiVersionInUri apiFullVersion]

|  |     |  |  +--rw nrf3gpp:apiVersionInUri    string

|  |     |  |  +--rw nrf3gpp:apiFullVersion     string

|  |     |  |  +--rw nrf3gpp:expiry?            yang:date-and-time

|  |     |  +--rw nrf3gpp:scheme                              UriScheme

|  |     |  +--rw nrf3gpp:nfServiceStatus                     NFServiceStatus

|  |     |  +--rw nrf3gpp:fqdn?                               inet:domain-name

|  |     |  +--rw nrf3gpp:interPlmnFqdn?                      inet:domain-name

|  |     |  +--rw nrf3gpp:ipEndPoints* [idx]

|  |     |  |  +--rw nrf3gpp:idx                  string

|  |     |  |  +--rw (nrf3gpp:address)?

|  |     |  |  |  +--:(nrf3gpp:ipv4Address)

|  |     |  |  |  |  +--rw nrf3gpp:ipv4Address?   inet:ipv4-address

|  |     |  |  |  +--:(nrf3gpp:ipv6Address)

|  |     |  |  |  |  +--rw nrf3gpp:ipv6Address?   inet:ipv6-address

|  |     |  |  |  +--:(nrf3gpp:ipv6Prefix)

|  |     |  |  |     +--rw nrf3gpp:ipv6Prefix?    inet:ipv6-prefix

|  |     |  |  +--rw nrf3gpp:transport?           TransportProtocol

|  |     |  |  +--rw nrf3gpp:port?                uint16

|  |     |  +--rw nrf3gpp:apiPrefix?                          string

|  |     |  +--rw nrf3gpp:defaultNotificationSubscriptions* [notificationType]

|  |     |  |  +--rw nrf3gpp:notificationType      NotificationType

|  |     |  |  +--rw nrf3gpp:callbackUri?          inet:uri

|  |     |  |  +--rw nrf3gpp:n1MessageClass?       N1MessageClass

|  |     |  |  +--rw nrf3gpp:n2InformationClass?   N2InformationClass

|  |     |  +--rw nrf3gpp:allowedPlmns* [mcc mnc]

|  |     |  |  +--rw nrf3gpp:mcc    Mcc

|  |     |  |  +--rw nrf3gpp:mnc    Mnc

|  |     |  +--rw nrf3gpp:allowedNfTypes*                     types3gpp:NfType

|  |     |  +--rw nrf3gpp:allowedNfDomains*                   string

|  |     |  +--rw nrf3gpp:allowedNssais*                      types3gpp:SNssai

|  |     |  +--rw nrf3gpp:priority?                           uint16

|  |     |  +--rw nrf3gpp:capacity?                           uint16

|  |     |  +--rw nrf3gpp:load?                               types3gpp:Load

|  |     |  +--rw nrf3gpp:recoveryTime?                       yang:date-and-time

|  |     |  +--rw nrf3gpp:chfServiceInfo* [primaryChfServiceInstance secondaryChfServiceInstance]

|  |     |  |  +--rw nrf3gpp:primaryChfServiceInstance      string

|  |     |  |  +--rw nrf3gpp:secondaryChfServiceInstance    string

|  |     |  +--rw nrf3gpp:supportedFeatures?                  SupportedFeatures

|  |     +--rw nrf3gpp:nfProfileChangesSupportInd?         boolean

|  |     +--rw nrf3gpp:nfProfileChangesInd?                boolean

|  |     +--rw nrf3gpp:defaultNotificationSubscriptions* [notificationType]

|  |        +--rw nrf3gpp:notificationType      NotificationType

|  |        +--rw nrf3gpp:callbackUri?          inet:uri

|  |        +--rw nrf3gpp:n1MessageClass?       N1MessageClass

|  |        +--rw nrf3gpp:n2InformationClass?   N2InformationClass

|  +--rw cep3gpp:EP_N27* [id]

|  |  +--rw cep3gpp:id            string

|  |  +--rw cep3gpp:attributes

|  |     +--rw cep3gpp:userLabel?                  string

|  |     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|  |     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  |     |  +--ro cep3gpp:measurementType    string

|  |     |  +--ro cep3gpp:supportedGPs*      uint32

|  |     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|  |     |  +--rw cep3gpp:ipAddress    inet:ip-address

|  |     |  +--rw cep3gpp:vlanId       uint16

|  |     +--rw cep3gpp:remoteAddress               inet:ip-address

|  +--rw cep3gpp:EP_SBI_X* [id]

|     +--rw cep3gpp:id            string

|     +--rw cep3gpp:attributes

|        +--rw cep3gpp:userLabel?                  string

|        +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|        +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|        |  +--ro cep3gpp:measurementType    string

|        |  +--ro cep3gpp:supportedGPs*      uint32

|        +--rw cep3gpp:localAddress* [ipAddress vlanId]

|        |  +--rw cep3gpp:ipAddress    inet:ip-address

|        |  +--rw cep3gpp:vlanId       uint16

|        +--rw cep3gpp:remoteAddress               inet:ip-address

|        +--ro cep3gpp:sBIService*                 string

+--rw n3iwf3gpp:N3IWFFunction* [id]

+--rw n3iwf3gpp:id            string

+--rw n3iwf3gpp:attributes

|  +--rw n3iwf3gpp:userLabel?           string

|  +--rw n3iwf3gpp:vnfParametersList!

|  |  +--rw n3iwf3gpp:vnfInstanceId    string

|  |  +--rw n3iwf3gpp:vnfdId?          string

|  |  +--rw n3iwf3gpp:flavourId?       string

|  |  +--rw n3iwf3gpp:autoScalable     boolean

|  +--rw n3iwf3gpp:peeParametersList!

|  |  +--rw n3iwf3gpp:siteIdentification    string

|  |  +--rw n3iwf3gpp:siteLatitude?         decimal64

|  |  +--rw n3iwf3gpp:siteLongitude?        decimal64

|  |  +--rw n3iwf3gpp:siteDescription       string

|  |  +--rw n3iwf3gpp:equipmentType         string

|  |  +--rw n3iwf3gpp:environmentType       string

|  |  +--rw n3iwf3gpp:powerInterface        string

|  +--rw n3iwf3gpp:priorityLabel        uint32

|  +--rw n3iwf3gpp:pLMNIdList* [mcc mnc]

|     +--rw n3iwf3gpp:mcc    Mcc

|     +--rw n3iwf3gpp:mnc    Mnc

+--rw cep3gpp:EP_N2* [id]

|  +--rw cep3gpp:id            string

|  +--rw cep3gpp:attributes

|     +--rw cep3gpp:userLabel?                  string

|     +--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

|     +--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|     |  +--ro cep3gpp:measurementType    string

|     |  +--ro cep3gpp:supportedGPs*      uint32

|     +--rw cep3gpp:localAddress* [ipAddress vlanId]

|     |  +--rw cep3gpp:ipAddress    inet:ip-address

|     |  +--rw cep3gpp:vlanId       uint16

|     +--rw cep3gpp:remoteAddress               inet:ip-address

+--rw cep3gpp:EP_N3* [id]

+--rw cep3gpp:id            string

+--rw cep3gpp:attributes

+--rw cep3gpp:userLabel?                  string

+--ro cep3gpp:farEndEntity?               types3gpp:DistinguishedName

+--ro cep3gpp:supportedMeasurementsGPs* [measurementType]

|  +--ro cep3gpp:measurementType    string

|  +--ro cep3gpp:supportedGPs*      uint32

+--rw cep3gpp:localAddress* [ipAddress vlanId]

|  +--rw cep3gpp:ipAddress    inet:ip-address

|  +--rw cep3gpp:vlanId       uint16

+--rw cep3gpp:remoteAddress               inet:ip-address

module: _3gpp-nr-nrm-eutranetwork

+--rw EUtraNetwork* [id]

+--rw id                                         string

+--rw attributes

|  +--rw dnPrefix?                   types3gpp:DistinguishedName

|  +--rw userLabel?                  string

|  +--rw userDefinedNetworkType?     string

|  +--ro supportedMeasurementsGPs* [measurementType]

|  |  +--ro measurementType    string

|  |  +--ro supportedGPs*      uint32

|  +--rw setOfMcc*                   types3gpp:Mcc

|  +--rw priorityLabel               uint32

|  +--rw parentSubNetwork?           -> ../../id

|  +--rw containedSubNetworks*       -> ../../id

+--rw extservgw3gpp:ExternalServingGWFunction* [id] {eutranet3gpp:ExternalsUnderEUtraNetwork}?

|  +--rw extservgw3gpp:id            string

|  +--rw extservgw3gpp:attributes

|     +--rw extservgw3gpp:userLabel?           string

|     +--rw extservgw3gpp:vnfParametersList!

|     |  +--rw extservgw3gpp:vnfInstanceId    string

|     |  +--rw extservgw3gpp:vnfdId?          string

|     |  +--rw extservgw3gpp:flavourId?       string

|     |  +--rw extservgw3gpp:autoScalable     boolean

|     +--rw extservgw3gpp:peeParametersList!

|     |  +--rw extservgw3gpp:siteIdentification    string

|     |  +--rw extservgw3gpp:siteLatitude?         decimal64

|     |  +--rw extservgw3gpp:siteLongitude?        decimal64

|     |  +--rw extservgw3gpp:siteDescription       string

|     |  +--rw extservgw3gpp:equipmentType         string

|     |  +--rw extservgw3gpp:environmentType       string

|     |  +--rw extservgw3gpp:powerInterface        string

|     +--rw extservgw3gpp:priorityLabel        uint32

+--rw extenb3gpp:ExternalENBFunction* [id] {eutranet3gpp:ExternalsUnderEUtraNetwork}?

|  +--rw extenb3gpp:id                              string

|  +--rw extenb3gpp:attributes

|  |  +--rw extenb3gpp:userLabel?           string

|  |  +--rw extenb3gpp:vnfParametersList!

|  |  |  +--rw extenb3gpp:vnfInstanceId    string

|  |  |  +--rw extenb3gpp:vnfdId?          string

|  |  |  +--rw extenb3gpp:flavourId?       string

|  |  |  +--rw extenb3gpp:autoScalable     boolean

|  |  +--rw extenb3gpp:peeParametersList!

|  |  |  +--rw extenb3gpp:siteIdentification    string

|  |  |  +--rw extenb3gpp:siteLatitude?         decimal64

|  |  |  +--rw extenb3gpp:siteLongitude?        decimal64

|  |  |  +--rw extenb3gpp:siteDescription       string

|  |  |  +--rw extenb3gpp:equipmentType         string

|  |  |  +--rw extenb3gpp:environmentType       string

|  |  |  +--rw extenb3gpp:powerInterface        string

|  |  +--rw extenb3gpp:priorityLabel        uint32

|  |  +--rw extenb3gpp:eNBId                int32

|  +--rw exteutrancell3gpp:ExternalEUtranCellFDD* [id] {eutranet3gpp:ExternalsUnderEUtraNetwork}?

|  |  +--rw exteutrancell3gpp:id            string

|  |  +--rw exteutrancell3gpp:attributes

|  |     +--rw exteutrancell3gpp:userLabel?           string

|  |     +--rw exteutrancell3gpp:vnfParametersList!

|  |     |  +--rw exteutrancell3gpp:vnfInstanceId    string

|  |     |  +--rw exteutrancell3gpp:vnfdId?          string

|  |     |  +--rw exteutrancell3gpp:flavourId?       string

|  |     |  +--rw exteutrancell3gpp:autoScalable     boolean

|  |     +--rw exteutrancell3gpp:peeParametersList!

|  |     |  +--rw exteutrancell3gpp:siteIdentification    string

|  |     |  +--rw exteutrancell3gpp:siteLatitude?         decimal64

|  |     |  +--rw exteutrancell3gpp:siteLongitude?        decimal64

|  |     |  +--rw exteutrancell3gpp:siteDescription       string

|  |     |  +--rw exteutrancell3gpp:equipmentType         string

|  |     |  +--rw exteutrancell3gpp:environmentType       string

|  |     |  +--rw exteutrancell3gpp:powerInterface        string

|  |     +--rw exteutrancell3gpp:priorityLabel        uint32

|  |     +--rw exteutrancell3gpp:pci                  int32

|  |     +--rw exteutrancell3gpp:plmnIdList* [mcc mnc]

|  |     |  +--rw exteutrancell3gpp:mcc    Mcc

|  |     |  +--rw exteutrancell3gpp:mnc    Mnc

|  |     +--rw exteutrancell3gpp:cellLocalId?         int32

|  |     +--rw exteutrancell3gpp:eNBId                int32

|  |     +--rw exteutrancell3gpp:earfcnDL             int32

|  |     +--rw exteutrancell3gpp:earfcnUL             int32

|  +--rw exteutrancell3gpp:ExternalEUtranCellTDD* [id] {eutranet3gpp:ExternalsUnderEUtraNetwork}?

|     +--rw exteutrancell3gpp:id            string

|     +--rw exteutrancell3gpp:attributes

|        +--rw exteutrancell3gpp:userLabel?           string

|        +--rw exteutrancell3gpp:vnfParametersList!

|        |  +--rw exteutrancell3gpp:vnfInstanceId    string

|        |  +--rw exteutrancell3gpp:vnfdId?          string

|        |  +--rw exteutrancell3gpp:flavourId?       string

|        |  +--rw exteutrancell3gpp:autoScalable     boolean

|        +--rw exteutrancell3gpp:peeParametersList!

|        |  +--rw exteutrancell3gpp:siteIdentification    string

|        |  +--rw exteutrancell3gpp:siteLatitude?         decimal64

|        |  +--rw exteutrancell3gpp:siteLongitude?        decimal64

|        |  +--rw exteutrancell3gpp:siteDescription       string

|        |  +--rw exteutrancell3gpp:equipmentType         string

|        |  +--rw exteutrancell3gpp:environmentType       string

|        |  +--rw exteutrancell3gpp:powerInterface        string

|        +--rw exteutrancell3gpp:priorityLabel        uint32

|        +--rw exteutrancell3gpp:pci                  int32

|        +--rw exteutrancell3gpp:plmnIdList* [mcc mnc]

|        |  +--rw exteutrancell3gpp:mcc    Mcc

|        |  +--rw exteutrancell3gpp:mnc    Mnc

|        +--rw exteutrancell3gpp:cellLocalId?         int32

|        +--rw exteutrancell3gpp:eNBId                int32

|        +--rw exteutrancell3gpp:earfcn               int32

+--rw eutraneteutranfreq3gpp:EUtranFrequency* [id] {eutranet3gpp:ExternalsUnderEUtraNetwork}?

+--rw eutraneteutranfreq3gpp:id            string

+--rw eutraneteutranfreq3gpp:attributes

+--rw eutraneteutranfreq3gpp:userLabel?                string

+--rw eutraneteutranfreq3gpp:vnfParametersList!

|  +--rw eutraneteutranfreq3gpp:vnfInstanceId    string

|  +--rw eutraneteutranfreq3gpp:vnfdId?          string

|  +--rw eutraneteutranfreq3gpp:flavourId?       string

|  +--rw eutraneteutranfreq3gpp:autoScalable     boolean

+--rw eutraneteutranfreq3gpp:peeParametersList!

|  +--rw eutraneteutranfreq3gpp:siteIdentification    string

|  +--rw eutraneteutranfreq3gpp:siteLatitude?         decimal64

|  +--rw eutraneteutranfreq3gpp:siteLongitude?        decimal64

|  +--rw eutraneteutranfreq3gpp:siteDescription       string

|  +--rw eutraneteutranfreq3gpp:equipmentType         string

|  +--rw eutraneteutranfreq3gpp:environmentType       string

|  +--rw eutraneteutranfreq3gpp:powerInterface        string

+--rw eutraneteutranfreq3gpp:priorityLabel             uint32

+--rw eutraneteutranfreq3gpp:earfcnDL                  uint32

+--ro eutraneteutranfreq3gpp:multiBandInfoListEutra*   uint16

module: _3gpp-nr-nrm-nrnetwork

+--rw NRNetwork* [id]

+--rw id                                        string

+--rw attributes

|  +--rw dnPrefix?                   types3gpp:DistinguishedName

|  +--rw userLabel?                  string

|  +--rw userDefinedNetworkType?     string

|  +--ro supportedMeasurementsGPs* [measurementType]

|  |  +--ro measurementType    string

|  |  +--ro supportedGPs*      uint32

|  +--rw setOfMcc*                   types3gpp:Mcc

|  +--rw priorityLabel               uint32

|  +--rw parentSubNetwork?           -> ../../id

|  +--rw containedSubNetworks*       -> ../../id

+--rw extgnbcucp3gpp:ExternalGNBCUCPFunction* [id] {nrnet3gpp:ExternalsUnderNRNetwork}?

|  +--rw extgnbcucp3gpp:id                   string

|  +--rw extgnbcucp3gpp:attributes

|  |  +--rw extgnbcucp3gpp:userLabel?           string

|  |  +--rw extgnbcucp3gpp:vnfParametersList!

|  |  |  +--rw extgnbcucp3gpp:vnfInstanceId    string

|  |  |  +--rw extgnbcucp3gpp:vnfdId?          string

|  |  |  +--rw extgnbcucp3gpp:flavourId?       string

|  |  |  +--rw extgnbcucp3gpp:autoScalable     boolean

|  |  +--rw extgnbcucp3gpp:peeParametersList!

|  |  |  +--rw extgnbcucp3gpp:siteIdentification    string

|  |  |  +--rw extgnbcucp3gpp:siteLatitude?         decimal64

|  |  |  +--rw extgnbcucp3gpp:siteLongitude?        decimal64

|  |  |  +--rw extgnbcucp3gpp:siteDescription       string

|  |  |  +--rw extgnbcucp3gpp:equipmentType         string

|  |  |  +--rw extgnbcucp3gpp:environmentType       string

|  |  |  +--rw extgnbcucp3gpp:powerInterface        string

|  |  +--rw extgnbcucp3gpp:priorityLabel        uint32

|  |  +--rw extgnbcucp3gpp:gNBId                int64

|  |  +--rw extgnbcucp3gpp:gNBIdLength          int32

|  |  +--rw extgnbcucp3gpp:pLMNId* [mcc mnc]

|  |     +--rw extgnbcucp3gpp:mcc    Mcc

|  |     +--rw extgnbcucp3gpp:mnc    Mnc

|  +--rw extnrcellcu3gpp:ExternalNRCellCU* [id] {nrnet3gpp:ExternalsUnderNRNetwork}?

|     +--rw extnrcellcu3gpp:id            string

|     +--rw extnrcellcu3gpp:attributes

|        +--rw extnrcellcu3gpp:userLabel?           string

|        +--rw extnrcellcu3gpp:vnfParametersList!

|        |  +--rw extnrcellcu3gpp:vnfInstanceId    string

|        |  +--rw extnrcellcu3gpp:vnfdId?          string

|        |  +--rw extnrcellcu3gpp:flavourId?       string

|        |  +--rw extnrcellcu3gpp:autoScalable     boolean

|        +--rw extnrcellcu3gpp:peeParametersList!

|        |  +--rw extnrcellcu3gpp:siteIdentification    string

|        |  +--rw extnrcellcu3gpp:siteLatitude?         decimal64

|        |  +--rw extnrcellcu3gpp:siteLongitude?        decimal64

|        |  +--rw extnrcellcu3gpp:siteDescription       string

|        |  +--rw extnrcellcu3gpp:equipmentType         string

|        |  +--rw extnrcellcu3gpp:environmentType       string

|        |  +--rw extnrcellcu3gpp:powerInterface        string

|        +--rw extnrcellcu3gpp:priorityLabel        uint32

|        +--rw extnrcellcu3gpp:cellLocalId          int32

|        +--rw extnrcellcu3gpp:nRPCI                int32

|        +--rw extnrcellcu3gpp:pLMNIdList* [mcc mnc]

|        |  +--rw extnrcellcu3gpp:mcc    Mcc

|        |  +--rw extnrcellcu3gpp:mnc    Mnc

|        +--rw extnrcellcu3gpp:nRFrequencyRef       types3gpp:DistinguishedName

+--rw extgnbcuup3gpp:ExternalGNBCUUPFunction* [id] {nrnet3gpp:ExternalsUnderNRNetwork}?

|  +--rw extgnbcuup3gpp:id            string

|  +--rw extgnbcuup3gpp:attributes

|     +--rw extgnbcuup3gpp:userLabel?           string

|     +--rw extgnbcuup3gpp:vnfParametersList!

|     |  +--rw extgnbcuup3gpp:vnfInstanceId    string

|     |  +--rw extgnbcuup3gpp:vnfdId?          string

|     |  +--rw extgnbcuup3gpp:flavourId?       string

|     |  +--rw extgnbcuup3gpp:autoScalable     boolean

|     +--rw extgnbcuup3gpp:peeParametersList!

|     |  +--rw extgnbcuup3gpp:siteIdentification    string

|     |  +--rw extgnbcuup3gpp:siteLatitude?         decimal64

|     |  +--rw extgnbcuup3gpp:siteLongitude?        decimal64

|     |  +--rw extgnbcuup3gpp:siteDescription       string

|     |  +--rw extgnbcuup3gpp:equipmentType         string

|     |  +--rw extgnbcuup3gpp:environmentType       string

|     |  +--rw extgnbcuup3gpp:powerInterface        string

|     +--rw extgnbcuup3gpp:priorityLabel        uint32

|     +--rw extgnbcuup3gpp:gNBId                int64

|     +--rw extgnbcuup3gpp:gNBIdLength          int32

+--rw extupf3gpp:ExternalUPFFunction* [id] {nrnet3gpp:ExternalsUnderNRNetwork}?

|  +--rw extupf3gpp:id            string

|  +--rw extupf3gpp:attributes

|     +--rw extupf3gpp:userLabel?           string

|     +--rw extupf3gpp:vnfParametersList!

|     |  +--rw extupf3gpp:vnfInstanceId    string

|     |  +--rw extupf3gpp:vnfdId?          string

|     |  +--rw extupf3gpp:flavourId?       string

|     |  +--rw extupf3gpp:autoScalable     boolean

|     +--rw extupf3gpp:peeParametersList!

|     |  +--rw extupf3gpp:siteIdentification    string

|     |  +--rw extupf3gpp:siteLatitude?         decimal64

|     |  +--rw extupf3gpp:siteLongitude?        decimal64

|     |  +--rw extupf3gpp:siteDescription       string

|     |  +--rw extupf3gpp:equipmentType         string

|     |  +--rw extupf3gpp:environmentType       string

|     |  +--rw extupf3gpp:powerInterface        string

|     +--rw extupf3gpp:priorityLabel        uint32

+--rw nrfreq3gpp:NRFrequency* [id] {nrnet3gpp:ExternalsUnderNRNetwork}?

|  +--rw nrfreq3gpp:id            string

|  +--rw nrfreq3gpp:attributes

|     +--rw nrfreq3gpp:userLabel?                  string

|     +--rw nrfreq3gpp:vnfParametersList!

|     |  +--rw nrfreq3gpp:vnfInstanceId    string

|     |  +--rw nrfreq3gpp:vnfdId?          string

|     |  +--rw nrfreq3gpp:flavourId?       string

|     |  +--rw nrfreq3gpp:autoScalable     boolean

|     +--rw nrfreq3gpp:peeParametersList!

|     |  +--rw nrfreq3gpp:siteIdentification    string

|     |  +--rw nrfreq3gpp:siteLatitude?         decimal64

|     |  +--rw nrfreq3gpp:siteLongitude?        decimal64

|     |  +--rw nrfreq3gpp:siteDescription       string

|     |  +--rw nrfreq3gpp:equipmentType         string

|     |  +--rw nrfreq3gpp:environmentType       string

|     |  +--rw nrfreq3gpp:powerInterface        string

|     +--rw nrfreq3gpp:priorityLabel               uint32

|     +--rw nrfreq3gpp:absoluteFrequencySSB        uint32

|     +--rw nrfreq3gpp:sSBSubCarrierSpacing        uint8

|     +--ro nrfreq3gpp:multiFrequencyBandListNR*   uint16

+--rw extamf3gpp:ExternalAMFFunction* [id] {nrnet3gpp:ExternalsUnderNRNetwork}?

|  +--rw extamf3gpp:id            string

|  +--rw extamf3gpp:attributes

|     +--rw extamf3gpp:userLabel?           string

|     +--rw extamf3gpp:vnfParametersList!

|     |  +--rw extamf3gpp:vnfInstanceId    string

|     |  +--rw extamf3gpp:vnfdId?          string

|     |  +--rw extamf3gpp:flavourId?       string

|     |  +--rw extamf3gpp:autoScalable     boolean

|     +--rw extamf3gpp:peeParametersList!

|     |  +--rw extamf3gpp:siteIdentification    string

|     |  +--rw extamf3gpp:siteLatitude?         decimal64

|     |  +--rw extamf3gpp:siteLongitude?        decimal64

|     |  +--rw extamf3gpp:siteDescription       string

|     |  +--rw extamf3gpp:equipmentType         string

|     |  +--rw extamf3gpp:environmentType       string

|     |  +--rw extamf3gpp:powerInterface        string

|     +--rw extamf3gpp:priorityLabel        uint32

+--rw extgnbdu3gpp:ExternalGNBDUFunction* [id] {nrnet3gpp:ExternalsUnderNRNetwork}?

+--rw extgnbdu3gpp:id            string

+--rw extgnbdu3gpp:attributes

+--rw extgnbdu3gpp:userLabel?           string

+--rw extgnbdu3gpp:vnfParametersList!

|  +--rw extgnbdu3gpp:vnfInstanceId    string

|  +--rw extgnbdu3gpp:vnfdId?          string

|  +--rw extgnbdu3gpp:flavourId?       string

|  +--rw extgnbdu3gpp:autoScalable     boolean

+--rw extgnbdu3gpp:peeParametersList!

|  +--rw extgnbdu3gpp:siteIdentification    string

|  +--rw extgnbdu3gpp:siteLatitude?         decimal64

|  +--rw extgnbdu3gpp:siteLongitude?        decimal64

|  +--rw extgnbdu3gpp:siteDescription       string

|  +--rw extgnbdu3gpp:equipmentType         string

|  +--rw extgnbdu3gpp:environmentType       string

|  +--rw extgnbdu3gpp:powerInterface        string

+--rw extgnbdu3gpp:priorityLabel        uint32

+--rw extgnbdu3gpp:gNBId                int64

+--rw extgnbdu3gpp:gNBIdLength          int32

+--rw extgnbdu3gpp:pLMNId* [mcc mnc]

+--rw extgnbdu3gpp:mcc    Mcc

+--rw extgnbdu3gpp:mnc    Mnc

module: _3gpp-ns-nrm-networkslice

+--rw NetworkSlice* [id]

+--rw attributes

|  +--rw dnPrefix?                   types3gpp:DistinguishedName

|  +--rw userLabel?                  string

|  +--rw userDefinedNetworkType?     string

|  +--ro supportedMeasurementsGPs* [measurementType]

|  |  +--ro measurementType    string

|  |  +--ro supportedGPs*      uint32

|  +--rw setOfMcc*                   types3gpp:Mcc

|  +--rw priorityLabel               uint32

|  +--rw parentSubNetwork?           -> ../../id

|  +--rw containedSubNetworks*       -> ../../id

|  +--ro nSSIId?                     types3gpp:DistinguishedName

|  +--ro operationalState?           types3gpp:OperationalState

|  +--rw administrativeState?        types3gpp:AdministrativeState

|  +--rw serviceProfile* [serviceProfileId]

|  |  +--rw serviceProfileId        types3gpp:DistinguishedName

|  |  +--rw sNSSAIList*             types3gpp:SNssai

|  |  +--rw pLMNIdList* [mcc mnc]

|  |  |  +--rw mcc    Mcc

|  |  |  +--rw mnc    Mnc

|  |  +--rw perfReq* [idx]

|  |  |  +--rw idx                           uint32

|  |  |  +--rw (SST)?

|  |  |     +--:(eMBB)

|  |  |     |  +--rw expDataRateDL?          uint16

|  |  |     |  +--rw expDataRateU?           uint16

|  |  |     |  +--rw areaTrafficCapDL?       uint16

|  |  |     |  +--rw areaTrafficCapUL?       uint16

|  |  |     |  +--rw userDensity?            uint16

|  |  |     |  +--rw activityFactor?         uint16

|  |  |     |  +--rw uESpeed?                uint16

|  |  |     |  +--rw coverage?               string

|  |  |     +--:(uRLLC)

|  |  |        +--rw e2eLatency?             uint16

|  |  |        +--rw jitter?                 uint16

|  |  |        +--rw survivalTime?           uint16

|  |  |        +--rw cSAvailability?         decimal64

|  |  |        +--rw reliability?            decimal64

|  |  |        +--rw expDataRate?            uint16

|  |  |        +--rw payloadSize?            string

|  |  |        +--rw trafficDensity?         uint16

|  |  |        +--rw connDensity?            uint16

|  |  |        +--rw serviceAreaDimension?   string

|  |  +--rw maxNumberofUEs          uint64

|  |  +--rw coverageAreaTAList*     types3gpp:Tac

|  |  +--rw latency                 uint16

|  |  +--rw uEMobilityLevel?        types3gpp:UeMobilityLevel

|  |  +--rw resourceSharingLevel?   types3gpp:ResourceSharingLevel

|  |  +--rw sst                     uint32

|  |  +--rw availability?           decimal64

|  +--rw usesNetworkSliceSubnet?     instance-identifier

+--rw id            string

module: _3gpp-ns-nrm-networkslicesubnet

+--rw NetworkSliceSubnet* [id]

+--rw attributes

|  +--rw dnPrefix?                       types3gpp:DistinguishedName

|  +--rw userLabel?                      string

|  +--rw userDefinedNetworkType?         string

|  +--ro supportedMeasurementsGPs* [measurementType]

|  |  +--ro measurementType    string

|  |  +--ro supportedGPs*      uint32

|  +--rw setOfMcc*                       types3gpp:Mcc

|  +--rw priorityLabel                   uint32

|  +--rw parentSubNetwork?               -> ../../id

|  +--rw containedSubNetworks*           -> ../../id

|  +--ro mFIdList*                       types3gpp:DistinguishedName

|  +--ro constituentNSSIIdList*          types3gpp:DistinguishedName

|  +--ro operationalState                types3gpp:OperationalState

|  +--rw administrativeState             types3gpp:AdministrativeState

|  +--ro nsInfo?                         string

|  +--rw sliceProfileList* [sliceProfileId]

|  |  +--rw sliceProfileId          types3gpp:DistinguishedName

|  |  +--rw sNSSAIList*             types3gpp:SNssai

|  |  +--rw pLMNIdList* [mcc mnc]

|  |  |  +--rw mcc    Mcc

|  |  |  +--rw mnc    Mnc

|  |  +--rw perfReq* [idx]

|  |  |  +--rw idx                           uint32

|  |  |  +--rw (SST)?

|  |  |     +--:(eMBB)

|  |  |     |  +--rw expDataRateDL?          uint16

|  |  |     |  +--rw expDataRateU?           uint16

|  |  |     |  +--rw areaTrafficCapDL?       uint16

|  |  |     |  +--rw areaTrafficCapUL?       uint16

|  |  |     |  +--rw userDensity?            uint16

|  |  |     |  +--rw activityFactor?         uint16

|  |  |     |  +--rw uESpeed?                uint16

|  |  |     |  +--rw coverage?               string

|  |  |     +--:(uRLLC)

|  |  |        +--rw e2eLatency?             uint16

|  |  |        +--rw jitter?                 uint16

|  |  |        +--rw survivalTime?           uint16

|  |  |        +--rw cSAvailability?         decimal64

|  |  |        +--rw reliability?            decimal64

|  |  |        +--rw expDataRate?            uint16

|  |  |        +--rw payloadSize?            string

|  |  |        +--rw trafficDensity?         uint16

|  |  |        +--rw connDensity?            uint16

|  |  |        +--rw serviceAreaDimension?   string

|  |  +--rw maxNumberofUEs          uint64

|  |  +--rw coverageAreaTAList*     types3gpp:Tac

|  |  +--rw latency                 uint16

|  |  +--rw uEMobilityLevel?        types3gpp:UeMobilityLevel

|  |  +--rw resourceSharingLevel?   types3gpp:ResourceSharingLevel

|  +--rw managedFunctions* [aggregatedManagedFunction]

|  |  +--rw aggregatedManagedFunction    instance-identifier

|  +--rw aggregatedNetworkSliceSubnet*   -> ../../id

+--rw id                    string

+--rw MeasurementControl* [id] {MeasurementsUnderNetworkSliceSubnet}?

+--rw id                   string

+--rw attributes

|  +--rw pMAdministrativeState?        types3gpp:AdministrativeState

|  +--ro pMOperationalState            types3gpp:OperationalState

|  +--rw defaultFileLocation?          string

|  +--rw defaultFileBasedGP            uint32

|  +--rw defaultFileReportingPeriod    uint32

|  +--rw defaultStreamTarget?          string {StreamingSupported}?

|  +--rw defaultStreamBasedGP          uint32 {StreamingSupported}?

+--rw MeasurementReader* [id]

+--rw id            string

+--rw attributes

+--rw measurementTypes*        -> ../../../../attributes/supportedMeasurementsGPs/measurementType

+--rw managedObjectDNsBasic*   types3gpp:DistinguishedName

+--rw managedObjectDNs*        types3gpp:DistinguishedName

+--rw fileLocation?            string

+--rw fileBasedGP              -> ../../../../attributes/supportedMeasurementsGPs/supportedGPs

+--rw fileReportingPeriod      uint32

+--rw streamTarget?            string

+--rw streamBasedGP            -> ../../../../attributes/supportedMeasurementsGPs/supportedGPs

module: _3gpp-subnetwork

+--rw SubNetwork* [id]

+--rw id                                         string

+--rw attributes

|  +--rw dnPrefix?                   types3gpp:DistinguishedName

|  +--rw userLabel?                  string

|  +--rw userDefinedNetworkType?     string

|  +--ro supportedMeasurementsGPs* [measurementType]

|  |  +--ro measurementType    string

|  |  +--ro supportedGPs*      uint32

|  +--rw setOfMcc*                   types3gpp:Mcc

|  +--rw priorityLabel               uint32

|  +--rw parentSubNetwork?           -> ../../id

|  +--rw containedSubNetworks*       -> ../../id

+--rw MeasurementControl* [id] {MeasurementsUnderSubNetwork}?

|  +--rw id                   string

|  +--rw attributes

|  |  +--rw pMAdministrativeState?        types3gpp:AdministrativeState

|  |  +--ro pMOperationalState            types3gpp:OperationalState

|  |  +--rw defaultFileLocation?          string

|  |  +--rw defaultFileBasedGP            uint32

|  |  +--rw defaultFileReportingPeriod    uint32

|  |  +--rw defaultStreamTarget?          string {StreamingSupported}?

|  |  +--rw defaultStreamBasedGP          uint32 {StreamingSupported}?

|  +--rw MeasurementReader* [id]

|     +--rw id            string

|     +--rw attributes

|        +--rw measurementTypes*        -> ../../../../attributes/supportedMeasurementsGPs/measurementType

|        +--rw managedObjectDNsBasic*   types3gpp:DistinguishedName

|        +--rw managedObjectDNs*        types3gpp:DistinguishedName

|        +--rw fileLocation?            string

|        +--rw fileBasedGP              -> ../../../../attributes/supportedMeasurementsGPs/supportedGPs

|        +--rw fileReportingPeriod      uint32

|        +--rw streamTarget?            string

|        +--rw streamBasedGP            -> ../../../../attributes/supportedMeasurementsGPs/supportedGPs

+--rw extservgw3gpp:ExternalServingGWFunction* [id] {subnet3gpp:ExternalsUnderSubNetwork}?

|  +--rw extservgw3gpp:id            string

|  +--rw extservgw3gpp:attributes

|     +--rw extservgw3gpp:userLabel?           string

|     +--rw extservgw3gpp:vnfParametersList!

|     |  +--rw extservgw3gpp:vnfInstanceId    string

|     |  +--rw extservgw3gpp:vnfdId?          string

|     |  +--rw extservgw3gpp:flavourId?       string

|     |  +--rw extservgw3gpp:autoScalable     boolean

|     +--rw extservgw3gpp:peeParametersList!

|     |  +--rw extservgw3gpp:siteIdentification    string

|     |  +--rw extservgw3gpp:siteLatitude?         decimal64

|     |  +--rw extservgw3gpp:siteLongitude?        decimal64

|     |  +--rw extservgw3gpp:siteDescription       string

|     |  +--rw extservgw3gpp:equipmentType         string

|     |  +--rw extservgw3gpp:environmentType       string

|     |  +--rw extservgw3gpp:powerInterface        string

|     +--rw extservgw3gpp:priorityLabel        uint32

+--rw extenb3gpp:ExternalENBFunction* [id] {subnet3gpp:ExternalsUnderSubNetwork}?

|  +--rw extenb3gpp:id                              string

|  +--rw extenb3gpp:attributes

|  |  +--rw extenb3gpp:userLabel?           string

|  |  +--rw extenb3gpp:vnfParametersList!

|  |  |  +--rw extenb3gpp:vnfInstanceId    string

|  |  |  +--rw extenb3gpp:vnfdId?          string

|  |  |  +--rw extenb3gpp:flavourId?       string

|  |  |  +--rw extenb3gpp:autoScalable     boolean

|  |  +--rw extenb3gpp:peeParametersList!

|  |  |  +--rw extenb3gpp:siteIdentification    string

|  |  |  +--rw extenb3gpp:siteLatitude?         decimal64

|  |  |  +--rw extenb3gpp:siteLongitude?        decimal64

|  |  |  +--rw extenb3gpp:siteDescription       string

|  |  |  +--rw extenb3gpp:equipmentType         string

|  |  |  +--rw extenb3gpp:environmentType       string

|  |  |  +--rw extenb3gpp:powerInterface        string

|  |  +--rw extenb3gpp:priorityLabel        uint32

|  |  +--rw extenb3gpp:eNBId                int32

|  +--rw exteutrancell3gpp:ExternalEUtranCellFDD* [id] {subnet3gpp:ExternalsUnderSubNetwork}?

|  |  +--rw exteutrancell3gpp:id            string

|  |  +--rw exteutrancell3gpp:attributes

|  |     +--rw exteutrancell3gpp:userLabel?           string

|  |     +--rw exteutrancell3gpp:vnfParametersList!

|  |     |  +--rw exteutrancell3gpp:vnfInstanceId    string

|  |     |  +--rw exteutrancell3gpp:vnfdId?          string

|  |     |  +--rw exteutrancell3gpp:flavourId?       string

|  |     |  +--rw exteutrancell3gpp:autoScalable     boolean

|  |     +--rw exteutrancell3gpp:peeParametersList!

|  |     |  +--rw exteutrancell3gpp:siteIdentification    string

|  |     |  +--rw exteutrancell3gpp:siteLatitude?         decimal64

|  |     |  +--rw exteutrancell3gpp:siteLongitude?        decimal64

|  |     |  +--rw exteutrancell3gpp:siteDescription       string

|  |     |  +--rw exteutrancell3gpp:equipmentType         string

|  |     |  +--rw exteutrancell3gpp:environmentType       string

|  |     |  +--rw exteutrancell3gpp:powerInterface        string

|  |     +--rw exteutrancell3gpp:priorityLabel        uint32

|  |     +--rw exteutrancell3gpp:pci                  int32

|  |     +--rw exteutrancell3gpp:plmnIdList* [mcc mnc]

|  |     |  +--rw exteutrancell3gpp:mcc    Mcc

|  |     |  +--rw exteutrancell3gpp:mnc    Mnc

|  |     +--rw exteutrancell3gpp:cellLocalId?         int32

|  |     +--rw exteutrancell3gpp:eNBId                int32

|  |     +--rw exteutrancell3gpp:earfcnDL             int32

|  |     +--rw exteutrancell3gpp:earfcnUL             int32

|  +--rw exteutrancell3gpp:ExternalEUtranCellTDD* [id] {subnet3gpp:ExternalsUnderSubNetwork}?

|     +--rw exteutrancell3gpp:id            string

|     +--rw exteutrancell3gpp:attributes

|        +--rw exteutrancell3gpp:userLabel?           string

|        +--rw exteutrancell3gpp:vnfParametersList!

|        |  +--rw exteutrancell3gpp:vnfInstanceId    string

|        |  +--rw exteutrancell3gpp:vnfdId?          string

|        |  +--rw exteutrancell3gpp:flavourId?       string

|        |  +--rw exteutrancell3gpp:autoScalable     boolean

|        +--rw exteutrancell3gpp:peeParametersList!

|        |  +--rw exteutrancell3gpp:siteIdentification    string

|        |  +--rw exteutrancell3gpp:siteLatitude?         decimal64

|        |  +--rw exteutrancell3gpp:siteLongitude?        decimal64

|        |  +--rw exteutrancell3gpp:siteDescription       string

|        |  +--rw exteutrancell3gpp:equipmentType         string

|        |  +--rw exteutrancell3gpp:environmentType       string

|        |  +--rw exteutrancell3gpp:powerInterface        string

|        +--rw exteutrancell3gpp:priorityLabel        uint32

|        +--rw exteutrancell3gpp:pci                  int32

|        +--rw exteutrancell3gpp:plmnIdList* [mcc mnc]

|        |  +--rw exteutrancell3gpp:mcc    Mcc

|        |  +--rw exteutrancell3gpp:mnc    Mnc

|        +--rw exteutrancell3gpp:cellLocalId?         int32

|        +--rw exteutrancell3gpp:eNBId                int32

|        +--rw exteutrancell3gpp:earfcn               int32

+--rw amfset3gpp:AMFSet* [id]

|  +--rw amfset3gpp:id            string

|  +--rw amfset3gpp:attributes

|     +--rw amfset3gpp:userLabel?           string

|     +--rw amfset3gpp:vnfParametersList!

|     |  +--rw amfset3gpp:vnfInstanceId    string

|     |  +--rw amfset3gpp:vnfdId?          string

|     |  +--rw amfset3gpp:flavourId?       string

|     |  +--rw amfset3gpp:autoScalable     boolean

|     +--rw amfset3gpp:peeParametersList!

|     |  +--rw amfset3gpp:siteIdentification    string

|     |  +--rw amfset3gpp:siteLatitude?         decimal64

|     |  +--rw amfset3gpp:siteLongitude?        decimal64

|     |  +--rw amfset3gpp:siteDescription       string

|     |  +--rw amfset3gpp:equipmentType         string

|     |  +--rw amfset3gpp:environmentType       string

|     |  +--rw amfset3gpp:powerInterface        string

|     +--rw amfset3gpp:priorityLabel        uint32

|     +--rw amfset3gpp:pLMNIdList* [mcc mnc]

|     |  +--rw amfset3gpp:mcc    Mcc

|     |  +--rw amfset3gpp:mnc    Mnc

|     +--ro amfset3gpp:nRTACList*           types3gpp:Tac

|     +--rw amfset3gpp:sNSSAIList*          types3gpp:SNssai

|     +--rw amfset3gpp:aMFRegion?           instance-identifier

|     +--rw amfset3gpp:aMFSetMemberList*    types3gpp:DistinguishedName

+--rw extgnbcucp3gpp:ExternalGNBCUCPFunction* [id] {subnet3gpp:ExternalsUnderSubNetwork}?

|  +--rw extgnbcucp3gpp:id                   string

|  +--rw extgnbcucp3gpp:attributes

|  |  +--rw extgnbcucp3gpp:userLabel?           string

|  |  +--rw extgnbcucp3gpp:vnfParametersList!

|  |  |  +--rw extgnbcucp3gpp:vnfInstanceId    string

|  |  |  +--rw extgnbcucp3gpp:vnfdId?          string

|  |  |  +--rw extgnbcucp3gpp:flavourId?       string

|  |  |  +--rw extgnbcucp3gpp:autoScalable     boolean

|  |  +--rw extgnbcucp3gpp:peeParametersList!

|  |  |  +--rw extgnbcucp3gpp:siteIdentification    string

|  |  |  +--rw extgnbcucp3gpp:siteLatitude?         decimal64

|  |  |  +--rw extgnbcucp3gpp:siteLongitude?        decimal64

|  |  |  +--rw extgnbcucp3gpp:siteDescription       string

|  |  |  +--rw extgnbcucp3gpp:equipmentType         string

|  |  |  +--rw extgnbcucp3gpp:environmentType       string

|  |  |  +--rw extgnbcucp3gpp:powerInterface        string

|  |  +--rw extgnbcucp3gpp:priorityLabel        uint32

|  |  +--rw extgnbcucp3gpp:gNBId                int64

|  |  +--rw extgnbcucp3gpp:gNBIdLength          int32

|  |  +--rw extgnbcucp3gpp:pLMNId* [mcc mnc]

|  |     +--rw extgnbcucp3gpp:mcc    Mcc

|  |     +--rw extgnbcucp3gpp:mnc    Mnc

|  +--rw extnrcellcu3gpp:ExternalNRCellCU* [id] {subnet3gpp:ExternalsUnderSubNetwork}?

|     +--rw extnrcellcu3gpp:id            string

|     +--rw extnrcellcu3gpp:attributes

|        +--rw extnrcellcu3gpp:userLabel?           string

|        +--rw extnrcellcu3gpp:vnfParametersList!

|        |  +--rw extnrcellcu3gpp:vnfInstanceId    string

|        |  +--rw extnrcellcu3gpp:vnfdId?          string

|        |  +--rw extnrcellcu3gpp:flavourId?       string

|        |  +--rw extnrcellcu3gpp:autoScalable     boolean

|        +--rw extnrcellcu3gpp:peeParametersList!

|        |  +--rw extnrcellcu3gpp:siteIdentification    string

|        |  +--rw extnrcellcu3gpp:siteLatitude?         decimal64

|        |  +--rw extnrcellcu3gpp:siteLongitude?        decimal64

|        |  +--rw extnrcellcu3gpp:siteDescription       string

|        |  +--rw extnrcellcu3gpp:equipmentType         string

|        |  +--rw extnrcellcu3gpp:environmentType       string

|        |  +--rw extnrcellcu3gpp:powerInterface        string

|        +--rw extnrcellcu3gpp:priorityLabel        uint32

|        +--rw extnrcellcu3gpp:cellLocalId          int32

|        +--rw extnrcellcu3gpp:nRPCI                int32

|        +--rw extnrcellcu3gpp:pLMNIdList* [mcc mnc]

|        |  +--rw extnrcellcu3gpp:mcc    Mcc

|        |  +--rw extnrcellcu3gpp:mnc    Mnc

|        +--rw extnrcellcu3gpp:nRFrequencyRef       types3gpp:DistinguishedName

+--rw extnssf3gpp:ExternalNSSFFunction* [id]

|  +--rw extnssf3gpp:id            string

|  +--rw extnssf3gpp:attributes

|     +--rw extnssf3gpp:userLabel?           string

|     +--rw extnssf3gpp:vnfParametersList!

|     |  +--rw extnssf3gpp:vnfInstanceId    string

|     |  +--rw extnssf3gpp:vnfdId?          string

|     |  +--rw extnssf3gpp:flavourId?       string

|     |  +--rw extnssf3gpp:autoScalable     boolean

|     +--rw extnssf3gpp:peeParametersList!

|     |  +--rw extnssf3gpp:siteIdentification    string

|     |  +--rw extnssf3gpp:siteLatitude?         decimal64

|     |  +--rw extnssf3gpp:siteLongitude?        decimal64

|     |  +--rw extnssf3gpp:siteDescription       string

|     |  +--rw extnssf3gpp:equipmentType         string

|     |  +--rw extnssf3gpp:environmentType       string

|     |  +--rw extnssf3gpp:powerInterface        string

|     +--rw extnssf3gpp:priorityLabel        uint32

|     +--rw extnssf3gpp:pLMNIdList* [mcc mnc]

|        +--rw extnssf3gpp:mcc    Mcc

|        +--rw extnssf3gpp:mnc    Mnc

+--rw extgnbcuup3gpp:ExternalGNBCUUPFunction* [id] {subnet3gpp:ExternalsUnderSubNetwork}?

|  +--rw extgnbcuup3gpp:id            string

|  +--rw extgnbcuup3gpp:attributes

|     +--rw extgnbcuup3gpp:userLabel?           string

|     +--rw extgnbcuup3gpp:vnfParametersList!

|     |  +--rw extgnbcuup3gpp:vnfInstanceId    string

|     |  +--rw extgnbcuup3gpp:vnfdId?          string

|     |  +--rw extgnbcuup3gpp:flavourId?       string

|     |  +--rw extgnbcuup3gpp:autoScalable     boolean

|     +--rw extgnbcuup3gpp:peeParametersList!

|     |  +--rw extgnbcuup3gpp:siteIdentification    string

|     |  +--rw extgnbcuup3gpp:siteLatitude?         decimal64

|     |  +--rw extgnbcuup3gpp:siteLongitude?        decimal64

|     |  +--rw extgnbcuup3gpp:siteDescription       string

|     |  +--rw extgnbcuup3gpp:equipmentType         string

|     |  +--rw extgnbcuup3gpp:environmentType       string

|     |  +--rw extgnbcuup3gpp:powerInterface        string

|     +--rw extgnbcuup3gpp:priorityLabel        uint32

|     +--rw extgnbcuup3gpp:gNBId                int64

|     +--rw extgnbcuup3gpp:gNBIdLength          int32

+--rw extnrf3gpp:ExternalNRFFunction* [id]

|  +--rw extnrf3gpp:id            string

|  +--rw extnrf3gpp:attributes

|     +--rw extnrf3gpp:userLabel?           string

|     +--rw extnrf3gpp:vnfParametersList!

|     |  +--rw extnrf3gpp:vnfInstanceId    string

|     |  +--rw extnrf3gpp:vnfdId?          string

|     |  +--rw extnrf3gpp:flavourId?       string

|     |  +--rw extnrf3gpp:autoScalable     boolean

|     +--rw extnrf3gpp:peeParametersList!

|     |  +--rw extnrf3gpp:siteIdentification    string

|     |  +--rw extnrf3gpp:siteLatitude?         decimal64

|     |  +--rw extnrf3gpp:siteLongitude?        decimal64

|     |  +--rw extnrf3gpp:siteDescription       string

|     |  +--rw extnrf3gpp:equipmentType         string

|     |  +--rw extnrf3gpp:environmentType       string

|     |  +--rw extnrf3gpp:powerInterface        string

|     +--rw extnrf3gpp:priorityLabel        uint32

|     +--rw extnrf3gpp:pLMNIdList* [mcc mnc]

|        +--rw extnrf3gpp:mcc    Mcc

|        +--rw extnrf3gpp:mnc    Mnc

+--rw amfr3gpp:AMFRegion* [id]

|  +--rw amfr3gpp:id            string

|  +--rw amfr3gpp:attributes

|     +--rw amfr3gpp:userLabel?           string

|     +--rw amfr3gpp:vnfParametersList!

|     |  +--rw amfr3gpp:vnfInstanceId    string

|     |  +--rw amfr3gpp:vnfdId?          string

|     |  +--rw amfr3gpp:flavourId?       string

|     |  +--rw amfr3gpp:autoScalable     boolean

|     +--rw amfr3gpp:peeParametersList!

|     |  +--rw amfr3gpp:siteIdentification    string

|     |  +--rw amfr3gpp:siteLatitude?         decimal64

|     |  +--rw amfr3gpp:siteLongitude?        decimal64

|     |  +--rw amfr3gpp:siteDescription       string

|     |  +--rw amfr3gpp:equipmentType         string

|     |  +--rw amfr3gpp:environmentType       string

|     |  +--rw amfr3gpp:powerInterface        string

|     +--rw amfr3gpp:priorityLabel        uint32

|     +--rw amfr3gpp:pLMNIdList* [mcc mnc]

|     |  +--rw amfr3gpp:mcc    Mcc

|     |  +--rw amfr3gpp:mnc    Mnc

|     +--ro amfr3gpp:nRTACList*           types3gpp:Tac

|     +--rw amfr3gpp:sNSSAIList*          types3gpp:SNssai

|     +--rw amfr3gpp:aMFRegionId          types3gpp:AmfRegionId

|     +--rw amfr3gpp:aMFSet*              instance-identifier

+--rw extupf3gpp:ExternalUPFFunction* [id] {subnet3gpp:ExternalsUnderSubNetwork}?

|  +--rw extupf3gpp:id            string

|  +--rw extupf3gpp:attributes

|     +--rw extupf3gpp:userLabel?           string

|     +--rw extupf3gpp:vnfParametersList!

|     |  +--rw extupf3gpp:vnfInstanceId    string

|     |  +--rw extupf3gpp:vnfdId?          string

|     |  +--rw extupf3gpp:flavourId?       string

|     |  +--rw extupf3gpp:autoScalable     boolean

|     +--rw extupf3gpp:peeParametersList!

|     |  +--rw extupf3gpp:siteIdentification    string

|     |  +--rw extupf3gpp:siteLatitude?         decimal64

|     |  +--rw extupf3gpp:siteLongitude?        decimal64

|     |  +--rw extupf3gpp:siteDescription       string

|     |  +--rw extupf3gpp:equipmentType         string

|     |  +--rw extupf3gpp:environmentType       string

|     |  +--rw extupf3gpp:powerInterface        string

|     +--rw extupf3gpp:priorityLabel        uint32

+--rw nrfreq3gpp:NRFrequency* [id] {subnet3gpp:ExternalsUnderSubNetwork}?

|  +--rw nrfreq3gpp:id            string

|  +--rw nrfreq3gpp:attributes

|     +--rw nrfreq3gpp:userLabel?                  string

|     +--rw nrfreq3gpp:vnfParametersList!

|     |  +--rw nrfreq3gpp:vnfInstanceId    string

|     |  +--rw nrfreq3gpp:vnfdId?          string

|     |  +--rw nrfreq3gpp:flavourId?       string

|     |  +--rw nrfreq3gpp:autoScalable     boolean

|     +--rw nrfreq3gpp:peeParametersList!

|     |  +--rw nrfreq3gpp:siteIdentification    string

|     |  +--rw nrfreq3gpp:siteLatitude?         decimal64

|     |  +--rw nrfreq3gpp:siteLongitude?        decimal64

|     |  +--rw nrfreq3gpp:siteDescription       string

|     |  +--rw nrfreq3gpp:equipmentType         string

|     |  +--rw nrfreq3gpp:environmentType       string

|     |  +--rw nrfreq3gpp:powerInterface        string

|     +--rw nrfreq3gpp:priorityLabel               uint32

|     +--rw nrfreq3gpp:absoluteFrequencySSB        uint32

|     +--rw nrfreq3gpp:sSBSubCarrierSpacing        uint8

|     +--ro nrfreq3gpp:multiFrequencyBandListNR*   uint16

+--rw extamf3gpp:ExternalAMFFunction* [id]

|  +--rw extamf3gpp:id            string

|  +--rw extamf3gpp:attributes

|     +--rw extamf3gpp:userLabel?           string

|     +--rw extamf3gpp:vnfParametersList!

|     |  +--rw extamf3gpp:vnfInstanceId    string

|     |  +--rw extamf3gpp:vnfdId?          string

|     |  +--rw extamf3gpp:flavourId?       string

|     |  +--rw extamf3gpp:autoScalable     boolean

|     +--rw extamf3gpp:peeParametersList!

|     |  +--rw extamf3gpp:siteIdentification    string

|     |  +--rw extamf3gpp:siteLatitude?         decimal64

|     |  +--rw extamf3gpp:siteLongitude?        decimal64

|     |  +--rw extamf3gpp:siteDescription       string

|     |  +--rw extamf3gpp:equipmentType         string

|     |  +--rw extamf3gpp:environmentType       string

|     |  +--rw extamf3gpp:powerInterface        string

|     +--rw extamf3gpp:priorityLabel        uint32

|     +--rw extamf3gpp:pLMNIdList* [mcc mnc]

|     |  +--rw extamf3gpp:mcc    Mcc

|     |  +--rw extamf3gpp:mnc    Mnc

|     +--rw extamf3gpp:aMFIdentifier!

|        +--rw extamf3gpp:amfRegionId?   AmfRegionId

|        +--rw extamf3gpp:amfSetId?      AmfSetId

|        +--rw extamf3gpp:amfPointer?    AmfPointer

+--rw eutraneteutranfreq3gpp:EUtranFrequency* [id] {subnet3gpp:ExternalsUnderSubNetwork}?

|  +--rw eutraneteutranfreq3gpp:id            string

|  +--rw eutraneteutranfreq3gpp:attributes

|     +--rw eutraneteutranfreq3gpp:userLabel?                string

|     +--rw eutraneteutranfreq3gpp:vnfParametersList!

|     |  +--rw eutraneteutranfreq3gpp:vnfInstanceId    string

|     |  +--rw eutraneteutranfreq3gpp:vnfdId?          string

|     |  +--rw eutraneteutranfreq3gpp:flavourId?       string

|     |  +--rw eutraneteutranfreq3gpp:autoScalable     boolean

|     +--rw eutraneteutranfreq3gpp:peeParametersList!

|     |  +--rw eutraneteutranfreq3gpp:siteIdentification    string

|     |  +--rw eutraneteutranfreq3gpp:siteLatitude?         decimal64

|     |  +--rw eutraneteutranfreq3gpp:siteLongitude?        decimal64

|     |  +--rw eutraneteutranfreq3gpp:siteDescription       string

|     |  +--rw eutraneteutranfreq3gpp:equipmentType         string

|     |  +--rw eutraneteutranfreq3gpp:environmentType       string

|     |  +--rw eutraneteutranfreq3gpp:powerInterface        string

|     +--rw eutraneteutranfreq3gpp:priorityLabel             uint32

|     +--rw eutraneteutranfreq3gpp:earfcnDL                  uint32

|     +--ro eutraneteutranfreq3gpp:multiBandInfoListEutra*   uint16

+--rw extamf3gpp:ExternalAMFFunction* [id] {subnet3gpp:ExternalsUnderSubNetwork}?

|  +--rw extamf3gpp:id            string

|  +--rw extamf3gpp:attributes

|     +--rw extamf3gpp:userLabel?           string

|     +--rw extamf3gpp:vnfParametersList!

|     |  +--rw extamf3gpp:vnfInstanceId    string

|     |  +--rw extamf3gpp:vnfdId?          string

|     |  +--rw extamf3gpp:flavourId?       string

|     |  +--rw extamf3gpp:autoScalable     boolean

|     +--rw extamf3gpp:peeParametersList!

|     |  +--rw extamf3gpp:siteIdentification    string

|     |  +--rw extamf3gpp:siteLatitude?         decimal64

|     |  +--rw extamf3gpp:siteLongitude?        decimal64

|     |  +--rw extamf3gpp:siteDescription       string

|     |  +--rw extamf3gpp:equipmentType         string

|     |  +--rw extamf3gpp:environmentType       string

|     |  +--rw extamf3gpp:powerInterface        string

|     +--rw extamf3gpp:priorityLabel        uint32

+--rw extgnbdu3gpp:ExternalGNBDUFunction* [id] {subnet3gpp:ExternalsUnderSubNetwork}?

+--rw extgnbdu3gpp:id            string

+--rw extgnbdu3gpp:attributes

+--rw extgnbdu3gpp:userLabel?           string

+--rw extgnbdu3gpp:vnfParametersList!

|  +--rw extgnbdu3gpp:vnfInstanceId    string

|  +--rw extgnbdu3gpp:vnfdId?          string

|  +--rw extgnbdu3gpp:flavourId?       string

|  +--rw extgnbdu3gpp:autoScalable     boolean

+--rw extgnbdu3gpp:peeParametersList!

|  +--rw extgnbdu3gpp:siteIdentification    string

|  +--rw extgnbdu3gpp:siteLatitude?         decimal64

|  +--rw extgnbdu3gpp:siteLongitude?        decimal64

|  +--rw extgnbdu3gpp:siteDescription       string

|  +--rw extgnbdu3gpp:equipmentType         string

|  +--rw extgnbdu3gpp:environmentType       string

|  +--rw extgnbdu3gpp:powerInterface        string

+--rw extgnbdu3gpp:priorityLabel        uint32

+--rw extgnbdu3gpp:gNBId                int64

+--rw extgnbdu3gpp:gNBIdLength          int32

+--rw extgnbdu3gpp:pLMNId* [mcc mnc]

+--rw extgnbdu3gpp:mcc    Mcc

+--rw extgnbdu3gpp:mnc    Mnc

module: ietf-yang-schema-mount

+--ro schema-mounts

+--ro namespace* [prefix]

|  +--ro prefix    yang:yang-identifier

|  +--ro uri?      inet:uri

+--ro mount-point* [module label]

+--ro module                 yang:yang-identifier

+--ro label                  yang:yang-identifier

+--ro config?                boolean

+--ro (schema-ref)

+--:(inline)

|  +--ro inline!

+--:(shared-schema)

+--ro shared-schema!

+--ro parent-reference*   yang:xpath1.0

End of Change