3GPP TSG-SA5 Meeting #128	S5-197045

# Zhuhai, China, 18th Nov 2019 - 22nd Nov 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.255 | CR | 0133 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-16 CR 32.255 PDU Session modification procedure with I-SMF involvement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | ETSUN |  |  |  |  |  | Date: |  |  | 2019-11-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Missing I-SMF involvement in PDU Session modification procedure charging message flow for ETSUN scenario, need to add I-SMF information with charging events/triggers in charging information and usage reporting. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding I-SMF information charging information and usage reporting in PDU Session modification charging message flow for ETSUN scenario . |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Can’t support ETSUN PDU Session modification charging with I-SMF insertion . |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.x.3(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

##### 5.2.2.x.3	PDU Session modification procedure with I-SMF involvement

The following figure 5.2.2.x.3.1 describes a PDU session modification charging message flow, for the case where the service area of the selected SMF does not include the location where the UE camps, with simplified I-SMF with UE and Serving Network message flows , based on TS 23.502 figure 4.3.3.3-1, where the V-SMF and V-UPF are replaced by I-SMF and I-UPF, and H-SMF and H-UPF are replaced by SMF and UPF (PSA) respectively:

Figure 5.2.2.x.3-1: UE or network requested PDU Session Modification charging with I-SMF

0.ch.	PDU Session served by both I-SMF and SMF has been established, with the ongoing Charging session.

1a.	UE or serving network AMF requested PDU Session Modification Request to I-SMF, I-SMF invokes an Nsmf_PDUSession_Update Request message to SMF, including information related to SMF change of charging conditions triggers. SMF accept with Nsmf_PDUSession_Update_Response message to I-SMF.

.

2.ch-a. All counts are closed in SMF and a Charging Data Request [Update] message including I-SMF information is sent to CHF, if required by immediate report category trigger. New counts and time stamps for all active service data flows are started in the SMF.

2.ch-b. The CHF updates CDR for this PDU session.

2.ch-c. The CHF acknowledges by sending Charging Data Response[Update] message to the SMF.

3.	With PCF or UDM or SMF Network requested PDU Session Modification Request, The SMF invokes the Nsmf_PDUSession_Update Request , contains QoS Information related N4 information to the I-SMF.

4. I-SMF with Serving Network message flows contains QoS flow information.

5a.	I-SMF send N4 Session Modification Request to I-UPF, with QoS information updates.

5b. I-UPF send N4 Session Modification Response to I-SMF, with usage reporting.

6.	I-SMF with Serving Network message flows updates with QoS flow information.

7.	I-SMF responds to the SMF with an Nsmf_PDUSession_Update response carrying the information	of both UE and Serving Network information, also with usage reporting information with trigger condition matched.

8.ch-a. All counts are closed in SMF and a Charging Data Request [Update] message including I-SMF information is sent to CHF, if required by immediate report category trigger. New counts and time stamps for all active service data flows are started in the SMF.

8.ch-b. The CHF updates CDR for this PDU session.

8.ch-c. The CHF acknowledges by sending Charging Data Response[Update] message to the SMF.

End of change