3GPP TSG-SA5 Meeting #128 	S5-197162

# 18-22 Nov 2019, Zhuhai, China

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.532 | CR | 0078 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Discover MnS instance |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GDSM |  |  |  |  |  | Date: |  |  | 2019-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | One of the objectives of 5GMSD WI is to enhance and extend the existing management service discovery. The result of MnS discovery is to get the URI of MnS instance which can provide management capabilities. How to discover the URI of MnS instance should be described. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add description of getting URI of MnS instance. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The management service discovery solution will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 11.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of 1st modified section in TS 28.532

2	References

-	The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.526: "Telecommunication management; Life Cycle Management (LCM) for mobile networks that include virtualized network functions; Procedures".

[3]	3GPP TS 28.541: "Management and orchestration ; 5G Network Resource Model (NRM); Stage 2 and stage3".

[4]	ITU-T Recommendation X.733 (02/92): "Information technology - Open Systems Interconnection - Systems Management: Alarm reporting function".

[5]	3GPP TS 28.531: "Management and orchestration ; Provisioning; ".

[6]	3GPP TS 28.554: "Management and orchestration ; 5G end to end Key Performance Indicators (KPI)".

[7]	3GPP TS 22.261: "Technical Specification Group Services and System Aspects; Service requirements for the 5G system; Stage 1".

[8]	3GPP TS 23.501: "Technical Specification Group Services and System Aspects; System Architecture for the 5G System; Stage 2".

[9]	3GPP TS 23.003: "Technical Specification Group Core Network and Terminals; Numbering, addressing and identification".

[10]	ETSI GS NFV-IFA 013 V2.4.1 (2018-02) "Network Function Virtualization (NFV); Management and Orchestration; Os-Ma-nfvo Reference Point - Interface and Information Model Specification".

[11]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[12]	ETSI GS NFV-IFA 015 (V2.4.1): "Network Function Virtualisation (NFV); Management and Orchestration; Report on NFV Information Model".

[13]	3GPP TS 28.533: "Management and orchestration; Architecture framework"

[14]	ITU-T Recommendation X.734 (1992): "Information technology - Open Systems Interconnection - Systems management: Event report management function".

[15]	3GPP TS 32.158: "Management and orchestration; Design rules for REpresentational State Transfer (REST) Solution Sets (SS)".

[16]	3GPP TS 32.302:	 "Telecommunication management; Configuration Management (CM); Notification Integration Reference Point (IRP); Information Service (IS)".

[17]	3GPP TS 32.401: "Telecommunication management; Performance Management (PM); Concept and requirements".

[18]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[19]	3GPP TS 32.401: "Telecommunication management; Perfomance Measurement (PM); Concept and requirements".

[20]	ISO 8601:2004: "Data elements and interchange formats – Information interchange – Representation of dates and times".

[21]	Text Attribution: Creator: ONAP, under Creative Commons Attribution 4.0 International License, https://creativecommons.org/licenses/by/4.0/, URI to access the text: https://docs.onap.org/en/latest/_downloads/2c2b5962df52a0c1f2862f3bba3d67c7/CommonEventFormat_30.1_ONAP.json, accessed 21.03.2019.

[22]	Figure Attribution: Creator: ONAP, under Creative Commons Attribution 4.0 International License, https://creativecommons.org/licenses/by/4.0/, URI to access the figure: https://docs.onap.org/en/latest/submodules/vnfsdk/model.git/docs/files/ves7_1spec.html?highlight=heartbeatIntervalChange#resource-structure, accessed 21.03.2019).

[23]	Text Attribution: Creator: ONAP, under Creative Commons Attribution 4.0 International License, https://creativecommons.org/licenses/by/4.0/, URI to access the text:	https://docs.onap.org/en/latest/submodules/vnfsdk/model.git/docs/files/VESEventListener_7_0_1.html?highlight=ves%207#naming-standards-for-eventname, accessed 11.04.2019).

[24]	Text Attribution: Creator: ONAP, under Creative Commons Attribution 4.0 International License, https://creativecommons.org/licenses/by/4.0/, URI to access the text:	https://docs.onap.org/en/latest/submodules/vnfsdk/model.git/docs/files/VESEventListener_7_0_1.html?highlight=ves%207#datatype-commoneventheader, accessed 11.04.2019).

[25]	3GPP TS 32.300: "Telecommunication management; Configuration Management (CM); Name convention for Managed Objects ".

[26]	W3C REC-xmlschema-0-20010502: "XML Schema Part 0: Primer".

[27]	W3C REC-xmlschema-1-20010502: "XML Schema Part 1: Structures".

[28]	W3C REC-xmlschema-2-20010502: "XML Schema Part 2: Datatypes".

[29]	W3C REC-xml-names-19990114: "Namespaces in XML".

[w] 		3GPP TS 32.158: "Management and orchestration; Design rules for REpresentational State Transfer (REST) Solution Sets (SS)".

[x] 	IETF RFC 3958: "Domain-Based Application Service Location Using SRV RRs and the Dynamic Delegation Discovery Service (DDDS)".

[y] 	IETF RFC 3403: "Dynamic Delegation Discovery System (DDDS). Part Three: The Domain Name System (DNS) Database".

[z] 	IETF RFC 2782: "A DNS RR for specifying the location of services (DNS SRV)".

Start of 2nd modified section in TS 28.532

11	Management services – Stage 2

11.x	Management service instance discovery

If there is no MnS instance pre-configuration, MnS consumer may get the resource URI for demanded management capability by MnS instance discovery.

The URI of MnS instance includes the following mandatory parts (see clause 4.4 in TS 28.158[w]):

- {URI-DN-PREFIX}

- {URI-LDN}.

The URI-DN-PREFIX (i.e. FQDN) of each MnS instance is provisioned to the name resolution service (e.g. DNS) as resource records associated with the MnS name or operation or both. MnS consumer may get the URI-DN-prefix of MnS instance URI through S-NAPTR procedure (see IETF RFC 3958 [x]) using NAPTR resource records (see IETF RFC 3403 [y]) and optional SRV resource records (see IETF RFC 2782 [z]).

MnS consumer may get DN of base object instance by MnSquery operation with the URI-DN-PREFIX and base object class.  MnS consumer may get DN of subordinate attributes and object instance by getMOIAttributes operation. Then MnS consumer maps DN to URI-LDN (see clause 4.2.3 in TS 28.158[w]).

MnS consumer constructs the URI of MnS instance from URI-DN-PREFIX and URI-LDN according to the resource URI structure defined for each MnS.

End of modification in TS 28.532